3GPP TSG RAN WG1 Meeting #109-e                                                              	    R1-2204503

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	9.3.3

Source:	Lenovo

Title:	Potential enhancements on dynamic/flexible TDD

Document for:	Discussion and decision

# Introduction

In RAN#94e, a new SID: study on evolution of NR duplex operation [1] was agreed. Following objectives will be studied from RAN1 perspective,

- Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1).

From the objectives, it could be observed that the potential enhancements on dynamic/flexible TDD are mostly regarding CLI handling. In this contribution, we provide analysis on the CLI handling for dynamic TDD and also for the systems with gNB side full duplex.

# Discussion

In dynamic/flexible TDD, the CLI includes UE-to-UE CLI and gNB-to-gNB CLI. The former is a UL-to-DL interference coming from UL signal from UEs in neighboring cells interfering serving cell’s DL signal, while the latter is a DL-to-UL interference coming from DL signal from one gNB interfering UL signal of another gNB. Figure 1 illustrates the two kinds of interference.

Figure 1 Inter-UE CLI and inter-gNB CLI in dynamic/flexible TDD

To handle UE-to-UE CLI, Rel-16 introduced two enhancement schemes, which include UE side interference measurement and reporting, and inter-gNB coordination of intended DL and UL configurations. One UE from the serving cell could be configured to measure the signals from UEs in a neighboring cell. The measurement could be SRS-RSRP or CLI-RSSI based on configuration. The UE will report the measurement to the gNB so that the gNB could understand the interference situation and take this factor into consideration when perform scheduling.

The gNBs could coordinate the intended DL/UL configurations though e.g., Xn interface. The intended DL/UL configurations include resources for fixed transmission directions (DL or UL) and flexible resources, which the gNB might use for either DL or UL. Such information could be taken into consideration by the scheduler. For example, the CLI can be excluded when performing scheduling in the resources with fixed transmission directions.

The gNB-to-gNB interference measurement was not specified in Rel-16 and it was agreed that CLI measurement is based on gNB implementation. On the other hand, the inter-gNB coordination of intended DL/UL configurations is helpful for mitigating inter-gNB interference.

For Rel-18 CLI  handling, it would be natural that Rel-16 specified schemes shall be taken as baseline for comparison, and any potential enhancements shall provide convincible performance gain over the baseline.

Proposal 1: Any potential area for further enhancements of CLI mitigation shall take the Rel-16 enhancement as baseline for comparison, and should provide noticeable performance gain.

For further enhancements of UE-to-UE CLI mitigation in Rel-18, one potential area is more dynamic interference measurement and reporting. This is different with Rel-16 SRS-RSRP and CLI-RSSI measurement, both of which are based on linear averaging of a long-term measurement. Therefore, they cannot reflect interference level timely. In commercial network with UE mobility, more dynamic interference measurement and reporting might be useful.

Proposal 2: Consider more dynamic interference measurement and reporting as one potential area for further enhancements for inter-UE CLI mitigation in dynamic/flexible TDD.

gNB-to-gNB CLI measurement was not specified in Rel-16, and the measurement is based on gNB implementation. It has been listed in the SID to further study CLI handling between the gNBs of the same or different operators to enable the dynamic/flexible TDD in commercial networks. One potential area for enhancements is to have dedicated resources for inter-gNB CLI measurement. This may include muting some resources for measurement purpose.

Proposal 3: Consider using dedicated resources for inter-gNB CLI measurement as one potential area for inter-gNB CLI mitigation in dynamic/flexible TDD.

The CLI mitigation for dynamic/flexible TDD can be reused for the CLI mitigation in TDD system with full duplex capable gNB. However, depending on scenarios, the CLI might be more complex in the system with full duplex. For example, in dynamic/flexible TDD with gNB capable full duplex, the CLI includes not only inter-cell inter-UE CLI (as in dynamic/flexible TDD), but also intra-cell inter-UE CLI (inter-subband CLI). As another example, if it is allowed to configure different subbands for full duplex in different cells, the inter-cell inter-UE CLI shall consider finer granularity in frequency domain on CLI handling, e.g., from whole frequency resources to subband level CLI handling.

Figure 2 Inter-UE CLI and inter-gNB CLI in full duplex

Proposal 4: The CLI handling for dynamic/flexible TDD can be reused as much as possible for CLI handling for full duplex. On the other hand, specific schemes are needed to handle the more complex CLI in full duplex.

Finally, Rel-16 CLI measurement and reporting mechanisms do not consider potentially different interference levels measured by a UE when different spatial filters are used for interference measurements. In Rel-18, it may be beneficial to enable a UE to report impact of spatial filters to be used by the UE for intended communications on observed interference levels. Accordingly, gNB can use the reported information for interference handling with proper scheduling of UEs and corresponding serving beams.

Proposal 5: Consider spatially differentiated CLI measurement and reporting in Rel-18 NR.

# Conclusions

As a summary, we have the following proposals for CLI mitigation in dynamic/flexible TDD,

Proposal 1: Any potential area for further enhancements of CLI mitigation shall take the Rel-16 enhancement as baseline, and provide noticeable performance gain.

Proposal 2: Consider more dynamic interference measurement and reporting as one potential area for further enhancements for inter-UE CLI mitigation in dynamic/flexible TDD.

Proposal 3: Consider using dedicated resources for inter-gNB CLI measurement as one potential area for inter-gNB CLI mitigation in dynamic/flexible TDD.

Proposal 4: The CLI handling for dynamic/flexible TDD can be reused as much as possible for CLI handling for full duplex. On the other hand, specific schemes are needed to handle the more complex CLI in full duplex.

Proposal 5: Consider spatially differentiated CLI measurement and reporting in Rel-18 NR.

# References

RP-213591, “New SI: Study on evolution of NR duplex operation”, RAN#94-e.