3GPP TSG RAN WG1 Meeting #109-e                                                              	    R1-2204504

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	9.6.1

Source:	Lenovo

Title:	Potential solutions to further reduce UE complexity

Document for:	Discussion and decision

# Introduction

In RAN#94e, a new SID on study on further NR RedCap UE complexity reduction [1] was agreed. Following objectives will be studied from RAN1 perspective,

- Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1] - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact - Potential solutions, which may complement each other, for reducing device complexity are focusing on: - UE bandwidth reduction to 5MHz in FR1, - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - reduced UE peak data rate in FR1, - Possibly including restricted bandwidth for PDSCH and/or PUSCH - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - Notes: - Rel-15 SSB should be reused and L1 changes minimized. - Operation in BWP with/without SSB and without/with RF retuning should be considered. - It is not precluded that some solutions for FR1 can be applied to FR2 in WI stage. - Aim to define a single Rel-18 RedCap UE type for further UE complexity reduction.

In this contribution, we provide analysis on the potential solutions for further reduction of UE complexity, including the cost reduction of each solution and potential standard impacts.

# Discussion

Rel.17 RedCap with 20MHz maximum UE BW for FR1 could meet the data rate requirement up to 150Mbps DL and 50Mbps UL, while devices like industrial sensors, economic videos, etc., require much lower reference bit rate even less than 5Mbps. From this point of view, Rel-17 provides far more capability to support such low-tier RedCap devices. Therefore, there are rooms for further cost/complexity breakdown for such low-tier devices.

As indicated in the justification part of the SID, Rel-18 RedCap should provide NR support for low-tier devices between existing LPWA UEs and the capabilities of Rel-17 RedCap UEs. The supported peak data rate targets 10Mbps and Rel-18 RedCap should not overlap with existing LPWA solutions. Two potential solutions for reducing device complexity are listed in the objectives including UE bandwidth reduction to 5MHz in FR1 and reduced UE peak data rate in FR1.

UE bandwidth reduction to 5MHz in FR1

With UE bandwidth down to 5MHz in FR1, lower cost/complexity is achieved by having less operations on basedband and RF processing, including ADC/DAC, FFT/IFFT, etc. From our initial evaluations in [2], the cost reduction of 5MHz BW over Rel.17 RedCap UEs with 20MHz BW is around 12%. Together with other cost reduction solutions such as relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI, the combined cost reduction is expected to be noticeable and meaningful for low-tier UEs, especially considering that these UEs are supposed to be more sensitive to cost reduction.

Observation 1: Together with other solutions of relaxed UE processing timeline for PDSCH/PUSCH/CSI, the solution of UE BW reduction to 5MHz in FR1 could provide noticeable cost saving gain.

Regarding the standard impact, it is noted that 5MHz UE BW has already been supported by NR as the minimum UE BW. The channels/signals such as those for initial access, e.g., SS/PBCH block, SIB1, RACH and paging can be reused. On the other hand, similar with the discussion in Rel.17 RedCap, the standard impacts are mostly due to the co-existence of RedCap UEs and legacy UEs in the system, where some of the channels might be configured with a larger BW and thus not compatible with low BW UEs. For example, the SSB subcarrier spacing might be configured as 30kHz, and the SSB BW would be 7.2MHz in this case. In Table 1, we provide NR supported BW of SSB and CORESET#0. It is observed that in some scenarios the SSB and/or CORESET#0 are not compatible with UEs with 5MHz BW.

Table 1 compatibility of SSB/CORESET#0 with 5MHz UEs

| Scenario | SSB SCS | SSB BW (MHz) | CORESET#0 SCS | CORESET#0 BW(RB) | Compatible with SSB(Y/N) | Compatible with CORESET#(Y/N) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 15kHz | 3.6MHz | 15kHz | 24RBs | Y | Y |
| 2 | 15kHz | 3.6MHz | 15kHz | 48/96RBs | Y | N |
| 3 | 15kHz | 3.6MHz | 30kHz | 24/48/96RBs | Y | N |
| 4 | 30kHz | 7.2MHz | 15kHz | 24/48/96RBs | N | N |
| 5 | 30kHz | 7.2MHz | 30kHz | 24/48/96RBs | N | N |

As a result, and again similar with the discussion in Rel-17 RedCap, the network might need to configure dedicated channels/signals for lower BW UEs, e.g., dedicated CORESET#0 and/or NCD-SSB. Furthermore, dedicated CD-SSB (with 15kHz SCS) might be needed if the performance of 7.2MHz BW SSB (with 30kHz SCS) is a bottleneck.

As mentioned, since 5MHz UE BW has already been supported by NR, the potential dedicated CORESET#0 and CD-SSB for such UEs could reuse the legacy design. Therefore, we expect moderate standard impact is needed if supporting UE bandwidth down to 5MHz. Besides, Rel-17 design on handling the coexistence issue of RedCap UEs with legacy UEs could be reused as much as possible.

Observation 2: By reusing legacy design on channel/signals for initial access and on the handling of coexistence of RedCap UEs and legacy UEs, moderate standard impacts are expected if supporting UE bandwidth down to 5MHz.

Reduced UE peak data rate

Another potential solution of cost reduction is to reduce the UE peak data rate by restricting the BW for PDSCH and/or PUSCH transmission. In our understanding, the UE BW is assumed to be still 20MHz, same with Rel.17 RedCap UEs. The cost reduction of this solution comes from less baseband operations such as less decoding operations, and less HARQ buffer.

Compared with the solution of UE BW reduction of 5MHz, which could also provide reduced UE peak data rate, it is clear that this solution would have less cost reduction since less bandband and RF operations are reduced [2].

Besides, if considering that most time the RedCap UEs would be in RRC idle/inactive state, reduction of scheduling BW of PDSCH/PUSCH might provide little power saving gain over Rel.17 RedCap UEs. Comparatively, the power saving gain of reduced UE BW is expected to be higher.

Observation 3: Compared with UE bandwidth reduction, reducing the peak data rate by restricting PUSCH/PDSCH scheduling bandwidth has lower cost reduction and power saving gain.

The standard impact of reduced UE peak data rate might be low. One potential scheme to achieve reduced UE peak data rate is by introducing lower scaling factor. This was discussed in Rel.17 RedCap but was not accepted. It could be discussed in Rel-18 RedCap if reduced UE peak data rate is taken as the solution for cost reduction.

It is noted that some standard impact might be shared by both solutions, e.g., a new type UEs for Rel-18 RedCap, early identification of the new type of UEs, cell barring of new type of UEs, etc. The Rel-17 design principles could be reused as much as possible for these areas.

Observation 4: Compared with UE bandwidth reduction, reducing the peak data rate by restricting PUSCH/PDSCH scheduling bandwidth has lower standard impact.

Based on the observations, we have following proposal for potential solutions to further reduce UE complexity,

Proposal: If further UE complexity reduction techniques will be introduced in Rel-18 RedCap, the solution of UE bandwidth reduction to 5MHz in FR1 is preferred in Rel-18 RedCap, considering its cost reduction and power saving gain, the potential standard impact and the time units allocated for Rel-18 RedCap discussion.

# Conclusions

As a summary, we have the following observations and proposals on potential solutions to further reduce UE complexity,

Observation 1: Together with other solutions of relaxed UE processing timeline for PDSCH/PUSCH/CSI, the solution of UE BW reduction to 5MHz in FR1 could provide noticeable cost saving gain.

Observation 2: By reusing legacy design on channel/signals for initial access and on the handling of coexistence of RedCap UEs and legacy UEs, moderate standard impacts are expected if supporting UE bandwidth down to 5MHz.

Observation 3: Compared with UE bandwidth reduction, reducing the peak data rate by restricting PUSCH/PDSCH scheduling bandwidth has lower cost reduction and power saving gain.

Observation 4: Compared with UE bandwidth reduction, reducing the peak data rate by restricting PUSCH/PDSCH scheduling bandwidth has lower standard impact.

Proposal: If further UE complexity reduction techniques will be introduced in Rel-18 RedCap, the solution of UE bandwidth reduction to 5MHz in FR1 is preferred in Rel-18 RedCap, considering its cost reduction and power saving gain, the potential standard impact and the time units allocated for Rel-18 RedCap discussion.

# References

RP-213661, “New SID on Study on further NR RedCap UE complexity reduction”, RAN#94-e.

R1-2204505, “Evaluation needs and assumptions for further NR RedCap”, Lenovo, RAN1#109e