3GPP TSG RAN WG1 #109-e	R1-2204507

e-Meeting, May 9th – May 20th, 2022

Source:	Sharp

Title:	Two TAs for multi-DCI

Agenda Item:	9.1.1.2

Document for:	Discussion and Decision

Background

In RAN#94e, the working item to enhance both downlink and uplink MIMO operations in Rel-18 was agreed [1].

According to the WID, the following item needs to be studied, and if justified, specified:

- Two TAs for UL multi-DCI for multi-TRP operation

Initial discussion

In Rel-17, the single DCI and single TA have been specified for UL MTRP transmission to enhance reliability. This assumes that the relative propagation delay between TRPs is within CP length. For multi-DCI based multi-TRP operation, it is more realistic and beneficial in terms of network deployment that UL transmissions to multi-TRP are associated with different TAs. This means not only the network flexibility can be achieved but also more UEs can obtain the benefits to more UEs. This is because the same TA leads to restrict a position/distance relation between each TRP and UE. Therefore, in Rel-18, two TAs for multi-DCI based multi-TRP UL transmission should be supported.

For example, as shown in Figure 1, each UE is freely located in a cell to transmit uplink to two TRPs, if two TAs are supported.

Figure 1: Two TAs for UL

Proposal 1: Two TAs for multi-DCI based multi-TRP UL transmission should be supported in Rel-18.

To calculate a TA for each TRP, the UE needs to transmit a UL channel to each TRP. In Rel-15-17, if a TA for a serving cell is not provided or not adjusted, only PRACH preamble can be transmitted because it has guard period to consider interference. Consequently, a TA is calculated at the gNB side based on the received PRACH preamble and the TA is indicated by a RAR for a serving cell. Likewise, to calculate each TA, the UE should transmit a PRACH preamble to each TRP.

Proposal 2: The UE should transmit a PRACH preamble to each TRP.

Conclusion

In this contribution, we have the following proposal:

Proposal 1: Two TAs for multi-DCI based multi-TRP UL transmission should be supported in Rel-18.

Proposal 2: The UE should transmit a PRACH preamble to each TRP.

References

- [1]	RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Dec., 2021.