3GPP TSG RAN WG1 #109-e	R1-2204510

e-Meeting, May 9th – May 20th, 2022

Source:	Sharp

Title:	SRS enhancement targeting TDD CJT and 8 TX operation

Agenda Item:	9.1.3.2

Document for:	Discussion and Decision

Background

In RAN#94e, the working item to enhance both downlink and uplink MIMO operations in Rel-18 was agreed [1].

According to the WID, the following item needs to be studied, and if justified, specified:

- enhancement of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints the 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

- SRS enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle Industrial devices

SRS enhancement for TDD CJT

The benefits of coherent JT (CJT) are to achieve higher reliability and coverage performance. This is because a UE can be scheduled by synchronized multi-TRPs in cooperative area. However, for TDD CJT operation, there are many UEs in the cooperative area for SRS transmission and cross-SRS interference frequently occurs. Therefore, we support SRS enhancement targeting TDD CJT (e.g., SRS capacity enhancement).

Proposal 1: Support SRS enhancement for TDD CJT (e.g., SRS capacity enhancement).

As a simple idea to enhance SRS capacity, the number of SRS sequences multiplexed into an SRS resource should be increased. For example, orthogonal cover code (OCC) can be used to spread in time domain, and it is expected to suppress interference and improve SINR and capacity. For OCC, additional resources, new SRS comb structure, and new SRS root sequences are unnecessary.

Proposal 2: To enhance SRS capacity, OCC should be introduced in the time domain.

SRS enhancement for 8Tx operation

8 Tx UL operation (4 or more layers transmission) is beneficial for CPE/FWA/vehicle/industrial devices. Furthermore, to realize PUSCH transmission with 8 antenna ports for codebook-based transmission and non-codebook-based transmission, SRS ports need to be increased because uplink precoding/spatial domain filter are based on one of SRS resources. Therefore, we support SRS enhancement for 8Tx operation.

Proposal 3: Support SRS enhancement for 8Tx operation.

In Rel-15-17, SRS sequence with 8 ports cannot be generated because at least cyclic shift for SRS sequence is determined based on each SRS port. Therefore, SRS sequence generation should be extended for 8 SRS ports.

Proposal 4: SRS sequence generation should be extended for 8 SRS ports.

Conclusion

In this contribution, we have the following proposal:

Proposal 1: Support SRS enhancement for TDD CJT (e.g., SRS capacity enhancement).

Proposal 2: To enhance SRS capacity, OCC should be introduced in the time domain.

Proposal 3: Support SRS enhancement for 8Tx operation.

Proposal 4: SRS sequence generation for 8 ports should be studied and specified

References

- [1]	RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Dec., 2021.