3GPP TSG RAN WG1 #109-e	R1-2204511

e-Meeting, May 9th – May 20th, 2022

Source:	Sharp

Title:	Views on UL multi-panel transmission

Agenda Item:	9.1.4.1

Document for:	Discussion and Decision

Background

In RAN#94e, the working items to enhance both downlink and uplink MIMO operations in Rel-18 were agreed [1].

According to the WID, the following items regarding to multi-panel UL transmission needs to be studied, and if justified, specified:

  - The following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

    - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission

      - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

    - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation

      - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

In this contribution, we share our views on multi-panel UL transmission.

Discussions

First of all, we should recognize specification or function needed for simultaneous multi-panel UL transmission. In the WID, there is description as ‘the following item needs to be studied, and if justified, specified’. In our view, simultaneous UL multi-panel transmission should be introduced in Rel-18 because simultaneous use of different beams can potentially achieve higher rank in UL MIMO even in single TRP and have high affinity to UL MTRP.

Proposal 1: Simultaneous multi-panel UL transmission should be introduced in Rel-18.

Regarding the number of TRPs and the number of UL panels, the WID is describing that up to 2 TRPs and up to 2 panels are assumed in the WID. In our view, in Rel-18, RAN1 should focus on up to 2 TRPs and up to 2 panels because more than 2 TRPs and 2 panels does not seem very practical at this time for UE to simultaneous use of more than 2 TRPs or more than 2 panels. Thus, the both number of TRPs and panels should be up to 2 for simultaneous multi-panel UL transmission in Rel-18.

Proposal 2: The both number of TRPs and panels should be up to 2 for simultaneous multi-panel UL transmission in Rel-18.

In the WID, no new codebook should be introduced for multi-panel simultaneous transmission. The simplest method is to divert existing codebooks, i.e., codebooks applied for current single panel transmission.

Proposal 3: The codebook applied for each panel on multi-panel transmission should basically be based on the one for single panel transmission.

Unlike TRPs and the number of panels, anything is not mentioned in the WID regarding the configuration of panels. In Rel-16 discussion, it has been discussed how to identify the panel (e.g. SRS resource set ID,  UL antenna group). In our view, we need to discuss how to specify “panel” if we introduce simultaneous UL multi-panel transmission from signalling perspective.

Proposal 4: We should discuss how to specify “panel”.

Considering that no new codebooks will be introduced, the operation of each panel in a multi-panel transmission should be the same as in a single-panel transmission.

Proposal 5: Multi-panel UL transmisison of each panel follows the single-panel UL transmission scheme.

Conclusion

In this contribution, we have the following proposal:

Proposal 1: Simultaneous multi-panel UL transmission should be introduced in Rel-18.

Proposal 2: The both number of TRPs and panels should be up to 2 for simultaneous multi-panel UL transmission in Rel-18.

Proposal 3: The codebook applied for each panel on multi-panel transmission should basically be based on the one for single panel transmission.

Proposal 4: We should discuss how to specify “panel”.

Proposal 5: Multi-panel UL transmisison of each panel follows the single-panel UL transmission scheme.

References

- [1]	RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Dec., 2021.