3GPP TSG RAN WG1 #109-e	R1-2204512

e-Meeting, May 9th – May 20th, 2022

Source:	Sharp

Title:	Views on 8 TX UL transmission

Agenda Item:	9.1.4.2

Document for:	Discussion and Decision

Background

In RAN#94e, the working item to enhance both downlink and uplink MIMO operations in Rel-18 was agreed [1].

According to the WID, the following item needs to be studied, and if justified, specified:

- SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices.

Discussions

  - Support of 8Tx UL operations

The current specifications for TPMI and SRI assume only 4 Tx. In our view, UEs such as CPE/FWA/vehicle/Industrial devices don’t need to have restriction of antenna placing of UEs such us smartphones and 8 Tx antenna can be supported.

Proposal 1: 8 Tx UL operation should be introduced in Rel-18.

  - Support of UEs supporting more than 4 layers

At the RAN#94e, one of the discussion points is whether more than 4 layers is supported or not. In our view, more than 4 layers should be introduced for UEs targeting CPE/FWA/vehicle/Industrial devices because these types of UEs need symmetric maximum data rate of DL and UL is preferable from the specification support point of view. Therefore we believe that more than 4 layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices in Rel-18.

Proposal 2: More than 4 layers should be supported in Rel-18.

Regarding the number of layers, we should focus on discussion for 8 Tx UL operation, assuming 4 and more than layers in Rel-18. To clarify what is being discussed, a specific number of layers should be agreed upon rather than the phrase “4 and more layers”. In the current specification, 4 layers have been specified in up to Rel-17. Therefore, in Rel-18, RAN1 should study the necessity of 5, 6, 7, and 8 layers in this work item..

Proposal 3: The number of layers should be 5, 6, 7 and 8 for study purpose in Rel-18.

Conclusion

In this contribution, we have the following proposal:

Proposal 1: 8 Tx UL operation should be introduced in Rel-18.

Proposal 2: More than 4 layers should be supported in Rel-18.

Proposal 3: The number of layers should be 5, 6, 7 and 8 for study purpose in Rel-18.

References

- [1]	RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Dec., 2021.