3GPP TSG RAN WG1 Meeting #109                                                                 	     R1-2204516

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	9.12.2

Source:	Lenovo, Motorola Mobility

Title:	Disabling of HARQ feedback for IoT NTN

Document for:	Discussion and decision

Introduction

In the RAN#94 plenary meeting, an enhancement work item for Rel.18 IoT NTN was approved. One of the objectives is to specify the following HARQ enhancements to IoT NTN.

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below:

-	Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2]

-	Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption [RAN1]

In this contribution, detailed considerations of potential enhancement for IoT NTN, especially HARQ disabling enhancement are presented.

Discussion

Need of disabling HARQ feedback in IoT NTN

IoT NTN Disabling HARQ feedback for downlink transmission was discussed in Rel.17 study item phase[1]. The following observation was captured in TR36.763.

TR36.763 Section 6.6.4

Disabling HARQ feedback: It was concluded that from a physical layer perspective, there is no consensus on disabling HARQ feedback for NTN IoT in Rel-17. Disabling HARQ feedback for NB-IoT and for eMTC in NTN is recommended not to be supported in Rel-17.

-	The disabling of HARQ feedback for downlink transmission was discussed. This can benefit UE power consumption and latency. Disabling HARQ feedback for a DL transmission can improve uplink throughput in NTN as more resource would be available in uplink. If HARQ feedback is disabled, the L1 reliability of the downlink transmission may degrade due to the lack of HARQ feedback. Therefore, more UL resources may will be needed for RLC status reporting, which may will partly consume the UL resources made available by disabled HARQ feedback.

-	According to some companies' views, disabling HARQ feedback for downlink transmission can mitigate HARQ stalling for NB-IoT which is due to the large RTT in NTN and benefit UE power consumption and latency.

-	Means to mitigate the potential throughput/latency penalties due to the large RTT in NTN that are already supported in the current specification were also discussed. An eNB can ensure improved DL throughput for eMTC by scheduling new DL TBs for a given HARQ process without waiting for reception of the previous TB HARQ ACK/NACK of that HARQ process, even when the UE transmits a HARQ ACK for TBs scheduled on that HARQ process. This mitigates the throughput/latency penalties significantly. The UE is still required to always transmit a HARQ-ACK (which is no longer used for the primary purpose of physical layer acknowledgment, but may have secondary benefits, e.g., in link adaptation aspects), thereby requiring more UE power expenditure than the feedback-disabled case.

-	When the HARQ feedback is not disabled, an uplink resource will be always needed for HARQ ACK feedback. Considering HD-FDD processing for IoT UEs, always-enabled HARQ ACK feedback will impact DL scheduling and resource allocation in time domain and impact DL throughput/data rate, especially for large coupling losses in the uplink that necessitate large number of repetitions in the uplink.

Benefit and drawback of disabling HARQ

Disabling HARQ feedback for downlink transmission is supported in NR NTN by avoiding stop-and-wait in HARQ procedure and relying on RLC ARQ for reliability. The same benefit can be achieved for IoT NTN.

- Disabling HARQ feedback for a DL transmission can benefit UE power consumption/latency as less uplink transmission is needed for feedback to downlink transmission.

- Disabling HARQ feedback for a DL transmission can improve uplink throughput as more resource would be available in uplink transmission due to less feedback transmission for DL.

- Disabling HARQ feedback for downlink transmission can improve the downlink throughput by facilitating the scheduling of a new TB without waiting for a HARQ-ACK feedback for a previous TB for the same HARQ process and mitigating HARQ stalling, which is due to the large RTT in NTN, especially for the scheduling with small repetition number.

As for the drawback of disabling HARQ feedback for downlink transmission, it may degrade the L1 reliability of the downlink transmission and impact link adaptation in some IoT NTN scenarios due to the lack of feedback information.

In IoT NTN study item phase, some companies provided the analysis that the peak data rate requirements of IoT NTN can be met even without disabling HARQ feedback feature[3][4], however, the reduction in peak data rate due to HARQ stalling with all HARQ processes feedback can’t be negligible, especially for GEO scenarios. Some companies observed that disabling HARQ feedback for downlink transmission can’t mitigate HARQ stalling since the current specification does not require eNB to wait for reception of HARQ-ACK for a DL HARQ process before scheduling a new TB for that HARQ process. eNB can maintain downlink peak data rate by scheduling new TB for a DL HARQ process immediately after the UE transmits the HARQ-ACK feedback. However, this requires UE to always transmit HARQ-ACK in uplink resource and requires more UE power expenditure than explicit indicated feedback-disabled case.

Based on the above observation, since it had been agreed that increasing the number of HARQ processes for NB-IoT and for eMTC in NTN is recommended not to be supported in Rel.17 and even in Rel.18 potentially, disabling HARQ feedback can be one way to mitigate HARQ stalling and provide wide coverage in some cases. Disabling HARQ feedback for downlink transmission should be supported in Rel.18 NTN IoT as in WID.

Proposal 1: Disabling HARQ feedback for downlink transmission should be supported in Rel.18 NTN IoT as in WID.

Scenarios/cases to support disabling HARQ feedback

eMTC and NBIoT relevant to HARQ operation have been enhanced since Rel.13, scenarios/cases applied with the disabling HARQ feedback for downlink transmission in IoT NTN should be further discussed. The motivation of the disabling HARQ feedback for downlink transmission is to solve the HARQ stalling issues on data rate, so the following scenarios/cases should be considered/studied if the problems identified on HARQ stalling.

eMTC

- Rel.13 CE mode A (maximal FDD DL HARQ process number of 8)

- Rel.13 CE mode B (maximal FDD DL HARQ process number of 2)

- Rel.14 CE mode A (maximal H-FDD DL HARQ process number of 10) as shown in Figure 1

- Rel.16 CE mode A (maximal FDD DL HARQ process of 8 for multiple TB scheduling) as shown in Figure 2

- Rel.16 CE mode B (maximal FDD DL HARQ process of 4 for multiple TB scheduling)

- Rel.17 CE mode A (maximal H-FDD DL HARQ process number of 14) as shown in Figure 3

NBIoT

- Rel.13 NBIoT (maximal DL HARQ process number of 1)

- Rel.14 NBIoT (maximal DL HARQ process number of 2)

Figure 1 Rel.14 CE mode A HARQ-ACK bundling

Figure 2 Rel.16 CE mode A Multiple TB scheduling

Figure 3 Rel.17 CE mode A HARQ-ACK bundling

Disabling of HARQ feedback in case of single HARQ process and all HARQ feedback

Regarding disabling of HARQ feedback in case of single HARQ process and disabling of all HARQ feedback, it was discussed in NR NTN, for ensuring the efficiency and reliability of transmission carrying the signaling, e.g., RRC configuration, at least one HARQ process with feedback should also be kept for IoT NTN. However, the configuration restriction can be up to eNB scheduling implementation as NR NTN.

LEO and GEO scenarios

Disabling HARQ feedback for all LEO, MEO and GEO scenarios was studied in study item phase. For IoT NTN, the reference value of RTT for LEO and GEO are 25.77 ms and 541.46 ms separately. The RTT is much larger than the IoT NTN UE supported number of HARQ processing. Although the repetition-based transmission and LEO scenarios with smaller RTT will mitigate the impact of HARQ stalling, the issue can’t be solved essentially. So, we slightly prefer to support disabling HARQ feedback for downlink transmission for both LEO and GEO scenarios to achieve a unified solution.

Proposal 2: Scenarios/cases applied with the disabling HARQ feedback for downlink transmission in IoT NTN should be further discussed.

Indication/configuration of disabling HARQ feedback in IoT NTN

In NR NTN, the disabling HARQ feedback for downlink transmission is semi-static configured in RRC signaling. The configuration is indicated per HARQ process number by bitmap manner, e.g., 32bit bitmap for configured HARQ process number of 32. For the indication and configuration of disabling HARQ feedback for IoT NTN, NR NTN configuration is the baseline solution.

downlinkHARQ-FeedbackDisabled

Used to disable the DL HARQ feedback, sent in the uplink, per HARQ process ID. The first/leftmost bit corresponds to HARQ process ID 0, the next bit to HARQ process ID 1 and so on. Bits corresponding to HARQ process IDs that are not configured shall be ignored. The bit(s) set to one identify HARQ processes with disabled DL HARQ feedback and the bit(s) set to zero identify HARQ processes with enabled DL HARQ feedback.

Proposal 3：Disabling HARQ feedback is configured by RRC signaling, and NR NTN configuration is the baseline solution. FFS: per UE per HARQ process

(N)PDSCH/(N)PDCCH scheduling restriction

In NR NTN, additional gap is considered to avoid the continuous reception of PDSCH with same HARQ process at UE side as specified in TS38.214.

In NBIoT, there is NPDCCH monitoring restriction between the reception of NPDSCH and corresponding HARQ-ACK feedback transmission for UE complexity consideration since UE is not expected to have extra burden of decoding NPDCCH while decoding NPDSCH and encoding NPUSCH format 2 simultaneously. Furthermore, for NPDSCH of RAR, paging and multi-cast without HARQ-ACK feedback where only one HARQ process number is assumed, there is NPDCCH monitoring restriction that UE is not required to monitor NPDCCH in a period (e.g., 12ms) after the NPDSCH reception. The NPDCCH monitoring restriction may be reconsidered or removed in disabling HARQ feedback for downlink transmission for IoT NTN.

TS38.214 Section 5.1

When HARQ feedback for the HARQ process ID is disabled, the UE is not expected to receive another PDCCH carrying a DCI scheduling a PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process or to receive another PDSCH without corresponding PDCCH for the given HARQ process that starts until Tproc,1 after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process.

TS36.213 Section 16.6

If a NB-IoT UE detects NPDCCH with DCI Format N1 ending in subframe n, and if the corresponding NPDSCH transmission starts from n+k, and

-	for FDD, if the corresponding NPUSCH format 2 transmission starts from subframe n+m (accounting for uplink transmission timing), the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1.

If a NB-IoT UE receives a NPDSCH transmission ending in subframe n, and if the UE is not required to transmit a corresponding NPUSCH format 2, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12.

For IoT NTN DL HARQ process with disabled HARQ feedback, (N)PDCCH/(N)PDSCH scheduling restriction should be imposed to allow enough time for UE decoding processing considering the UE complexity and power saving.

- For NBIoT UE support one DL HARQ process, after reception PDSCH with HARQ process disabled, if supported, UE is not required to monitor NPDCCH in a period of X from the end of reception of the last NPDSCH similar as scenarios that NPDSCH of RAR, paging and multi-cast without HARQ-ACK feedback.

- For eMTC/NBIoT UE support more than one DL HARQ processes, after reception PDSCH with HARQ process disabled, UE needs to keep monitoring (N)PDCCH for new (N)PDSCH with another HARQ process(es). However, the restriction is imposed to (N)PDSCH scheduling for the same HARQ process similar as NR NTN. An PDSCH scheduling gap should be inserted that UE is not expected to receive another (N)PDCCH carrying a DCI scheduling a (N)PDSCH for a given HARQ process or to receive another PDSCH without corresponding PDCCH for the given HARQ process that starts until X after the end of the reception of the last (N)PDSCH for that HARQ process. X is determined by UE capability for NBIoT and eMTC separately.

Proposal 4: For IoT NTN DL HARQ process with disabled HARQ feedback, (N)PDCCH/(N)PDSCH scheduling restriction should be imposed to allow enough time for UE decoding processing.

SPS PDSCH

SPS PDSCH is configured for semi-persistent services by RRC signaling, the determination of HARQ process number is related to the subframe index and supported HARQ process number for eMTC. The per HARQ process disabling configuration for IoT NTN is still valid for the SPS PDSCH transmission with proper network implementation. Configuration of HARQ enabling/disabling per HARQ process can also apply to SPS PDSCH.

Proposal 5: Configuration of HARQ enabling/disabling per HARQ process can also apply to SPS PDSCH.

Regarding the impacts for HARQ disabling on SPS activation, the feedback for the first SPS PDSCH is critical to ensure the common understanding between gNB and UE. Follow NR NTN solution, ACK/NACK is reported by UE for the first SPS PDSCH based on the decoding result regardless of network configuration of enabled/disabled for this HARQ process if additional signal indicated. For other SPS PDSCH transmission, the ACK/NACK feedback or not is determined by the HARQ process number and corresponding HARQ enabling/disabling configuration. For DCI indicating SPS PDSCH release, HARQ-ACK report is as legacy.

Note that only uplink SPS (e.g., SPS for BSR) with only single HARQ process is supported in NB-IoT, so there is no SPS PDSCH HARQ disabling issue for NB-IoT.

TS36.321 Section 5.3.1

For configured downlink assignments, the HARQ Process ID associated with this TTI is derived from the following equation:

-	if the TTI is a subframe TTI:

-	HARQ Process ID = [floor(CURRENT_TTI/semiPersistSchedIntervalDL)] modulo numberOfConfSPS-Processes,

where CURRENT_TTI=[(SFN * 10) + subframe number].

-	else:

-	HARQ Process ID = [floor(CURRENT_TTI/semiPersistSchedIntervalDL-sTTI)] modulo numberOfConfSPS-Processes-sTTI,

where CURRENT_TTI = [(SFN * 10 * sTTI_Number_Per_Subframe) + subframe number * sTTI_Number_Per_Subframe + sTTI_number]. Refer to 5.10.1 for sTTI_Number_Per_Subframe and sTTI_number.

For BL UEs or UEs in enhanced coverage, CURRENT_TTI refers to the TTI where first transmission of repetition bundle takes place.

TS36.213 Section 9.2

Table 9.2-1B: Special fields for Semi-Persistent Scheduling Activation MPDCCH Validation

|  | DCI format 6-0A | DCI format 6-1A |
| --- | --- | --- |
| HARQ process number | set to '000' | Set to all '0's |
| Redundancy version | set to '00' | set to '00' |
| TPC command for scheduled PUSCH | set to '00' | N/A |
| TPC command for scheduled PUCCH | N/A | set to '00' |

Proposal 6: For SPS PDSCH, ACK/NACK is reported by UE for the first SPS PDSCH regardless of network configuration of enabled/disabled for this HARQ process if additional signal indicated.

Conclusions

In this contribution, considerations of HARQ enhancement for IoT NTN are provided. The following proposals are given.

Proposal 1: Disabling HARQ feedback for downlink transmission should be supported in Rel.18 NTN IoT as in WID.

Proposal 2: Scenarios/cases applied with the disabling HARQ feedback for downlink transmission in IoT NTN should be further discussed.

Proposal 3：Disabling HARQ feedback is configured by RRC signaling, and NR NTN configuration is the baseline solution.

Proposal 4: For IoT NTN DL HARQ process with disabled HARQ feedback, (N)PDCCH/(N)PDSCH scheduling restriction should be imposed to allow enough time for UE decoding processing.

Proposal 5: Configuration of HARQ enabling/disabling per HARQ process also applies to SPS PDSCH.

Proposal 6: For SPS PDSCH, ACK/NACK is reported by UE for the first SPS PDSCH regardless of network configuration of enabled/disabled for this HARQ process if additional signal indicated.

- References

3GPP TR 36.763 V1.0.0 (2021-06)

RP-213596, “New WID on IoT NTN enhancements”, MediaTek Inc, RAN#94e

R1-2104570, “HARQ in IoT-NTN”, MediaTek Inc, RAN1#105e

R1-2104817, “On HARQ enhancements for IoT NTN”, Ericsson, RAN1#105e