3GPP TSG RAN WG1 Meeting #109                                                                 	     R1-2204517

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	9.12.3

Source:	Lenovo, Motorola Mobility

Title:	Improved GNSS operations for IoT NTN

Document for:	Discussion and decision

Introduction

In the RAN#94 plenary meeting, an enhancement work item for Rel.18 IoT NTN was approved. One of the objectives is to specify the following GNSS operations enhancements to IoT NTN.

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below:

-	Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2]

-	Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption [RAN1]

In this contribution, detailed considerations of potential enhancement for IoT NTN, especially GNSS operation improvements are presented.

Discussion

Review of Rel.17 GNSS operation

In Rel.17 IoT NTN discussion, simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed, that is, GNSS and IoT modules should operate alternatively. GNSS measurement is related to valid GNSS position fix that UE uses to determine its own geolocation, which is critical towards maintaining uplink synchronization (e.g., the pre-compensation behavior).

Agreement

- For sporadic short transmission, UE in RRC_CONNECTED should go back to idle mode and re-acquire a GNSS position fix if GNSS becomes outdated

- The UE autonomously determines its GNSS validity duration X and reports information associated with this valid duration to the network via RRC signalling.

  - X = {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity}

- Note: The duration of the short transmission is not longer than the “validity timer for UL synchronization” referred to in the WID objective (but which still needs further discussion for specifying further details)

Based on the UE’s reported validity duration of GNSS position fix (e.g., 10s, 20s…) as specified in Rel.17 IoT NTN, the network can decide whether to move the UE to RRC_IDLE or perform an uplink scheduling. In typical case, the network should ensure uplink transmission to be terminated before GNSS position fix outdate, which can reduce the resource waste by ensuring that GNSS position fix is valid for a given transmission. The mechanism is suitable for sporadic short transmission as prioritized in Rel.17 IoT NTN.

Need of GNSS operation improvement

In Rel.17 IoT NTN discussion, only sporadic short transmission is in the scope of study. In TR36.763, the definition of sporadic short transmission is specified as

For sporadic short transmission:

- The idle UE wakes up from idle DRX / PSM, access the network, perform uplink and/or downlink communications for a short duration of time and go back to idle.

- Before accessing the network, the UE acquires GNSS position fix and does not need to re-acquire a GNSS position fix for the transmission of the packets.

Details of the duration of the short transmission, acquisition of the GNSS position and validity of the GNSS position can be discussed in normative phase.

However, in scenario of converge enhancement for NB-IoT, a total duration of NPUSCH transmission may span tens of seconds, which may be another traffic scenarios in IoT NTN (e.g., IoT transmission with larger repetition and coverage enhancement).  Table 1 indicates the maximum total durations of PUSCH transmissions and NPUSCH transmission can span up to 40s, which is not exactly same as typical traffic model of sporadic short transmission in Rel.17 IoT NTN.

Table 1 NPUSCH transmission duration in converge enhancement

As described above, one NPUSCH transmission may span up to 40s, which may be longer than the GNSS validity duration (e.g. when the GNSS validity duration is 10s or 20s, etc). It means that, during the one PUSCH transmission (that may span up to 40s), UE can’t complete the NPUSCH transmission, and the UE shall go back to idle mode and re-acquire a GNSS position fix as specified UE behavior in Rel.17 IoT NTN. GNSS operation improvement should be supported to facilitate IoT potential long uplink transmission.

Proposal 1: GNSS operation improvement should be supported to facilitate IoT potential long uplink transmission.

GNSS operation improvement

GNSS operation should be improved for long connection and high-velocity UEs. Several solutions have been discussed in Rel.17. The solutions include:

- Moving UE to RRC_IDLE

- Scheduling a gap to allow UE to refresh its GNSS position fix

- UE re-acquire GNSS in connected DRX

- UE closed-loop time and frequency corrections issued by the base-station

For option i), when the GNSS validity is invalid, UE in RRC_CONNECTED suspends the uplink transmission and move to RRC_IDLE (e.g., re-acquire a GNSS position fix). The solution is not feasible for UE with potential long uplink transmission. UE needs to perform frequent RACH procedure when the GNSS becomes outdated, which will significantly increase the UE cost. Furthermore, once the UE moves to RRC_IDLE mode, then all repetitions for the ongoing transmission will be wasted, even with new GNSS position fix outdate, there is a risk that the uplink transmission can’t be completed (e.g., uplink transmission spans up to tens of seconds while the GNSS validity duration is ~10s).

For option ii), similar as NBIoT uplink transmission gap (e.g., with length of 40ms) during uplink transmission, a new scheduling gap (e.g., much longer than legacy uplink transmission gap) after a transmission duration is configured to UE. The new scheduling gap length is determined by the GNSS Time To First Fix (TTFF), the GNSS TTFF parameter is typically 1~2 seconds with hot start or around 5~9 seconds with warm or cold start, which can be configured by eNB based on the UE reported capability. The transmission duration can be configured by eNB based on the UE reported validity duration of GNSS position fix. During the new transmission gap, UE can refresh its GNSS position fix, may need to maintain DL synchronization, and may even need to perform contention-free/contention-based based RACH procedure to confirm the serving cell and corresponding TA. Some of the UE behaviors in the new transmission gap are up to UE implementation (e.g., GNSS position fix, DL SYNC).

Figure 1: Scheduling new gap for uplink transmission

For option iii), eNB configures long DRX (e.g., maximal DRX period of 2.56 s or maximal eDRX period of 10.24s) and UE re-acquire GNSS in the connected DRX, which allows UE to make a GNSS position measurement with a typical hot fix of 1 second. It is up to UE implementation to re-acquire a GNSS position fix during any DRX OFF period without any specification impact. However, it will make some scheduling restriction with the long DRX configuration, and it can’t be suitable to all traffic models.

For option iv), UE timing and frequency adjustment is based on UE closed-loop corrections, which may be applicable for stationary UEs or UEs with low speed. But for high-velocity UEs, the closed loop correction may not be able to track the variation. Furthermore, if there is no update GNSS position fix, it is hard for UE to calculate the UE-specific TA based on the satellite ephemeris information and UE GNSS position fix.

Based on the above analysis, option ii and option iii can work for long duration and high-velocity UEs, especially the detail of option ii) needs further study.

Proposal 2: Scheduling new gap solution and configure long DRX solution can be supported for UE with long uplink transmission.

Proposal 3: Solution of scheduling new gap to allow UE to refresh its GNSS position fix needs further study.

Conclusions

In this contribution, considerations of GNSS operation enhancement for IoT NTN are provided. The following proposals are given.

Proposal 1: GNSS operation improvement should be supported to facilitate IoT potential long uplink transmission.

Proposal 2: Scheduling new gap solution and configure long DRX solution can be supported for UE with long uplink transmission.

Proposal 3: Solution of scheduling new gap to allow UE to refresh its GNSS position fix needs further study.

- References

3GPP TR 36.763 V1.0.0 (2021-06)

RP-213596, “New WID on IoT NTN enhancements”, MediaTek Inc, RAN#94e