3GPP TSG RAN WG1 #109-e		R1-2204518

e-Meeting, May 9th – 20th, 2022

Agenda Item:     8.17

Source:	Spreadtrum Communications

Title:	      	     Remaining issues on RAN1 aspects for small data transmission

Document for:	Discussion and decision

Introduction

In this paper, we will provide one text proposal for small data transmission.

Text proposals

Reason for changes

For Rel-15/Rel-16, for single cell operation or for operation with carrier aggregation in a same frequency band, when the QCL-TypeD property of PDCCH in Type-0/0A/2/3 CSS or USS set are different from the QCL-TypeD property of PDCCH in Type-1 CSS and, and both PDCCHs or associated PDSCH are overlapping or partially overlapping in time, a UE does not expect to monitor the PDCCH in a Type-0/0A/2/3 CSS or USS set.

In Rel-17, Type-1A CSS is introduced to support small data transmission. In our understanding, although the type of SS of Type-1A is common, the scheduling behaviour is kind of UE specific data scheduling, for the sake of PDCCH in Type-1A CSS with CRC scrambled by a C-RNTI or a CS-RNTI. Thus, taking USS as reference, the priority of Type-1A CSS can be regarded as being lower than Type-1 CSS when referring to collision issue of the QCL-Type D property of overlapped PDCCHs each in Type-1 CSS or Type-1A CSS.

Summary of changes

When the QCL-TypeD property of PDCCH in Type-1A CSS is different from the QCL-TypeD property of PDCCH in Type-1 CSS and, and both PDCCHs or associated PDSCH are overlapping or partially overlapping in time, a UE does not expect to monitor the PDCCH in a Type-1A CSS.

Consequences if not approved:

The UE behavior is not clear, when the QCL-TypeD property of PDCCH in Type-1A CSS is different from the QCL-TypeD property of PDCCH in Type-1 CSS and, and both PDCCHs or associated PDSCH are overlapping or partially overlapping in time.

Based on the above analysis, we have the following proposal:

Proposal 1: Suggest to endorse the Text Proposal#1 in TS 38.213

------------------------------------------Start of Text Proposal#1 for TS 38.213--------------------------------------

- 10.1	UE procedure for determining physical downlink control channel assignment

------------------------------------------Unchanged part omitted--------------------------------------

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not expect to monitor a PDCCH in a Type0/0A/0B/1A/2/3-PDCCH CSS set or in a USS set if a DM-RS for monitoring a PDCCH in a Type1-PDCCH CSS set is not configured with same qcl-Type set to 'typeD' properties [6, TS 38.214] with a DM-RS for monitoring the PDCCH in the Type0/0A/0B/1A/2/3-PDCCH CSS set or in the USS set, and if the PDCCH or an associated PDSCH overlaps in at least one symbol with a PDCCH the UE monitors in a Type1-PDCCH CSS set or with an associated PDSCH.

------------------------------------------End of Text Proposal#1 for TS 38.213--------------------------------------

Conclusion

In this contribution, we present one proposal on SDT.

Proposal 1: Suggest to endorse the Text Proposal#1 in TS 38.213

------------------------------------------Start of Text Proposal#1 for TS 38.213--------------------------------------

- 10.1	UE procedure for determining physical downlink control channel assignment

------------------------------------------Unchanged part omitted--------------------------------------

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not expect to monitor a PDCCH in a Type0/0A/0B/1A/2/3-PDCCH CSS set or in a USS set if a DM-RS for monitoring a PDCCH in a Type1-PDCCH CSS set is not configured with same qcl-Type set to 'typeD' properties [6, TS 38.214] with a DM-RS for monitoring the PDCCH in the Type0/0A/0B/1A/2/3-PDCCH CSS set or in the USS set, and if the PDCCH or an associated PDSCH overlaps in at least one symbol with a PDCCH the UE monitors in a Type1-PDCCH CSS set or with an associated PDSCH.

------------------------------------------End of Text Proposal#1 for TS 38.213--------------------------------------

Reference

- 3GPP TS 38.213: "NR; Physical layer procedures for control", h10