3GPP TSG RAN WG1 #109-e		                     R1- 2204519

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.4

Source:	LG Electronics

Title:	Remaining issues on UL time and frequency synchronization enhancements in
NTN

Document for:	   Discussion and Decision

- Introduction

In the Rel-17 NR NTN, appropriate agreements were made to support UL time and frequency synchronization enhancements, but there are some remaining issues. So, in this contribution, we discuss on the remaining issues of UL time and frequency synchronization in NTN, and also provide the corresponding proposal.

- Discussion

Issue on closed loop TA (i.e., NTA)

In RAN1 #106bis-e meeting, it was concluded that a TA margin is not defined in Rel-17 NR NTN. Based on this conclusion, the parameter “NTA_old” is no longer necessary for the equation of closed loop TA (i.e., NTA), when TAC (TA) in msg2/msgB is received. Moreover, following working assumption was made in RAN1 107-e meeting [1]:

Working assumption: When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as: - Option 1: NTA=TA⋅16⋅642μ. Where, TA is the TAC field in msg2/msgB

Therefore, we think there is no need to discuss this issue further, so it is preferred to confirm the above working assumption.

Proposal 1. Confirm the following working assumption:

Working assumption:

When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as:

- Option 1: NTA=TA⋅16⋅642μ.

where, TA is the TAC field in msg2/msgB

Issue on UE behavior w.r.t Validity timer expiry

In RAN1 #108-e meeting [2], the UE behavior for validity timer expiration has been discussed, but it is fail to achieve consensus. In [3], some companies proposed that additional UE behavior regarding this issue should be specified, but we think that it can be handled by network implementation. For example, network can appropriately set the Epoch time of additional information (e.g., common TA parameters and/or ephemeris information) before expiry of validity timer. As a result, UE can operate well as shown in Figure 1. In summary, it is desirable to set the Epoch time of additional information appropriately by network.

Figure 1. An example of timeline between Epoch time, validity duration, and validity timer.

Proposal 2. The NTN UE shall re-acquire new assistance information before expiry of UL validity timer.

- The Epoch time of additional information (e.g., common TA parameters and/or ephemeris information) should be set before expiry of validity timer.

- Conclusion

In this contribution, we discussed on the remaining issues of UL time and frequency synchronization in NTN and also provide the corresponding proposals. Based on the above discussion, we have following proposals.

Proposal 1. Confirm the following working assumption:

Working assumption:

When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as:

- Option 1: NTA=TA⋅16⋅642μ.

where, TA is the TAC field in msg2/msgB

Proposal 2. The NTN UE shall re-acquire new assistance information before expiry of UL validity timer.

- The Epoch time of additional information (e.g., common TA parameters and/or ephemeris information) should be set before expiry of validity timer.

- Reference

[1] RAN1 #107-e Chairman’s note

[2] RAN1 #108-e Chairman’s note

[3] R1-2202553 	FL Summary #3: Enhancements on UL time and frequency synchronization for NR NTN 	Moderator (Thales)