3GPP TSG RAN WG1 #109-e		                     R1- 2204521

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.12.1

Source:	LG Electronics

Title:	Discussion on coverage enhancement for NR NTN

Document for:	   Discussion and Decision

- Introduction

In the WID for NR NTN enhancement [1], the following objectives are relevant to the physical layer design.

- 6-month study phase - Evaluate the coverage performance and identify the candidate physical radio channels that have coverage issues specific to NTN with following target services taking into account the studies in TR38.830 where appropriate, as well as general coverage enhancement techniques specified in Rel-18 [RAN1,RAN2,RAN4] - VoIP and low-data rate services for commercial handset terminals - Next step of study phase - NTN-specific repetitions enhancements beyond techniques covered in Rel-17 CovEnh WI for the relevant channels - NTN-specific techniques for improved diversity and/or reduced polarization loss - Improved performance of low-rate codecs in link budget limited situation including reducing RAN protocol overhead for VoNR - NOTE: Intent is not to introduce a new codec.

In this contribution, we discuss on the evaluation methodology and potential specification impact for NR NTN enhancement.

- Discussion

Evaluation of coverage performance

- Performance metric and general aspect

During Rel-17 discussion, NTN WI and CE WI had conducted evaluation in order to identify issue and required performance enhancement, for each environment, with different evaluation methodology. In order to identify the candidate physical radio channels that have coverage issues specific to NTN, it would be necessary to consider evaluation methodology and assumptions used for CE WI.

To identify coverage issues for NTN environment, the metric of coverage performance should be defined first. In Rel-16 NTN study phase, carrier-to-noise-and-interference ratio (CNIR) was used for link budget performance. However, for coverage enhancement purpose, we may consider another metric, which can represent required link budget for specific channel given required SNR/throughput.

In Rel-17 CE WI, three of metrics had been used; MIL, MCL and MPL. Each metric are defined in TR 38.830. MCL and MIL can be used to identify the coverage bottleneck among candidate physical radio channels. Some of candidate physical radio channels can be exempted from coverage bottleneck, based on a value of MPL, which can define deployment dependent target in given scenarios. Therefore, it is proposed to re-use this method for coverage bottleneck identification in NTN enhancement. Considering objectives of NR NTN enhancement, MIL can be considered mainly for reflecting impact of different UE antenna gain and polarization loss.

Proposal 1. Reuse the metric of coverage bottleneck identification defined in TR 38.830.

For coverage bottleneck identification, it is necessary to derive MIL for all combinations of deployment scenarios and physical channels. To reduce the simulation burden, reduced number of scenarios and candidate physical channel would be desirable. Therefore, it would be beneficial to limit the number of scenario and channel combinations for efficient evaluation.

In terms of deployment scenarios, LEO-600/1200 and GEO can be considered basically. Considering that VoIP and low-data rate services for commercial handset terminals, the LEO can be considered as a baseline. In addition to this, handheld type of UE should be considered for the worst scenario which shows the bottleneck. Note that smart phone form factor of about -5dB should be take into account when handheld type is considered.

To cover various scenarios, MEO can be also considered for identifying bottleneck with different antenna configurations, but it should be optional scenario in order to reduce the simulation burden. Other detailed evaluation assumptions can be re-used from TR 38.821 according to scenarios.

Proposal 2. LEO-600/1200 and handheld UE characteristics should be baseline scenario for NTN CE.

- GEO and MEO satellite can be optionally considered

Proposal 3. For other evaluation assumption, the evaluation assumption defined in TR 38.821 is used as a baseline for the NTN CE.

- Candidate physical channels for the evaluation

For target channel of coverage enhancement, we can refer to evaluation result of Rel-17 CE. Though the evaluation was conducted with different assumption, PUSCH and PRACH were identified as bottleneck channels that have coverage issues, in most of scenarios. In other words, which scenario is assumed may have no significant impacts for identifying bottleneck channel. Therefore, it can be expected similar tendencies to Rel-17 CE evaluation even with NTN scenarios and assumption. Thus, we propose following:

Proposal 4. PUSCH/PRACH should be considered as a candidate physical radio channels for the NTN CE.

In addition, the traffic assumption for each channel should be considered as well. Since the traffic assumption would determine required SNR, it has direct impact on the coverage performance.  According to evaluation result in Rel-17 CE, derived coverage performance metric also were highly dependent to channel-specific parameter; for example, payload, target BLER , time-frequency resource allocation, and DMRS configuration.

Proposal 5. Channel specific parameter (e.g., payload, target BLER, time-frequency resource allocation and etc.) should be defined with the consideration of use cases in NTN environment.

- Consideration on Rel-17 CE techniques in terms of the evaluation

One purpose of the evaluation is to introduce Rel-17 CE techniques to NTN environment. In this point of view, Rel-17 CE techniques can be considered as a baseline scheme for evaluation assumption. For example, the PUSCH/PUCCH repetitions and TB over multiple slots (TBoMS) can be considered as one of scenarios. In terms of repetition, it may be necessary to consider larger number (e.g., 32), in order to find proper number of repetitions. For efficient evaluation, the number of case should be limited to few combinations between the number of repetition and the number of allocated slots for TBoMS. Since NTN considers FDD mainly, it may not be necessary to consider available slot counting.

Proposal 6. Consider Rel-17 CE techniques (e.g., repetition, TBoMS) as a baseline scheme.

Potential enhancement points

- Initial access procedure (PRACH preamble and/or Msg. 3 PUSCH)

In Rel-17 CE study item phase, it was proposed that the coverage enhancement for UL signal/channel in initial access procedure is necessary. Therefore, both PRACH preamble repetition and Msg. 3 PUSCH repetition were considered in this phase. However, due to lack of discussion time, repeated transmission of Msg. 3 PUSCH, which had a high priority, was preferentially introduced in Rel-17 CE work item phase, and repeated transmission of PRACH preamble, which had a low priority, will be additionally introduced in Rel-18 CE work item phase.

Regarding this point, it may be suggested that repeated transmission of Msg. 3 PUSCH should be introduced to Rel-18 NTN preferentially, since this feature has already introduced in Rel-17 CE for initial access procedure. However, we think that there is any reason that Rel-18 NTN cannot simultaneously introduce repeated transmission of PRACH preamble, which are scheduled to be introduced in Rel-18 CE. Furthermore, simultaneous discussion of PRACH preamble repetition and Msg. 3 PUSCH repetition on Rel-18 NTN can be a good way to efficiently use insufficient discussion time. For these reasons, it can be desirable to consider introducing both PRACH preamble repetition and Msg. 3 PUSCH repetition in Rel-18 NTN.

Proposal 7. It can be desirable to consider introducing both PRACH preamble repetition and Msg. 3 PUSCH repetition in Rel-18 NTN.

We believe that repeated transmission of Msg. 3 PUSCH introduced in Rel-17 CE can be supported in Rel-18 NTN without further enhancement. However, since repeated transmission of PRACH preamble is not supported in NR, it would be necessary to consider the following discussion points to support PRACH preamble repetition.

First of all, it is necessary to determine the maximum repetition number. For example, the maximum repetition number of PRACH preamble can be determined as a similar level of the maximum repetition number of Msg. 3 PUSCH (e.g., 8). Moreover, this repetition number can be provided to the UE via higher layer signaling (e.g., SIB).

Secondly, since the UE can be defined to repeatedly transmit PRACH preamble over multiple ROs, it is necessary to define a reference RO for the UE and network to use the same RA-RNTI value. For example, the last RO among multiple ROs used for PRACH preamble repetition can be defined as a reference RO, and the RA-RNTI would be calculated based on the reference RO.

Lastly, it is necessary to define additional UE behaviour for determining starting point of the RAR window. In this case, similar to the previous example, starting point of RAR window may be determined by UE based on the last RO among multiple ROs used for PRACH preamble repetition.

Proposal 8. If repeated transmission of PRACH preamble is supported in Rel-18 NTN, RAN1 should at least consider the following discussion points.

- Maximum repetition number

- RA-RNTI

- RAR window starting point

- Normal UL transmission (PUSCH and/or PUCCH)

Repeated transmission of PUSCH and/or PUCCH introduced in Rel-17 CE (e.g., Type-A PUSCH repetition, TBoMS, and PUSCH/PUCCH repetition number) can be supported in Rel-18 NTN without NTN specific enhancement. However, it is possible to discuss whether to support the DMRS bundling in Rel-18 NTN for joint channel estimation. In order to perform the joint channel estimation at the network, the UE should transmit the UL signal/channel to a fixed UL TA value (i.e., Total TA including the UE specific TA and common TA). But, in NTN system, especially for LEO case, since UL TA value depends heavily on the movement of the satellite, the UE behavior of maintaining the UL TA during UL transmission may be undesirable. Therefore, it seems necessary to discuss whether to support the DMRS bundling in Rel-18 NTN for joint channel estimation.

Proposal 9. It is necessary to discuss whether to support the DMRS bundling in Rel-18 NTN for joint channel estimation.

In case when it is necessary to modify the open loop TA (e.g., UE specific TA and/or common TA) during repeated transmission of UL signal/channel, one of the following solutions may be considered. First of all, the UL segment introduced in Rel-17 IoT NTN can be considered forRel-18 NTN. If it is supported, the UE can change the open loop TA at UL segment boundary. In addition, it may be considered to support update period of open loop TA, so that UE specific TA and/or common TA can be updated periodically.

Proposal 10. It is necessary to discuss how to support changing open loop TA (e.g., UE specific TA and/or common TA) during repeated transmission of UL signal/channel in Rel-18 NTN.

- Conclusion

In this contribution, we discussed on the evaluation methodology and potential specification impact for NR NTN enhancement. Based on the above discussion, we have following proposals.

Proposal 1. Reuse the metric of coverage bottleneck identification defined in TR 38.830.

Proposal 2. LEO-600/1200 and handheld UE characteristics should be baseline scenario for NTN CE.

- GEO and MEO satellite can be optionally considered

Proposal 3. For other evaluation assumption, the evaluation assumption defined in TR 38.821 is used as a baseline for the NTN CE.

Proposal 4. PUSCH/PRACH should be considered as a candidate physical radio channels for the NTN CE.

Proposal 5. Channel specific parameter (e.g., payload, target BLER, time-frequency resource allocation and etc.) should be defined with the consideration of use cases in NTN environment.

Proposal 6. Consider Rel-17 CE techniques (e.g., repetition, TBoMS) as a baseline scheme.

Proposal 7. It can be desirable to consider introducing both PRACH preamble repetition and Msg. 3 PUSCH repetition in Rel-18 NTN.

Proposal 8. If repeated transmission of PRACH preamble is supported in Rel-18 NTN, RAN1 should at least consider the following discussion points.

- Maximum repetition number

- RA-RNTI

- RAR window starting point

Proposal 9. It is necessary to discuss whether to support the DMRS bundling in Rel-18 NTN for joint channel estimation.

Proposal 10. It is necessary to discuss how to support changing open loop TA (e.g., UE specific TA and/or common TA) during repeated transmission of UL signal/channel in Rel-18 NTN.

- Reference

[1] RP-213690, “New WI: NR NTN (Non-Terrestrial Networks) enhancements”, RAN Vice-Chair (AT&T), Thales