3GPP TSG RAN WG1 #109-e		                         R1- 2204523

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.2.1

Source: 	LG Electronics

Title: 	Discussion on integrity of RAT dependent positioning techniques

Document for:	Discussion and decision

- Introduction

In RAN#94-e meeting, a study item on evolution of OFDM based carrier phase measurement in NR is approved and the corresponding description is provided in [1]. According to the SID, objectives are identified in the cope as follows.

- Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]: - Identify the error sources, [RAN1, RAN2]. - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2] - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

In this contribution, we discuss on potential error sources of RAT dependent positioning techniques.

- Discussion

According to the TS 22.872, definition of integrity is defined as bellow:

-	Integrity: measure of the trust in the accuracy of the position-related data provided by the positioning system and the ability to provide timely and valid warnings to the UE and/or the user when the positioning system does not fulfil the condition for intended operation.

As described above, integrity could be used as measure of the trust that UE and positioning system have. Moreover, it can provide timely and valid warning to UE when the positioning error beyond allowable limit is occur. Positioning integrity is already supported by GNSS services. Furthermore, in Rel-17 NR, integrity for RAT-dependent positioning with GNSS was introduced. In TS 38.305[2], integrity operation for RAT-dependent positioning was captured, and error sources and associated parameters are listed. For RAT-dependent integrity, same or similar methodology with RAT-independent integrity can be considered in Rel-18. However, error source between RAT-dependent and RAT-independent positioning system could be different from each other because of differences in positioning environments and scenarios. From RAN1 perspective, study for identifying physical layer level error sources would be required.

In NR positioning system, especially in RAT-dependent positioning, several types of positioning methods are supported. For example, the NR positioning system supports time-based positioning methods, such as DL/UL-TDOA and Multi-RTT, and angle-based positioning methods, such as DL-AoD, UL-AoA. Due to the different characteristic of each positioning method, each has a different source of error that is important to be considered. For example, in measurement error perspective, TOA measurement error or reporting delay could be error sources in time-based positioning methods, while departure or arrival beam angle measurement error could be considered to be error sources in angle-based positioning methods. In case of using time-based positioning, the effect of synchronization errors between TRPs can be considered, whereas angle-based positioning methods do not. Besides, study on positioning based on the NR carrier phase measurement has started in companion item in Rel-18. Therefore, it is also worth considering the error source corresponding to the carrier phase measurement of the RAN dependent positioning system.

On the other hand, we can also consider some error sources that are common for all kind of positioning methods. Multipath is one of the most significant error source in measurement for positioning system. For example, received signal from LOS path can be distorted by the NLOS component, which may degrade sequence correlation property and/or induce phase distortion. Moreover, when blockage or shadowing of the LOS path occurs, there is a possibility that receiver tracks NLOS path for measuring distance and/or direction. In this case error will be biased to NLOS path which is not desirable for positioning. However, it should be noted that since each positioning method has different measurement characteristics, the resulting error types due to multipath environment may be different.

Observation 1: Each positioning method of RAT-dependent positioning can have different error sources

Observation 2: For study of identification of the error sources for time-based positioning methods (e.g. TDOA and Multi-RTT), timing measurement, TRP synchronization and multipath environment could be considered.

Observation 3: For study of identification of the error sources for angle-based positioning methods (e.g. AoD and AoA), angle measurement and multipath environment could be considered.

- Conclusion

In this contribution, we discussed on potential error sources on RAT dependent positioning. From the discussion, following observations are obtained.

Observation 1: Each positioning method of RAT-dependent positioning can have different error sources

Observation 2: For study of identification of the error sources for time-based positioning methods (e.g. TDOA and Multi-RTT), timing measurement, TRP synchronization and multipath environment could be considered.

Observation 3: For study of identification of the error sources for angle-based positioning methods (e.g. AoD and AoA), angle measurement and multipath environment could be considered.

- Reference

[1] Intel, “Revised SID on Study on expanded and improved NR positioning” RP-213588, e-Meeting, December 6th – 17th, 2021

[2] 3GPP TS 38.305, V17.0.0 (2022-03): “NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN”