3GPP TSG RAN WG1 #109-e	R1-2204527

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.8.1

Source: 	LG Electronics

Title: 	Remaining details on PUSCH enhancements

Document for:	Discussion and decision

Introduction

This contribution provides our views on several issues need to be discussed and clarified for coverage enhancements for PUSCH.

The available slot determination for multiple TRPs

In RAN1#107bis-e meeting, the issue on the available slot counting for inter-cell mTRPs was discussed and the discussion is deferred until further progress on the collision handling between UL channels/signals and multiple SSBs for inter-cell mTRPs is made in feMIMO session. [1]

In this regard, feMIMO session made a related agreement in the last meeting as follow [2].

Agreement For inter-cell mTRP, UE does not transmit PUCCH/PUSCH/PRACH in a slot or SRS in the symbols if in time domain the PUCCH/PUSCH/PRACH/SRS overlaps with an SSB of a serving cell PCI or an SSB associated with the active additional PCI.

In inter-cell mTRP, an UE can perform uplink transmission to a serving cell PCI or an active non-serving PCI in inter-cell mTRP. When PUCCH/PUSCH/PRACH/SRS transmission of the UE overlaps with an SSB symbol of a serving cell PCI or the active PCI, the UE omits the uplink transmission based on the agreement.

Regarding the determination of NPUCCHRepeat slots for a PUCCH transmission in inter-cell mTRP, there is no change in the specification, so only the SSB symbols for the serving cell PCI is used for determining the NPUCCHRepeat slots.

Based on the above understanding on PUCCH transmission in inter-cell mTRP environment, it is preferred to align the determination of available slots for PUSCH with the operation of PUCCH. At this stage, the optimization of PUSCH repetitions with available slot based counting for the mTRP environment seems not essential.

Proposal 1: Specification change for available slot determination for PUSCH in inter-cell mTRP is not necessary.

Summary

In this contribution, we provide our views on the available slot determination for multiple TRPs as follow.

Proposal 1: Specification change for available slot determination for PUSCH in inter-cell mTRP is not necessary.

Reference

- RAN1 Chair’s Notes, RAN1 #107bis-e, e-Meeting, January 17th – 25th, 2022

- RAN1 Chair’s Notes, RAN1 #108-e, e-Meeting, February 21st – March 3rd, 2022