3GPP TSG RAN WG1 #109-e	R1-2204528

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.10

Source: 	LG Electronics

Title: 	Remaining details on enhancements to IAB

Document for:	Discussion and decision

Introduction

This contribution provides our views on several issues need to be discussed and clarified for Rel-17 IAB enhancements.

Resource multiplexing

Frequency resource for cell-specific/semi-static signals and channels

During the Rel-16 IAB WI, it was agreed that the symbol resource which is configured with cell-specific signals/channels is treated to be Hard DU resource [1], therefore following is supported in TS 38.213.

A symbol of a slot is equivalent to being configured as hard if an IAB-DU would transmit a SS/PBCH block, PDCCH for Type0-PDCCH CSS sets configured by pdcchConfigSIB1, or a periodic CSI-RS in the symbol of the slot, or would receive a PRACH or a SR in the symbol of the slot.

Also, the equivalent agreement applied for frequency domain H/S/NA determination was made in RAN1#104-e meeting as follow.

Agreement Regardless of simultaneous operation, the same cell-specific/semi-static signals and channels of the IAB-DU considered as hard time/frequency resources in Rel-16 are also considered as hard time/frequency resources in Rel-17. FFS: IAB-MT behavior in case of conflicts between cell-specific signals/channels and other resource configurations of the IAB-MT (e.g., dedicated slot configurations)

According to the agreement, in the symbols for cell-specific/semi-static signals and channels, PRB resources for the cell-specific/semi-static signals and channels of the IAB-DU is treated as Hard. However, it seems not captured in the corresponding specification yet.

Thus, we suggest to reflect the agreement in TS 38.213 and provide corresponding TP.

Proposal 1: Capture the following TP for TS 38.213 on frequency resource for cell-specific/semi-static signals and channels.

- 14. Integrated access-backhaul operation -------------------------------------------------------- Omitted ----------------------------------------------------- With reference to slots of an IAB-DU cell, the IAB-DU can be provided an indication of hard, soft or unavailable type per RB set for symbols configured as downlink, uplink or flexible in a slot by Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config [16, TS 38.473]. The RB set size and the number of RB sets are configured by RB-Set-Configuration [16, TS 38.473]. If an indication of hard, soft or unavailable type is not provided for an RB set of a symbol in a slot, the IAB-DU applies the configuration of hard, soft or unavailable type provided by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] for the RB set of the symbol in the slot. If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config [16, TS 38.473] when the IAB-node uses simultaneous transmission and reception in the slot. When an RB set of a downlink, uplink, or flexible symbol is configured as hard, the IAB-DU cell can respectively transmit, receive, or either transmit or receive on the RB set in the symbol. When an RB set of a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU cell can respectively transmit, receive or either transmit or receive on the RB set in the symbol only if - the IAB-MT does not transmit or receive on the RB set during the symbol of the IAB-DU cell, or - with respect to all serving cells, the IAB-MT would transmit or receive on the RB set during the symbol of the IAB-DU cell, and the transmission or reception on the RB set or any RB set that is configured as unavailable or configured as soft and not indicated as available during the symbol of the IAB-DU cell is not changed due to a use of the RB set in the symbol by the IAB-DU, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft RB set as available if the IAB-MT is not configured with an SCG, or - the IAB-MT detects two DCI formats 2_5 with an AI index field value indicating the soft RB set as available from the MCG and SCG, respectively, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft RB set as available from one cell group and with respect to all serving cells of the other cell group, the IAB-MT would transmit or receive on the RB set during the symbol of the IAB-DU cell, and the transmission or reception on the RB set during the symbol of the IAB-DU cell does not change due to a use of the RB set in the symbol by the IAB-DU. When an RB set of a downlink, uplink, or flexible symbol is configured as unavailable, the IAB-DU neither transmits nor receives in the RB set in the symbol. If Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config is applied in is a slot, a RB of a symbol in the slot is equivalent to being configured as hard if an IAB-DU would transmit a SS/PBCH block, PDCCH for Type0-PDCCH CSS sets configured by pdcchConfigSIB1, or a periodic CSI-RS in the symbol of the slot, or would receive a PRACH or a SR in the symbol of the slot. Otherwise, a symbol of the slot is equivalent to being configured as hard if an IAB-DU would transmit a SS/PBCH block, PDCCH for Type0-PDCCH CSS sets configured by pdcchConfigSIB1, or a periodic CSI-RS in the symbol of the slot, or would receive a PRACH or a SR in the symbol of the slot. -------------------------------------------------------- Omitted -----------------------------------------------------

Adaptation of an IAB-node’s multiplexing operation

In the last meeting, there was a discussion about potential specification impact for conflict resolution between parent and child IAB nodes in case when the Rel-17 H/S/NA configuration is provided, however the IAB node needs the Rel-16 H/S/NA configuration to be applied instead of the Rel-17 H/S/NA configuration [3]. During the meeting, several alternatives for TDM/non-TDM conflict resolution between parent and child IAB nodes were discussed, but consensus was not reached.

In our view, even for an IAB-node which can perform simultaneous operation, there can be various situations that simultaneous operation is not allowed or available. And, in an IAB-node perspective, transition between simultaneous and non-simultaneous operation is expected to be happened frequently. When the IAB-node is operating in TDM, limiting the operating frequency resources by frequency domain H/S/NA attributes is not desirable in terms of resource utilization. It is hard to properly be reflected by semi-static configuration of frequency domain H/S/NA considering on-going situation of IAB-node since it is configured by CU, therefore dynamic indication from child node to parent node for switching between TDM and non-TDM operation is necessary. A new signaling based on MAC-CE can be introduced to support it.

Proposal 2: Support MAC-CE signaling from child node to parent node to indicate switching between TDM/non-TDM.

Resource availability indication for TDM and FDM in DCI format 2_5

The details on the configuration of DCI format 2_5 to support both of TDM and FDM indication was one of the main discussion in the last meeting [3].

In our view, it is necessary to support both of TDM and FDM indication in a DCI format 2_5 for simplicity which enables one of the availability indication for TDM or FDM can be applied in a slot. If the IAB-node is operating in a TDM multiplexing mode, the availability indication for TDM is applied, so the availability of the soft can be determined by DCI format 2_5. On the other hand, if the IAB-node is operating in a non-TDM multiplexing mode, the availability indication for FDM is applied, so the availability of each soft RB set can be determined by DCI format 2_5. No ambiguity is remained, simple as it is.

Regarding the configuration of DCI format 2_5 both of availability indication for TDM and FDM, following two alternatives can be considered.

- For the first alternative, each availability combination in an availabilityCombinations would include one TDM and N FDM configurations. Then, the availability combination corresponds to an availabilityCombinationId is applied, where the availabilityCombinationId is indicated in an AI index filed in DCI format 2_5. This alternative would require less specification impact, but have limited indication flexibility.

- For the other alternative, two availabilityCombinations would be configured where one is for TDM and the other one is for FDM. Both of availabilityCombinations for TDM and FDM have multiple availability combinations where each availability combination corresponds to an availabilityCombinationId.

For details, a single or separated AI index value can be used to indicate the applied availability combination for TDM and FDM.

If a single AI index field value is used, it can point both of availability combination for TDM and availability combination for FDM. Therefore, an availability combination for TDM and FDM correspond to the same availabilityCombinationId can be indicated always. In this case, there is no technical difference between it and the first alternative.

Instead, two separated AI index values where one is for TDM and the other is for FDM can be used in this alternative. Then, availability combination for TDM and FDM correspond to the different availabilityCombinationId can be applied, so flexible indication compared to the first alternative can be obtained.

To indicate two separated availabilityCombinationId for TDM and FDM, two AI index fields are required. The location of the AI index field for TDM can be determined by positionInDCI-AI as before. In addition, to indicate the AI index for FDM, the next DCI field of the AI index field for TDM can be used. In other word, the location of the AI index field for FDM can be determined as ‘positionInDCI-AI + maxlog2maxAIindex+1,1’, where maxAIindex is the maximum of the values provided by corresponding availabilityCombinationId for TDM.

In conclusion, the benefit of indication flexibility by the second alternative is valuable given the difference of specification impact between two alternatives is not expected to be large. Therefore, we suggest to adopt the second alternative with two AI fields.

Proposal 3: An IAB node is configured with two availabilityCombinations tables, one for TDM and one for FDM.

- The location of AI index field for TDM in DCI format 2_5 is determined by positionInDCI-AI.

- The location of AI index field for FDM in DCI format 2_5 is determined by positioInDCI-AI + maxlog2maxAIindex+1,1.

The corresponding proposed TP based on Proposal 3 is provided in Annex 1.

RB resource not covered by RB sets of frequency domain H/S/NA

The IAB-DU can be provided an indication of hard, soft or unavailable type per RB set by Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config. If the configured RB sets do not cover the entire carrier bandwidth, there can be RB resource not included any configured PRB sets. The discussion on this issue was made during the last meeting [3], and it is deferred until this meeting.

It is not sure to us whether this optimization issue should be resolved. If this issue should be discussed, adopting similar principle to the agreement made in the previous meeting will be the likely solution. In RAN1#106-e, it was agreed that ‘For a given RB set at a symbol, if Rel-17 frequency domain H/S/NA configuration is not provided, the Rel-16 time domain H/S/NA is applied.’. Thus, the same approach can be applied in this case.

Proposal 4: If necessary, the RBs within the carrier bandwidth but not covered by any RB set are treated according to the corresponding Rel-16 H/S/NA configuration.

The corresponding proposed TP based on Proposal 4 is provided in Annex 2.

Power control

In the last meeting, it was agreed that the IAB-MT shall account for the provided DL Tx power adjustment as indicated by parent-node in addition to Pc when deriving CSI feedback [5]. Since downlink CSI-RS EPRE is currently derived from the ratio of CSI-RS EPRE to SS/PBCH block EPRE, which is indicated via RRC parameter powerControlOffsetSS. The provided DL Tx power adjustment for IAB-MT is indicated via MAC-CE and updates the power of CSI-RS. Thus, combining them together, we suggest to reflect the agreement in TS 38.214 and provide corresponding TP.

Proposal 5: Capture the following TP in TS38.214 for CSI feedback accounting for provided power adjustment.

- 4.1 Power allocation for downlink -------------------------------------------------------- Omitted ----------------------------------------------------- The downlink CSI-RS EPRE can be derived from the SS/PBCH block downlink transmit power given by the parameter ss-PBCH-BlockPower and CSI-RS power offset given by the parameter powerControlOffsetSS provided by higher layers and provided downlink power adjustment of CSI-RS via MAC-CE if indicated, where the CSI-RS is QCLed with the SS/PBCH block, and the SS/PBCH block can be associated with serving cell PCI or additional PCI different from serving cell PCI. The downlink reference-signal transmit power is defined as the linear average over the power contributions (in [W]) of the resource elements that carry the configured CSI-RS within the operating system bandwidth. -------------------------------------------------------- Omitted -----------------------------------------------------

Summary

In this contribution, we provide our views on remaining details on IAB enhancements as follows.

Proposal 1: Capture the following TP for TS 38.213 on frequency resource for cell-specific/semi-static signals and channels.

- 14. Integrated access-backhaul operation -------------------------------------------------------- Omitted ----------------------------------------------------- With reference to slots of an IAB-DU cell, the IAB-DU can be provided an indication of hard, soft or unavailable type per RB set for symbols configured as downlink, uplink or flexible in a slot by Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config [16, TS 38.473]. The RB set size and the number of RB sets are configured by RB-Set-Configuration [16, TS 38.473]. If an indication of hard, soft or unavailable type is not provided for an RB set of a symbol in a slot, the IAB-DU applies the configuration of hard, soft or unavailable type provided by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] for the RB set of the symbol in the slot. If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config [16, TS 38.473] when the IAB-node uses simultaneous transmission and reception in the slot. When an RB set of a downlink, uplink, or flexible symbol is configured as hard, the IAB-DU cell can respectively transmit, receive, or either transmit or receive on the RB set in the symbol. When an RB set of a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU cell can respectively transmit, receive or either transmit or receive on the RB set in the symbol only if - the IAB-MT does not transmit or receive on the RB set during the symbol of the IAB-DU cell, or - with respect to all serving cells, the IAB-MT would transmit or receive on the RB set during the symbol of the IAB-DU cell, and the transmission or reception on the RB set or any RB set that is configured as unavailable or configured as soft and not indicated as available during the symbol of the IAB-DU cell is not changed due to a use of the RB set in the symbol by the IAB-DU, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft RB set as available if the IAB-MT is not configured with an SCG, or - the IAB-MT detects two DCI formats 2_5 with an AI index field value indicating the soft RB set as available from the MCG and SCG, respectively, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft RB set as available from one cell group and with respect to all serving cells of the other cell group, the IAB-MT would transmit or receive on the RB set during the symbol of the IAB-DU cell, and the transmission or reception on the RB set during the symbol of the IAB-DU cell does not change due to a use of the RB set in the symbol by the IAB-DU. When an RB set of a downlink, uplink, or flexible symbol is configured as unavailable, the IAB-DU neither transmits nor receives in the RB set in the symbol. If Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config is applied in is a slot, a RB of a symbol in the slot is equivalent to being configured as hard if an IAB-DU would transmit a SS/PBCH block, PDCCH for Type0-PDCCH CSS sets configured by pdcchConfigSIB1, or a periodic CSI-RS in the symbol of the slot, or would receive a PRACH or a SR in the symbol of the slot. Otherwise, a symbol of the slot is equivalent to being configured as hard if an IAB-DU would transmit a SS/PBCH block, PDCCH for Type0-PDCCH CSS sets configured by pdcchConfigSIB1, or a periodic CSI-RS in the symbol of the slot, or would receive a PRACH or a SR in the symbol of the slot. -------------------------------------------------------- Omitted -----------------------------------------------------

Proposal 2: Support MAC-CE signaling from child node to parent node to indicate switching between TDM/non-TDM.

Proposal 3: An IAB node is configured with two availabilityCombinations tables, one for TDM and one for FDM.

- The location of AI index field for TDM in DCI format 2_5 is determined by positionInDCI-AI.

- The location of AI index field for FDM in DCI format 2_5 is determined by positioInDCI-AI + maxlog2maxAIindex+1,1.

Proposal 4: If necessary, the RBs within the carrier bandwidth but not covered by any RB set are treated according to the corresponding Rel-16 H/S/NA configuration.

Proposal 5: Capture the following TP in TS38.214 for CSI feedback accounting for provided power adjustment.

- 4.1 Power allocation for downlink -------------------------------------------------------- Omitted ----------------------------------------------------- The downlink CSI-RS EPRE can be derived from the SS/PBCH block downlink transmit power given by the parameter ss-PBCH-BlockPower and CSI-RS power offset given by the parameter powerControlOffsetSS provided by higher layers and provided downlink power adjustment of CSI-RS via MAC-CE if indicated, where the CSI-RS is QCLed with the SS/PBCH block, and the SS/PBCH block can be associated with serving cell PCI or additional PCI different from serving cell PCI. The downlink reference-signal transmit power is defined as the linear average over the power contributions (in [W]) of the resource elements that carry the configured CSI-RS within the operating system bandwidth. -------------------------------------------------------- Omitted -----------------------------------------------------

Reference

- RAN1 Chairman’s Notes, RAN1 #97, Reno, USA, May 13th – 17th, 2019

- RAN1 Chairman’s Notes, RAN1 #104-e, e-Meeting, January 25th – February 5th, 2021

- Moderator (AT&T), “Feature Lead Summary #4 of 8.10.1,” R1-2202874, e-Meeting, February 21st – March 3rd, 2022

- RAN1 Chair’s Notes, RAN1 #106bis-e, e-Meeting, October 11th – 19th, 2021

- RAN1 Chair’s Notes, RAN1 #108-e, e-Meeting, February 21st – March 3rd, 2022

- Annex 1. Text proposals for 2.3

- 14. Integrated access-backhaul operation -------------------------------------------------------- Omitted ----------------------------------------------------- If an IAB-node is provided an AvailabilityIndicator, the IAB-node is provided an AI-RNTI by ai-RNTI and a payload size of a DCI format 2_5 by dci-PayloadSizeAI. The IAB-node is also provided a search space set configuration, by SearchSpace, for monitoring PDCCH. For each cell of an IAB-DU in a set of cells of the IAB-DU, the IAB-DU can be provided: - an identity of the IAB-DU cell by iab-DU-CellIdentity - a location of an availability indicator (AI) index field in DCI format 2_5 by positionInDCI-AI - a set of availability combinations by availabilityCombinations, where each availability combination in the set of availability combinations includes - resourceAvailability indicating availability of soft symbols in one or more slots for the IAB-DU cell, and - a mapping for the soft symbol, and/or for soft resources, availability combinations provided by resourceAvailability to a corresponding AI index field value in DCI format 2_5 provided by availabilityCombinationId - a set of availability combinations by availabilityCombinations-freq, where each availability combination in the set of availability combinations includes - one or multiple resourceAvailability with each resourceAvailablity indicating availability of soft resources in one or more slots for one RB set group where one RB set group includes one or multiple RB sets, and - a mapping for the soft symbol, and/or for soft resources, availability combinations provided by resourceAvailability to a corresponding AI index field value in DCI format 2_5 provided by availabilityCombinationId The IAB-DU can assume a same SCS configuration for availabilityCombinations for a cell as an SCS configuration provided by gNB-DU Cell Resource Configuration for the cell. An AI index field value in a DCI format 2_5 indicates to an IAB-DU a soft symbol availability in each slot for a number of slots starting from the earliest slot of the IAB-DU which overlaps in time with the slot of the IAB-MT where the IAB-MT detects the DCI format 2_5. The number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 2_5 as provided by SearchSpace. The AI index field includes maxlog2maxAIindex+1,1 bits where maxAIindex is the maximum of the values provided by corresponding availabilityCombinationId. An AI index field value for frequency domain in a DCI format 2_5 indicates to an IAB-DU a soft RB set in an RB set group availability in each slot for a number of slots starting from the earliest slot of the IAB-DU which overlaps in time with the slot of the IAB-MT where the IAB-MT detects the DCI format 2_5. The number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 2_5 as provided by SearchSpace. A location of the AI index field for frequency domain is positionInDCI-AI+maxlog2maxAIindex+1,1 and field includes maxlog2maxAIindex+1,1 bits where maxAIindex is the maximum of the values provided by corresponding availabilityCombinationId. An availability for a soft symbol or a soft RB set in an RB set group in a slot is identified by a corresponding value resourceAvailability as provided in Table 14-3. -------------------------------------------------------- Omitted -----------------------------------------------------

- Annex 2. Text proposals for 2.4

- 14. Integrated access-backhaul operation -------------------------------------------------------- Omitted ----------------------------------------------------- With reference to slots of an IAB-DU cell, the IAB-DU can be provided an indication of hard, soft or unavailable type per RB set for symbols configured as downlink, uplink or flexible in a slot by Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config [16, TS 38.473]. The RB set size and the number of RB sets are configured by RB-Set-Configuration [16, TS 38.473]. If an indication of hard, soft or unavailable type is not provided for an RB set of a symbol in a slot, the IAB-DU applies the configuration of hard, soft or unavailable type provided by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] for the RB set of the symbol in the slot. If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config [16, TS 38.473] when the IAB-node uses simultaneous transmission and reception in the slot. For any RB not included by RB set indicated by RB-Set-Configuration [16, TS 38.473], the IAB-DU applies the configuration of hard, soft or unavailable type provided by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] for the RB of the symbol in the slot. -------------------------------------------------------- Omitted -----------------------------------------------------