3GPP TSG RAN WG1 #109-e	R1-2204529

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.3.1

Source: 	LG Electronics

Title: 	Study on Evaluation for NR duplex evolution

Document for:	Discussion and decision

Introduction

In RAN#94-e meeting, a study item on evolution of NR duplex operation is approved and the corresponding description is provided in [1].

According to the SID, the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD are studied. Also, identification of deployment scenarios and developing evaluation methodology are also included in the scope as follows.

In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this contribution, we discuss on the deployment scenarios and evaluation methodology for the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD. In addition, we provide our initial evaluation results on subband non-overlapping full duplex.

Deployment scenarios

For the deployment scenario of NR duplex evolution, at least FR1 and FR2-1 can be considered. In order to discuss the deployment scenario for subband non-overlapping full duplex (i.e., SB-FD) and dynamic/flexible TDD operation in FR1 and FR2-1, the discussion in flexible duplex of the NR study in Rel-14 can be a reference. In TR 38.802, carrier frequencies of 4GHz (FR1) and 30GHz (FR2-1) were considered for the study of flexible duplex, and three deployment scenarios were considered: Dense Urban, Urban macro, and Indoor hotspot.

Additionally, it is necessary to discuss whether FR2-2 will be included in the deployment scenario for NR duplex evolution. FR2-2 can be a better environment for making isolated cells due to severe pathloss. Therefore, it can be a more favorable environment for using SB-FD and dynamic/flexible TDD since it is less affected by CLI. According to TR 38.808, deployment scenarios of indoor hotspots, dense urban areas, and factory halls were included in the FR2-2 study. Therefore, these deployment scenarios can be considered for evaluation of NR duplex evolution in FR 2-2.

Proposal 1: For deployment scenarios of study on NR duplex evolution, deployment scenarios (i.e., Dense urban, Urban macro, and Indoor hotspot) which were applied for flexible duplex evaluation in Rel-14 NR SI are considered.

Proposal 2: Evaluation for NR duplex evolution is performed at FR1 and FR2-1. Also, it can be discussed whether or not to consider FR 2-2 for study of NR duplex evolution.

Evaluation methodology

In this section, we discuss details on evaluation methodology for the SB-FD and dynamic/flexible TDD.

Interference

In the SB-FD and dynamic/flexible TDD environment considered in NR duplex evolution, it is affected by various interferences.

If SB-FD is applied in a cell, the gNB has the advantage of being able to perform DL transmission and UL reception simultaneously. When the gNB performs SB-FD for DL ​​transmission and UL reception by separating subbands in the spectrum, emission occurs in the UL subband due to the signal of the subband for DL ​​transmission. This emission becomes self-interference (SI) from the gNB’s point of view and BS to BS CLI from the adjacent gNB’s point of view. This CLI occurred between different subbands so becomes inter-subband CLI.

On the other hand, in the case of dynamic/flexible TDD or when the DL/UL subband configuration between cells is different for SB-FD, the aggressor and the victim operate in the same subband/band. Such CLI becomes intra-band/intra-subband CLI.

Cross-Link Interference

In the case of intra-subband CLI, the signal transmitted by the aggressor within the same subband becomes interference. However, in the case of the inter-subband CLI, the emission of the signal to the adjacent subband has an interference effect. In order to perform evaluation reflecting inter-subband CLI, it is necessary to model the emission from the DL Tx subband to the UL Rx subband in gNB perspective and the UL Tx subband to the DL Rx subband in UE perspective.

For this, the adjacent subband interference ratio (i.e., ASIR) can be considered for evaluation. Our initial evaluation results in Section 4 show the performance trend according to the ASIR value. According to the evaluation results, especially the UL performance is greatly affected by the ASIR.

It seems that the adjacent channel interference ratio (ACIR) modelling applied for co-existence study in RAN4 can be a reference for the assumption of inter-subband emission of SB-FD evaluation. In the RAN4 study of Rel-16 CLI handling (TR 38.828), inter-channel emission was considered, and the same amount of emission was assumed for the entire wideband region for evaluation. In order to apply ASIR in evaluation for SB-FD, it is necessary to discuss the value of ASIR and whether the value is the same or different for each frequency resource.

Self-Interference

Unlike CLI that cannot be reduced by receiver, in case of SI, various implementation methods that reduce SI emission from DL subband to UL subband can be considered. For example, antenna separation, RF self-interference cancellation (SIC), base-band SIC, subband filtering, etc. can be applied.

However, it would not be desirable to directly perform SIC through evaluation to reduce SI. Instead, it would be better to assume the amount of self-interference that can be reduced through SIC or the residual SI value remaining after cancellation and reflect it for calculating the SINR. As a result, UL performance will depend on the SIC ratio or residual SI value. Therefore, it would be desirable to study UL performance of SB-FD by assuming various values.

In conclusion, to consider the SI impact on SB-FD evaluation, it is necessary to apply ASIR to DL transmission to obtain the amount of SI emitted in the UL subband, and to reflect residual SI to UL reception by additionally applying the SIC ratio.

Proposal 3:  Apply Adjacent Subband Interference Ratio (ASIR) from BS to BS and from UE to UE for evaluation of subband non-overlapped full duplex. Discuss on exact values of ASIR for evaluation.

Proposal 4: Apply residual Self-Interference (SI) after self-interference cancelation or mitigation for evaluation of subband non-overlapped full duplex. Discuss on definition and exact values of residual SI.

gNB DL Tx power

BS to BS CLI, which can cause UL performance degradation in SB-FD and dynamic/flexible TDD, can be reduced by DL Tx power control of gNB. In addition, since DL transmission of gNB can cause SI in SB-FD, DL TX power reduction can be considered which also reduces residual SI, which has potential to improve UL performance. However, since Tx power of a gNB is the most important factor determining DL coverage, reducing DL Tx power may affect DL performance.

In summary, Tx power control of gNB can be considered to reduce interference impact. But, it should be studied carefully considering the effect of DL/UL performance due to DL Tx power reduction.

Proposal 5: DL Tx power control can be applied for evaluation of subband non-overlapping full duplex and dynamic/flexible TDD.

DL/UL resource configuration

The DL/UL resource configuration is a factor that greatly affects the performance of SB-FD and dynamic/flexible TDD.

When SB-FD or dynamic/flexible TDD is applied, latency reduction thanks to increment in transmission opportunity can be expected. In case of UL of the legacy DL heavy TDD configuration, the interval between UL resources is considered to be large compared to SB-FD or dynamic/flexible TDD. Therefore, latency can be an issue due to the time required for obtaining an opportunity to transmit the UL packet becomes long.

However in SB-FD, simply increasing the UL resource in time domain can be considered to overcome such an issue. In such case, a UE can have more the opportunity to transmit UL packet compared to legacy TDD configuration, thereby latency can be reduced. In addition to that, since the number of UL transmission opportunities can be increased in SB-FD, it also can be a favorable environment for repeated transmission for UL coverage enhancement.

Therefore, for SB-FD operation, it will be necessary to discuss the ratio and location between the wideband HD resource, which is capable of only DL or UL operation, and the SB-FD resource, which is capable of DL and UL operation simultaneously.

Proposal 6: Discuss the location and ratio of wideband half-duplex resource and sub-band full duplex resource for evaluation of subband non-overlapping full duplex.

On the other hand, when DL/UL is transmitted within a subband, compared to DL/UL transmission using the wideband resource, reduced resource becomes a restriction on the size of information that can be transmitted at once. In particular, in the case of DL having a relatively large packet size, it may be more affected by the constraint on the DL resource constraint in the SB-FD.

In addition, in consideration of the influence of interference between sub-bands, a guard band may be required between the DL and UL subbands. This may reduce the available DL/UL resources for SB-FD operation.

Considering those aspects, it will be necessary to determine the amount of DL/UL subband in the SB-FD resource.

Proposal 7: Discuss DL/UL subband size, and guard band size for evaluation of subband non-overlapping full duplex.

Traffic model

For evaluation on NR duplex evaluation, it is necessary to discuss the assumption on the traffic model. According to our initial evaluation results in Section 4, it is observed that DL/UL packet size affects the both of DL and UL performance.

As in the NR study, FTP traffic model 3 can be used as a baseline for evaluation. Although it is necessary to consider the various sizes and arrival rates of DL and UL packet since the arrival rate and size of packet can affect performance.

In particular, performance differences in SB-FD and dynamic/flexible TDD is observed depending on whether CLI and SI is present or not. In order to evaluate UL performance reflecting the impact of SI or CLI, it is necessary to generate DL and UL traffic simultaneously.

Proposal 8: Discuss DL and UL packet size and packet arrival rate for evaluation of subband non-overlapping full duplex and dynamic/flexible TDD.

Proposal 9: Discuss whether DL and UL traffic is simultaneously generated for evaluation of subband non-overlapping full duplex and dynamic/flexible TDD.

Antenna configuration

For the evaluation of NR duplex evolution, the antenna configuration applied in the legacy evaluation assumption can be considered as a baseline.

However, in the conventional half duplex (HD) TDD, it is assumed that all antennas of the gNB are used for DL or UL operation. On the other hand, in SB-FD, when DL and UL are simultaneously performed, it is necessary to determine how many antennas will be used for DL and UL, respectively.

For this, it can be assumed that the same number of antennas is used for both of DL/UL in HD TDD and SB-FD. Alternatively, when DL transmission and UL reception are performed simultaneously, it can be assumed that the number of antennas used for DL and UL is smaller than when operating in HD and the sum of the number of antennas used for DL transmission and UL reception is not larger than the number of antennas used for HD operation.

Proposal 10: Discuss antenna configuration for half duplex and subband non-overlapping full duplex.

Performance metric

As a performance metric for system level simulation for NR duplex evaluation, the following can be considered.

- Average UE Throughput

In order to obtain the throughput performance gain, the distribution of average packet transmission throughput of each UE can be evaluated.

- Average UE latency

The distribution of average latency of each UE can be used for performance metric to obtain the effect of latency reduction required for packet transmission. For the evaluation, the latency may be defined to be a time interval from the time when a packet is generated to the time when the packet is successfully transmitted.

Additionally, a packet dropping timer can be considered. It can be assumed that a packet is in outage if this packet failed to be successfully received by destination receiver beyond ‘packet dropping timer’.

Proposal 11: Average UE throughput and average UE latency can be adopted as the performance metric for evaluation for subband non-overlapping full duplex and dynamic/flexible TDD. Discuss further details of definition of average UE throughput and average UE latency.

Initial performance evaluation results

This section provides our initial evaluation results on subband non-overlapping full duplex.

For the evaluation, throughput and latency performance between HD TDD and SB-FD are compared. For performance comparison, almost same ratio among DL and UL resource between HD TDD and SB-FD is assumed.

- For legacy TDD operation, legacy static TDD configuration is considered. For evaluation, DL and UL slot ratio of 3:2 is assumed as shown in Figure 1. (a).

- For SB-FD operation, a part of time resources is designated for HD operation and other part of time resources is designated for SB-FD operation. As depicted in Figure 1.(b), the ratio among DL slot, UL slot and SB-FD slot is 1:1:3.

Within a SB-FD slot, a ratio of 2:1 DL subband and UL subband is assumed. Also, total 14 PRBs of guard band is assumed.

(a) HD only                                                             (b) HD and SB-FD

Figure 1. DL/UL resource configuration for (a) HD only and (b) HD and SB-FD

For the evaluation, three packet sizes are considered for the evaluation. DL/UL packet sizes corresponding to packet sizes 1, 2, and 3 are shown in Table 1.

In addition, ASIR is assumed to reflect the effects of SI and CLI on the DL/UL performance. The ASIR implies the adjacent subband interference ratio and is defined as the ratio of the power transmitted on one subband to the total interference received by a receiver on the adjacent subband, due to both transmitter and receiver imperfections without considering channel. For the interpretation, concept of the ACIR(adjacent channel interference ratio) can be referred. The ASIR value may be different according to the subband filtering capability of gNB and UE and the frequency gap between the DL subband and the UL subband. In that sense, various ASIR values as shown in Table 2 are assumed for the evaluation.

Other detailed evaluation assumptions are provided in Table 3 in Annex 1.

Table 1. Evaluation assumption on DL/UL packet size

|  | DL packet size | UL packet size |
| --- | --- | --- |
| Packet size 1 | 6KB | 3KB |
| Packet size 2 | 60KB | 3KB |
| Packet size 3 | 60KB | 30KB |

Table 2. Evaluation assumption on ASIR (Adjacent Subband Interference Ratio)

|  | ASIR_BS_BS | ASIR_UE_UE |
| --- | --- | --- |
| ASIR_A | 43dB | 28dB |
| ASIR_B | 28dB | 18dB |
| ASIR_C | 14dB | 9dB |
| ASIR_D | 0dB | 0dB |

Under the assumptions, the evaluation results of HD TDD operation (i.e., HD only) and SB-FD operation (i.e., HD+SBFD) are obtained. Figures 2, 3, and 4 show the evaluation results according to Packet size 1, 2, and 3, respectively. Each result shows average UE packet delay and average UE throughput of HD TDD and SB-FD operation. In the case of SB-FD, four different ASIR values are applied.

Performance difference according to Packet size

If packet size of UL is small, UL performance enhancement can be achieved as shown in Figure 2 and 3. But, when packet size of UL is increased as in Figure 4, performance enhancement is not expected because UL transmission latency is increased due to the narrower bandwidth for UL.

If packet size of DL is small, DL performance enhancement can be achieved. However, as the packet size of DL is increased, DL performance is degraded.

Observation 1: In case of subband non-overlapping full duplex operation, DL/UL performance is affected by DL/UL packet size. As the packet size increases, the performance gain of subband non-overlapping full duplex decreases or deteriorates compared to HD TDD.

Performance difference according to ASIR

Throughout the evaluation, UL performance gap is observed according to the different values of ASIR. In the case of DL, the performance difference according to the ASIR seems negligible. But in the case of UL, the significant degradation of performance is observed due to the effect of interference. As the DL packet size increases, the interference increases due to the frequent occurrence, and thus the degradation of UL performance due to interference becomes more severe.

Observation 2: In case of subband non-overlapping full duplex operation, the DL performance degradation is marginal even though adjacent subband interference from other UE is increased, while the UL performance is significantly degraded due to increased BS2BS CLI from other gNB when adjacent subband interference ratio is increased.

|  | UE DL throughput (Mbps). 5%; 50%; 95%. HD only; 25.91; 28.51; 31.26. HD+SBFD, ASIR_A; 35.34; 38.40; 41.14. HD+SBFD, ASIR_B; 35.40; 38.40; 41.09. HD+SBFD, ASIR_C; 35.43; 38.40; 41.16. HD+SBFD, ASIR_D; 35.31; 38.32; 41.04. |
| --- | --- |

(a) Average UE DL packet delay (slot) and UE throughput (Mbps)

|  | UE UL throughput (Mbps). 5%; 50%; 95%. HD only; 9.06; 10.16; 11.27. HD+SBFD, ASIR_A; 16.67; 18.95; 20.35. HD+SBFD, ASIR_B; 15.95; 18.62; 20.15. HD+SBFD, ASIR_C; 14.91; 17.94; 19.86. HD+SBFD, ASIR_D; 12.61; 16.89; 19.53. |
| --- | --- |

(b) Average UE UL packet delay (slot) and UE throughput (Mbps)

Figure 2. DL and UL performance of HD TDD and SB-FD for Packet size 1 (DL: 6KB, UL: 3KB)

|  | UE DL throughput (Mbps). 5%; 50%; 95%. HD only; 67.31; 123.70; 150.42. HD+SBFD, ASIR_A; 70.12; 131.84; 182.30. HD+SBFD, ASIR_B; 68.23; 132.27; 184.47. HD+SBFD, ASIR_C; 68.02; 130.40; 183.60. HD+SBFD, ASIR_D; 67.24; 128.00; 183.46. |
| --- | --- |

(a) Average UE DL packet delay (slot) and UE throughput (Mbps)

|  | UE UL throughput (Mbps). 5%; 50%; 95%. HD only; 9.06; 10.16; 11.27. HD+SBFD, ASIR_A; 15.01; 18.31; 20.00. HD+SBFD, ASIR_B; 11.43; 17.09; 19.64. HD+SBFD, ASIR_C; 9.41; 14.29; 18.87. HD+SBFD, ASIR_D; 7.45; 11.47; 17.74. |
| --- | --- |

(b) Average UE UL packet delay (slot) and UE throughput (Mbps)

Figure 3. DL and UL performance of HD TDD and SB-FD for Packet size 2 (DL: 60KB, UL: 3KB)

|  | UE DL throughput (Mbps). 5%; 50%; 95%. HD only; 67.31; 123.70; 150.42. HD+SBFD, ASIR_A; 70.73; 132.46; 185.06. HD+SBFD, ASIR_B; 68.57; 131.15; 184.66. HD+SBFD, ASIR_C; 68.79; 130.40; 182.47. HD+SBFD, ASIR_D; 65.07; 128.18; 182.88. |
| --- | --- |

(a) Average UE DL packet delay (slot) and UE throughput (Mbps)

|  | UE UL throughput (Mbps). 5%; 50%; 95%. HD only; 20.73; 34.77; 55.17. HD+SBFD, ASIR_A; 19.95; 33.48; 52.42. HD+SBFD, ASIR_B; 17.50; 30.67; 51.14. HD+SBFD, ASIR_C; 16.29; 27.26; 48.54. HD+SBFD, ASIR_D; 14.31; 23.31; 43.01. |
| --- | --- |

(b) Average UE UL packet delay (slot) and UE throughput (Mbps)

Figure 4. DL and UL performance of HD TDD and SB-FD for Packet size 3 (DL: 60KB, UL: 30KB)

Summary

In this contribution, we discussed the deployment scenarios and evaluation methodology for NR duplex evolution. In addition, some initial evaluation results on subband non-overlapping full duplex were provided. From the discussion and evaluation, we obtained following proposals and observations.

Proposal 1: For deployment scenarios of study on NR duplex evolution, deployment scenarios (i.e., Dense urban, Urban macro, and Indoor hotspot) which were applied for flexible duplex evaluation in Rel-14 NR SI are considered.

Proposal 2: Evaluation for NR duplex evolution is performed at FR1 and FR2-1. Also, it can be discussed whether or not to consider FR 2-2 for study of NR duplex evolution.

Proposal 3:  Apply Adjacent Subband Interference Ratio (ASIR) from BS to BS and from UE to UE for evaluation of subband non-overlapped full duplex. Discuss on exact values of ASIR for evaluation.

Proposal 4: Apply residual Self-Interference (SI) after self-interference cancelation or mitigation for evaluation of subband non-overlapped full duplex. Discuss on definition and exact values of residual SI.

Proposal 5: DL Tx power control can be applied for evaluation of subband non-overlapping full duplex and dynamic/flexible TDD.

Proposal 6: Discuss the location and ratio of wideband half-duplex resource and sub-band full duplex resource for evaluation of subband non-overlapping full duplex.

Proposal 7: Discuss DL/UL subband size, and guard band size for evaluation of subband non-overlapping full duplex.

Proposal 8: Discuss DL and UL packet size and arrival rate for evaluation of subband non-overlapping full duplex and dynamic/flexible TDD.

Proposal 9: Discuss whether simultaneous generation of DL and UL traffic is performed for evaluation of subband non-overlapping full duplex and dynamic/flexible TDD.

Proposal 10: Discuss antenna configuration for half duplex and subband non-overlapping full duplex.

Proposal 11: Average UE throughput and average UE latency can be adopted as the performance metric for evaluation for subband non-overlapping full duplex and dynamic/flexible TDD. Discuss further details of definition of average UE throughput and average UE latency.

Observation 1: In case of subband non-overlapping full duplex operation, DL/UL performance is affected by DL/UL packet size. As the packet size increases, the performance gain of subband non-overlapping full duplex decreases or deteriorates than HD TDD.

Observation 2: In case of subband non-overlapping full duplex operation, the DL performance degradation is marginal even though adjacent subband interference from other UE is increased, while the UL performance is significantly degraded due to increased BS2BS CLI from other gNB when adjacent subband interference ratio is increased.

Reference

- CMCC, “New SI: Study on evolution of NR duplex operation,” RP-213591, e-Meeting, December 6th – 17th, 2021

- Annex 1.

Table 3. Evaluation assumption for SLS

| Parameters | Evaluation assumption |  |
| --- | --- | --- |
| Carrier Frequency | FR1: 2GHz (Macro layer) + 3.5GHz (Micro layer) |  |
| Layout | Two layers: - Macro layer: Hex. Grid, 19 BSs - Micro layer: Random drop, 3 micro cells per macro sector Min. distance btw macro-to-macro: 500m Min. distance btw macro-to-micro: 105m Min. distance btw micro-to-micro: 57.9m |  |
| UE distribution | 855 micro UEs (5 UEs per micro cell in average) 80% of indoor UEs, 20% of outdoor UEs Min. distance btw macro-to-UE: 35m Min. distance btw micro-to-UE: 10m |  |
| System bandwidth/ Subcarrier spacing | 3.5GHz: 50MHz / 15kHz (270RBs) |  |
| Tx power | Macro Tx power: 43dBm Micro Tx power: 33dBm UE max. Tx power: 23dBm |  |
| BS antenna configuration | FR1: (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ |  |
| UE antenna configuration | FR1: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = 0.5 |  |
| Large-scale channel parameters | Below 6GHz: - Macro-to-UE: 3D UMa - Micro-to-UE: 3D UMi - Macro-to-Macro: 3D UMa (hUE =25m) - Macro-to-Micro: 3D UMa (hUE =10m) - Micro-to-Micro: 3D UMi (hUE =10m) - UE-to-UE: A.2.1.2 in TR36.843, penetration loss between UEs follows Table A.2.1-13 in TS38.802 |  |
| Fast fading parameters | Below 6GHz: - Macro-to-UE: 3D UMa - Micro-to-UE: 3D UMi - Macro to Macro: 3D UMa O-to-O (hUE =25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro to Micro: 3D UMa O-to-O - Micro to Micro: 3D UMi O-to-O (hUE =10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - UE to UE: InH for indoor to indoor, and 3D UMi for other cases. ASD and ZSD statistics updated to be the same as ASA and ZSA. Dual mobility support. |  |
| Traffic model | FTP traffic model 3 Downlink: 10 packets/sec/UE, 6/60 KB/packet Uplink: 10 packets/sec/UE, 3/30 KB/packet |  |
| DL/UL resource pattern | TDD: DDDUUDDDUU | SB-FD: DSSSUDSSSU DL and UL PRBs in S slot - DL RB: 170 RBs - UL RB: 86 RBs - Guard RB: 14 RBs |
| Resource pattern flexibility | Static and common DL/UL resource pattern among cells |  |
| ASIR for CLI | - | ASIR BS-BS: 0/14/28/43 dB ASIR UE-UE: 0/9/18/28 dB |
| Residual self-interference | - | Residual SI = Tx power - ASIR – SIC - ASIR: 0/14/28/43 dB - SIC: 80dB |
| Packet dropping timer | 30 slots (A packet is in outage if this packet failed to be successfully received by destination receiver beyond “Packet dropping timer) |  |
| Output | UE average DL/UL packet delay (slot) - Packet delay: slot index of packet transmission completion – slot index of packet generation - Minimum packet delay: 1 slot UE average DL/UL packet throughput (Mbps) - UE average DL/UL throughput: Harmonic mean of packet size / packet delay |  |

- Annex 2.

Table 4. Evaluation assumption for LLS

| Parameters | Evaluation assumption |
| --- | --- |
| Carrier frequency | 2 GHz |
| Subcarrier spacing | 30 kHz |
| System BW | 150 RBs |
| BS antenna configuration | 2 Rx |
| UE antenna configuration | 1 Tx |
| Waveform | CP-OFDM |
| MCS | Index 5 in MCS Table 5.1.3.1-1 in TS38.214 |
| PRBs for PUSCH transmission | 150 RBs |
| Number of OS per repetition | 12 |
| Number of repetition | 2, 4, 8 |
| DMRS overhead | 3 DMRS symbols per slot |
| Frequency hopping | Inter-slot frequency hopping enabled |
| Channel model | TDL-C (NLOS) with delay spread 30 ns |
| UE speed | 3 km/h |
| Channel estimation | Practical |
| Receiver type | MMSE |
| Interference | No Self-interference, No Cross Link Interference |

Table 5. MCL

| Parameter | Value | Value | Value |
| --- | --- | --- | --- |
| Number of repetition | 2 | 4 | 8 |
| (2a) Number of transmit chains modelled in LLS | 1 | 1 | 1 |
| (3) Total transmit power (dBm) Note: total transmit power for system bandwidth | 23 | 23 | 23 |
| (3a) Occupied bandwidth for uplink (Hz) | 54000000 | 54000000 | 54000000 |
| (3b) Power Spectrum Density = (3) - 10 log( (3a) / 1000000 ) (dBm/MHz) | 5.68 | 5.68 | 5.68 |
| (3c) bandwidth used for the evaluated channel (Hz) Note: For uplink (3a)=(3c) | 54000000 | 54000000 | 54000000 |
| (3bis) Total transmit power for occupied bandwidth = (3b) + 10 log ( (3c) / 1000000 ) (dBm) | 23 | 23 | 23 |
| (5) total antenna gain at antenna gain component 2 of transmitter = (5a) - (5b) (dB) Note: zero for uplink | 0 | 0 | 0 |
| (5a) antenna gain at antenna gain component 2 of transmitter = 10 log( (2)/(2a)) (dB) Note: zero for uplink | 0 | 0 | 0 |
| (5b) antenna gain correction factor at antenna gain component 2 of transmitter (dB) Note: zero for uplink | 0 | 0 | 0 |
| Receiver |  |  |  |
| (10a) Number of receive TxRUs Note: this row is void (empty) for downlink | 2 | 2 | 2 |
| (10b) Number of receive chains modelled in LLS | 2 | 2 | 2 |
| (11bis) total antenna gain at antenna gain component 2 of receiver = (11bis-a) - (11bis-b) (dB) Note: zero for downlink | 0 | 0 | 0 |
| (11bis-a) antenna gain at antenna gain component 2 of receiver = 10 log( (10a)/(10b)) (dB) Note: zero for donwlink | 0 | 0 | 0 |
| (11bis-b) antena gain correction factor at antenna gain component 2 of receiver (dB) Note: zero for downlink | 0 | 0 | 0 |
| (13) Receiver noise figure (dB) | 5 | 5 | 5 |
| (14) Thermal noise density (dBm/Hz) | -174 | -174 | -174 |
| (15) Receiver interference density (dBm/Hz) | -165.7 | -165.7 | -165.7 |
| (16) Total noise plus interference density = 10 log (10^(( (13) + (14))/10) + 10^((15)/10)) (dBm/Hz) | -164.0 | -164.0 | -164.0 |
| (18) Effective noise power = (16) + 10 log((3c)) (dBm) | -86.68 | -86.68 | -86.68 |
| (19) Required SNR (dB) | -4.4 | -7.6 | -10.6 |
| (20) Receiver implementation margin (dB) | 2 | 2 | 2 |
| (21) Receiver sensitivity = (18) + (19) + (20) (dBm) | -89.08 | -92.28 | -95.28 |
| MCL = (3bis) - (21) + (5) + (11bis) (dB) | 112.08 | 115.28 | 118.28 |

|  | SNR (dB) of 10% BLER. dB. Repetition 2; -4.4. Repetition 4; -7.6. Repetition 8; -10.6. |
| --- | --- |

Figure 5. BLER performance of uplink LLS for MCS 5