3GPP TSG RAN WG1 #109-e	R1-2204530

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.3.2

Source: 	LG Electronics

Title: 	Study on Subband non-overlapping full duplex

Document for:	Discussion and decision

Introduction

In RAN#94-e meeting, a study item on evolution of NR duplex operation is approved and the corresponding description is provided in [1].

According to the SID, the subband non-overlapping Full Duplex (subband FD) and potential enhancements on dynamic/flexible TDD are studied. Also, identification of deployment scenarios and developing evaluation methodology are also included in the scope as follows.

In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this contribution, we discuss on the subband non-overlapping full duplex.

Discussion

For 5G, many of the new service types (e.g., XR service, AI based service, Self-driving Car) are characterized by dynamic variation of traffic in both DL and UL directions, requiring low latency in the packet delivery. For 5G services, demands on the traffic load will be exploding to support various emerging use cases. Existing semi-static or dynamic TDD UL/DL configuration has a limit of delay in time and inter-operator interference issue. SB-FD operation within a single cell can be expected to provide latency reduction and throughput enhancement. Also, from the initial result of performance evaluation in our companion documentation [2], it is observed that SB-FD shows better performance than HD TDD.

However, because SB-FD is not supported in current NR system, some modification is necessary and new functions seems to be introduced. In Rel-18 Duplex Evolution (DE) SI, followings should be studied for supporting SB-FD.

- Resource allocation

- Duplex scheme adaptation and time boundary alignment

- Interference handling

- MIMO operation

Resource allocation

In order to perform SB-FD, separated subband within a spectrum should be assigned for downlink and uplink. Also, guard frequency is required between subbands for DL and subbands for UL in order to prevent subband emission from DL subband to UL subband. Therefore, subband partitioning to support SB-FD should be studied considering on these basic element (i.e., DL subband, UL subband and guard subband).

The DL subband, UL subband and guard subband could be designated within a BWP or configured bandwidth, where subbands for DL/UL could be consecutive or not. Also, the subband partitioning could be semi-statically or dynamically configured.

Proposal 1: Study subband partitioning for supporting SB-FD. Followings can be studied.

- The DL subband, UL subband and guard subband could be designated within a BWP or configured bandwidth.

- Subbands for DL/UL could be consecutive or not.

- Semi-static / dynamic configuration for subband partitioning

In current TDD unpaired spectrum, a UE does not expected to receive a configuration where the center frequency of a DL BWP is different from the center frequency of an UL BWP since the center frequency is shared between BWPs with same IDs where the BWP-ID of DL BWP is same as the BWP-ID of the UL BWP. In Rel-18 DE SI, it is assumed that SB-FD is operated in TDD unpaired spectrum. In case of SB-FD in TDD unpaired spectrum, center frequency of a DL BWP could be same as or different from that of a UL BWP similar with BWP in paired spectrum. In Rel-18 DE SI, it needs to be discussed whether center frequency of a DL BWP of SB-FD should be same as or can be different from that of a UL BWP of SB-FD which has same BWP-ID. If a center frequency of DL and UL BWP should be same, it needs to be discussed whether discontinuous subband can be assigned for a BWP. Also, if a center frequency for DL and UL BWP can be different, gNB/UE implementation should be discussed for deciding frequency position of DL subband and UL subband.

Proposal 2: Discuss whether center frequency of a DL BWP of SB-FD should be same as or can be different from that of a UL BWP of SB-FD which has same BWP-ID.

- If a center frequency of DL and UL BWP should be same, it needs to be discussed whether discontinuous subband can be assigned for a BWP.

- If a center frequency of DL and UL BWP can be different, gNB/UE implementation should be discussed for deciding frequency position of DL subband and UL subband.

Legacy TDD configuration is designed for HD TDD operation, where time resource (slot or symbol) for downlink only, time resource for uplink only, and flexible resource can be designated in time domain. In Rel-18 DE SI, it was decided that gNB SB-FD and UE HD are assumed. That is, because UE operates in half duplex manner, current TDD configuration could be valid for UE with DE capability. But, in Rel-18 DE SI, it can be discussed whether TDD configuration mechanism should be updated for supporting SB-FD.

Proposal 3: Discuss whether TDD configuration mechanism should be updated for supporting SB-FD.

Duplex Scheme Adaptation and Time Boundary Alignment

Even if SB-FD is adopted, HD TDD seems to be required for supporting legacy operation which could be available within wider bandwidth or could be suitable for transmitting DL signal/cannel with higher transmission power than SB-FD. Therefore, it needs to be studied how to support both legacy HD TDD and SB-FD within time duration.

Proposal 4: Study how to support both legacy HD TDD and SB-FD within time duration.

For operating SB-FD, time boundary alignment between UL and DL within a slot for SB-FD should be studied. Following two candidate of time boundary alignment (i.e., symbol level and slot level) can be considered for study.

Firstly, symbol boundary alignment between DL signal/channel and UL signal/channel could be assumed. One of challenging point of operating SB-FD is how to reduce self-interference. Symbol level alignment could be helpful to adopt simplified implementation of self-interference cancelation (SIC) receiver in digital domain. On the other hand, in this case, since UL symbol boundary should be aligned to DL symbol boundary, the time grid of UL symbol within slot for SB-FD is different from the time grid of UL symbol within slot for HD TDD. Hence, two different values of NTA,offset for UL transmission in UE side could be required depending on slots where HD TDD is operated or SB-FD is operated.

Secondly, slot boundary alignment between DL and UL could be assumed. In this case, symbol boundary alignment between DL and UL within slots for SB-FD is not required. Depending on implementation of SIC receiver, SIC can be operated even if symbol boundary between DL and UL is not aligned. One of advantage of slot level alignment without symbol level alignment is to keep the time grid of UL symbol between slot for SB-FD and slot for HD TDD.

Proposal 5: Study time boundary alignment between UL and DL within a slot for SB-FD. Following options can be studied.

- Option 1: Symbol boundary alignment between DL signal/channel and UL signal/channel

- Option 2: Slot boundary alignment without symbol boundary alignment between DL signal/channel and UL signal/channel

Interference handling

In SB-FD, performance degradation could be caused due to various type of interference (i.e., self-interference, UE2UE Cross Link Interference (CLI), BS2BS CLI, inter-carrier interference between operators).  In order to fully take advantage from SB-FD, interference handling method should be studied in Rel-18 DE SI.

Cross Link Interference Handling

When SB-FD is operated, UL performance would be degraded due to severe inter-cell BS2BS CLI. In our companion contribution [2], UL performance due to inter-subband emission from DL subband of aggressor gNB is observed. In addition, intra-subband CLI could be appeared if frequency position for subbands designated for DL transmission and UL reception is not same at each gNBs. In order to handling inter-subband emission and/or intra-subband CLI from aggressor gNB, interference management schemes (e.g., long-term CLI measurement, scheduling, link adaption, power control, etc) can be studied.

In case of SB-FD, both intra-cell UE2UE CLI and inter-cell UE2UE CLI can be appeared, which is different from dynamic TDD scenario. For inter-cell UE2UE CLI handling, L3 based CLI measurement and reporting (i.e., L3 CLI-RSSI and SRS-RSRP) can be reused. On the other hand, for intra-cell UE2UE CLI handling, L1 based CLI measurement and reporting can be applied, which was proposed as one of candidates for UE2UE CLI handling in Rel-16 CLI/RIM WI.

Proposal 6: Study Cross Link Interference handling method for SB-FD. Followings can be studied.

- Interference management schemes (e.g., long-term CLI measurement, scheduling, link adaption, power control, etc) for handling inter-subband emission and/or intra-subband CLI from aggressor gNB

- L1 based CLI measurement and reporting for intra-cell UE2UE CLI management

- CLI handling frame work for inter-cell UE2UE CLI management

Self-Interference Cancellation/Mitigation

For Self-Inference (SI) cancellation/mitigation, information of transmitted signal/channel can be fully shared between transmitter and receiver via internal interface, which is different characteristic from CLI handling. For example, beam management for DL transmission and UL reception can be applied simultaneously. Also, DL power adjustment and UL power control can be applied simultaneously for reduction of self-interference and increasing of received power level of desired signal/channel which is transmitted with higher power from UE according to power control indication of gNB. In addition, various type of implementation (e.g., antenna separation, RF level SI cancellation, Baseband level SI cancellation, subband filtering, etc.) can be applied.

Proposal 7: Study Self-Interference cancellation/mitigation method for SB-FD. Followings can be studied.

- Simultaneous beam management for DL transmission and UL reception

- Simultaneous power control for DL transmission and UL reception

- gNB implementation for Self-Interference cancellation/mitigation (e.g., antenna separation, RF level SI cancellation, baseband level SI cancellation, subband filtering, etc.)

MIMO operation

According to duplex scheme operated in gNB side, antenna scheme and/or beam mange scheme can be differently applied.

In SB-FD case, antenna separation based self-interference reduction scheme can be applied. For example, some panel(s) among total panels can be used for DL transmission, and the other panel(s) which is geographically separated from the panel for DL transmission can be used for UL reception. But, in HD TDD case, following options for using multiple panels can be considered.

- Option 1: All panels can be used for DL transmission or UL reception in HD TDD case.

- Option 2: Designated panel(s) for DL transmission (or UL reception) in SB-FD case is used in HD TDD case.

Also, beam management among DL and UL can be applied for self-interference reduction in SB-FD case. On the other hand, in HD TDD case, beam management for DL and UL can be operated independently. In this case, UE may be configured different TCI state according to duplex scheme in gNB side.

Addition to that, in SB-FD case, reduced DL Tx power can be applied for reduction of self-interference. On the other hand, in HD TDD case, higher DL Tx power can be operated for maintaining DL coverage. If different level of DL Tx power is applied, CSI measurement/report could be required according to Tx power level.

Furthermore, if it is allowed that non-contiguous subband can be designated within a BWP for SB-FD case, several features (e.g., FDRA, reference signal, etc.) need to be modified for supporting SB-FD.

In this sense, it needs to be studied whether MIMO related configuration (e.g., antenna configuration, beam management, power control, CSI measurement/report, reference signal, etc.) can be differently applied according to duplex schemes (i.e., HD TDD and SB-FD) operated in gNB side.

Proposal 8: Study whether MIMO related configuration (e.g., antenna configuration, beam management, power control, CSI measurement/report, reference signal, etc.) can be differently applied according to duplex schemes (i.e., HD TDD and SB-FD) operated in gNB side.

Summary

In this contribution, we discussed on the subband non-overlapping full duplex.

Resource allocation

Proposal 1: Study subband partitioning for supporting SB-FD. Followings can be studied.

- The DL subband, UL subband and guard subband could be designated within a BWP or configured bandwidth.

- Subbands for DL/UL could be consecutive or not.

- Semi-static / dynamic configuration for subband partitioning

Proposal 2: Discuss whether center frequency of a DL BWP of SB-FD should be same as or can be different from that of a UL BWP of SB-FD which has same BWP-ID.

- If a center frequency of DL and UL BWP should be same, it needs to be discussed whether discontinuous subband can be assigned for a BWP.

- If a center frequency of DL and UL BWP can be different, gNB/UE implementation should be discussed for deciding frequency position for DL subband and UL subband.

Proposal 3: Discuss whether TDD configuration mechanism should be updated for supporting SB-FD.

Duplex Scheme Adaptation and Time Boundary Alignment

Proposal 4: Study how to support both legacy HD TDD and SB-FD within time duration.

Proposal 5: Study time boundary alignment between UL and DL within a slot for SB-FD. Following options can be studied.

- Option 1: Symbol boundary alignment between DL signal/channel and UL signal/channel

- Option 2: Slot boundary alignment without symbol boundary alignment between DL signal/channel and UL signal/channel

Interference handling

Proposal 6: Study Cross Link Interference handling method for SB-FD. Followings can be studied.

- Interference management schemes (e.g., long-term CLI measurement, scheduling, link adaption, power control, etc) for handling inter-subband emission and/or intra-subband CLI from aggressor gNB

- L1 based CLI measurement and reporting for intra-cell UE2UE CLI management

- CLI handling frame work for inter-cell UE2UE CLI management

Proposal 7: Study Self-Interference cancellation/mitigation method for SB-FD. Followings can be studied.

- Simultaneous beam management for DL transmission and UL reception

- Simultaneous power control for DL transmission and UL reception

- gNB implementation for Self-Interference cancellation/mitigation (e.g., antenna separation, RF level SI cancellation, baseband level SI cancellation, subband filtering, etc.)

MIMO operation

Proposal 8: Study whether MIMO related configuration (e.g., antenna configuration, beam management, power control, CSI measurement/report, reference signal, etc.) can be differently applied according to duplex schemes (i.e., HD TDD and SB-FD) operated in gNB side.

Reference

- CMCC, “New SI: Study on evolution of NR duplex operation,” RP-213591, e-Meeting, December 6th – 17th, 2021

- LG Electronics, “Study on Evaluation for NR duplex evolution,” R1-2204529, e-Meeting, May 9th – 20th, 2022