3GPP TSG RAN WG1 #109-e	R1-2204531

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.3.3

Source: 	LG Electronics

Title: 	Potential enhancement on dynamic/flexible TDD

Document for:	Discussion and decision

Introduction

In RAN#94-e meeting, a study item on evolution of NR duplex operation is approved and the corresponding description is provided in [1].

According to the SID, the subband non-overlapping Full Duplex (SB-FD) and potential enhancements on dynamic/flexible TDD are studied. Also, identification of deployment scenarios and developing evaluation methodology are also included in the scope as follows.

In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

Discussion

In this contribution, we discuss on potential enhancements on dynamic/flexible TDD.

TDD configuration

Before starting discussion, it should be noted what the use case of Rel-18 dynamic TDD is. It is our understanding that major use case is uplink heavy traffic. It is the reason why dynamic TDD configuration is considered to be studied to support uplink heavy configuration. It is also notable that Rel-15 TDD configuration is defined with similar motivation. Therefore checking whether conventional TDD configuration can support uplink heavy traffic or not should be firstly considered.

TDD configuration in Rel-15

There are mainly three types of signaling for Rel-15 TDD configuration. According to hierarchy of the signaling, firstly there is TDD-UL-DL-ConfigurationCommon which is configured by cell-specific RRC configuration. Then TDD-UL-DL-ConfigDedicated is configured by UE-specific RRC configuration and lastly slot format indicator (SFI) is configured by DCI format 2_0. There are some exceptional rules, however there is a basic rule for configuration and it is as follows: by TDD-UL-DL-ConfigurationCommon, a periodicity is indicated and within it, downlink and uplink slots and symbols are configured. Rest of the slots and symbols are considered as flexible. For the flexible slots and symbols, similar mechanism is applied for TDD-UL-DL-ConfigDedicated to indicate downlink and uplink slots and symbols which overrides flexible slots and symbols. After that, final slot format within periodicity is configured by indication of SFI via group common DCI format 2_0. The total number of SFI index is 256, and 57 of them is used for indication including index of 255, so called “wild card”, while rest of them are reserved. [2]

Considering the structure of current specification, it is our understanding that the TDD configuration of Rel-15 is designed to have sufficient flexibility to support not only downlink heavy but also uplink heavy configuration. That is, it seems there is no difficulty in indicating the uplink heavy configuration with the conventional TDD configuration. Therefore the first thing to be clarified is finding the use case that the current configuration methodology cannot support.

Proposal 1: It should be prioritized to identify the issues that cannot be supported by the Rel-15 TDD configuration.

BS-to-BS CLI handling methods

One of the most typical problem to be solved considering dynamic TDD configuration is CLI handling issue. Regarding this issue, there are evaluation results given by Rel-14 NR study. So it is natural to be a starting point of the discussion by checking those results for dynamic TDD. The BS-to-BS CLI issue is one of the hardest issue to be solved in terms of uplink performance improvement in uplink heavy TDD configuration. Many candidate technical methodologies are proposed in Rel-14 NR SI for BS-to-BS CLI handling and evaluation results are given by some of them proving the benefit of them. The proposed candidate of cross link interference mitigation schemes are captured in section 10 of TS 38.801, and the list of them is as follows: [3]

- Advanced receiver (IC/IS)

- Hybrid dynamic/static UL/DL resource assignment

- Scheduling coordination

- Beam coordination

- Link adaptation

- Power control

- Sensing

- Cell/TRP clustering

- OTA signaling

- Co-channel multiple connectivity

- Dynamic TDD type definition

- Load/link-based resource/scheduling adaptation

It is difficult to describe detailed captured observations, however it can be summarized. Evaluation results are provided in various scenarios; indoor hotspot scenario, urban macro scenario, dense urban scenario. It is proved by those evaluations that cross-link interference mitigation schemes listed above showed improved (average) UPT or average/edge throughput compared to static UL/DL resource partition and/or duplexing flexibility without cross-link interference mitigation scheme in certain unpaired spectrum.

After such study, L3 UE2UE CLI measurement/report (i.e., CLI-RSSI, SRS-RSRP) and intended TDD configuration exchange are specified in Rel-16 CLI handling/RIM WI on the purpose of handling of UE-to-UE CLI in dynamic TDD configuration. On the other hand, BS-to-BS CLI handling is excluded in work scope by WID. In conclusion, candidate interference mitigation schemes provided in BS-to-BS CLI handling of Rel-14 and evaluation results provided in UE-to-UE CLI measurement/report of Rel-16 should be a starting point of discussion for Rel-18 dynamic TDD.

Proposal 2: BS-to-BS CLI handling in Rel-14 NR SI and evaluation results of UE-to-UE CLI handling in Rel-16 should be a starting point of discussion for CLI handling.

To prevent duplication, additional evaluations for CLI handling schemes studied in Rel-14 NR SI and Rel-16 CLI handling WI is not desirable in Rel-18 DE SI. Nevertheless it can be discussed which of item to be studied and evaluated to minimize the duplication in case further evaluations for those schemes is necessary. For example, introducing downlink power control to reduce BS-to-BS CLI and MCS selection based on measurement of long-term uplink CLI can be considered. In addition to that, achievable measurement schemes for BS-to-BS CLI handling can be discussed for potential study topics. As an example of those potential study topics, configuration for interference measurement resource (IMR) can be considered. That is, for long-term CLI measurement, puncturing some resources to provide better CLI measurements can be considered for RSRP/RSRQ like CLI measurement. Another example is that IMR on existing physical reference signals (e.g., DM-RS, CSI-RS, and SRS) and other REs (if necessary) for measuring CLI can be considered. For RSRP/RSRQ like CLI measurement, puncturing some resources to provide better CLI measurements can also be considered. Another candidate solution can be inter-TRP information exchange on the purpose of CLI measurement.

Proposal 3: If necessary, discuss how to enable measurement for BS-to-BS CLI handing.

Inter-operator Interference Handling

Another important problem to be considered for dynamic TDD is inter-operator interference handling issue. With same manner, the result of study in past release should be carefully investigated and considered. In Rel-16 CLI handling/RIM WI, co-existence evaluation for CLI handling for NR was studied by RAN4. [4] To briefly summarize the results, dynamic TDD in various scenario is evaluated to show potential benefit. Firstly, performance degradation was observed in macro-to-macro scenario in both of FR1 and FR2. For indoor scenario, i.e., indoor-to-macro and indoor-to-indoor scenario, performance degradation is not observed as long as care is taken, that is, when the sufficient isolation is provided in both of FR1 and FR2. And for FR2, it is concluded that dynamic TDD have potential to be used in micro-to-micro scenario as long as care is taken even the performance degradation exists or not depending on provided simulation results. Therefore it is our understanding that macro-to-macro scenario is the most challenging environment for dynamic TDD configuration. In that sense, in order to operate dynamic TDD properly, it is necessary to study a method that can support dynamic TDD even in difficult environments such as macro-to-macro scenarios. Such schemes can be studied in Rel-18 DE SI.

The hardest issue to be resolved for dynamic TDD configuration is the interference between co-located inter-operators. It is tricky since it is assumed that information for coordination is not shared among operators. Therefore, downlink to uplink interference from one operator leads significant degradation in uplink performance of another operator. However, the information exchange is hard to be expected. In that perspective, i.e., no information is shared for coordination between operators, there are two options to be considered.

- Uplink band suppression

- Sensing based dynamic TDD

The uplink band suppression is that, reducing inter-carrier emission by limiting the uplink bandwidth operated by the base station. It has similarity to sub-band full duplex (SBFD) that uplink is only transmitted in sub-band, or limited bandwidth, but has different point that in SBFD, downlink is transmitted at the same time. It is one of the simplest option that can be considered as a method of reducing the amount of interference while using less resources in one cell if coordination between inter-operators is not assumed. In addition, sensing based dynamic TDD is that a method similar to listen before talk (LBT) in the unlicensed band, first determining whether the inter-operator is using the band the base station wants to operate, and occupying it according to the interference level. It is necessary to discuss whether these methods are valid.

Proposal 4: If information exchange between inter-operators are not assumed, technical schemes to support dynamic TDD can be studied, and candidate solutions for interference handling (e.g., uplink band suppression, sensing, etc.) can be discussed.

Summary

In this contribution, we discussed on potential enhancements on dynamic/flexible TDD. From the discussion, we obtained following proposals:

TDD configuration

Proposal 1: It should be prioritized to identify the issues that cannot be supported by the Rel-15 TDD configuration.

BS-to-BS CLI handling methods

Proposal 2: BS-to-BS CLI handling in Rel-14 NR SI and evaluation results of UE-to-UE CLI handling in Rel-16 should be a starting point of discussion for CLI handling.

Proposal 3: If necessary, discuss how to enable measurement for BS-to-BS CLI handing.

Inter-operator Interference Handling

Proposal 4: If information exchange between inter-operators are not assumed, technical schemes to support dynamic TDD can be studied, and candidate solutions for interference handling (e.g., uplink band suppression, sensing, etc.) can be discussed.

Reference

- CMCC, “New SI: Study on evolution of NR duplex operation,” RP-213591, e-Meeting, December 6th – 17th, 2021

- 3GPP TS 38.213, V15.7.0 (2019-09): “NR; Physical layer procedures for control”

- 3GPP TR 38.802, V14.2.0 (2017-09): “Study on new radio access technology Physical layer aspects”

- 3GPP TR 38.828, V16.1.0 (2019-09): “Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR”