3GPP TSG RAN WG1 #109-e	R1-2204533

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.8.2

Source: 	LG Electronics

Title: 	Discussion on L1/L2 Signaling for Side Control Information

Document for:	Discussion and decision

Introduction

In RAN#94-e meeting, a study item on NR Network-Controlled Repeaters (NCR) was approved and the corresponding description is provided in [1]. According to the SID, the study on side control information is required for NCR and L1/L2 signaling to carry the side control information are included in the scope as follows.

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios. - For only single hop stationary network-controlled repeaters - Network-controlled repeaters are transparent to UEs - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters. Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority) Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1] Study the following aspects of network-controlled repeater management - Identification and authorization of network-controlled repeaters [RAN2, RAN3] NOTE2: Coordination with SA3 may be needed.

In this contribution, we discuss on the L1/L2 signaling to carry side control information for NR NCR.

Discussion

Unlike the conventional RF repeater that simply forwarding the received signal with power amplification, NR network-controlled repeater (NCR) considers more effective operation under the control of the network.

While the conventional RF repeater has a remote unit (RU) having RF layer only, it can be considered that the NCR is composed of a remote unit (RU) and a mobile terminal (MT) as illustrated in Figure 1.

- RU (Remote Unit): RU of NR simply forwards the received signal without direct data communication with gNB or UE. For the DL operation, the RU receives DL signals from the gNB and forwards it to the UE. In case of the UL operation, the RU forwards UL signals from UEs to the gNB. The RU only has an RF layer, so it simply operates as a forwarding subject but does not obtain any information for signal detection and demodulation directly from gNB.

- MT (Mobile Terminal): MT of NCR can perform transmission/reception with the gNB, and receives side control information from the gNB which is necessary for the operation of RU of the NCR. The MT receives side control information by L1/L2 signaling, then the NCR controls the operation of the RU based on the side control information.

Considering the NCR consists of MT and RU as described above, gNB-repeater link and repeater-UE link can be clarified as follows.

- gNB-Repeater link: The gNB-repeater link can be divided into two parts; gNB-MT link and gNB-RU link. The gNB-MT link can be used to deliver side control information from the gNB to the MT. The gNB-RU link can be used for the RU to receive and forward downlink signal from the gNB to UEs and uplink signal from UEs to the gNB.

- Repeater-UE link: Since the MT of the NCR is connected only to the gNB and not to the UE, the repeater-UE link may refer to an RU-UE link. The RU-UE link can be used only by the RU to forward downlink signal received from the gNB to the UEs and uplink signal from the UEs to the gNB.

Figure 1. Structure of NCR

As shown in Figure.1, it is assumed that side control information for NCR is delivered via gNB-MT link from gNB to MT of the NCR. Also, it can be assumed that the side control information is used for controlling RU of the NCR.

Hence, in order to carry side control information, structure of protocol for MT in NCR should be discussed, and potential candidate of signaling configuration should be studied. In addition, L1/L2 signaling for each side control information need to be studied

Configuration for L1/L2 signaling

In NCR, it is assumed that MT has a role to receive side control information that is to be applied to RU. Also, if it can be assumed that DL reception timing and UL transmission timing are aligned between MT and RU, MT based timing tracking can be considered. Therefore, it needs that NR related operation should be well defined for NCR.

Furthermore, for the operation of MT in NCR, configuration for MT in NCR is required. The monitoring and reception of MT can be the starting point for the discussion. For example, at least BWP/PDCCH/PDSCH and/or RNTI should be configured for monitoring/receiving L1/L2 signaling in MT side. Also, if beam management between gNB-repeater link is adopted, or even single beam (e.g., single SSB index) based operation is assumed for gNB-repeater link, beam related configuration (e.g., SSB, CSI-RS, TCI state, etc) is required. In addition, timing alignment perspective should be considered. Considerable options depending on timing alignment are as follows; if UL transmission timing adjustment is required, RACH procedure and/or SRS based timing advance (TA) adjustment should be operated, hence RACH and/or SRS related configuration needs to be provided. Therefore, in Rel-18 NCR SI, it should be studied which configuration is required for operating MT in NCR.

Proposal 1: Study required configuration for operating MT in NCR.

- E.g., BWP, PDCCH (e.g., search space, CORESET, TCI state), PDSCH (e.g., DMRS, antenna configuration, etc), RNTI, UL related channel (e.g., PUSCH, RACH)

One of the most important study topics for NCR is how to model MT in NCR. It should be discussed first because capability of MT and specification impact for MT are decided depending on the modelling of MT. Following two options can be considered:

The first option is that information for configuration is fully provided via RRC for MT in NCR, similar to legacy UE. In this case, the configuration for L1/L2 signalling to control RU can be fully delivered via RRC, which has an advantage of full flexibility for configuration.

Another option is that only limited information for configuration is provided for MT in NCR. That is, some of information for configuration is delivered via RRC, and the rest of information for configuration is obtained by OAM which can be provided by network implementation. In this case, operation of MT in NCR would be limited due to limited flexibility of configuration, but simplified configuration would be beneficial in terms of implementation.

Proposal 2: Study modelling of MT in NCR in terms of configuration. Following two options can be studied.

- Option 1: For MT in NCR, information for configuration is fully provided via RRC, similar to legacy UE.

- Option 2: For MT in NCR, limited configurations can be provided. Some of the information for configuration is provided via RRC and other some obtained by OAM.

Considerations on L1/L2 signaling of side control information

Similar with legacy L1/L2 signalling mechanism, side control information can be delivered to MT via DCI and MAC-CE. That is, DCI in PDCCH and MAC-CE in PDSCH can be a container of L1 and L2 signalling of side control information for RU, respectively. If this mechanism is adopted for NCR, new DCI format and new MAC-CE which contain side control information for RU control needs to be defined. Furthermore, in case suitable container needs differentiation depending on what sort of side control information should be delivered, new DCI format and/or new MAC-CE should be defined according to side control information. In Rel-18 NCR SI, it needs to be studied which side control information can be contained in DCI or MAC-CE.

Proposal 3: Side control information can be delivered to MT via DCI and MAC-CE. Study which side control information can be contained in DCI or MAC-CE.

One of study topic to be defined is an association between configured side control information and applicable time resource. For example, unit of time resource (e.g., multi-/single- slot level and/or symbol level) in which configured side control information is applied needs to be discussed. Also, timeline for side control information should be studied. For example, whether the side information is applied periodically or in aperiodic manner can be studied. The application timing is also notable. That is the timing difference between the time when MT obtained side control information and the time when side control information is actually applied. It should be considered accounting for processing time of MT, switching time, and so on.

One step further, followings can be studied.

- Whether different type of side control information can be combined for indication.

- Where side control information is indicated per RU carrier in order to support multi-carrier based RU operation

Proposal 4: Study an association between configured side control information and applicable time resource. Followings can be studied.

- Unit of time resource (e.g., multi-/single- slot level and/or symbol level) in which configured side control information is applied

- Whether the side information is applied periodically or in aperiodic manner.

- When side control information is applied accounting for processing time of MT, switching time, and so on.

Proposal 5: Followings can be studied further.

- Whether different type of side control information can be combined for indication.

- Where side control information is indicated per RU carrier in order to support multi-carrier based RU operation

Summary

In this contribution, we discussed on the L1/L2 signaling to carry side control information for NR network-controlled repeaters, and obtained the following proposals.

Proposal 1: Study required configuration for operating MT in NCR.

- E.g., BWP, PDCCH (e.g., search space, CORESET, TCI state), PDSCH (e.g., DMRS, antenna configuration, etc), RNTI, UL related channel (e.g., PUSCH, RACH)

Proposal 2: Study modelling of MT in NCR in terms of configuration. Following two options can be studied.

- Option 1: For MT in NCR, information for configuration is fully provided via RRC, similar to legacy UE.

- Option 2: For MT in NCR, limited configurations can be provided. Some of the information for configuration is provided via RRC and other some obtained by OAM.

Proposal 3: Side control information can be delivered to MT via DCI and MAC-CE. Study which side control information can be contained in DCI or MAC-CE.

Proposal 4: Study an association between configured side control information and applicable time resource. Followings can be studied.

- Unit of time resource (e.g., multi-/single- slot level and/or symbol level) in which configured side control information is applied

- Whether the side information is applied periodically or in aperiodic manner.

- When side control information is applied accounting for processing time of MT, switching time, and so on.

Proposal 5: Followings can be studied further.

- Whether different type of side control information can be combined for indication.

- Where side control information is indicated per RU carrier in order to support multi-carrier based RU operation

Reference

- ZTE Corporation, “New SI: Study on NR Network-controlled Repeaters,” RP-213700, e-Meeting, December 6th – 17th, 2021