3GPP TSG RAN WG1 #109-e														       R1-2204536

e-Meeting, May 9th – 20th , 2022

Agenda item:		8.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Maintenance of Enhancements on Multi-TRP deployment

Document for:		Discussion and Decision

# Multi-TRP for PDCCH, PUCCH and PUSCH

In this section, we discuss one potential remaining open point on the multi-TRP enhancements for PUSCH.

## 1.1 M-TRP PUSCH operation with pathlossReferenceLinking

As captured in Sec. 7 of TS 38.213, for PUSCH pathloss RS determination, when pathlossReferenceLinking is provided, the UE determines the RS resource on a serving cell indicated by a value of pathlossReferenceLinking. In other words, only a single RS resource determination seems to be covered in this case. This can be noticed, for example, in the highlighted text copied from the latest TS 38.213 version (V17.1.0). This may be ambiguous as there is no reason not to also support the determination of two RS resources in this case. Also, in the highlighted text below, the ambiguity is for the case where pathlossReferenceLinking is not provided.

- If the UE is provided SRI-PUSCH-PowerControl and more than one values of PUSCH-PathlossReferenceRS-Id, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field, or for first and second SRI fields if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook', or values for a first SRI field and values associated with a second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', in a DCI format scheduling the PUSCH transmission and a set of PUSCH-PathlossReferenceRS-Id values and determines the RS resource index qd, or respective first and second RS resource indexes qd, from the value of PUSCH-PathlossReferenceRS-Id that is mapped to the SRI field value, or from the values of PUSCH-PathlossReferenceRS-Id that are mapped to respective first and second SRI field values if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook', or from the values of PUSCH-PathlossReferenceRS-Id that are mapped to respective first SRI field value and a value associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', where the RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

The ambiguities in the above text would need to be corrected in such a way to also cover the case with ‘first and second RS resources’. We thus propose:

Proposal 1-1: For multi-TRP PUSCH, to determine two RS resources for the case where pathlossReferenceLinking is provided, adopt the following text proposal to TS 38.213 Sec. 7:

Reason for change: To determine two RS resources for the case where pathlossReferenceLinking is provided for multi-TRP PUSCH. Summary of change: Editorial correction to cover more than one RS resource Consequences if not approved: UE operation is not clearly defined when pathlossReferenceLinking is provided for multi-TRP PUSCH ==================== Start of Text proposal ============================ ### 7.1.1 UE behaviour <Unchanged text is omitted> PLb,f,c(qd)is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index qd for the active DL BWP, as described in clause 12, of carrier f of serving cell c - If the UE is not provided PUSCH-PathlossReferenceRS and enableDefaultBeamPL-ForSRS, or before the UE is provided dedicated higher layer parameters, the UE calculates PLb,f,c(qd) using a RS resource from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB - If the UE is configured with a number of RS resource indexes, up to the value of maxNrofPUSCH-PathlossReferenceRSs, and a respective set of RS configurations for the number of RS resource indexes by PUSCH-PathlossReferenceRS, the set of RS resource indexes can include one or both of a set of SS/PBCH block indexes, each provided by ssb-Index when a value of a corresponding pusch-PathlossReferenceRS-Id maps to a SS/PBCH block index, and a set of CSI-RS resource indexes, each provided by csi-RS-Index when a value of a corresponding pusch-PathlossReferenceRS-Id maps to a CSI-RS resource index. The UE identifies a RS resource index qd in the set of RS resource indexes to correspond either to a SS/PBCH block index or to a CSI-RS resource index as provided by pusch-PathlossReferenceRS-Id in PUSCH-PathlossReferenceRS - If the PUSCH transmission is scheduled by a RAR UL grant as described in clause 8.3, or for a PUSCH transmission for Type-2 random access procedure as described in clause 8.1A, the UE uses the same RS resource index qd as for a corresponding PRACH transmission - If the UE is provided SRI-PUSCH-PowerControl and more than one values of PUSCH-PathlossReferenceRS-Id, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field, or for first and second SRI fields if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook', or values for a first SRI field and values associated with a second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', in a DCI format scheduling the PUSCH transmission and a set of PUSCH-PathlossReferenceRS-Id values and determines the RS resource index qd, or respective first and second RS resource indexes qd, from the value of PUSCH-PathlossReferenceRS-Id that is mapped to the SRI field value, or from the values of PUSCH-PathlossReferenceRS-Id that are mapped to respective first and second SRI field values if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook', or from the values of PUSCH-PathlossReferenceRS-Id that are mapped to respective first SRI field value and a value associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', where the RS resource(s) is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking ==================== End of Text proposal ============================

# Enhancements on beam management

Here, we discuss the remaining issues related to the beam failure recovery in multi-TRP operation.

## 2.1 Update of explicit BFD-RS set configuration for CORESET

RAN1 have agreed to support for updating explicit BFD-RS set via MAC-CE, however, the implemented text in the latest CR was interpreted this agreement as activation of BFD-RS set. In this case, there is an ambiguity of the UE operation with configured BFD-RS resources before activation. To clear the operation, the operation should be first the configuration of two BFD-RS sets and updating of them afterword.

A MAC CE is used for activating/deactivating reference signals in a list/set provided by the RRC for a BFD-RS set. As two BFD-RS sets can be configured, two separate RRC configured lists may be needed.

Proposal 2-1: In order to remove the ambiguity of UE operation for BFD-RS set determination, MAC-CE operation is supported for updating one or more existing BFD-RS resources (per failureDetectionSet) without any restriction on whether one or two BFD-RS sets are configured for BWP. Adopt following TP below.

Reason for change: In RAN1 #108e, new MAC-CE for updating explicit BFD-RS was agreed without details. There is ambiguity if MAC-CE can add/release of CR provide for clarifying unclear UE assumption. Summary of change: Clarify explicit BFD-RS is RRC configured and can be updated by MAC-CE. Consequences if not approved: UE operation is not clearly defined, and ambiguos when explicit BFD-RS set(s) are configured but not activated. ==================== Start of Text proposal ============================ TS38.213, Section 6 <Unchanged text is omitted> A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes by [RRC name] that can be activated by a MAC CE [11 TS 38.321] and corresponding two sets q1,0 and q1,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList1 and candidateBeamRSList2, respectively, for radio link quality measurements on the BWP of the serving cell. The set q0,0 is associated with the set q1,0 and the set q0,1 is associated with the set q1,1. The [set q0 or] two sets q0,0 and q0,1 can be updated by a MAC-CE [11, TS 38.213]. ==================== End of Text proposal ============================

## 2.2 Beam/power update for PUCCH after receiving gNB response

After reception of gNB response, it is required to update spatial relation or power control parameter for PUCCH. We prefer introducing association between PUCCH resource and BFD-RS.

Proposal 2-2: Support beam/power update for PUCCH after receiving gNB response.

•	Introduce association between PUCCH and BFD-RS.

## 2.3. Update of CORESETs for BFD-RS set of S-DCI after receiving gNB response

For S-DCI, after reception of gNB response, because there is no association to CORESET to TRP, UE cannot know which CORESET to be updated. However, fixed association to BFD-RS set to CORESET restrict the gNB’s scheduling flexibility due to limited number of CORESETs can be supported for S-DCI. The simplest method is to associate them based on the active TCI states of CORESETs and BFD-RS sets.

Proposal 2-3: For S-DCI, after receiving gNB response, the UE assumes that

- The QCL assumption of CORESETs associated with the failed BFD-RS sets is updated to the latest reported qnew.

- UE can determine the CORESETs based on the RS indicated by the active TCI states and the associated BFD-RS set for the explicitly configured BFD-RS

## 2.4 Single TRP fallback operation after gNB response

In addition, when UE is not able to indicate suitable candidate (qnew) for the failed TRP from the provided candidate beam list, after receiving gNB response, UE would not be required to monitor the CORESETs associated with the failed BFD-RS set for PDCCH until reconfigured by network. This could also apply for the UL transmission associated with the failed TRP. In one option, UE could fallback to single TRP mode. Fallback to single TRP mode could be considered also when both TRPs fail.

Proposal 2-4: If UE indicates that no suitable candidate for the failed BFD-RS set was found, UE is not required to monitor PDCCH on the CORESET(s) associated with failed BFD-RS set and falls back to single TRP operation.

Reason for change: In RAN1 #108e, UE is required continue monitoring for PDCCH on the CORESETs associated with failed BFD-RS set even if no suitable candidate can be found.

Summary of change: UE is not required to monitor PDCCH on CORESETs associated with failed BFD-RS set if no suitable candidate is indicated.

Consequences if not approved: Increased power consumption and potential effect on the monitoring of the CORESETs associated with non-failed BFD-RS set.

==================== Start of Text proposal ============================

TS38.213

For serving cells associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, and having radio link quality worse than Qout,LR, after 28 symbols from a last symbol of a first PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for transmission of the second PUSCH and having a toggled NDI field value, the UE assumes antenna port quasi-collocation parameters

- corresponding to qnew from q1,0, if any, for the first CORESETs,

-corresponding to qnew from q1,1, if any, for the second CORESETs

where the SCS configuration for the 28 symbols is the smallest of the SCS configurations of the active DL BWP for the PDCCH reception and of the active DL BWP(s) of the serving cells. If the UE is not able to provide any qnew  from  q1,0, The UE is not required to monitor PDCCH on the first CORESETs. If the UE is not able to provide any qnew  from  q1,1, The UE is not required to monitor PDCCH on the second CORESETs.

==================== Start of Text proposal ============================

# Enhancements on HST-SFN deployment

Here, we discuss the remaining issues related to the enhancements for HST-SFN deployment.

## 3.1 SFN PDCCH for CORESET associated with CSS type 0/0A/1/2

One of the issues is related to whether or not PDCCH candidates in CSS type 0/0A/1/2 can be associated with a CORESET activated with two TCI states and if allowed, how to determine the TCI states for reception of the CSS. In RAN1 #108-e, three alternatives were discussed:

- Alt 1: UE doesn’t expect PDCCH candidates in CSS type 0/0A/1/2 to be associated with CORESET activated with two TCI states.

- Alt 2: If PDCCH candidates in CSS 0/0A/1/2 are associated with CORESET that activated with two TCI states, the first TCI state is applied for the CSS reception.

- Alt 3: If PDCCH candidates in CSS 0/0A/1/2 are associated with CORESET that activated with two TCI states, both TCI states are applied for the CSS reception.

Alt 1 is very restrictive because it implies that if a CORESET is configured with CSS type 0/0A/1/2 as well USS, it cannot be activated with two TCI states and therefore the benefit of SFN configuration cannot be achieved for USS in this CORESET. Therefore, this alternative is not preferred.

Our view with respect to Alt 3 is that the there is too much complexity associated with supporting it given the need to consider different cases corresponding to whether the UE supports one or two monitoring occasions for CSS0 and the different sub-alternatives that can be considered for each case. Besides, Alt 3 may present ambiguity for UE operation if the gNB needs to configure different CORESET#0 according to UE capability. Furthermore, the impact to legacy UE that expects a non-SFN transmission but receives an SFN transmission needs to be considered. SFN transmission may increase the delay spread, and UE’s incorrect assumption (that doesn’t account for SFN transmission) may impact the demodulation performance. Lastly, if support of Alt 3 is a UE optional feature and the default is Alt 2, Alt 2 would anyway need to be supported. On the other hand, Alt 2 is simpler and enables the sharing of a CORESET between CSS and USS.

Proposal 3-1: Support Alt 2 – If PDCCH candidates in CSS 0/0A/1/2 are associated with CORESET that activated with two TCI states, the first TCI state is applied for the CSS reception.

## 3.2 BFR support for SFN PDCCH

When a CORESET is indicated with two TCI states, it was agreed that one BFD RS pair for SFN CORESET is counted as two BFD RSs. However, it is not fully clear that the UE can consider such assumption when there is a CORESET with a single TCI state and the number of BFD RSs are limited to two. When UE is configured with both single and two TCI state CORESETs but the UE is only capable of the current maximum number of BFD-RS (UE assumes set of q0 to include up to two RS), the total number of BFD RSs is exceeded in the implicit configuration if the UE selects all RSs in the two CORESETs. In this case, the UE should select one TCI (and one RS) from the CORESET with two TCI states and one RS from the CORESET with one TCI state. In this manner, the current maximum number for one set of q0 is not exceeded and both CORESETs are monitored for failure detection. In one option, the selected RS from the CORESET with two TCI states could be the one with shorter periodicity.

Proposal 3-2: When configured with two CORESETs with one or two active TCI States and the number of monitored BFD-RSs is 2, UE selects one RS from one CORESET and one RS from another CORESET.

In RAN1 #106bis agreement, it is also mentioned that whether to increase the maximum number X of monitored BFD RSs based on UE capability can be further studied (values of X = 2, 3, 4 are proposed for initial consideration) [3]. The maximum number of BFD-RSs monitored by the UE in the current specification is 2. This is, however, with a maximum of one TCI state for up to three configured CORESETs. With the possibility to configure CORESETs with two TCI states, it would be useful to increase the maximum number of monitored BFD RSs based on UE capability.

Proposal 3-3: Support increasing the maximum number of monitored BFD-RSs to up to 4 based on UE capability.

When supporting more than 2 monitored BFD-RSs, it is also required to clarify how the UE selects these BFD-RSs from activated TCI state(s) of CORESETs. Here, it seems reasonable to consider the following scenarios.

- Case 1: All CORESETs are activated with two TCI states.

- Case 2: Some CORESETs are activated with single TCI state and others are activated with two TCI states.

If the number of monitored BFD-RSs is equal to 3, the UE may select two RS from one SFN CORESET and the first RS from another SFN CORESET or select two RS from one SFN CORESET and one RS from another non-SFN CORESET for Case 1 or Case 2, respectively. This may be a bit different when the number of monitored BFD-RSs is equal to 4, where the UE may need to select two RSs from one SFN CORESET and two RSs from another SFN CORESET for Case 1. For Case 2, the UE may select two RS from one SFN CORESET, one RS from a non-SFN CORESET, and the last RS from remaining CORESET (SFN or non-SFN CORESET). In summary, for X = 3 or 4 BFD-RSs, the UE can select RSs from both SFN and non-SFN (if available) CORESETs. The rule is generalized in the following proposal.

Proposal 3-4: When supporting the number of monitored BFD-RSs X where X = 3 or 4 based on UE capability:

- If all CORESETs are SFN-CORESETs, one BFD RS pair for one SFN CORESET is counted as two BFD RSs and remaining one or two BFD-RSs are selected from the first TCI state (for X = 3) or both TCI states (for X = 4) of another SFN CORESET, respectively.

- If CORESETs are a mix of SFN-CORESET(s) and non-SFN CORESET(s),

  - if X = 3, one BFD RS pair for an SFN CORESET is counted as two BFD RSs and the remaining one BFD-RS is selected from the TCI state of a non-SFN CORESET.

  - if X = 4, one BFD RS pair for an SFN CORESET is counted as two BFD RSs and the remaining two BFD-RSs are selected from the TCI state of a non-SFN CORESET and one TCI state from the remaining CORESET.

For explicit configuration of BFD RS, it was agreed in RAN1 #105-e to down-select from the following two alternatives.

  - Alt 2-1: Support defining CSI-RS resource or SSB pairs as BFD RS

    - FFS other details

  - Alt 2-2: Reuse the existing Rel-15/Rel-16 approach for BFD RS configuration

In our view, the Rel-15/16 approach is simple enough and sufficient, and it is not necessary to support defining CSI-RS resource or SSB pairs as BFD RS.

Proposal 3-5: For explicit configuration of RS for BFD, support Alt 2-2 to reuse the existing Rel-15/Rel-16 approach for BFD RS configuration.

The following agreement was made in RAN1 #106bis-e.

Agreement

When two TCI states are activated for a CORESET, NBI RS can be configured as follows

- Alt 4-1: Using the existing Rel-15 NBI configuration based on single SSB / CSI-RS resource

- FFS addition support of Alt 4-2: two new beam identification CSI-RS resource sets / new beam identification CSI-RS resource pairs or SSB pairs

Even though RAN1 agreed not to include enhanced MAC-CE signalling when CORESETPoolindex is configured, there may be extensions in general of per-TRP BFR with S-DCI based M-TRP modes. With SFN, there can be two alternative options for a case where CORESET is activated with two TCI states. The first option is corresponding to Rel-15 principle, using both CSI-RS as the q0 BFD-RSs and determining beam failure when all the RSs in the set of q0 are in failure condition, i.e., below the quality threshold. The second option is applying Rel-17 multi-TRP BFR principle: defining two sets of BFD-RSs for two TRPs and deriving BFD-RS per TRP based on further clarification of BFD-RS to TRP association.

For beam failure recovery in SFN operation, UE could be configured with the information regarding which of the DL RS of a cell can be assumed to be valid for SFN operation within the serving cell. When UE performs beam failure recovery, it could utilize the information for candidate beam selection. In Rel-17 the BFR is enhanced to cover beam failure recovery for M-TRP operation where UE determines failure per BFD-RS set and can indicate candidate for the failed BFD-RS set (up to 2 sets can be configured). If the RSs indicated by the 2 active TCI states for a CORESET are included in different sets of BFD-RS, UE could recover the failed RS and indicate a new candidate that is suitable for continuing SFN operation. If UE indicates new candidate beam for recovery that is not suitable for continuing the SFN operation, or UE is not able to select any candidate beam, UE may not assume SFN to continue.

Proposal 3-6: When two TCI states are activated for a CORESET,

- Support Alt 4-2: two new beam identification CSI-RS resource sets / new beam identification CSI-RS resource pairs or SSB pairs.

- The UE can be configured with additional information that indicates a candidate beam or a candidate beam pair (pairs) that support SFN operation. If the UE indicates candidate beam or candidate beam pair for beam failure that supports SFN operation, the UE can continue with SFN operation after BFR.

In case of 2 BFD-RS sets (with corresponding NBI-RS sets/candidate beam sets), the additional information could indicate the resource sets that can be used for SFN operation i.e., if UE recovers one failed beam it can select candidate beam that is suitable for continuing SFN operation based on the additional information. In case of 1 BFR-RS set (that includes RSs indicated by both TCI states), the candidate beam /NBI-RS set can indicate an RS index that points to the resource pair that is suitable for SFN operation.

Furthermore, the issue of RLM-RS selection was discussed in RAN1 #107bis-e. Since the radio link monitoring is performed on a single set of RSs similar to BFD case (and opposed to mTRP that has agreed that up to 2 sets are used), in our view the same selection rule can be used for RLM-RS and BFD-RS. The maximum number of RLM-RS may be different than the UE capability for maximum number of BFD-RS; however, the same selection could be used.

For explicit configuration of RLM-RS, it could be assumed that network may not configure the number of RLM-RS beyond the maximum possible value (based on Lmax as defined in Rel-15).

Proposal 3-7: For implicit configuration of failure detection RS for BFD and RLM, apply the same selection rule for RLM-RS and BFD-RS if the number of configured RS exceeds the respective maximum value.

# Conclusion

In this contribution, we discuss remaining details related to multi-TRP/panel transmission. The following proposals are made.

Proposal 1-1: For multi-TRP PUSCH, to determine two RS resources for the case where pathlossReferenceLinking is provided, adopt the following text proposal to TS 38.213 Sec. 7:

Reason for change: To determine two RS resources for the case where pathlossReferenceLinking is provided for multi-TRP PUSCH. Summary of change: Editorial correction to cover more than one RS resource Consequences if not approved: UE operation is not clearly defined when pathlossReferenceLinking is provided for multi-TRP PUSCH ==================== Start of Text proposal ============================ ### 7.1.1 UE behaviour <Unchanged text is omitted> PLb,f,c(qd)is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index qd for the active DL BWP, as described in clause 12, of carrier f of serving cell c - If the UE is not provided PUSCH-PathlossReferenceRS and enableDefaultBeamPL-ForSRS, or before the UE is provided dedicated higher layer parameters, the UE calculates PLb,f,c(qd) using a RS resource from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB - If the UE is configured with a number of RS resource indexes, up to the value of maxNrofPUSCH-PathlossReferenceRSs, and a respective set of RS configurations for the number of RS resource indexes by PUSCH-PathlossReferenceRS, the set of RS resource indexes can include one or both of a set of SS/PBCH block indexes, each provided by ssb-Index when a value of a corresponding pusch-PathlossReferenceRS-Id maps to a SS/PBCH block index, and a set of CSI-RS resource indexes, each provided by csi-RS-Index when a value of a corresponding pusch-PathlossReferenceRS-Id maps to a CSI-RS resource index. The UE identifies a RS resource index qd in the set of RS resource indexes to correspond either to a SS/PBCH block index or to a CSI-RS resource index as provided by pusch-PathlossReferenceRS-Id in PUSCH-PathlossReferenceRS - If the PUSCH transmission is scheduled by a RAR UL grant as described in clause 8.3, or for a PUSCH transmission for Type-2 random access procedure as described in clause 8.1A, the UE uses the same RS resource index qd as for a corresponding PRACH transmission - If the UE is provided SRI-PUSCH-PowerControl and more than one values of PUSCH-PathlossReferenceRS-Id, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field, or for first and second SRI fields if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook', or values for a first SRI field and values associated with a second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', in a DCI format scheduling the PUSCH transmission and a set of PUSCH-PathlossReferenceRS-Id values and determines the RS resource index qd, or respective first and second RS resource indexes qd, from the value of PUSCH-PathlossReferenceRS-Id that is mapped to the SRI field value, or from the values of PUSCH-PathlossReferenceRS-Id that are mapped to respective first and second SRI field values if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook', or from the values of PUSCH-PathlossReferenceRS-Id that are mapped to respective first SRI field value and a value associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', where the RS resource(s) is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking ==================== End of Text proposal ============================

Proposal 2-1: In order to remove the ambiguity of UE operation for BFD-RS set determination, MAC-CE operation is supported for updating one or more existing BFD-RS resources (per failureDetectionSet) without any restriction on whether one or two BFD-RS sets are configured for BWP. Adopt following TP below.

Reason for change: In RAN1 #108e, new MAC-CE for updating explicit BFD-RS was agreed without details. There is ambiguity if MAC-CE can add/release of CR provide for clarifying unclear UE assumption. Summary of change: Clarify explicit BFD-RS is RRC configured and can be updated by MAC-CE. Consequences if not approved: UE operation is not clearly defined, and ambiguos when explicit BFD-RS set(s) are configured but not activated. ==================== Start of Text proposal ============================ TS38.213, Section 6 <Unchanged text is omitted> A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes by [RRC name] that can be activated by a MAC CE [11 TS 38.321] and corresponding two sets q1,0 and q1,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList1 and candidateBeamRSList2, respectively, for radio link quality measurements on the BWP of the serving cell. The set q0,0 is associated with the set q1,0 and the set q0,1 is associated with the set q1,1. The [set q0 or] two sets q0,0 and q0,1 can be updated by a MAC-CE [11, TS 38.213]. ==================== End of Text proposal ============================

Proposal 2-2: Support beam/power update for PUCCH after receiving gNB response.

- Introduce association between PUCCH and BFD-RS.

Proposal 2-3: For S-DCI, after receiving gNB response, the UE assumes that

•	The QCL assumption of CORESETs associated with the failed BFD-RS sets is updated to the latest reported qnew.

•	UE can determine the CORESETs based on the RS indicated by the active TCI states and the associated  BFD-RS set for the explicitly configured BFD-RS

Proposal 2-4: If UE indicates that no suitable candidate for the failed BFD-RS set was found, UE is not required to monitor PDCCH on the CORESET(s) associated with failed BFD-RS set and falls back to single TRP operation.

Reason for change: In RAN1 #108e, UE is required continue monitoring for PDCCH on the CORESETs associated with failed BFD-RS set even if no suitable candidate can be found.

Summary of change: UE is not required to monitor PDCCH on CORESETs associated with failed BFD-RS set if no suitable candidate is indicated.

Consequences if not approved: Increased power consumption and potential effect on the monitoring of the CORESETs associated with non-failed BFD-RS set.

==================== Start of Text proposal ============================

TS38.213

For serving cells associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, and having radio link quality worse than Qout,LR, after 28 symbols from a last symbol of a first PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for transmission of the second PUSCH and having a toggled NDI field value, the UE assumes antenna port quasi-collocation parameters

- corresponding to qnew from q1,0, if any, for the first CORESETs,

-corresponding to qnew from q1,1, if any, for the second CORESETs

where the SCS configuration for the 28 symbols is the smallest of the SCS configurations of the active DL BWP for the PDCCH reception and of the active DL BWP(s) of the serving cells. If the UE is not able to provide any qnew  from  q1,0, The UE is not required to monitor PDCCH on the first CORESETs. If the UE is not able to provide any qnew  from  q1,1, The UE is not required to monitor PDCCH on the second CORESETs.

==================== Start of Text proposal ============================

Proposal 3-1: Support Alt 2 – If PDCCH candidates in CSS 0/0A/1/2 are associated with CORESET that activated with two TCI states, the first TCI state is applied for the CSS reception.

Proposal 3-2: When configured with two CORESETs with one or two active TCI States and the number of monitored BFD-RSs is 2, UE selects one RS from one CORESET and one RS from another CORESET.

Proposal 3-3: Support increasing the maximum number of monitored BFD-RSs to up to 4 based on UE capability.

Proposal 3-4: When supporting the number of monitored BFD-RSs X where X = 3 or 4 based on UE capability:

- If all CORESETs are SFN-CORESETs, one BFD RS pair for one SFN CORESET is counted as two BFD RSs and remaining one or two BFD-RSs are selected from the first TCI state (for X = 3) or both TCI states (for X = 4) of another SFN CORESET, respectively.

- If CORESETs are a mix of SFN-CORESET(s) and non-SFN CORESET(s),

  - if X = 3, one BFD RS pair for an SFN CORESET is counted as two BFD RSs and the remaining one BFD-RS is selected from the TCI state of a non-SFN CORESET.

  - if X = 4, one BFD RS pair for an SFN CORESET is counted as two BFD RSs and the remaining two BFD-RSs are selected from the TCI state of a non-SFN CORESET and one TCI state from the remaining CORESET.

Proposal 3-5: For explicit configuration of RS for BFD, support Alt 2-2 to reuse the existing Rel-15/Rel-16 approach for BFD RS configuration.

Proposal 3-6: When two TCI states are activated for a CORESET,

- Support Alt 4-2: two new beam identification CSI-RS resource sets / new beam identification CSI-RS resource pairs or SSB pairs.

- The UE can be configured with additional information that indicates a candidate beam or a candidate beam pair (pairs) that support SFN operation. If the UE indicates candidate beam or candidate beam pair for beam failure that supports SFN operation, the UE can continue with SFN operation after BFR.

Proposal 3-7: For implicit configuration of failure detection RS for BFD and RLM, apply the same selection rule for RLM-RS and BFD-RS if the number of configured RS exceeds the respective maximum value.

# References

- 3GPP RAN1 #108-e Chairman notes