3GPP TSG RAN WG1 #109-e Meeting	R1- 2204539

e-Meeting, May 9th – May 20th, 2022

Agenda item:		9.1.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Two TAs for UL multi-DCI multi-TRP operation

Document for:		Discussion and Decision

# Introduction

Rel-18 MIMO scope was finalized in RAN#94 where the corresponding objectives were captured in RP-213598. One of these objectives is to study and, if justified, specify the use of two TAs (timing advance) considering UL multi-DCI for multi-TRP operation. This objective is copied below:

7. Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In this contribution, we discuss the motivation to introduce two TAs for UL multi-DCI for multi-TRP operation as well as potential enablers and directions that would need to be considered and investigated.

# Discussion

## Justification for introducing multiple TAs for multi-TRP

Legacy (Rel-15/Rel-16) timing advance (TA) procedures defined the use of a single TA per serving cell. Hence, for multi-TRP operation within a serving cell, it has so far been assumed that two transmitted signals towards (or from) different TRPs are within the CP length.

On the other side, Rel-16/Rel-17 introduced multi-TRP operations where the main goal was to improve the reliability and robustness of DL and UL channels. Rel-18 MIMO objectives include the extension of the Rel-17 unified TCI framework to multi-TRP use cases, and also include enabling simultaneous UL transmissions for higher UL throughput/reliability assuming up to two TRPs. Under multi-TRP cases, including both intra-cell and inter-cell cases, a UE may communicate with or be connected to (at least) two TRPs at a time.

However, as can be seen in Figure 1, the links from a UE to two TRPs could have different propagation delays, as these links typically correspond to different propagation paths.

The difference between the propagation delays may be important, especially in mmWave systems with different UE beams/panels used for transmissions towards different TRPs. Also, TRPs may belong to different cells in the case of inter-cell multi-TRP. In addition, TRPs could be non-ideal synchronized, especially considering the multi-DCI mode as this mode is more suitable for non-ideal backhaul cases.

Figure 1: UE served by two TRPs with different propagation paths/delays.

Therefore, considering at least the difference in propagation delays between TRPs (for both inter-cell and intra-cell multi-TRP) and the synchronization precision between TRPs, it may not be possible to guarantee that UL signals towards different TRPs are within the CP length if a single TA is used, i.e., relying on a single TA for UL transmissions towards different TRPs may not be sufficient. Otherwise, using a single TA in this case would lead to UL performance degradation due to at least inter-symbol interference. Furthermore, although the UE may be able to adjust TA autonomously, the adjustment can be done with a limited rate and may not be enough for operating UL links to different TRPs.

In the following, we provide an example to show the restriction caused by using a single TA for multi-TRP, and thus why two TAs would be needed. For this example, we consider 120 SCS, and a CP length of 590 ns. We assume that a single TA is used for UL transmissions towards different TRPs, and the propagation difference between the two TRPs in addition to delay spread should not exceed 2/3 CP length. In practice, this leads to a propagation delay difference constrained to about 30-60 meters (depending on the delay spread assumption), if a single TA is used. However, there are many scenarios where a larger difference in distance is required for multi-TRP, especially considering inter-cell use cases. Examples are provided in Figure 2, based on the following calculations, where the difference in propagation delay between TRPs corresponds to 43 m:

| TRP2-device distance | Cluster separation | a | d1 | d2 | AoA separation intra-cell (beta) | b |  | a+b | d1+d2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60,0 | 40,0 | 42,6 | 14,6 | 45,4 | 41,4 | 60,5 |  | 103,1 | 60,0 |

Figure 2: Examples requiring multiple TA loops for intra-cell and inter-cell mTRP scenarios.

Furthermore, considering the TA autonomous adjustments are only up to 80 ns per 200 ms according to TS38.133 section 7.1.2.1, which corresponds to 24 m, this may not be sufficient to address UL transmissions to different TRPs in all scenarios.

Observation 1: Multi-TA is necessary for multi-TRP operation in intra-cell and inter-cell use cases.

Proposal 1: RAN1 supports multi-TA operation for multi-TRP use cases in Rel-18 NR.

## TRP synchronization assumption

One aspect that needs to be considered in the Rel-18 TA discussion is the synchronization between TRPs. Specifically, depending on the TRP synchronization assumption, two cases can be considered:

- Case 1: the two TRPs are assumed to have ideal synchronization. An example of this case is illustrated in Figure 3.

- Case 2: the two TRPs are assumed to have non-ideal synchronization. An example of this case is illustrated in Figure 4.

Figure 3: Example illustrating two TAs in case of ideal synchronization between TRPs.

Figure 4: Example illustrating two TAs in case of non-ideal synchronization between TRPs.

It should be discussed and decided whether both Cases 1 and 2 would be allowed so that we start the discussions on two TAs for multi-TRP with a common understanding/assumption between companies – as this may impact the assumption regarding the DL Tx timing of the two TRPs. Case 1, even if called “ideal”, it is understood as experiencing some level of non-idealities. In our view, the assumption under which the TRPs are assumed to have non-ideal synchronization is more realistic.

Proposal 2: For the Rel-18 TA enhancements for multi-TRP, RAN1 should prioritize the assumption that the TRPs have non-ideal synchronization.

## Enablers for multiple TAs for multi-TRP

### Enablers for the determination of TA values for multi-TRP

In legacy procedures, the network can determine the TA value for a UE based on the measurement of its UL transmissions. Although sounding reference signals (SRS) can be used as a regular UL signal to perform such measurements, generally speaking, the network can use any signal transmitted from the UE for this purpose. In addition, when the gNB detects a problem in the UL synchronization of a UE, the network can indicate the UE to initiate a random-access procedure through a PDCCH order. The PDCCH order is triggered by DCI format 1_0 (with CRC scrambled by C-RNTI). This PDCCH can trigger either contention-free random access (CFRA) procedure or contention-based random access (CBRA) procedure.

Considering now the multi-TRP scenarios with two TRPs, the DL Rx (reception) timing at the UE corresponding from two TRPs are different (e.g., synchronized TRPs with different propagation delays to the UE as explained in section 2.1). Therefore, in order to operate independent TA loops, the UE shall consider two DL Rx timings, each DL Rx timing corresponding to a TRP, and the TA per TRP is calculated with respect to the DL Rx timing corresponding to the same TRP.

Observation 2: To support TA per TRP, the UE needs to maintain two DL Rx timings, each corresponding to a TRP.

For multi-DCI multi-TRP scenario, as the UE should be configured with CoresetPoolIndex, it is somewhat evident that two sets of CORESETs shall be monitored from two different TRPs, and DL RX timing corresponding to each TRP can be derived. Also, as most of the UL channels would consider an indirect association to a CORESETPoolIndex, it is clear how the TAs would be applied to those UL channels (e.g., dedicated PUCCH, DG PUSCH). Anyhow, some discussion seems to be needed on how to determine the applicable TA for other UL channels that may not have a clear association with a CORESETPoolIndex.

Observation 3: DL Rx timing can be associated with a CORESETPoolIndex as DL channels can be separated into two different groups based on multi-DCI multi-TRP configuration of CORESETPoolIndex.

With multi-TRP considerations, the network would for a UE configure or trigger SRS transmissions towards each TRP and then determine the TA value corresponding to each TRP. As previously mentioned, the network could use other UL signals/transmissions towards a TRP to determine the TA value corresponding to this TRP. Given legacy multi-TRP operations and Rel-18 objective to extend the unified TCI framework to multi-TRP, it seems there is no special enhancement required for the support of determining two TAs for multi-TRP scenarios based on UL transmissions/signals, excluding the PRACH transmissions (e.g., triggered by PDCCH order).

Observation 4: Except for PRACH, no special enhancements seem to be required for the support of determining two TAs for multi-TRP scenarios based on UL transmissions/signals considering the Rel-18 objective of extending the unified TCI framework to multi-TRP use cases.

Proposal 3: To enable the determination of two TAs for multi-TRP scenarios, two different DL Rx timings should be maintained by the UE, where each DL Rx timing corresponding to DL receptions associated with a CORESETPoolIndex.

Proposal 4: Per TRP UL channels shall be used to maintain TA loop per TRP, where each TA is determined relative to the DL Rx timing of the corresponding TRP.

On the other hand, the operation of triggering random-access procedure through PDCCH order may require more discussions to see whether some enhancements on this operation are required considering the presence of two TRPs, as the legacy operation doesn’t account for the TRP dimension. And the discussion should consider both CFRA and CBRA procedures, as the PDCCH order could trigger any of these procedures. Note that CBRA procedure could also be used in case of UE assisted triggering for multiple TA operation.

Further, in legacy, RRC configures the UE with a timer (timeAlignmentTimer) per group of cells referred to as TAG (time advance group). The UE restarts the timer when it receives a new TA value. If the timer expires, the UE will need to perform a random-access procedure before it’s possible for the UE to perform UL transmissions within these cells. Considering the multi-TRP aspect now, it should be discussed whether some enhancements would be needed considering the aspects related to the time alignment timer and the corresponding random-access procedure (triggered when the timer expires).

Proposal 5: To enable the determination of two TAs for multi-TRP scenarios, discuss whether enhancements are needed on the operation of triggering random-access procedure through PDCCH order, considering CFRA and CBRA procedures.

Proposal 6: To enable the determination of two TAs for multi-TRP scenarios, discuss whether enhancements are needed on the aspects related to the time alignment timer and the corresponding random-access procedure.

### Indication of one TA value or two TA values corresponding to two TRPs

First, it should be noted that for multi-TRP scenarios, it would be up to the network to determine whether, in addition to a first TA, a second TA is needed. Thus, if two TAs are supported for multi-TRP, the network may not necessarily always indicate the UE with two TACs (timing advance commands).

In legacy, the UE receives the initial (absolute) timing advance value in a RAR (random access response) message. Timing advance value can then be updated with a MAC CE giving a TAC with a relative TA value. Considering the Rel-18 TA enhancements, when the network decides to indicate two TACs for the UE, similar ways of carrying one TAC in legacy can be essentially used, i.e., through RAR and MAC CE.

For MAC-CE updates of the relative TA, the legacy MAC-CE could be for instance associated with CORESETPoolIndex0 and a new MAC-CE, following a similar design as the legacy, could be introduced and be associated with CORESETpoolIndex1.

Proposal 7: Discuss the indications of one or two TA values for multi-TRP scenarios.

## Feasibility of applying two TAs for multi-TRP

Although the support of two TAs for multi-TRP scenarios is needed (as argued above), it’s important to discuss the feasibility of applying two TAs (and maintaining two TA loops) from a UE implementation perspective in some cases.

Specifically, from a UE implementation perspective, it may be easier to consider a scenario where the UE is using two TAs and applying them only for TDMed UL transmissions at the UE. In such a scenario, the UE may switch from one UL transmission timing assumption to another UL transmission timing assumption with a certain delay (delay may be at least associated with switching of timing modes) or without any delay. For example, in legacy multi-TRP UL enhancements, the transmission modes defined follow TDM operation, and only one TA is assumed for all TDMed UL transmissions. Extending such operations with two different UL timing seems a more reasonable assumption from the UE implementation perspective.

However, as the same UE may also support simultaneous UL transmissions in Rel-18, it should be clarified whether applying two TAs would be feasible from UE implementation perspective and, if needed, how the different TA loops can be used to derive proper UL timing for the simultaneous UL transmission. Although here we mention about simultaneous UL transmissions, similar observations can be made on the feasibility of applying two TAs for TDM UL transmissions towards different TRPs when the switching period between these UL transmissions is too tight or when the TA adjustment would exceed the autonomous adjustment rate.

Proposal 8: Discuss the feasibility of applying two TAs for different multi-TRP UL operations (e.g., simultaneous UL transmissions, TDM UL transmissions when the switching period between UL transmissions is too tight, etc.) from the UE implementation perspective.

# Conclusion

In this contribution, we discuss the use of two TAs considering UL multi-DCI for multi-TRP operation. The following observations and proposals are made:

Observation 1: Multi-TA is necessary for multi-TRP operation in intra-cell and inter-cell use cases.

Proposal 1: RAN1 supports multi-TA operation for multi-TRP use cases in Rel-18 NR.

Proposal 2: For the Rel-18 TA enhancements for multi-TRP, RAN1 should prioritize the assumption that the TRPs have non-ideal synchronization.

Observation 2: To support TA per TRP, the UE needs to maintain two DL Rx timing, each corresponding to a TRP.

Observation 3: DL Rx timing can be associated with a CORESETPoolIndex as DL channels can be separated into two different groups based on multi-DCI multi-TRP configuration of CORESETPoolIndex.

Observation 4: Except for PRACH, no special enhancements seem to be required for the support of determining two TAs for multi-TRP scenarios based on UL transmissions/signals considering the Rel-18 objective of extending the unified TCI framework to multi-TRP use cases.

Proposal 3: To enable the determination of two TAs for multi-TRP scenarios, two different DL Rx timings should be maintained by the UE, where each DL Rx timing corresponding to DL receptions associated with a CORESETPoolIndex.

Proposal 4: Per TRP UL channels shall be used to maintain TA loop per TRP, where each TA is determined relative to the DL Rx timing of the corresponding TRP.

Proposal 5: To enable the determination of two TAs for multi-TRP scenarios, discuss whether enhancements are needed on the operation of triggering random-access procedure through PDCCH order, considering CFRA and CBRA procedures.

Proposal 6: To enable the determination of two TAs for multi-TRP scenarios, discuss whether enhancements are needed on the aspects related to the time alignment timer and the corresponding random-access procedure.

Proposal 7: Discuss the indications of one or two TA values for multi-TRP scenarios.

Proposal 8: Discuss the feasibility of applying two TAs for different multi-TRP UL operations (e.g., simultaneous UL transmissions, TDM UL transmissions when the switching period between UL transmissions is too tight, etc.) from the UE implementation perspective.