3GPP TSG RAN WG1 Meeting #109-e		R1- 2204542

e-Meeting, May 9th – 20th, 2022

Agenda item:       9.1.3.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	SRS enhancement for TDD CJT and 8Tx operation

Document for:     Discussion and Decision

# 1	Introduction

In RAN#94-e it was agreed to study and, if justified, specify two enhancements related to SRS [1], to address CSI acquisition for coherent joint transmission (CJT) in TDD and UL MIMO transmission with 8Tx antennas:

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this paper we discuss the evaluation methodology for the two enhancements and present our views on the schemes that should be studied.

# 2	SRS Enhancement for 8 Tx Antenna Ports

The work item aims to specify the enhancements identified for NR MIMO. The detailed RAN1 objectives are as follows: … Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study. …

## 2.1	 Discussion on Resource Configuration for >4 TX Antenna Ports

Higher peak data rate for UL could play a significant role in short-range applications such as home entertainment, video surveillance/monitoring in industrial/healthcare/ safety, IAB, and other applications where devices power/form-factor/cost are not as stringent as in traditional handheld devices. Building a specification support for UL transmission with >4Tx antenna ports can be seen as essential component in Rel-18 to bridge the gap between DL and UL spectral efficiencies, in both FR1 and FR2.

Current Rel-17 specification provides a support for up to 4 APs with different UL SRS comb-types and maximum cyclic-shift values.  Table 1 provides summary of Rel-17 UL SRS comb-types and maximum cyclic-shift values and the number of supported antenna ports.

Table 1 Rel-17 UL SRS comb-types, maximum cyclic-shift values and number of antenna ports per resource.

| Comb-Type | Max. CSs | #Antenna Ports per Resource |
| --- | --- | --- |
| Comb-2 | 8 | up to 4 APs |
| Comb-4 | 12 | up to 4 APs |
| Comb-8 | 6 | up to 4 APs (w/ 2 comb offsets) |

Up to now, to enable simultaneous up to 4APs UL SRS transmission, UE can be configured with UL SRS resource up to 4 APs associated with a resource set, where the UL SRS resource set can have one or more resources with same or different number of APs in each resource, depending on SRS usage type. To increase the number of UL SRS APs for simultaneous transmission, three different ways can be identified to obtain 8-APs per UE as follows:

- Option 1) Increasing number of APs per resource

- Option 2) Enabling simultaneous transmission over multiple resources including APs within a resource set

- Option 3) Enabling simultaneous transmission over resources including APs distributed across resource sets

In case of Option 1,a straightforward way of increasing the number of antenna ports is to configure second comb-offset for the UE by using same root sequence as the first comp-offset.  For example, 8 AP UL SRS  resource can be supported with comb-4 and CS=12 by locating AP0-AP3 to in first comb-offset with four different cyclic values and locating AP4-AP7 into the second comb-offset = (first comb-offset+2)mod 4. It is worth noting in this case same mother sequency is applied for all APs.  It is also possible to support 6 UL SRS APs by reducing the number of CS values up 3 per comb-offset. By applying same principle, the number of antenna ports can be extended also with comb-2 and comb-8. Clearly, actual channel estimation performance of different options is heavily subject to channel conditions, i.e. frequency selectivity and inter-cell interference. Based on the above discussion, Option 1 can been as potential candidate for  Rel-18 UL SRS with usage of antenna switching and codebook without increasing  significantly resource overhead.

Observation 1: Option 1 facilitates a straightforward way to extend number of SRS APs up to 8 without increasing the number of resources or resource sets.

Observation 2: Option 1 can be seen as  a potential candidate for Rel-18 UL SRS w/ usage antenna switching and codebook .

In case of Option 2, the number of antenna ports can be extended by increasing the number of resources with same or different number of antenna ports (≥1) and defining set of resources to be simultaneously transmitted in a UL SRS resource set. It is worth noting that no new root sequence or comb-structures is needed. However, resource overhead is higher with respect to Option 1.  For example, to support  UL SRS with 8-APs, the UE can be configured with UL SRS resource set with two resources with comb-2/4/8 where each resource consist of 4-APs. Similar to Option1, it also possible to obtain 6-AP for example with two 3-AP resources or three 2-AP resources in the resource set. Option 2 can be seen as potential candidate for  Rel-18 UL SRS with codebook  and non-codebook usages.

Observation 3: Option 2 enables to extend number of SRS APs up to 8 by increasing the number of resources within a resource set.

Observation 4: Option 2 can be seen as a potential candidate for Rel-18 UL SRS w/ usage codebook and non-codebook.

In case of Option 3, the number of antenna ports can be extended by increasing the number of resource sets, where each resource set can have one resource with multiple of antenna ports (≥1) and defining resource sets to be simultaneously transmitted across configured SRS resource sets. It is worth noting that no new root sequence or comb-structures is needed. However, resource overhead is higher with respect to Option 1. For example, to support  UL SRS with 8-APs, the UE can be configured with two UL SRS resource sets where each set can  have one resource   with comb-2/4/8 with 4-APs. By following same logic, also other number of antenna ports can be obtained, e.g. 6-AP.   Option 2 can be seen as potential candidate for  Rel-18 UL SRS with codebook, non-codebook usages and beam management.

Observation 5: Option 3 enables to extend number of SRS APs up to 8 by increasing the number of resource sets.

Observation 6: Option 3 can be seen as a potential candidate for Rel-18 UL SRS w/ usage codebook, non-codebook and beam management.

Proposal 1: Study further the feasibility and performance of different Options 1-3 to increase the number of UL SRS antenna ports for different usages, i.e. antenna switching, codebook, non-codebook and beam management.

## 2.2	SRS Antenna Switching Enhancements for DL CSI Acquisition with up to 8Tx Antenna Ports

By facilitating specification support for up to 8 Tx UL operation, it enables to reduce UL SRS resource overhead and latency associated with  DL CSI acquisition via UL SRS antenna switching. From a network perspective, reduction of UL resource overhead is a highly important aspect. Therefore, we prefer to extend specification support for UL SRS  xTyR antenna switching configurations with 4 > UL TX antenna ports, for example where x = {6,8} and y = {6, 8},

Observation 7: Support for 8 Tx UL SRS operation enables to reduced UL SRS resource overhead and transmission latency related to antenna switching.

Proposal 2: Support in Rel-18 UL SRS  xTyR antenna switching configurations with 4 > UL TX antenna ports, for example xTyR. where x = {6,8} and y = {6, 8}.

# 3	SRS Enhancement for TDD CJT

The work item aims to specify the enhancements identified for NR MIMO. The detailed RAN1 objectives are as follows: … Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences …

Objective 4 of the MIMO WID for Rel-18 targets a possible extension of Type-II CSI reporting to coherent joint transmission (CJT) from up to 4 distributed remote radio heads (RRH) or TRPs, in TDD operation in FR1.

Rel-17 has introduced support for multi-TRP CSI reporting for the first time, with Type-I single panel (SP) for non-coherent joint transmission (NCJT) from two TRPs. CJT differs from NCJT in that it assumes phase synchronization between TRPs, such that a MIMO layer can be mapped to transmit antennas of multiple TRPs, whereas in NCJT, a MIMO layer can only be transmitted from a single TRP. Note that NCJT CSI reporting in Rel-17 assumes full overlap of MIMO layers in time and frequency resources, i.e., it assumes perfect time synchronization between TRPs, which is also assumed for CJT transmission.

Observation 8: CJT differs from NCJT in that it assumes phase synchronization between TRPs, such that a MIMO layer can be mapped to transmit antennas of multiple TRPs. Both CJT and NCJT assume full overlap of MIMO layers in time and frequency resources.

Let us consider a downlink single-DCI multi-TRP system formed by 2≤NTRP≤4 TRPs connected via perfect backhaul link and transmitting coherently (CJT) to one or more co-scheduled UEs. We assume that all RRH/TRPs have the same array geometry, N1×N2, with N1 antenna ports in azimuth and N2 in elevation, for each of the two polarisations, and 2N1N2 antenna ports in total per TRP. Figure 1 shows an example of a single-DCI multi-TRP setup with four RRH/TRPs where each TRP is equipped with a 2×2 dual-polarised array. The total number of transmit antenna ports across the four TRPs is 32.

Figure 1. Example of single-DCI multi-TRP configuration with ideal backhaul with 4 RRH/TRPs. The total number of tx antenna ports is 32.

## 3.1	Capacity enhancement

Figure 2 shows an example of Rel-17 UL SRS resource block (RB)-level partial frequency sounding configuration without hopping for UL SRS antenna switching purposes to be used in TDD C-JT. It is assumed that all UEs are configured with  frequency scaling factor, PF, =2 and comb-8.The first group of UEs associated with resources  is configured with  startRBIndex=0 and the second group of UEs with startRBIndex=1 with corresponding resources. Furthermore, it is assumed that in each sub-hop associated with group of UEs up to 24 APs are available (i.e 1-24 UEs subject to antenna ports per UE). As result of this, there are up to 48 APs in total for UL SRS antenna switching purposes, i.e. 1-48 UEs (subject to number of antenna ports per UE).  When UEs are configured with frequency scaling factor, PF, =4, the total number of available antenna ports for UL SRS antenna-switching can be doubled up to 96 APs leading to 1-96 UEs (subject to number of antenna ports per UE). Therefore, Rel-17 RB-level partial frequency sounding with PF, =4  without hopping can enable up to four times higher multiplexing capacity with respect to configuration without partial frequency sounding.

Figure 2 An example of Rel-17 SRS with RB-level partial frequency hopping without hopping with PF=2.

Observation 9: Rel-17 UL SRS RB-level with partial frequency sounding without hopping enables to increase UE multiplexing capacity with respect to UL SRS resource configuration without partial frequency sounding.

Proposal 3: Study further the feasibility and performance of  Rel-17 partial frequency sounding in the context of Rel-18 TDD C-JT.

## 3.3	Evaluation methodology

For reference, the table with the SLS parameters is found in Appendix A. Some modifications/additions are needed to define the relevant CJT scenarios. reuse Appendix A, except CSI part, with modifications given in the TRP layout of the table provided in sec. 3.6, plus the SRS of Rel-17, i.e. antenna switching configuration 2T2R and 4T4R are assumed to be used.

Proposal 4: For system level performance evaluation of CSI enhancement schemes for CJT in TDD operations, adopt the SLS assumptions from Rel-16 eType-II with the following modifications

| Parameters | Scenarios |  |
| --- | --- | --- |
|  | A: Intra-site (Macro + RRH) | B: Inter-site (Macro-only) |
| Inter-site distances | 1.7 km | 200 m |
| Carrier frequencies | 0.7 GHz | 2 GHz, 4 GHz (optional) |
| Channel type | Rural (RMa) | Urban Macro (Uma) |
| Simulation bandwidth | 10MHz |  |
| BS Transmit Power | Macro: 46 dBm RRH: 46 dBm | Macro: 46 dBm |
| BS Height | Macro: 35 m RRH: 35m | Macro: 25m |
| BS Antenna Configuration | 4 ports: (M,N,P,Mg,Ng,N1,N2) = (8,2,2,1,1,1,2) 100 mechanical elevation tilt | 16 ports: (M,N,P,Mg,Ng,N1,N2) = (8,4,2,1,1,2,4) 100 mechanical elevation tilt |
| UE Distribution | 100% outdoor | 20% outdoor |
| UE Antenna Configuration | 2 Rx: (M,N,P) = (1,1,2) 4 Rx: (M,N,P) = (1,2,2) |  |
| UE speed | 3 kmph |  |
| Traffic Model | FTP Model 1: 20/50% target RU |  |
| Receiver | Non-ideal 2RX MMSE | Non-ideal 4RX MMSE |
| CJT Scheduling Set Size | Intra-sector: 4 TRPs (1 Macro + 3 RRHs) Intra-site: 12 TRPs (3 Macros + 9 RRHs) | Intra-site: 3 TRPs Inter-site: 6 or 9 TRPs |

# 4	Conclusion

Hereafter is a summary of proposals for SRS enhancement for 8Tx UL MIMO.

Proposal 1: Study further the feasibility and performance of different Options 1-3 to increase the number of UL SRS antenna ports for different usages, i.e. antenna switching, codebook, non-codebook and beam management.

Proposal 2: Support in Rel-18 UL SRS  xTyR antenna switching configurations with 4 > UL TX antenna ports, for example xTyR. where x = {6,8} and y = {6, 8}.

Hereafter is a summary of proposals for SRS enhancement for TDD CJT.

Proposal 3: Study further the feasibility and performance of  Rel-17 partial frequency sounding in the context of Rel-18 TDD C-JT.

Proposal 4: For system level performance evaluation of CSI enhancement schemes for CJT in TDD operations, adopt the SLS assumptions from Rel-16 eType-II with the following modifications

| Parameters | Scenarios |  |
| --- | --- | --- |
|  | A: Intra-site (Macro + RRH) | B: Inter-site (Macro-only) |
| Inter-site distances | 1.7 km | 200 m |
| Carrier frequencies | 0.7 GHz | 2 GHz, 4 GHz (optional) |
| Channel type | Rural (RMa) | Urban Macro (Uma) |
| Simulation bandwidth | 10MHz |  |
| BS Transmit Power | Macro: 46 dBm RRH: 46 dBm | Macro: 46 dBm |
| BS Height | Macro: 35 m RRH: 35m | Macro: 25m |
| BS Antenna Configuration | 4 ports: (M,N,P,Mg,Ng,N1,N2) = (8,2,2,1,1,1,2) 100 mechanical elevation tilt | 16 ports: (M,N,P,Mg,Ng,N1,N2) = (8,4,2,1,1,2,4) 100 mechanical elevation tilt |
| UE Distribution | 100% outdoor | 20% outdoor |
| UE Antenna Configuration | 2 Rx: (M,N,P) = (1,1,2) 4 Rx: (M,N,P) = (1,2,2) |  |
| UE speed | 3 kmph |  |
| Traffic Model | FTP Model 1: 20/50% target RU |  |
| Receiver | Non-ideal 2RX MMSE | Non-ideal 4RX MMSE |
| CJT Scheduling Set Size | Intra-sector: 4 TRPs (1 Macro + 3 RRHs) Intra-site: 12 TRPs (3 Macros + 9 RRHs) | Intra-site: 3 TRPs Inter-site: 6 or 9 TRPs |

# References

- RP-213598, Samsung, “New WID: MIMO Evolution for Downlink and Uplink,” RAN#94e.

# Appendix A	SLS assumptions for TDD-CJT

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, 4GHz. |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Type II overhead reduction Other configuration is not precluded. |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| BS receiver noise figure |  | 5dB (IMT 2020/3GPP model) |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation is a baseline for overhead reduction. For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed for higher rank extension. For medium/high RU, SU/MU-MIMO with rank adaptation is assumed for higher rank extension. |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI sounding |  | assumption at least for baseline scheme - SRS periodicity : 5 ms, - xTyR, where x=y, y=2 or 4 |
| Overhead |  | Companies shall provide the downlink overhead assumption |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. |
| Traffic load (Resource utilization) |  | - 50/70 % for CSI overhead reduction - 20/50 % for high rank extension Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and SRS overhead as baseline metrics. |
| Baseline for performance evaluation |  | Rel-15 Type II Codebook is the baseline for performance and overhead evaluation for overhead reduction. (Type I Codebook can be considered at least for performance evaluation) - Companies are encouraged to compare the proposed overhead reduction scheme with Rel-15 overhead reduction scheme, Rel-15 Type I Codebook is the baseline for performance and overhead evaluation for higher rank codebook. |