3GPP TSG RAN WG1 #109-e															R1-2204543

e-meeting, May  9th – 20th May, 2022

Agenda item:		9.1.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Precoder Indication for Multi-Panel UL Transmission

Document for:		Discussion and Decision

# 1	Introduction

In RAN#94, enhancements on UL simultaneous multi-panel transmission were approved to be considered and specified as part of the MIMO evolution for Downlink and Uplink WID [1]. The objectives for the UL multi-panel simultaneous enhancements are stated as follows [1]:

The work item aims to specify the enhancements identified for NR MIMO. The detailed RAN1 objectives are as follows: … - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. …

# 2	Discussion on UL Simultaneous Multi-Panel Transmission

Enhanced UL peak data rates can enable short-range applications such as home entertainment, video surveillance/monitoring in industrial/healthcare/ safety, IAB, and other applications where power/form-factor/cost with devices are not as stringent as in traditional handheld user equipment. To reduce difference between DL and UL spectral efficiencies in NR, specification support for simultaneous UL transmission with >4 Tx antenna ports can be seen as an attractive approach for both FR1 and FR2. In this contribution, different aspects related to simultaneous UL multi-panel transmission with >4TX antenna ports with legacy codebook for FR2 is discussed.

## 2.1 Motivation for Simultaneous UL Multi-Panel Transmission

To consider the feasibility of the uplink simultaneous multi-panel transmission, initial system level simulations were carried out to consider observed L1-RSRP difference between the best and second best panels. In the results the best possible TX and RX beam pair between the TRP and UE panel is considered. The simulation assumptions are provided in Annex A. Figure 1 illustrates the CDF of L1-RSRP difference between the best and nth best panel (where n={2,3,4}). It can be observed that L1-RSRP difference between the best and second best panel is within 5 dB with probability of 0.85. That means that in the analysed scenario it’s common that UE would have two panels almost with similar path loss towards the TRPs available simultaneously. It’s to be noted that actual panels that are within 2 best panels are changing over time.

Figure 1 CDF of L1-RSRP difference between the best and nth best panel where n = {2,3,4}.

Observation 1: In the analyzed scenario it’s common that UE would have two panels almost with similar path loss towards the TRPs available simultaneously.

Next, we analyse the potential throughput gain in link simulator with the following setup:

- UE with two Tx panels (in opposite directions) and one TRP with two Rx panels, see Figure 2

- Rank-2 transmission, same (rank-2) data transmitted through both Tx antenna panels

- Simultaneous transmission with both Tx panels, Ptx_total=Ppanel or 2xPpanel

- ’Tx panel selection’, i.e., selecting best single Tx panel (out of two panels) for transmission, Ptx_total = Ppanel

- Link throughput results averaged over different UE antenna rotations (in azimuth domain)

Figure 2 Setup of link simulations.

The link simulation analysis shows the potential of the throughput increase the simultaneous multi-panel UL transmission can provide (black curves represent simultaneous transmission with different transmit power assumptions and red dashed line single panel transmission). The results indicate that the higher the operating SNR region the higher the gains are from multi-panel transmission.

Multi-panel simultaneous UL transmission in Rel-18 focuses on CPE/FWA/vehicle/industrial devices that are having higher transmission power levels than handheld UEs/devices. That further increases the feasibility of the simultaneous multi-panel transmission. However, the current description of target devices is lacking the technical assumptions with respect to: power class definition, amount of panels and their ability to operate simultaneously (including the assumption of per-panel PA), panel switching latency, antenna ports coherency between panels, just to name a few technical characteristics. Indeed, one may consider the high-end device which is able to transmit simultaneously with full power from multiple panels, have zero switching latency between panels, if for example the device has more than panels and keeps 2 active panels simultaneously.

The 38.101-2 RAN4 specification is defining the UE maximum output power for various UE types in FR2, as listed in Table 1below.

Table 1 38.101-2 assumptions of UE types.

Table 1: 38.101-2 assumption of UE Types (Table 6.2.1.0-1:)

| UE Power class | UE type |
| --- | --- |
| 1 | Fixed wireless access (FWA) UE |
| 2 | Vehicular UE |
| 3 | Handheld UE |
| 4 | High power non-handheld UE |
| 5 | Fixed wireless access (FWA) UE |
| 6 | High Speed Train Roof-Mounted UE |
| 7 | RedCap UE |

Power class 3 (hence handheld UEs) is the typical/default power class which was assumed in legacy RAN1 designs. However, the Rel18 WID scope is opening the discussion on additional UE power classes and it should be clarified if the assumptions of UE power class 3 are in the scope of Rel18 or not.

Proposal 1: RAN1 should first and foremost clarify the assumptions under which the simultaneous multi-panel UL transmission are going to be considered in Rel-18.

Proposal 2: RAN1 should refer in the upcoming discussion to the UE power classes defined by RAN4 rather than to the UE types.

## 2.2	Current Specification Status for Simultaneous UL Multi-Panel Transmission

According to [TS 38.306] UE indicates with higher layer parameter, maxNumberSRS-Ports-PerResource, its capability for the maximum number of UL SRS antenna port per each SRS resource. Moreover, the UE also indicates via higher layer parameter, maxNumberMIMO-LayersCB-PUSCH, its capability for the maximum number of MIMO layers for PUSCH with codebook based transmission. Furthermore, the UE indicates its PUSCH TX coherence capability via higher layer parameter, pusch-TransCoherence, which defines support of the uplink codebook subset by the UE for UL precoding for PUSCH transmission as described in clause 6.1.1.1 of TS 38.214 [12]. Depending on UE’s transmission coherency capability, the UE can support one of the following three capabilities, i.e. fullyandPartiaAndNoncoherent or partialAndNonCoherent or nonCoherent. In other words, in current specification, reported single pusch-TransCoherence determines phase coherence restriction for different TPMI hypothesis as well as restriction for TPMI indication associated with all PUSCH transmissions. We note that these existing coherency definitions are related to the antenna ports of a single panel.

Observation 2: Existing RRC based capability signaling defines only single PUSCH-TransCoherence value to be applied for all UL resources associated with a single panel PUSCH transmission.

Rel-15 UL codebooks provide support for uplink transmission using 1 or 2 or 4 antenna ports with 1, 2, 3 or 4 layers. It is worth noting that single antenna port transmission is a special case of UL codebook transmission with 2 and 4 APs. Depending on the reported pusch-TransCoherence capability, the network may restrict the number of hypotheses per resource with specific antenna port configuration for UL SRS resource.

Observation 3: Legacy UL codebooks can provide support for up to 4 APs and 4 layers.

Figure 3 provides an example of Rel-17 capability set index value reporting with two TRPs and a device equipped with four TX antenna panels  with different number of antenna ports. The device can be configured with DL SSBs or NZP-CSI-RS resources to perform DL CSI measurements (i.e. L1-RSRP or L1-SINR) enabling Rel-17 capability set index reporting. Furthermore, the device is configured with NZP-CSI-RS resource sets #1 and #2 with corresponding resources (e.g. CRI#23, CRI#14, etc.). Moreover, the device is pre-configured with four UL SRS resource sets with usage of beam management, i.e. SRS-set#3, SRS-set#5, SRS-set#7, and SRS resource-set#9. Each of these resource sets have been configured with one DL RS resource, i.e. DL NZP-CSI-RS, as spatial relation for UL transmission. Furthermore, it is assumed that the device has reported during capability reporting signaling phase, different capability value set indices, i.e. capability-set index#0= 2 APs SRS, capability-set index#1= 1AP SRS, capability-set index#2 = 2 APs SRS, capability-set index#3 = 2 APs SRS.

According to Rel-17 specification, the device shall measure DL SSB/NZP-CSI-RS resources and report up to four-best SSB/NZP-CSI-RS resources associated with UL TX capability set index. For example, reported single CSI reporting instance consist of the following values, N=2: CRI#23, CRI#43, RSRP(#23), RSRP(#43), c-value-set-ind#2, c-value-set-ind#3. As a result of the CSI reporting, the network may have awareness how many UL TX antenna ports are associated with reported DL RS resources that used as spatial source for preconfigured UL SRS resources. By using capability index reporting, the network can configure UL SRS resource set with resources with usage ‘codebook’ associated with single antenna panel with corresponding antenna ports for single TRP transmission In other words, according to current Rel-17 specification, the device can be configured with one UL SRS resource set with usage ‘codebook’ with two resources with 2-AP, with CRI#23, CRI#43 as spatial source for UL transmission, respectively. As a result of this, the network can perform TRP specific TPMI hypotheses for precoder and rank selection antenna panel specifically for PUSCH transmissions by using legacy uplink codebooks.

Observation 4: Rel-17 L1-based CSI capability index reporting provides support only for antenna panel specific UL codebook based PUSCH transmission for each TRP.

Figure 3 An example of Rel-17 capability index reporting.

Rel-17 provides support for multi-TRP TDM PUSCH repetition operations/schemes with codebook and non-codebook based operation. By using Rel-17 specification, Rel-17 M-TRP PUSCH repetition operation, two SRS resource sets are configured (each of resource corresponds to a TRP), and a second SRI field is added to UL DCI when the number of SRS resource sets when the value of associated SRS resource set indicator is greater than one. The same number of SRS resources for the two SRS resource sets is assumed. Also, the same number of layers is assumed for all the PUSCH (TDMed) repetitions. In other words, both S-DCI and M-DCI options for precoder indication are supported for M-TRP operation with single antenna panel transmission in Rel-17.

Observation 5: Rel-17 provides support for codebook based TDM PUSCH repetition with two different TRPs where up to two different TPMIs and SRIs are associated with two TRPs.-

## 2.3	Rel-18 Assumptions for Simultaneous Multi-Panel Codebook based UL TX

By following Rel-18 WID, the UE may be equipped with multiple TX antenna panels for operation with FR2 where up to two antenna panels are used for simultaneous codebook based PUSCH transmission up to two TRPs. In other words, the UE can have multiple TX antenna panel pairs, where each pair of antenna panels is used for PUSCH transmission up to two TRPs. It is also worth noting that legacy codebook, i.e. up to 4 AP and 4 layers, is assumed to be used. Furthermore, the WID says that the total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

As discussed earlier, in legacy operation, single PUSCH-TransCoherence capability defines restrictions for TPMI selection and indication. As one form of operation, the two panels may operate in an independent manner, hence in a non-coherent way. In this case, the layers can be also split across the panels in various combinations and per panel number-of-layer adaptation would be a first solution to consider.Another way of operating two panels poses the question of phase coherence between the antenna ports of the two panels, with a target of coherent transmission across the two panels. In Rel-18, due to potential different vendor specific implementation aspects, different antenna panel pairs may also have different phase coherency assumptions. More specifically, to enable utilization of phase coherency information with different antenna panel pairs, it might be beneficial to investigate the phase coherency between antenna ports within the pair of antenna panels. It is worth noting that phase coherency between antenna ports can be seen as a super-set for antenna panel (i.e. coherency between antenna ports within antenna panel) specific phase coherency. Otherwise, if not defined, it is difficult to use different UEs with different multi-panel transmission capabilities in full extend for simultaneous UL codebook based multi-panel transmission.

Observation 6: The phase coherency between APs within the pair of antenna panels is an enabler for coherent transmission across multiple panels.

Observation 7: Phase coherency between APs within pair of antenna panels works as super-set for antenna panel specific phase coherency (i.e. coherency between APs within antenna panel ).

Figure 4 shows an example of potential antenna panel pair combinations with different phase coherency assumptions for codebook based simultaneous UL multi-panel transmission. It is worth noting that all possible combinations are not shown. For example, antenna panel pair#3 can consist of antenna panels #2 and #3 with 2-APs in each panel resulting in 4 APs in total.

Figure 4 An example of different antenna panel pair combinations for codebook based simultaneous multi-panel UL TX.

Observation 8: Rel-18 device  can be equipped with multiple antenna panel pairs for simultaneous UL TX.

Table 2 summaries different antenna panel pair combinations with different panel specific number of antenna ports. As can be seen, only two antenna panel pair combinations can be used for simultaneous UL multi-panel transmission (marked with bold font). Other combinations lead to single panel UL transmission. To configure up to two layers simultaneous transmission across multiple antenna panels, each antenna panel within the antenna panel pair needs to have 1-AP SRS capability. Similarly, to configure up to 4 layers simultaneous transmission across multiple antenna panels, each antenna panel within the antenna panel pair needs to have 2-AP SRS capability.

Table 2 Possible antenna port combinations for a pair of antenna panels with legacy codebook (simultaneous multi-panel options marked with bold font).

| Total #APs w/ legacy codebook | Total number of layers | #APs per Antenna Panel#x | #APs per Antenna Panel#y |
| --- | --- | --- | --- |
| 1 | 1 | 1 | 0 |
| 1 | 1 | 0 | 1 |
| 2 | 1,2 | 1 | 1 |
| 2 | 1,2 | 2 | 0 |
| 2 | 1,2 | 0 | 2 |
| 4 | 1,2,3,4 | 4 | 0 |
| 4 | 1,2,3,4 | 0 | 4 |
| 4 | 1,2,3,4 | 2 | 2 |

Observation 9: To configure up to 2 layer transmission across antenna panel pair, each antenna panel within the antenna panel pair needs to have 1-AP SRS capability.

Observation 10: To configure up to 4 layer transmission across antenna panel pair, each antenna panel within the antenna panel pair needs to have 2-AP SRS capability.

Table 3 shows a summary of antenna port combinations and their coherency assumptions for antenna ports between antenna panels and coherency between antenna ports within antenna panel.

Table 3 Possible antenna port combinations for a pair of antenna panels with legacy codebook with different coherency assumptions (simultaneous multi-panel options marked with bolding).

| Total #APs w/ legacy codebook | Total #layers | #APs per Antenna Panel#x | #APs per Antenna Panel#y | Coherency between antenna ports within antenna panel pair | Coherency between antenna ports within antenna panel |
| --- | --- | --- | --- | --- | --- |
| 2 | 1,2 | 1 | 1 | non-coherent/fully coherent | non-coherent |
| 4 | 1,2,3,4 | 2 | 2 | non-coherent/partial-coherent/fully coherent | non-coherent/ /fully coherent |

4 summarizes different legacy codebook options for simultaneous multi-panel UL TX case with coherency between APs within pair of antenna panels with TPMI options-

Table 4 legacy codebook options for simultaneous multi-panel TX with coherence between APs within pair of antenna panels.

| #total layers | codebook option | TPMIs |
| --- | --- | --- |
| 1 | single layer transmission using 2-APs (TS 38.211, Table 6.3.1.5-1) | 2-5 |
| 1 | single layer transmission using 4-APs (TS 38.211,Table 6.3.1.5-2) | 4-27 |
| 2 | two layer transmission using 2-APs (TS 38.211, Table 6.3.1.5-4) | 0-2 |
| 2 | transmission using 4-APs (TS 38.211, Table 6.3.1.5-5), | 0-21 |
| 3 | three layer transmission using 4-APs (TS 38.211, Table 6.3.1.5-6) | 1-6 Note:TPMI 0 exluded because only 3APs simultaneously used |
| 4 | four layer transmission using 4-APs (TS 38.211, Table 6.3.1.5-7), | 0-4 |

Observation 11: Legacy codebook with coherence between APs with pair of antenna panels enables the use of layers 1-4 and up to six different legacy codebook options.

Table 5 legacy codebook options for simultaneous multi-panel TX with coherence between APs within antenna panel.

| #total layers | codebook option | TPMI for antenna panel #x | TPMI for antenna panel #y |
| --- | --- | --- | --- |
| 2 (restriction 1 layer per TRP) | single layer transmission using 2-APs (TS 38.211, Table 6.3.1.5-1) | 2-5 | 2-5 |
| 4 (restriction 2 layers per TRP) | two layer transmission using 2-APs (TS 38.211, Table 6.3.1.5-4) | 0-2 | 0-2 |

Observation 12: Legacy codebook with coherence between APs within antenna panel enables to use the number of layers up to 4 (2 per antenna panel) and use of 2 different legacy codebook options.

From system operation perspective, it would be beneficial to consider the usage of different coherency assumptions in the context different use cases (i.e. single TRP and multi-TRP).

Proposal 3: Consider usage of different coherency assumptions in the context different use cases (i.e. single TRP and multi-TRP).

## 2.4	Key Components for Rel-18 Codebook based Simultaneous Multi-Panel UL TX

Based on previous discussion, the following list of key components may need further enhancements in Rel-18 for enabling efficient simultaneous UL multi-panel codebook based PUSCH transmission with up to two TRPs

- Capability signalling

- L1-based capability index reporting

- single DCI and multi-DCI enhancements for the indication of TPMI, SRI with legacy codebook

- CSI reporting

### 2.4.1		Capability Signalling Enhancements

As discussed earlier, current Rel-17 specification does not provide support for the use of different phase coherence values (or different combinations) associated with different antenna panels. Therefore, it is not possible to use efficiently different UL TX antenna panels with different capabilities as well as possible antenna panel pair combinations for simultaneous multi-panel transmission. To enable mutual understanding between network and UE about simultaneous multi-panel transmission capabilities, it would be beneficial to study different capability signaling enhancements for different antenna panel pair coherency assumptions.

Proposal 4: Study different capability signaling mechanisms to indicate different coherency assumptions for different antenna panel combinations.

### 2.4.2		CSI Reporting Enhancements

To enable mutual understanding between network and UE about simultaneous multi-panel transmission capability, from a network perspective, it would be beneficial to enhance Rel-17 L1-based reporting to cover also simultaneous UL multi-panel aspects including phase coherency assumption between different UL antenna ports associated with multiple antenna panels

Observation 13: It is beneficial to enable dynamic reporting of coherency assumption between antenna panels to indicate coherency for simultaneous antenna panel candidates at the time.

UL SRS/PUSCH resources limiting to its use to single indicated TX coherency value for all applied for all UL resources. Therefore, it is not possible to use efficiently different UL TX antenna panels with different capabilities as well as possible antenna panel combinations for simultaneous multi-panel transmission with codebook based precoding (only single panel transmission at the time supported).

Proposal 5: Study different L1-capability index reporting enhancements to indicate different coherency assumptions for different antenna panel combinations.

### 2.4.3		UL Precoder Indication for single-DCI & multi-DCI

As indicated in the WID, it is required to study how the precoder indication for PUSCH works for simultaneous UL transmission, and such discussion should address both s-DCI and m-DCI mTRP schemes. As discussed in section 2.2.2 of [2], there can be different Rel-18 candidate uplink transmission schemes or modes for simultaneous uplink transmission (in time and frequency). In detail, the simultaneous UL transmission modes can categorize as below,

PUSCH SDM and FDM schemes:

- Single-DCI mode where one DCI schedules two simultaneous PUSCH transmission/repetition occasions:

- Time and frequency domains overlap:

  - PUSCH FDM schemes would consist of two PUSCH transmission/repetition occasions for which the resources are fully overlapping in time and non-overlapping in frequency

  - PUSCH SDM schemes would consist of two PUSCH transmission/repetition occasions for which the resources are fully overlapping in time and frequency.

- Single-DCI PUSCH FDM/SDM Scheme 1: one PUSCH with two PUSCH transmission occasions would be transmitted simultaneously; for FDM, the two PUSCH transmission occasions would carry coded bits of a same codeword which would be mapped across the two PUSCH allocations; for SDM, each PUSCH transmission occasion would carry a different layer(s) of a same codeword.

  - This is a single codeword transmission, where SDM scheme may assume at least 2 layers and FDM may work with any number of layers.

- Single-DCI PUSCH FDM/SDM Scheme 2: one codeword with one RV would be transmitted in each PUSCH transmission/repetition occasion in FDM or SDM manner. RVs corresponding to the two occasions can be the same or different.

  - This is TB repetition mode, where each repetition (codeword) may consider equal number of layers   for both SDM and FDM modes.

- Single-DCI PUSCH SDM Scheme 3: same layer(s) of a same codeword would be transmitted using both PUSCH transmission occasions (from different UE panels) where different degrees of coherencies may be assumed.

- Multi-DCI mode where two DCIs schedule two simultaneous PUSCH transmission occasions:

- Time and frequency domains overlap:

  - The two PUSCH transmissions for which the resources may be fully/partially overlapping in time and fully/partially/non-overlapping in frequency.

- Multi-DCI PUSCH FDM/SDM Scheme: two TBs each of which would be carried in one PUSCH transmission.

We think some discussion on the above modes for simultaneous transmissions may be needed either in AI 9.1.1.1 or AI 9.1.4.1. It is required to define these simultaneous UL transmission schemes, and we have the following proposals.

Proposal 6: RAN1 to define the Rel-18 PUSCH FDM/SDM schemes for single-DCI mode, where one DCI schedules two simultaneous PUSCH transmission/repetition occasions, from the following:

- Single-DCI PUSCH FDM/SDM Scheme 1: one PUSCH with two PUSCH transmission occasions would be transmitted simultaneously.

  - for FDM, the two PUSCH transmission occasions would carry coded bits of a same codeword which would be mapped across the two PUSCH allocations.

  - for SDM, each PUSCH transmission occasion would carry a different layer(s) of a same codeword.

- Single-DCI PUSCH FDM/SDM Scheme 2: one codeword with one RV would be transmitted in each of the two PUSCH transmission/repetition occasions in FDM or SDM manner, where RVs corresponding to the two occasions can be the same or different.

- Single-DCI PUSCH SDM Scheme 3: same layer(s) of a same codeword would be transmitted using both PUSCH transmission occasions (from different UE panels) where different degrees of coherencies may be assumed.

Proposal 7: For multi-DCI, PUSCH simultaneous UL transmission schemes shall consider all possibilities of overlaps among separately scheduled PUSCH transmissions (i.e., fully/partially in time and fully/partially/non-overlapping in frequency).

For single-DCI PUSCH FDM/SDM Scheme 1, both FDM and SDM modes use a single codeword. Therefore, it is sufficient to indicate a single TPMI to apply across two PUSCH transmission occasions. We see a similar situation in single-DCI PUSCH SDM Scheme 3, where intended transmission is mainly towards the same TRP, and a single codeword is transmitted across two panels.

Observation 14: When the simultaneous UL transmission occasions are from a single codeword, the UE is only required to be indicated with a single TPMI.

For single-DCI PUSCH FDM/SDM Scheme 2, the UE may use two different codewords (even with the same TB), and different precoders may be applied for each transmission. The framework we have here is similar to the Rel-17 PUSCH repetition (mTRP TDMed PUSCH) scenario, where DCI can indicate up to two SRIs and TPMIs, and each transmission occasion corresponds to one SRI/TPMI field. Also, Rel-17 also supports a dynamic switch between s-TRP and m-TRP PUSCH transmission modes, where the operation is controlled by the SRS resource indicator field (2 bits). From our view, it may be easier to follow the same framework as the mTRP TDMed PUSCH scenario to indicate the different SRIs/TPMIs for the UE.

Observation 15: When the simultaneous two UL transmissions are corresponding to two codewords, the UE is required to be indicated with two TPMIs.

Observation 16: It is feasible to reuse DCI fields (i.e., second SRI/second TPMI/SRS resource set indicator) of Rel-17 mTRP PUSCH repetition also for Rel-18 mTRP PUSCH simultaneous transmission.

Proposal 8: For precoder indication of single-DCI PUSCH FDM/SDM schemes, reuse the DCI fields (i.e., second SRI/second TPMI/SRS resource set indicator) introduced in Rel-17 mTRP PUSCH repetition schemes.

For multi-DCI schemes, the discussion is easier than the single-DCI scenario as two different PUSCHs are scheduled by PDCCHs corresponding to different CORESETPoolIndex. As each DCI can indicate corresponding TPMI that shall be used for corresponding PUSCH, it may not require any enhancement from legacy (Rel-15/16) methods of indicating one TPMI.

Proposal 9: For precoder indication of multi-DCI PUSCH FDM/SDM schemes, no additional enhancement is required compared to legacy (Rel-15/16) TPMI indication.

# 3	Conclusions

In the previous sections, the following observations and proposal have been made:

Observation1: In the analyzed scenario it’s common that UE would have two panels almost with similar path loss towards the TRPs available simultaneously.

Observation 2: Existing RRC based capability signaling defines only single PUSCH-TransCoherence value to be applied for all UL resources associated with a single panel PUSCH transmission.

Observation 3: Legacy UL codebooks can provide support for up to 4 APs and 4 layers

Observation 4: Rel-17 L1-based CSI capability index reporting provides support only for antenna panel specific UL codebook based PUSCH transmission for each TRP.

Observation 5: Rel-17 provides support for codebook based TDM PUSCH repetition with two different TRPs where up to two different TPMIs and SRIs are associated with two TRPs-

Observation 6: The phase coherency between APs within the pair of antenna panels is an enabler for coherent

Observation 7: Phase coherency between APs within pair of antenna panels works as super-set for antenna panel specific phase coherency (i.e. coherency between APs within antenna panel ).

Observation 8: Rel-18 UE can be equipped with multiple antenna panel pairs for simultaneous UL TX.

Observation 9: To configure up to 2 layer transmission across antenna panel pair, each antenna panel within the antenna panel pair needs to have 1-AP SRS capability.

Observation 10: To configure up to 4 layer transmission across antenna panel pair, each antenna panel within the antenna panel pair needs to have 2-AP SRS capability.

Observation 11: Legacy codebook with coherence between APs with pair of antenna panels enables the use of layers 1-4 and up to six different legacy codebook options.

Observation12: Legacy codebook with coherence between APs within antenna panel enables to use the number of layers up to 4 (2 per antenna panel) and use of 2 different legacy codebook options .

Observation 13: It is beneficial to enable dynamic reporting of coherency assumption between antenna panels to indicate coherency for simultaneous antenna panel candidates at the time.

Observation 14: When the simultaneous UL transmission occasions are from a single codeword, the UE is only required to be indicated with a single TPMI.

Observation 15: When the simultaneous two UL transmissions are corresponding to two codewords, the UE is required to be indicated with two TPMIs.

Observation 16: It is feasible to reuse DCI fields (i.e., second SRI/second TPMI/SRS resource set indicator) of Rel-17 mTRP PUSCH repetition also for Rel-18 mTRP PUSCH simultaneous transmission.

Proposal 1: RAN1 should first and foremost clarify the assumptions under which the simultaneous multi-panel UL transmission are going to be considered in Rel-18.

Proposal 2: RAN1 should refer in the upcoming discussion to the UE power classes defined by RAN4 rather than to the UE types.

Proposal 3: Consider usage of different coherency assumptions in the context different use cases (i.e. single TRP and multi-TRP).

Proposal 4: Study different capability signaling mechanisms to indicate different coherency assumptions for different antenna panel combinations.

Proposal 5: Study different L1-capability index reporting enhancements to indicate different coherency assumptions for different antenna panel combinations.

Proposal 6: RAN1 to define the Rel-18 PUSCH FDM/SDM schemes for single-DCI mode, where one DCI schedules two simultaneous PUSCH transmission/repetition occasions, from the following:

- Single-DCI PUSCH FDM/SDM Scheme 1: one PUSCH with two PUSCH transmission occasions would be transmitted simultaneously.

  - for FDM, the two PUSCH transmission occasions would carry coded bits of a same codeword which would be mapped across the two PUSCH allocations.

  - for SDM, each PUSCH transmission occasion would carry a different layer(s) of a same codeword.

- Single-DCI PUSCH FDM/SDM Scheme 2: one codeword with one RV would be transmitted in each of the two PUSCH transmission/repetition occasions in FDM or SDM manner, where RVs corresponding to the two occasions can be the same or different.

- Single-DCI PUSCH SDM Scheme 3: same layer(s) of a same codeword would be transmitted using both PUSCH transmission occasions (from different UE panels) where different degrees of coherencies may be assumed.

Proposal 7: For multi-DCI, PUSCH simultaneous UL transmission schemes shall consider all possibilities of overlaps among separately scheduled PUSCH transmissions (i.e., fully/partially in time and fully/partially/non-overlapping in frequency).

Proposal8: For precoder indication of multi-DCI PUSCH FDM/SDM schemes, no additional enhancement is required compared to legacy (Rel-15/16) TPMI indication.

Proposal 9: For precoder indication of multi-DCI PUSCH FDM/SDM schemes, no additional enhancement is required compared to legacy (Rel-15/16) TPMI indication.

# 4  References

- RP-213517, “MIMO Evolution for Downlink and Uplink”, Samsung.

- R1-2204538, “Unified TCI framework extension for multi-TRP”, Nokia, NSB