3GPP TSG RAN WG1 Meeting #109-e		R1-2204544

e-Meeting, May 9th – 20th, 2022

Agenda item:       9.1.4.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	UL enhancements for enabling 8Tx UL transmission

Document for:     Discussion and Decision

# 1	Introduction

In RAN#94-e two new objectives were agreed related for UL MIMO enhancements in Rel-18 NR [1], to address SRI/TPMI enhancements for enabling 8Tx UL transmission:

5.	 Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

- Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

  - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission

    - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

  - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation

    - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

In this contribution, we are addressing the Objective 5 on the uplink enhancements to enable 8Tx UL transmission.

# 2	Codebook based transmission to support UL 8Tx

## 2.1	New NR 8Tx codebook

Based on the WID, 8Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices shall be under the Rel-18 study, and if justified, for specifying. Since there is no 8Tx UL codebook in NR, the first step shall consider introducing 8Tx codebook for NR uplink.

NR has already defined 8Tx codebook for downlink transmission. Naturally, we may consider using DL 8Tx codebook to support uplink transmission. Compared to the NR uplink codebooks up to 4Tx, there is a clear difference: 2Tx and 4Tx NR uplink codebooks are single-stage codebook, while the 8Tx DL codebook is a 2-stage codebook. We may need to study the downlink two-stage codebook for possible application for uplink transmission.

Given the precoding is defined with

y=Wx,

where precoded signal y is a P×1 vector with P antenna ports, x is a rank-v data symbol vector with size of v×1, and the matrix W of size P×v is the precoder related with P antenna ports and rank v. For Rel-15 NR, uplink codebooks are single-stage codebooks which define codewords W. With the two-stage codebook design, the precoder W has the form

W=W1W2

where W1 contains the wideband component of the precoder and W2 contains the sub-band specific components of the precoder.

For type-I codebook for NR downlink MIMO, there are some general assumptions:

- The codebook design is based on 2-D DFT beam grid with oversampling.

- Each 2-D DFT beam in the type-I codebook has two components corresponding to an azimuth and elevation direction with a 2-D antenna array.

- Cross-polarized antennas are assumed.

The downlink Type-I codebook is a good reference for 8Tx codebook design. However, some of the DL codebook design assumptions are not necessarily true for UE’s implementation. Because of restricted form factor at the UE, even for CPE/FWA/vehicle/Industrial devices, half-lambda antenna spacing usually would be hard to maintain for UE implementation, if not impossible, especially for FR1 with lower carrier frequency where radio wavelength is larger. We may need to address the potential impact of an almost arbitrary placement of the UE’s antenna elements.

Proposal 1: Study NR DL 8Tx codebooks for uplink transmission, with consideration of uplink antenna implementations for CPE/FWA/vehicle/Industrial devices.

Following 2-D DFT beam grid, Type-I single beam codebook with rank 1 has its choice for W1 and W2 as

W1(1)i11,i12=vi11,i1200vi11,i12

W2(1)i2=1P1φ1(1)i2

where φ1(1)∈1,j,-1,-j, and vi11,i12 is defined as a DFT matrix. Note that a single beam is selected at wideband, and co-phasing across polarizations is performed per subband.

One 2-D DFT beam has an azimuth component and an elevation component, as

tl=1, ej2πlO1N1, ⋯, ej2πlN1-1O1N1T

um=1, ej2πmO2N2, ⋯, ej2πmN2-1O2N2T

Each 2-D beam is defined as

vl,m=tl⊗um

The antenna port configuration per polarization is given with (N1, N2), and oversampling factors O1 and O2 for two components, respectively. For Type-I codebook, these configurations are used:

Table 1    Type-I codebook configuration: 4Tx and 8Tx

| # of CSI-RS Ports | (N1,N2) | (O1,O2) |
| --- | --- | --- |
| 4 | (2, 1) | (4, 1) |
| 8 | (2, 2) | (4, 4) |
|  | (4, 1) | (4, 1) |

There are two (N1,N2) configurations for Type-I 8Tx codebook, as (2, 2) and (4, 1). The configuration of (2, 2) provides better elevation capability. For uplink codebook, we may start with configuration (4, 1) because UE’s antenna location might be arbitrary due to form factor limitation.

Proposal 2: Consider 8Tx codebook design with azimuth/elevation antenna port configuration N1,N2=(4,1) of Type-I 8Tx codebook.

Type I Multi-panel codebook

NR downlink supports Type-I multi-panel codebook. When UE is using more than one panel for 8Tx transmission, the multi-panel codebook shall be used. However, in the WID under Objective 6, it is indicated that:

“UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission”

This statement seems to indicate that multi-panel codebook is out of the scope for Object 6. However, it is not quite clear when 8Tx codebook is introduced, whether the 8Tx codebook can be applied for multi-panel operation under Objective 5. Therefore, RAN1 needs to discuss whether 8Tx multi-panel codebook is within the Rel-18 scope.

Proposal 3: RAN1 shall discuss on whether 8Tx multi-panel codebook should be studied in Rel-18 under Objective 5.

The Type-I multi-panel 8Tx codebook has this antenna configuration as shown in Table 2.

Table 2    Type-I multi-panel codebook configuration: 8Tx

| # of CSI-RS Ports | (Ng, N1,N2) | (O1,O2) |
| --- | --- | --- |
| 8 | (2, 2, 1) | (4, 1) |

This codebook is based on the Type-I 4Tx codebook with co-phasing across panels (2 panels). The antenna placement with two panels can be illustrated as

Single beam codebook shall be used and identical beams can be chose for each panel.

## 2.2	Support of frequency selective precoding for 8Tx

Similar to the DL, frequency selective precoding in uplink would provide better throughput gain for 8Tx. When two-stage codebook is introduced, the subband component W2 of the precoding matrix provides frequency selective precoding, which has potential to provide better gain than single-stage codebook, where a wideband precoding matrix is applied.

During NR Rel-15 specification phase, there were efforts to evaluate frequency selective precoding for the uplink. The evaluation efforts included simulation campaign for system-level performance. Following the similar evaluation path, we should study uplink performance for 8Tx codebook, together with its frequency selective precoding performance. Therefore,

Proposal 4: RAN1 shall study system level performance for 8Tx codebook design with frequency selective precoding.

For system-level performance, RAN1 needs to agree on simulation assumptions. One possible set of simulation assumptions is listed in Table 3

Table 3    System-level Simulation Assumptions

| Parameters | Values |
| --- | --- |
| Scenario | Urban macro |
| Carrier freq. | 4 GHz |
| System Bandwidth | 100MHz |
| Sub-band bandwidth | 10MHz (10 sub-bands over the system bandwidth), and 5MHz (20 sub-bands over system bandwidth), or other alternatives |
| UE Tx power | 23 dBm over 100MHz |
| Channel model | According to 38.901 |
| TRP antenna config. | (M,N,P,Mg,Ng) = (8,4,2,1,1), (dV,dH) = (0.5, 0.5) λ |
| BS TXRU mapping | Single TXRU per antenna element. |
| Down-tilt | 90 degrees |
| UE antenna config. | 8 Transmit antennas = Omni UE with (M,N,P)=(1,4,2), 8 TXRUs |
| UE # | 5 users per TRP (average) |
| UE distribution | 20% outdoor (3km/h), 80% indoor (3km/h) |
| Traffic model | Full buffer and bursty traffic |
| CSI | CSI either wideband or sub-band based. PMI: 8TX codebook |
| MIMO mode | SU-MIMO with Rank adaptation |

Proposal 5: RAN1 shall agree on simulation assumptions for system-level evaluation for 8Tx performance.

### 2.2.1	Signalling aspect for sub-band feedback

To support frequency selective precoding, gNB shall provide UE specific uplink precoder information for all related sub-bands, as well as wideband. Such feedback information can be signalled to UE via UL grant in DCI or other downlink control channel.

If downlink UL grant in DCI is used, there might have overhead problem. For two-stage (W1, W2) UL codebook, the indication of both W1 and W2 shall be signalled to the UE. W1 indication is usually a wideband parameter, and the indication of W2 is sub-band based. The overhead of signalling W2 is high due to various sub-bands. Allocation of a large portion of DCI payload for W2 signalling has a significant overhead problem for precoder feedback.

Observation 1: The overhead of signalling subband component W2 is high.

If the sub-band size is fixed, the signalling overhead is a function of UE PRB allocation size. A larger UE PRB allocation needs a larger signalling overhead. If the DCI size is designed to cover the full system bandwidth, it won’t be efficient because some UE won’t need its sub-band precoder information at its un-allocated sub-bands. If gNB only indicates the precoder information over the allocated PRB, the UE grant payload size will be variable, which is undesirable for DCI design.

One solution to reduce the overhead of sub-band precoder feedback is the introduction of 2-step DCI process. At the first step, wideband precoder information is signalled to the UE. Depending on the reception of Step-1 DCI, the second step DCI might be needed to provide sub-band precoder information. The DCI size of Step-1 can be short; and Step-2 provides lengthier precoder information with sub-band specifics. This type of design provides some flexibility on DCI efficiency with Step-1. However, the step-2 DCI still have the problem of variable size or inefficient indication of precoder information. Besides, two-step DCI will certainly increase uplink latency.

A simple approach for single-step DCI design can have a common DCI format for sub-band precoder indication with different UL resource allocation sizes. Since the DCI format is common, the DCI size is fixed.

Assume that the overall DCI payload size for sub-band precoding in the UL grant is N bits. The size of precoder indication per sub-band is m bits/sub-band. The number of PRBs allocated to a specific UE is M. If K is the number of sub-bands that the M PRBs span, we have N=m∙K. If M PRBs comprise J sub-bands, then, K=MJ (assuming M is a multiple of J). Therefore, we have

N=m∙MJ

We may fix the DCI payload size N and distribute these bits for precoder indication differently depending on the UL PRB allocation size M. Thus, for different PRB allocation sizes M, either m or J or both can be varied to keep N the same.

With a given precoder indication size per sub-band of m bits/sub-band, the number of sub-bands is determined as K=Nm. If the allocation size is M PRBs, each sub-band will have J=MK PRBs (assuming M is a multiple of K). This approach fixes the number of sub-bands to K, regardless of the PRB allocation size. The sub-band size will be variable, depending on the allocated UE-specific PRB allocation size M. A larger PRB allocation M will result in a wider sub-band size.

Proposal 6: Consider a common DCI format for sub-band precoder indication for various UL resource allocation sizes.

# 3	Considerations with 8Tx UE coherence levels

Due to UE’s different RF chain implementation, coherence level cross multiple antenna ports could be different. Between two uplink Tx ports, UE may transmit coherently or non-coherently, depending on whether the signal phase between the two ports are calibrated. Rel-15 2Tx codebook has two coherence levels, to support full coherent and non-coherent transmission. For 4Tx, there are three levels of transmission coherence level: full coherent, partial coherent, and non-coherent.

With the introduction of 8Tx, there will be various combination of coherence levels. For example, there will have 2x4 where 2-coherent Tx ports as a pair, and non-coherent among the 4 pairs of ports. Alternatively, 4x2 where 4-coherent Tx ports as a group, and non-coherent between the two groups. The legacy approach of using partial coherence to indicate this case is no longer valid.

Observation 2:  The legacy approach of using partial coherence to indicate 8Tx coherence levels is no longer valid.

One possible approach is to group coherent antenna ports together.

Table 5    Coherence Group

| Tx ports | Rel-15 indications | Number of coherence group | Number of ports per coherence group |
| --- | --- | --- | --- |
| 2Tx | Coherent | 1 | 2 |
|  | Non-coherent | 2 | 1 |
| 4Tx | coherent | 1 | 4 |
|  | Partial-coherent | 2 | 2 |
|  | Non-coherent | 4 | 1 |
| 8Tx |  | 1 | 8 |
|  |  | 2 | 4 |
|  |  | 4 | 2 |
|  |  | 8 | 1 |

Proposal 7: Consider to group coherence antenna ports to support the indication of 8Tx coherence levels.

# 4	Conclusion

In this contribution, we are addressing the Objective 5 of the Rel-WID [1] on the uplink enhancements to enable 8Tx UL transmission. NR Type-I 8Tx codebook is reviewed with discussion on frequency selective precoding and subband based signalling. 8Tx coherence levels are discussed. We have these observations:

Observation 1: The overhead of signalling subband component W2 is high.

Observation 2:  The legacy approach of using partial coherence to indicate 8Tx coherence levels is no longer valid.

These are our proposals:

Proposal 1: Study NR DL 8Tx codebooks for uplink transmission, with consideration of uplink antenna implementations for CPE/FWA/vehicle/Industrial devices.

Proposal 2: Consider 8Tx codebook design with azimuth/elevation antenna port configuration N1,N2=(4,1) of Type-I 8Tx codebook.

Proposal 3: RAN1 shall discuss on whether 8Tx multi-panel codebook should be studied in Rel-18 under Objective 5.

Proposal 4: RAN1 shall study system level performance for 8Tx codebook design with frequency selective precoding.

Proposal 5: RAN1 shall agree on simulation assumptions for system-level evaluation for 8Tx performance.

Proposal 6: Consider a common DCI format for sub-band precoder indication for various UL resource allocation sizes.

Proposal 7: Consider to group coherence antenna ports to support the indication of 8Tx coherence levels.

# References

- RP-213598, “New WID: MIMO Evolution for Downlink and Uplink,” RAN#94e.