3GPP TSG RAN WG1 Meeting #109-e  								            R1-2204545

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.12.1

Source:	Thales

Title:	Discussion on NR NTN coverage enhancement

Document for:	Discussion and decision

# Introduction

In RAN95# e-meeting, a new Rel-18 work item defining enhancements for NG-RAN based Non-Terrestrial Networks was approved [1]. Among other objectives, this  new work item aims to introduce optimized performance and improve the NTN coverage especially when addressing handset terminals, including smartphones with more realistic assumptions on antenna gain instead of 0 dBi antenna gain usually considered for link budget analysis.

The detailed description of the WID objective on NR NTN coverage enhancement is recopied hereafter:

[RP-220953]: The Rel-18 NTN objectives are focused on the applicability of the solutions developed by general NR coverage enhancement to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage). The work needs to cover the use case of voice and low-data rate services using commercial smartphones with more realistic assumptions on antenna gains instead of 0dBi currently assumed for link budget analysis for non-terrestrial networks. The specific realistic antenna gain assumption will be determined at the working group level. The evaluation should also take into account any related regulatory requirements, e.g., ITU limitation of power flux density. Have a 1-TU 6-month study phase focusing on the following (to derive clear & limited scope): - Evaluate the coverage performance and identify the candidate physical radio channels that have coverage issues specific to NTN with following target services taking into account the studies in TR38.830 where appropriate, as well as general coverage enhancement techniques specified in Rel-18 [RAN1,RAN2,RAN4] - VoIP and low-data rate services for commercial handset terminals The following items are shown as examples of areas to consider in the next step of the study. The actual items for study will be based on the evaluation of coverage issues specific to NTN identified above. - NTN-specific repetitions enhancements beyond techniques covered in Rel-17 CovEnh WI for the relevant channels - NTN-specific techniques for improved diversity and/or reduced polarization loss - Improved performance of low-rate codecs in link budget limited situation including reducing RAN protocol overhead for VoNR - NOTE: Intent is not to introduce a new codec. RAN to determine by RAN#97 (for RAN1 items) and RAN#98 (for RAN2 items) whether the study phase has identified any need for NTN-specific coverage enhancements in Rel-18. If needed, the set of NTN-specific work item objectives will be updated.

In this contribution, we discuss the following aspects related to the coverage enhancements for NTN:

- The target scenarios/services and date rates,

- The evaluation approaches, the criteria and simulation parameters that should be used for coverage study,

- The link budget template and preliminary assessment of the coverage performance and identification coverage bottlenecks.

# Target scenarios and services requirements

## Target data rate

Handheld devices are identified as the UE type in several important use cases for NTN [2]. Typical frequency bands for handheld devices are S band and L band. Smartphones with their ubiquity are the most important handheld devices. Thereby, providing direct access connectivity to mass market smartphones through NTN will resolve the Zero G (0G) coverage problem in unserved or under-served geographical areas. Early support of mass market smartphones is also expected to foster speedy deployment of NR NTN networks.

Important service capabilities for mass market smartphones over NTN are:

- Voice over NR which requires a high reliability with a low rBLER and high mean opinion score to ensure good experience.

- (e)MBB services which require a data rate at least few Mbps/few hundreds of kbps (DL/UL) as per Table 7.4.2-1 in [3, TS 22.261] : for pedestrian user, the required data rate is 1 Mbit/s in the downlink and 100 kbit/s in the uplink.

For VoNR performance evaluation based on link-level simulation we propose to consider the Enhanced Voice Services (EVS) codec which is the current recommendation for voice over NR: The EVS 13.2 kbps can be used in this study which corresponds to data rate of 17.2 kbps at layer 1 or a packet size of 344 bits every 20ms as depicted in  Figure 1.

Figure 1 VoNR: EVS 13.2 kbps

The following Table provides the target services and target data rates for the coverage study:

Table 1 Target data rates

|  | Target data rate |  |
| --- | --- | --- |
| VoNR (EVS codec) | 13.2 kbps |  |
| MBB | DL: 1 Mbit/s | UL: 100 kbit/s |

Proposal 1:

The following target data rates are considered for MBB and VoNR performance evaluation:

- For VoNR,  a packet size of 344 bits every 20ms i.e. a data rate of 17.2 kbps

- For MBB, 1 Mbit/s in the DL and 100 kbit/s in the UL.

## Satellite parameters sets

The two sets of satellite parameters shown in Table 2 and Table 3 are considered as baseline for coverage performance evaluation and the performance target for coverage enhancement.

Proposal 2:

The satellite parameters used for the NTN coverage enhancement study are given by the following two tables:

Table 2 Set-1 satellite parameters

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture | S-band | 22 m | 2 m | 2 m |
| Satellite EIRP density |  | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter |  | 250 km | 90 km | 50 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture | S-band | 22 m | 2 m | 2 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |

Table 3 Set-2 satellite parameters

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture | S-band | 12 m | 1 m | 1 m |
| Satellite EIRP density |  | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |
| 3dB beamwidth |  | 0.7353 deg | 8.8320 deg | 8.8320 deg |
| Satellite beam diameter |  | 450 km | 190 km | 90 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture | S-band | 12 m | 1 m | 1 m |
| G/T |  | 14 dB K-1 | -4.9 dB K-1 | -4.9 dB K-1 |
| Satellite Rx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |

## UE characteristics

The UE characteristics to be considered in this study are given in Table 4. The UE is considered to be a commercial smartphones with more realistic assumptions on antenna gain instead of 0dBi currently assumed for link budget analysis for non-terrestrial networks. The specific realistic antenna gain assumption is yet to be determined at RAN1. As a baseline assumption on antenna gain we propose to consider  -5 dBi.

Proposal 3:

The UE characteristics used for the NTN coverage enhancement study are given by the following table:

Table 4 UE characteristics

| Frequency band | S band (i.e. 2 GHz) |
| --- | --- |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
| Polarisation | Linear: +/-45°X-pol |
| Rx Antenna gain | -5 dBi |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 23 dBm (200 mW) |
| Tx antenna gain | -5 dBi |

## List of coverage study cases

The  following study cases are considered as baseline for coverage performance evaluation and the performance target for coverage enhancement:

Proposal 4:

The coverage study cases used for the NTN coverage enhancement and targeting the smartphones UEs are given by the following table:

Table 5 List of coverage study cases

| Satellite orbit | Satellite parameter set | Beam elevation | Frequency Band |
| --- | --- | --- | --- |
| LEO-600 | Set 2 | 30 deg | S-band |
| LEO-1200 | Set 2 | 30 deg | S-band |
| GEO | Set 1 | 12.5 deg | S-band |

# Evaluation methodology

For NTN coverage enhancement study, the evaluation methodology can be based on link-level simulation but as system level simulation provides realistic results for coverage analysis we propose to conduct also system-level simulation for this study. We made the following proposal:

Proposal 5:

The following two options should be considered for the evaluation methodology:

Option 1: Based on link-level simulation:

- Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements.

- Obtain the baseline performance based on required SINR and link budget template.

- Identify target performance and coverage bottlenecks based on target performance metric.

Option 2: Based on link- level and system-level simulation

- Obtain the required SINR for the given target data rate based on link-level simulation.

- Obtain the target performance based on system-level simulation (i.e. the 5th percentile downlink or uplink SINR value in CDF curve).

# Link level Simulation assumptions

## Simulation assumptions for PRACH

The LLS parameters for PRACH performance evaluation are summarized within the following table:

Proposal 6:

For link level simulation, adopt the following parameters for PRACH:

Table 6 Simulation assumptions for PRACH

| Configurations | S-band |
| --- | --- |
| Carrier Frequency | 2 GHz |
| Channel Model | Baseline TDL/CDL-D model |
| Satellite Antenna | 1 Rx |
| Antenna Configuration at the UE | Omni-directional antenna with single linearly polarized antenna element |
| UE antenna gain | -5 dBi |
| UE speed | 3 km/h |
| PRACH format | PRACH format 2 and PRACH format B4 |
| Performance metric | 1% missed detection at 0.1% false alarm probability |

## Simulation assumptions for PUSCH and PUCCH

The parameters for PUSCH and PUCCH to be used for link level simulation are given within the following table:

Proposal 7:

For link level simulation, adopt the following parameters for PUSCH and PUCCH.

Table 7 Simulation assumptions for PUSCH and PUCCH

| Parameters | S-band |
| --- | --- |
| Carrier frequency | 2 GHz |
| Channel coding scheme | NR channel coding |
| Subcarrier spacing | 15 kHz, 30 kHz |
| Channel estimation | Realistic estimation |
| Frequency offset | Residual Frequency error after DL synchronisation: [0.1] ppm assuming UL pre-compensation |
| Frequency drift | Doppler rate values provided in Table 6.1.1.1-8 in [TR 38.821] |
| Frequency tracking | drift pre-compensation is assumed |
| UE speed | 3 km/h |
| Channel model | For GEO: Baseline TDL/CDL model in [TR 38.811], with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor based on the selected channel conditions. These parameters should be provided by the companies. For LEO: Baseline TDL/CDL model in [TR 38.811], with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor based on the selected channel conditions. These parameters should be provided by the companies. |
| Satellite antenna configuration | 1Tx/Rx |
| UE antenna configuration | (1, 1, 2) with omni-directional antenna element |
| Phase noise Model | S-band phase noise modelling (optional) |

### Channel-specific parameters for PUSCH

Apart from the parameters listed in section 4.2, the following channel-specific parameters for PUSCH should be considered for the coverage study:

Proposal 8:

For link level simulation, adopt the following channel-specific parameters for PUSCH.

Table 8 Channel-specific parameters for PUSCH

| Parameter | Value |
| --- | --- |
| BLER | For eMBB, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains | 1 |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. |
| Waveform | DFT-s-OFDM |
| PUSCH duration | 14 OS |
| Repetitions | w/ or w/o repetition for PUSCH The following value set for repetition factor: {1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32} |
| HARQ configuration | w/ and w/o HARQ |
| PRBs/TBS/MCS for eMBB | 30 PRBs for 1Mbps, 4 PRBs for 100kbps TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | 4 PRBs for VoIP. QPSK, pi/2 BPSK |

### Channel-specific parameters for PUCCH

Apart from the parameters listed in section 4.2, the following channel-specific parameters for PUCCH should be considered for the coverage study:

Proposal 9:

For link level simulation, adopt the following channel-specific parameters for PUCCH.

Table 9 Channel-specific parameters for PUCCH

| Parameters | Values |
| --- | --- |
| PUCCH format type | Format 1, 2bits UCI Format 3, 11/22 bits UCI |
| BLER for PUCCH | For PUCCH format 1: - DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. - ACK missed detection probability: 1%. For PUCCH format 3: - BLER for Ack/Nack, SR: 1% - BLER for CSI: 1%, optional for 10%. |
| Number of PRBs for PUCCH | 1 PRB |
| Number of UE antennas for PUCCH | 1 |
| Number of UE TRXUs for PUCCH | 1 |
| PUCCH duration | 14 OS |
| Number of repetitions for PUCCH | w/ or w/o repetition for PUCCH. Reuse the Rel-16 PUCCH repetition factors 2, 4, 8. |

## Simulation assumptions for PUSCH msg3

The LLS parameters for PUSCH msg3 performance evaluation are summarized within the following table:

Proposal 10:

For link level simulation, adopt the following parameters for PUSCH msg3:

Table 10 Simulation assumptions for PUSCH msg3

| Number of UE transmit chains | 1 |
| --- | --- |
| Number of DMRS symbol | 3 |
| Waveform | DFT-s-OFDM |
| SCS | 15kHz |
| HARQ configuration | For eMBB, w/o HARQ For VoIP, w/o HARQ. |
| PUSCH duration | 14 OS |
| Number of PRBs | 2 |
| TBS | 56 bits |

## Simulation assumptions for SSB

The LLS parameters for SSB performance evaluation are summarized within the following table:

Proposal 11:

For link level simulation, adopt the following parameters for SSB:

Table 11 Simulation assumptions for SSB

| Carrier Frequency | S-band / 2 GHz |
| --- | --- |
| Channel Model | For GEO: Baseline TDL/CDL model in [TR 38.811], with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO: Baseline TDL/CDL model in [TR 38.811], with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg |
| SCS | 15kHz, 30kHz |
| Periodicity | 20ms |
| DL RS | SSB |
| Antenna Configuration at the TRP (satellite) | 1Tx |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element |
| UE speed | 3 km/h |
| UE elevation angle | For GEO (optional): 10°, For LEO: 30° |
| Phase noise model | S-band phase noise modelling (optional) |
| Performance metrics | One-shot initial cell detection accuracy of PCID; CDF of timing and frequency residual offset at SNIR point corresponding to 90% likelihood for one-shot detection accuracy of cell ID. Note: FAR of PCID detection requirement = 1% |

## Simulation assumptions for PDSCH

The LLS parameters for PDSCH performance evaluation are summarized within the following table:

Proposal 12:

For link level simulation, adopt the following parameters for PDSCH.

Table 12 Simulation assumptions for PDSCH

| Parameters | S-band |
| --- | --- |
| Carrier frequency | 2 GHz |
| Channel coding scheme | NR channel coding |
| Subcarrier spacing | 15 kHz, 30 kHz |
| Channel estimation | Realistic estimation |
| Frequency offset | Residual Frequency error after DL synchronisation: [0.1] ppm assuming UL pre-compensation |
| Frequency drift | Doppler rate values provided in Table 6.1.1.1-8 in [TR 38.821] |
| Frequency tracking | drift pre-compensation is assumed |
| UE speed | 3 km/h |
| Channel model | For GEO: Baseline TDL/CDL model in [TR 38.811], with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor based on the selected channel conditions. These parameters should be provided by the companies. For LEO: Baseline TDL/CDL model in [TR 38.811], with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor based on the selected channel conditions. These parameters should be provided by the companies. |
| Satellite antenna configuration | 1Tx/Rx |
| UE antenna configuration | (1, 1, 2) with omni-directional antenna element |
| Phase noise Model | S-band phase noise modelling (optional) |

### Channel-specific parameters for PDSCH

The following channel-specific parameters for PDSCH should be considered for the coverage study:

Proposal 13:

For link level simulation, adopt the following channel-specific parameters for PDSCH.

Table 13 Channel-specific parameters for PDSCH

| Parameter | Value |
| --- | --- |
| BLER | For eMBB, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 1 |
| HARQ configuration | w/ and w/o HARQ |
| DMRS configuration | 3 DMRS symbols is used for PDSCH of Msg.2. Type I, 1 or 2 DMRS symbol, no multiplexing with data. |
| PDSCH duration | 12 OS For PDSCH of Msg.4, 12 OS |
| Payload size for PDSCH of Msg.4 | 1040 bits |

## Simulation assumptions for PDCCH

Apart from the parameters provided in section 4.5, the following channel-specific parameters are to be considered for PDCCH.

Proposal 14:

For link level simulation, adopt the following channel-specific parameters for PDCCH.

Table 14 Simulation assumptions for PDCCH

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 1 |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48 PRBs |
| BLER | 1% BLER |

# Link Budget baseline for NTN UL

For NTN coverage enhancement study the link budget assumptions are based on the parameters listed in Table 6.1.3.2-1 in [TR 38.821] with UE antenna gain of -5dBi.  The Link budgets results are shown in the Tables below for  LEO 600- Set 2, LEO 1200- Set 2 and GEO- Set1 study cases.

Based on the link budget analysis, for a 30-degree elevation angle, 1200 km orbit we made the following observations:

- VoNR with 720 kHz bandwidth, and satellite receive antennas with 24 dB gain, the achievable UL SNR is -16,62dB.

- PUSCH msg3 might be also one of the coverage bottlenecks as the SNR is -13,61dB

- The achievable UL SNR for PRACH long format 2 is -18,25dB. Repetition number of PRACH transmission may need to be increased.

Clearly, like in NR terrestrial networks, uplink is the coverage bottleneck in NTN due to the following limitations:

- Limit of transmit power.

- Polarization loss. Typical handheld devices including smartphones use linear polarized antennas while satellites typically employ circular polarized antennas.

- A small number of receive antennas in the UL. In NTN, satellite antennas can be a significant contributor of launching cost. Typically, only one receive antenna is used per satellite beam although two receive antennas per beam may be used in some cases, one with right-hand and another with left-hand circular polarization.

To support smartphones in NTN, a significantly lower link budget needs to be supported by considering the following:

- Only one satellite receive antenna instead of two.

- Smaller satellite antenna gains and smaller elevation angles. When elevation angle decreases from 30 to 10 degrees, the path loss increases by about 4 to 5 dB for satellite orbits in 600km to 1200 km.

- Smaller UE antenna gains. Internal antennas of smartphones are typically negative (e.g. -5dBi).

The following coverage enhancements for NR NTN may be considered.

- PUSCH: Same as considered for NR coverage enhancements with larger number of repetitions/aggregation slots.

- PUCCH: Same as considered for NR coverage enhancements with larger number of repetitions/aggregation slots.

- PUSCH Msg3: Enabling PUSCH coverage enhancements during RACH. Unlike in terrestrial network, re-scheduling of Msg3 is not desirable due to large round-trip delay in NTN.

- PRACH: PRACH format 2. Evaluation is needed to determine the  number of repetitions that may be required

The following Table provides the UL Link budgets results in case of LEO 600- Set 2:

Table 15 UL Link budgets results LEO 600- Set 2

|  | PUSCH Msg3 | PUSCH VoNR | PUCCH | PRACH long format 2 | PRACH format B4 |
| --- | --- | --- | --- | --- | --- |
| Orbit | LEO 600 | LEO 600 | LEO 600 | LEO 600 | LEO 600 |
| Satellite parameters | Set 2 | Set 2 | Set 2 | Set 2 | Set 2 |
| UE antenna gain | -5dBi | -5dBi | -5dBi | -5dBi | -5dBi |
| Bandwidth (MHz) | 0,36 | 0,72 | 0,18 | 1,048 | 2,16 |
| Frequency [GHz] | 2,00 | 2,00 | 2,00 | 2,00 | 2,00 |
| TX: EIRP [dBm] | 18,01 | 18,01 | 18,01 | 18,01 | 18,01 |
| RX: G/T [dB/T] | -4,90 | -4,90 | -4,90 | -4,90 | -4,90 |
| Bandwidth [MHz] | 0,36 | 0,72 | 0,18 | 1,05 | 2,16 |
| Free space path loss [dB] | 159,10 | 159,10 | 159,10 | 159,10 | 159,10 |
| Atmospheric loss [dB] | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 |
| Shadow fading margin [dB] | 3,00 | 3,00 | 3,00 | 3,00 | 3,00 |
| Scintillation Loss [dB] | 2,20 | 2,20 | 2,20 | 2,20 | 2,20 |
| Polarization loss [dB] | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 |
| Additional losses [dB] | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 |
| CNR [dB] | -8,22 | -11,23 | -5,21 | -12,86 | -16,00 |

The following Table provides the UL Link budgets results in case of LEO 1200- Set 2:

Table 16 UL Link budgets results LEO 1200- Set 2

|  | PUSCH Msg3 | PUSCH VoNR | PUCCH | PRACH long format 2 | PRACH format B4 |
| --- | --- | --- | --- | --- | --- |
| Orbit | LEO 1200 | LEO 1200 | LEO 1200 | LEO 1200 | LEO 1200 |
| Satellite parameters | Set 2 | Set 2 | Set 2 | Set 2 | Set 2 |
| UE antenna gain | -5dBi | -5dBi | -5dBi | -5dBi | -5dBi |
| Bandwidth (MHz) | 0,36 | 0,72 | 0,18 | 1,048 | 2,16 |
| Frequency [GHz] | 2,00 | 2,00 | 2,00 | 2,00 | 2,00 |
| TX: EIRP [dBm] | 18,01 | 18,01 | 18,01 | 18,01 | 18,01 |
| RX: G/T [dB/T] | -4,90 | -4,90 | -4,90 | -4,90 | -4,90 |
| Bandwidth [MHz] | 0,36 | 0,72 | 0,18 | 1,05 | 2,16 |
| Free space path loss [dB] | 164,48 | 164,48 | 164,48 | 164,48 | 164,48 |
| Atmospheric loss [dB] | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 |
| Shadow fading margin [dB] | 3,00 | 3,00 | 3,00 | 3,00 | 3,00 |
| Scintillation Loss [dB] | 2,20 | 2,20 | 2,20 | 2,20 | 2,20 |
| Polarization loss [dB] | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 |
| Additional losses [dB] | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 |
| CNR [dB] | -13,61 | -16,62 | -10,60 | -18,25 | -21,39 |

The following Table provides the UL Link budgets results in case of GEO- Set 1:

Table 17 UL Link budgets results GEO - Set 1

|  | PUSCH Msg3 | PUSCH VoNR | PUCCH | PRACH long format 2 | PRACH format B4 |
| --- | --- | --- | --- | --- | --- |
| Orbit | GEO | GEO | GEO | GEO | GEO |
| Satellite parameters | Set 1 | Set 1 | Set 1 | Set 1 | Set 1 |
| UE antenna gain | -5dBi | -5dBi | -5dBi | -5dBi | -5dBi |
| Bandwidth (MHz) | 0,36 | 0,72 | 0,18 | 1,048 | 2,16 |
| Frequency [GHz] | 2,00 | 2,00 | 2,00 | 2,00 | 2,00 |
| TX: EIRP [dBm] | 18,01 | 18,01 | 18,01 | 18,01 | 18,01 |
| RX: G/T [dB/T] | 19,00 | 19,00 | 19,00 | 19,00 | 19,00 |
| Bandwidth [MHz] | 0,36 | 0,72 | 0,18 | 1,05 | 2,16 |
| Free space path loss [dB] | 190,58 | 190,58 | 190,58 | 190,58 | 190,58 |
| Atmospheric loss [dB] | 0,16 | 0,16 | 0,16 | 0,16 | 0,16 |
| Shadow fading margin [dB] | 3,00 | 3,00 | 3,00 | 3,00 | 3,00 |
| Scintillation Loss [dB] | 2,20 | 2,20 | 2,20 | 2,20 | 2,20 |
| Polarization loss [dB] | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 |
| Additional losses [dB] | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 |
| CNR [dB] | -15,89 | -18,90 | -12,88 | -20,54 | -23,68 |

# Link Budget baseline for NTN DL

The following Table provides the DL Link budgets results in case of LEO 600- Set 2:

Table 18 DL Link budgets results LEO 600- Set 2

|  | SSB | PDCCH | PDSCH MBB | PDSCH VoNR |
| --- | --- | --- | --- | --- |
| Orbit | LEO 600 | LEO 600 | LEO 600 | LEO 600 |
| Satellite parameters | Set 2 | Set 2 | Set 2 | Set 2 |
| UE antenna gain | -5dBi | -5dBi | -5dBi | -5dBi |
| Bandwidth (MHz) | 3,60 | 8,65 | 5,40 | 0,72 |
| Frequency [GHz] | 2,00 | 2,00 | 2,00 | 2 |
| TX: EIRP [dBm] | 63,56 | 67,37 | 65,32 | 56,57332496 |
| RX: G/T [dB/T] | -31,60 | -31,60 | -31,60 | -31,6 |
| Bandwidth [MHz] | 3,60 | 8,65 | 5,40 | 0,72 |
| Free space path loss [dB] | 159,10 | 159,10 | 159,10 | 159,0972636 |
| Atmospheric loss [dB] | 0,07 | 0,07 | 0,07 | 0,070880411 |
| Shadow fading margin [dB] | 3,00 | 3,00 | 3,00 | 3 |
| Scintillation Loss [dB] | 2,20 | 2,20 | 2,20 | 2,2 |
| Polarization loss [dB] | 0,00 | 0,00 | 0,00 | 0 |
| Additional losses [dB] | 5,00 | 5,00 | 5,00 | 5 |
| CNR [dB] | -4,37 | -4,37 | -4,37 | -4,37 |

The following Table provides the DL Link budgets results in case of LEO 1200- Set 2:

Table 19 DL Link budgets results LEO 1200- Set 2

|  | SSB | PDCCH | PDSCH MBB | PDSCH VoNR |
| --- | --- | --- | --- | --- |
| Orbit | LEO 1200 | LEO 1200 | LEO 1200 | LEO 1200 |
| Satellite parameters | Set 2 | Set 2 | Set 2 | Set 2 |
| UE antenna gain | -5dBi | -5dBi | -5dBi | -5dBi |
| Bandwidth (MHz) | 3,60 | 8,65 | 5,40 | 0,72 |
| Frequency [GHz] | 2,00 | 2,00 | 2,00 | 2,00 |
| TX: EIRP [dBm] | 69,56 | 73,37 | 71,32 | 62,57 |
| RX: G/T [dB/T] | -31,60 | -31,60 | -31,60 | -31,60 |
| Bandwidth [MHz] | 3,60 | 8,65 | 5,40 | 0,72 |
| Free space path loss [dB] | 164,48 | 164,48 | 164,48 | 164,48 |
| Atmospheric loss [dB] | 0,07 | 0,07 | 0,07 | 0,07 |
| Shadow fading margin [dB] | 3,00 | 3,00 | 3,00 | 3,00 |
| Scintillation Loss [dB] | 2,20 | 2,20 | 2,20 | 2,20 |
| Polarization loss [dB] | 0,00 | 0,00 | 0,00 | 0,00 |
| Additional losses [dB] | 5,00 | 5,00 | 5,00 | 5,00 |
| CNR [dB] | -3,76 | -3,76 | -3,76 | -3,76 |

The following Table provides the DL Link budgets results in case of GEO- Set 1:

Table 20 DL Link budgets results GEO - Set 1

|  | SSB | PDCCH | PDSCH MBB | PDSCH VoNR |
| --- | --- | --- | --- | --- |
| Orbit | GEO | GEO | GEO | GEO |
| Satellite parameters | Set 1 | Set 1 | Set 1 | Set 1 |
| UE antenna gain | -5dBi | -5dBi | -5dBi | -5dBi |
| Bandwidth (MHz) | 3,60 | 8,65 | 5,40 | 0,72 |
| Frequency [GHz] | 2,00 | 2,00 | 2,00 | 2,00 |
| TX: EIRP [dBm] | 94,56 | 98,37 | 96,32 | 87,57 |
| RX: G/T [dB/T] | -31,60 | -31,60 | -31,60 | -31,60 |
| Bandwidth [MHz] | 3,60 | 8,65 | 5,40 | 0,72 |
| Free space path loss [dB] | 190,58 | 190,58 | 190,58 | 190,58 |
| Atmospheric loss [dB] | 0,07 | 0,07 | 0,07 | 0,07 |
| Shadow fading margin [dB] | 3,00 | 3,00 | 3,00 | 3,00 |
| Scintillation Loss [dB] | 2,20 | 2,20 | 2,20 | 2,20 |
| Polarization loss [dB] | 0,00 | 0,00 | 0,00 | 0,00 |
| Additional losses [dB] | 5,00 | 5,00 | 5,00 | 5,00 |
| CNR [dB] | -4,85 | -4,85 | -4,85 | -4,85 |

# Conclusion

In this contribution. we made the following observations and proposals:

Observation 1.	To support smartphones in NTN, a significantly lower link budget needs to be supported by considering the following:

- Only one satellite receive antenna instead of two.

- Smaller satellite antenna gains and smaller elevation angles. When elevation angle decreases from 30 to 10 degrees, the path loss increases by about 4 to 5 dB for satellite orbits in 600km to 1200 km.

- Smaller UE antenna gains. Internal antennas of smartphones are typically negative (e.g. -5dBi).

Target scenarios and services requirements:

Proposal 1:

The following target data rates are considered for MBB and VoNR performance evaluation:

- For VoNR,  a packet size of 344 bits every 20ms i.e. a data rate of 17.2 kbps

- For MBB, 1 Mbit/s in the DL and 100 kbit/s in the UL.

Satellite parameters sets:

Proposal 2:

The satellite parameters used for the NTN coverage enhancement study are provided by the two tables in section 2.2 of this contribution.

UE characteristics:

Proposal 3:

The UE characteristics used for the NTN coverage enhancement study are provided by the table in section 2.3 of this contribution.

List of coverage study cases:

Proposal 4:

The coverage study cases used for the NTN coverage enhancement study and targeting the smartphones UEs are provided by the table in section 2.4  of this contribution.

Evaluation methodology:

Proposal 5:

The following two options should be considered for the evaluation methodology:

Option 1: Based on link-level simulation:

- Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements.

- Obtain the baseline performance based on required SINR and link budget template.

- Identify target performance and coverage bottlenecks based on target performance metric.

Option 2: Based on link- level and system-level simulation

- Obtain the required SINR for the given target data rate based on link-level simulation.

- Obtain the target performance based on system-level simulation (i.e. the 5th percentile downlink or uplink SINR value in CDF curve).

Simulation assumptions for PRACH

Proposal 6:

For link level simulation, adopt the parameters for PRACH provided by the table in section 4.1

Simulation assumptions for PUSCH and PUCCH

Proposal 7:

The parameters for PUSCH and PUCCH to be used for link level simulation are given the table in section 4.2 of this contribution.

Channel-specific parameters for PUSCH

Proposal 8:

For link level simulation, adopt the channel-specific parameters for PUSCH provided by the Table in section 4.2.1 of this contribution.

Channel-specific parameters for PUCCH

Proposal 9:

For link level simulation, adopt the channel-specific parameters for PUCCH provided by the Table in section 4.2.2 of this contribution.

Simulation assumptions for PUSCH msg3

Proposal 10:

For link level simulation, adopt the parameters for PUSCH msg3 provided by the Table in section 4.3 of this contribution.

Simulation assumptions for SSB

Proposal 11:

For link level simulation, adopt the parameters for SSB provided by the Table in section 4.4 of this contribution

Simulation assumptions for PDSCH

Proposal 12:

For link level simulation, adopt the parameters for PDSCH provided by the Table in section 4.5 of this contribution.

Channel-specific parameters for PDSCH

Proposal 13:

For link level simulation, adopt the channel-specific parameters for PDSCH provided by the Table in section 4.5.1 of this contribution.

Simulation assumptions for PDCCH

Proposal 14:

For link level simulation, adopt the channel-specific parameters for PDCCH provided by the Table in section 4.6 of this contribution.

# References

- RP-220953 Rel-18 WI NR-NTN-enh, Thales

- 3GPP TR 38.821 Solutions for NR to support non-terrestrial networks (NTN) Release 16

- 3GPP TS 22.261 Service requirements for the 5G system; Stage 1, Release 16