3GPP TSG RAN WG1 #109-e			R1-2204549

e-Meeting, May 9th – 20th, 2022

Source:	WILUS Inc.

Title:	Remaining issues on Joint channel estimation for PUCCH and PUSCH

Agenda item:	8.8.2

Document for:	Discussion/Decision

Introduction

In the RAN1#107-e meeting, the following agreement was made on PUCCH enhancements for coverage enhancement [1].

Agreement at RAN1#107-e: For the interaction between inter-slot frequency hopping and DMRS bundling for PUCCH/PUSCH repetitions, a UE performs the “hopping intervals determination”, “configured TDW determination”, and “actual TDW determination” in a sequential ordering, based on the following option 1. - Option 1: “hopping intervals determination” -> “configured TDW determination” -> “actual TDW determination” - DMRS bundling shall be restarted at the beginning of each frequency hop - DMRS bunding is per actual TDW - FFS: Frequency hopping pattern is determined by physical slot indices. - FFS: different FH pattern determination for PUCCH and PUSCH - FFS: details of FH pattern design - Support separate RRC configuration(s) for hopping interval and configured TDW length. - if hopping interval is not configured, the default hopping interval is the same as the configured TDW length - FFS: if both hopping interval and TDW length are not configured - Note: hopping interval is only determined by the configuration of hopping interval if hopping interval is configured

In this contribution, we provide our views on joint channel estimation for PUCCH and PUSCH.

Remaining issues on joint channel estimation for PUCCH and PUSCH

Inter-slot frequency hopping with inter-slot bundling.

In the RAN1#107-e meeting, it was agreed to support Option 1 for inter-slot frequency hopping with inter-slot bundling. Thus, hopping interval determination does not depend on configured TDW determination, and separate RRC configurations are supported. Here, we would like to raise an issue when the configured value of L’ is larger than L (i.e., L’ > L), where L’ is the configured value of hopping interval and L is the configured value of configured TDW length.

Figure 1. Determination of hopping interval and configured TDW length if L’ > L.

In Figure 1, a UE can be configured to transmit PUCCH with 8 repetitions, L’=4, and L=2. Also, it is assumed that configured values of L’ and L are smaller than UE’s maximum duration. Accordingly, the first 4 slots are determined as the 1st hop, and subsequent 4 slots are determined as the 2nd hop. Meanwhile, consecutive 2 slots are determined as the same configured TDW. In this case, PUCCH repetitions in CTDW#1 (CTDW#3) and CTDW#2 (CTDW#4) are likely to be transmitted by being satisfied with conditions for joint channel estimation. However, since the UE is configured as L=2 from a gNB, the gNB may not try to jointly estimate the channel of PUCCH repetitions in CTDW#1 (CTDW#3) and CTDW#2 (CTDW#4) together.

To address this issue, a gNB can configure a value of L’ smaller than or equal to a value of L. Thus, a UE does not expect to be configured as L’ > L.

Otherwise, a UE can determine the configured TDW length based on L’ if the configured hopping interval value of L’ is larger than configured TDW length value of L. Therefore, both hopping interval and configured TDW length can be determined based on L’.

- Proposal 1: Following methods can be further considered to maximize the gain of joint channel estimation in case of both hopping interval (i.e., L’) and configured TDW length (i.e., L) are configured:

  - Alt 1: A UE does not expect to be configured as hopping interval (i.e., L’) > configured TDW length (i.e., L).

  - Alt 2: Hopping interval (i.e., L’) is used for determination of configured TDW length if configured hopping interval value of L’ is larger than configured TDW length value of L.

Conclusion

In this contribution, we provided our views on joint channel estimation for PUCCH and PUSCH and the following was proposed:

- Proposal 1: Following methods can be further considered to maximize the gain of joint channel estimation in case of both hopping interval (i.e., L’) and configured TDW length (i.e., L) are configured:

  - Alt 1: A UE does not expect to be configured as hopping interval (i.e., L’) > configured TDW length (i.e., L).

  - Alt 2: Hopping interval (i.e., L’) is used for determination of configured TDW length if configured hopping interval value of L’ is larger than configured TDW length value of L.

References

- Final Report of 3GPP TSG RAN WG1 #107-e v1.0.0