3GPP TSG RAN WG1 #109-e			R1-2204550

e-Meeting, May 9th – 20th, 2022

Source:	WILUS Inc.

Title:	Discussion on sub-band non overlapping full duplex

Agenda item:	9.3.2

Document for:	Discussion

Introduction

In the RAN#95-e meeting, revised SID on evolution of NR duplex operation was approved [1], and the objective for evolution of NR duplex operation is listed as follows.

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we provide our views on sub-band non overlapping full duplex.

Discussion on sub-band non overlapping full duplex

Spectrum partitioning

To support sub-band non overlapping full duplex operation, several issues should be studied, such as which time domain resources are used for sub-band operation and how to flexibly partition a single TDD spectrum.

For the issue on time domain resources for sub-band operation, cell-common flexible slots or symbols (i.e., flexible slots or symbols by TDD-UL-DL-ConfigCommon) can be considered. Cell-common downlink/uplink slots or symbols are not suitable to be used for sub-band operation considering co-existence with UEs that does not support sub-band operation. Also, even for UEs that support sub-band operation in a same cell, cell-common downlink/uplink slots or symbols are better to not divide into sub-bands to minimize inter-UE interferences.

A guard band may be required between a downlink sub-band and a uplink sub-band to reduce DL/UL interferences. For this, it can be studied whether/how to support flexible sub-band which can be used as a guard band. It is worth noting that flexible sub-band also might be considered for the scenarios to support flexible scheduling (i.e., downlink reception and uplink transmission).

- Proposal 1: RAN1 to study flexible slots or symbols by TDD-UL-DL-ConfigCommon as time domain resources for sub-band non overlapping full duplex.

  - RAN1 to study flexible sub-band which can be used as a guard band between downlink sub-band and uplink sub-band.

For the issue on how to flexibly partition a single TDD spectrum, there are two potential options for spectrum partitioning for sub-band non overlapping full duplex operation.

Option 1: Single sub-band per downlink/uplink sub-band

A TDD spectrum can be partitioned with single sub-band per each downlink and uplink sub-band. In other words, up to one sub-band can be allocated for downlink and uplink, respectively as shown in Figure 1 (a). Regarding that a guard band is required to minimize the DL/UL interference between the downlink sub-band and the uplink sub-band, Option 1 requires at least one guard band between DL/UL. With Option 1, efficient frequency resource utilization can be achieved. There are more available resources for downlink reception and uplink transmission with Option 1. Also, since more frequency resources are available when the CORESET resource for PDCCH monitoring of the UE is configured, CORESET can be flexibly configured in the downlink sub-band.

Option 2: Multiple sub-bands per downlink/uplink sub-band

A TDD spectrum can be partitioned with multiple sub-bands per each downlink and uplink sub-band. Thus, more than one sub-band can be allocated for downlink and uplink, respectively as shown in Figure 1 (b). As discussed in Option 1, a guard is required between the downlink sub-band and the uplink sub-band, thus Option 2 would require multiple guard bands. However, Option 2 can achieve flexible sub-band allocation by considering multiple sub-bands allocation.

Figure 1. Spectrum partitioning options for sub-band non overlapping full duplex operation.

Based on above discussion, it can be studied which option to support for spectrum partitioning for sub-band non overlapping full duplex operation.

- Proposal 2: RAN1 to study following options for spectrum partitioning for sub-band non overlapping full duplex operation.

  - A single TDD spectrum can be divided into sub-bands consisting of one downlink sub-band and one uplink sub-band.

  - A single TDD spectrum can be divided into sub-bands consisting of multiple downlink sub-bands and uplink sub-bands.

Conclusion

In this contribution, we discussed sub-band non overlapping full duplex and summarize our views as the followings:

- Proposal 1: RAN1 to study flexible slots or symbols by TDD-UL-DL-ConfigCommon as time domain resources for sub-band non overlapping full duplex.

  - RAN1 to study flexible sub-band which can be used as a guard band between downlink sub-band and uplink sub-band.

- Proposal 2: RAN1 to study following options for spectrum partitioning for sub-band non overlapping full duplex operation.

  - A single TDD spectrum can be divided into sub-bands consisting of one downlink sub-band and one uplink sub-band.

  - A single TDD spectrum can be divided into sub-bands consisting of multiple downlink sub-bands and uplink sub-bands.

References

- RP-220633, “Revised SID: Study on evolution of NR duplex operation,” CMCC, RAN#95-e