3GPP TSG RAN WG1 #109-e			R1-2204551

e-Meeting, May 9th – 20th, 2022

Source:	WILUS Inc.

Title:	Discussion on potential enhancements on dynamic/flexible TDD

Agenda item:	9.3.3

Document for:	Discussion

Introduction

In the RAN#95-e meeting, revised SID on evolution of NR duplex operation was approved [1], and the objective for evolution of NR duplex operation is listed as follows.

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we provide our views on potential enhancements on dynamic/flexible TDD.

Discussion on potential enhancements on dynamic/flexible TDD

Sub-band format configuration/indication

In RAN1, it should be studied how to semi-statically configure and/or dynamically indicate sub-band to UE which supports Rel-18 duplex operation. Existing mechanisms in frequency domain, e.g., FDRA, BWP can be reused for the indication. In addition to the indication in the frequency domain, TDD slot format configuration/indication mechanism can be further considered by switching slot format to sub-band format. Also, as we discussed in our contribution in A.I. 9.3.2 [2], flexible slots or symbols can be regarded as time domain resources for sub-band operation.

Semi-static sub-band format configuration

Semi-static slot format configuration in TDD operation can be introduced for the semi-static sub-band format configuration. Signaling methods considered in the time domain (e.g., # of DL/UL symbols) can be reused in the frequency domain (e.g., # of DL/UL RBs) with little modifications.

In TDD semi-static slot format configuration, both cell-specific configuration by TDD-UL-DL-ConfigCommon and UE-specific configuration by TDD-UL-DL-ConfigDedicated are supported in Rel-15 NR. For Rel-18 duplex operation, it should be studied whether to configure sub-band format cell-specifically, UE-specifically, or both. Considering that different sub-band configuration among UEs may introduce DL/UL interference in a cell, at least the cell-specific sub-band format configuration can be used. However, the UE-specific sub-band format configuration also can be further studied during Rel-18 study phase.

In a semi-static sub-band format configuration, a UE may be configured with the number of consecutive DL RBs for downlink sub-band and consecutive UL RBs for uplink sub-band as shown in Figure 1. In this case, remaining RBs which are not configured can be determined as a guard band (or flexible sub-band) implicitly.

Figure 1. Semi-static sub-band format configuration.

- Proposal 1: RAN1 to study semi-static sub-band format configuration based on semi-static TDD slot format configuration.

  - RAN1 to study cell-specific and/or UE-specific sub-band format configuration.

Dynamic sub-band format indication

In Rel-15, dynamic SFI (slot format indication) was introduced for dynamic and flexible slot format indication in time domain. For Rel-18 duplex operation, it should be studied whether/how to support dynamic sub-band format indication in frequency domain.

However, different from the semi-static configuration which can reuse the TDD slot format configuration, it might be hard to utilize a method used in dynamic SFI for sub-band format indication in frequency domain. Dynamic SFI indicates slot format by utilizing the pre-defined slot format table in specification, it may be difficult to define the pre-defined sub-band format table since the number of RBs for sub-band is not fixed and can be flexibly allocated for DL and UL in a dynamic manner.

Instead, some RA mechanisms in frequency domain, such as RIV can be reused for dynamic sub-band format indication. It should be further studied whether/how to support dynamic sub-band format indication.

- Proposal 2: RAN1 to study whether/how to support dynamic sub-band format indication.

Overwriting rule for sub-band format configuration/indication

In Rel-15 TDD operation, UE behavior (i.e., overwriting rule) was specified among semi-statically configured symbols and dynamically indicated symbols. Semi-static DL/UL symbol is not overridden by dynamic SFI, scheduling DCI, or configured signal/channel.

However, semi-static flexible slots or symbols can be overridden, thus the UE behavior was specified by depending on whether or not a UE is configured to monitor PDCCH with DCI format 2_0 with CRC scrambled by SFI-RNTI, and whether or not the UE detects DCI format 2_0. In detail, the UE behavior was specified for PDCCH monitoring in configured CORESET symbols, DCI-indicated DL reception or UL transmission, and higher layer-configured DL reception or UL transmission when a UE is not configured to monitor PDCCH with dynamic SFI, or a UE is configured to monitor PDCCH with dynamic SFI and detect DCI format 2_0, or a UE is configured to monitor PDCCH with dynamic SFI and does not detect DCI format 2_0.

For Rel-18 duplex operation, if flexible sub-band is introduced as we discussed in [2], and if dynamic sub-band format indication is introduced, there can be both semi-static and dynamic flexible sub-bands. Then, a UE behavior for the cases discussed above (PDCCH monitoring, DL reception or UL transmission by DCI, and DL reception or UL transmission by higher layer configuration) should also be discussed as in Rel-15 TDD operation.

- Proposal 3: RAN1 to study the UE behavior for following cases on semi-static flexible sub-band.

  - PDCCH monitoring in configured CORESET symbols.

  - DCI-indicated DL reception or UL transmission.

  - Higher layer configured DL reception or UL transmission.

Conclusion

In this contribution, we discussed potential enhancements on dynamic/flexible TDD and summarize our views as the following:

- Proposal 1: RAN1 to study semi-static sub-band format configuration based on semi-static TDD slot format configuration.

  - RAN1 to study cell-specific and/or UE-specific sub-band format configuration.

- Proposal 2: RAN1 to study whether/how to support dynamic sub-band format indication.

- Proposal 3: RAN1 to study the UE behavior for following cases on semi-static flexible sub-band.

  - PDCCH monitoring in configured CORESET symbols.

  - DCI-indicated DL reception or UL transmission.

  - Higher layer configured DL reception or UL transmission.

References

- RP-220633, “Revised SID: Study on evolution of NR duplex operation,” CMCC, RAN#95-e

- R1-2204550, “Discussion on sub-band non overlapping full duplex,” WILUS, RAN1#109-e