3GPP TSG RAN WG1 #109-e			R1-2204552

e-Meeting, May 9th – 20th, 2022

Source:	WILUS Inc.

Title:	Discussion on channel access mechanism for SL on unlicensed spectrum

Agenda item:	9.4.1.1

Document for:	Discussion/Decision

- Introduction

At RAN#95-e meeting, revised WID on NR sidelink evolution was approved with one of the objectives related to NR sidelink support on unlicensed spectrum as follows [1]:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this contribution, we discuss a channel access mechanism for NR sidelink on unlicensed spectrum and provide our views.

Discussion on channel access for NR sidelink on unlicensed spectrum

According to the RAN guidance, RAN1 study should focus on FR1 unlicensed bands such as n46 and n96/n012. Since NR sidelink in unlicensed is expected to be deployed on FR1 unlicensed band, it would be desirable to guarantee fair-coexistence on shared spectrum with other existing technologies such as Wi-Fi, LAA, and NR-U. And as described in the WID, channel access mechanisms from NR-U shall be reused for sidelink unlicensed operations from the channel access perspective.

In Rel-16 NR-U, there are two channel access modes which are LBE (i.e. dynamic channel access mode) and FBE (i.e. semi-static channel access mode). For LBT mode, there are multiple LBT types to be defined for each DL/UL transmission. A gNB or a UE use Type 1 channel access type to obtain a channel occupancy for initiating the DL or UL transmission respectively. Depending on the time gap between consecutive transmissions, information type of transmission burst, and a restriction of the transmission burst length, Type 2A/2B/2C are defined in NR-U.

Under the assumption that NR sidelink PHY channels/signals such as S-SSB, PSBCH, PSCCH/PSSCH and PSFCH are reused in sidelink on unlicensed spectrum, it should be discussed whether and how to apply the NR-U channel access mechanism for NR sidelink PHY channels/signals according to the sidelink resource allocation scheme. However, as initial channel access mechanism at least Type 1 channel access should be reused to initialize the sidelink transmission to obtain COT for fair-coexistence on shared spectrum with other existing technologies such as Wi-Fi, LAA, and NR-U.

- Proposal 1: At least Type 1 channel access should be reused to initialize the sidelink transmission to obtain COT for fair-coexistence on shared spectrum with other existing technologies such as Wi-Fi, LAA, and NR-U.

In NR-U, when a gNB or a UE use Type 1 channel access procedure, a contention window should be determined based on HARQ-ACK feedback according to contention window adjustment procedure before step 1 (i.e. random number selection) of Type 1 channel access procedure. However, in the case of sidelink transmission, depending on the transmission type, that is, for unicast/groupcast transmission, HARQ-ACK may exist in a case where A/N can be reported as HARQ-ACK for improved reliability and NACK-only can be reported for groupcast transmission and for sidelink broadcast transmission, CWS adjustment method based on sidelink HARQ-ACK feedback is unavailable. Therefore, it might be necessary to study and investigate the CWS adjustment method especially for both the groupcast transmission with NACK-only and the broadcast transmission with no HARQ-ACK feedback when Type 1 channel access procedure is applied to NR SL-U.

- Proposal 2: Similar to CWS adjustment in NR-U, CWS adjustment based on the HARQ-ACK feedback via PSFCH might be used for Type 1 channel access for PSSCH transmission if HARQ-ACK is available for unicast transmission with A/N as HARQ-ACK feedback

- Proposal 3: It might be necessary to further study and investigate the CWS adjustment method especially for both the groupcast transmission with NACK-only feedback and the broadcast transmission with no HARQ-ACK feedback when Type 1 channel access procedure is applied to NR SL-U.

To increase the channel access opportunity and to improve the resource utilization efficiency on unlicensed spectrum, COT sharing mechanism has been supported in NR-U. The COT was defined as the total time for which the gNB and the UE can occupy a given channel and perform transmissions after carrying out the required channel access procedures. More specifically, a UE can share a COT initiated by a gNB, and then the UE can transmit UL channels to gNB in the shared channel occupancy. And a UE also can share the channel occupancy initiated by the UE to a gNB, then the gNB can transmit DL channels, at least including this UE as the receiver, in the shared channel occupancy. Similar to how gNB and UE were given an opportunity to continuously guarantee transmission through COT sharing in NR-U, it may be desirable to provide an opportunity to ensure continuous PSSCH transmission and PSFCH transmission as feedback transmission between UEs also in the NR sidelink on unlicensed spectrum. Therefore, whether and how to support UE-to-UE COT sharing mechanism for sidelink transmission in unlicensed spectrum should be studied.

- Proposal 4: Whether and how to support UE-to-UE COT sharing mechanism for sidelink transmission in unlicensed spectrum should be studied.

According to the ETSI harmonized standard [2], it is allowed to transmit the transmission as short control signaling if the specific conditions are met below.

- Within an observation period of 50 ms, the number of Short Control Signalling Transmissions by the equipment shall be equal to or less than 50; and

- The total duration of Short Control Signalling Transmissions shall be less than 2500 μs within an observation period.

At least for the case of S-SSB transmission in the NR sidelink on unlicensed spectrum, whether it can be regarded as short control signaling by satisfying the above two conditions should be discussed. And similar to that at least for a discovery burst including SSB in NR-U, Type 2A could be supported, it might be desirable to apply Type 2A channel access for S-SSB transmission in the NR sidelink on unlicensed spectrum.

- Proposal 5: For S-SSB transmission in NR sidelink on unlicensed, it should be further studied whether it can be regarded as short control signaling transmission.

Different from the sidelink HARQ-ACK on PSFCH in Rel-16, PSFCH in sidelink unlicensed might not be transmitted due to LBT failure. However, in Rel-16 NR-U, HARQ-ACK retransmission was specified which makes possible to retransmit HARQ-ACK along with other HARQ-ACK later. Therefore, it should be further investigated how to support sidelink HARQ-ACK retransmission via PSFCH.

- Proposal 6: For PSFCH in sidelink unlicensed which might not be transmitted due to LBT failure, it should be further investigated how to support sidelink HARQ-ACK retransmission via PSFCH.

Conclusion

In this contribution, we have discussed channel access mechanisms for NR sidelink on unlicensed spectrum. As a conclusion, we summarize our view as follows:

- Proposal 1: At least Type 1 channel access should be reused to initialize the sidelink transmission to obtain COT for fair-coexistence on shared spectrum with other existing technologies such as Wi-Fi, LAA, and NR-U.

- Proposal 2: Similar to CWS adjustment in NR-U, CWS adjustment based on the HARQ-ACK feedback via PSFCH might be used for Type 1 channel access for PSSCH transmission if HARQ-ACK is available for unicast transmission with A/N as HARQ-ACK feedback

- Proposal 3: It might be necessary to further study and investigate the CWS adjustment method especially for both the groupcast transmission with NACK-only feedback and the broadcast transmission with no HARQ-ACK feedback when Type 1 channel access procedure is applied to NR SL-U.

- Proposal 4: Whether and how to support UE-to-UE COT sharing mechanism for sidelink transmission in unlicensed spectrum should be studied.

- Proposal 5: For S-SSB transmission in NR sidelink on unlicensed, it should be further studied whether it can be regarded as short control signaling transmission.

- Proposal 6: For PSFCH in sidelink unlicensed which might not be transmitted due to LBT failure, it should be further investigated how to support sidelink HARQ-ACK retransmission via PSFCH.

References

[1] RP-220300, “WID revision: NR sidelink evolution”, OPPO

[2] ETSI EN 301 893 (V2.1.1) :"5 GHz RLAN; Harmonised Standard covering the essential requirements

of article 3.2 of Directive 2014/53/EU".