3GPP TSG RAN WG1 #109-e			R1-2204553

e-Meeting, May 9th – 20th, 2022

Source:	WILUS Inc.

Title:	Discussion on PHY channel design framework for SL on unlicensed spectrum

Agenda item:	9.4.1.2

Document for:	Discussion/Decision

- Introduction

At RAN#95-e meeting, revised WID on NR sidelink evolution was approved with one of the objectives related to NR sidelink support on unlicensed spectrum as follows [1]:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this contribution, we discuss a PHY channel design for NR sidelink on unlicensed spectrum and provide our views.

Discussion on channel access for NR sidelink on unlicensed spectrum

From the RAN guidance, RAN1 study should focus on FR1 unlicensed bands such as n46 and n96/n012 and is to be completed by RAN#98. According to the regulation on FR1 unlicensed spectrum, devices shall meet Occupied Channel Bandwidth (OCB) requirement that transmissions in a channel shall occupy 80% or more of the channel bandwidth. From the coverage perspective, it might be beneficial for the device to maintain the high output power and fulfill the regulations on maximum EIRP, and power spectral density. For these aspects, the interlaced allocation as one of resource allocation mechanisms was adopted in NR-U Rel-16 and LTE-LAA Rel-14. Considering available subchannel size(s) for each transmission of PSCCH/PSSCH which can be allocated on one or more contiguous sub-channels, PSCCH/PSSCH in sidelink unlicensed cannot satisfy the OCB requirement. Therefore, interlaced transmission for PSCCH, PSSCH on unlicensed spectrum should be supported. The details how to support interlaced transmission such as RB based interlaced transmission or sub-channel based interlaced transmission should be further studied.

- Proposal 1: To meet OCB requirement, interlaced transmission for PSCCH, PSSCH on unlicensed spectrum should be supported.

  - The details how to support interlaced transmission such as RB based interlaced transmission or sub-channel based interlaced transmission should be further studied.

Considering that LBT is performed per 20 MHz channel bandwidth for fair-coexistence on shared spectrum with other existing technologies such as Wi-Fi, LAA, and NR-U, interlaced allocation should be reused for PSSCH resource allocation and RB set indication might be also reused if intra-cell guard configuration is configured for PSSCH resource allocation.

Different from PSCCH/PSSCH which can be allocated on one or more contiguous sub-channels, each PSFCH associated with PSSCH is occupied with 1 PRB using PUCCH format 0. To meet OCB requirement, it may be desirable to allocate the PSFCH in the form of an RB-based interlaced transmission. In Rel-16 NR-U, enhanced PUCCH format 0 can be considered as a starting point for RB-based interlaced PSFCH transmission. Unlike the method in NR-U in which PUCCH resources are dynamically indicated by scheduling DCI, in NR sidelink, a set of continuous PRBs for PSFCH occasions is used for PSFCH. This is to simplify the sidelink HARQ-ACK feedback procedure by allowing all sidelink UEs to have a common understanding of HARQ-ACK feedback, and to improve the reliability of SL HARQ-ACK feedback by avoiding interference from non-HARQ feedback SL transmission to HARQ feedback SL transmission. Therefore, PUCCH format 0 is considered as a starting point for RB-based interlace PSFCH transmission, but it should be further studied and investigated how to indicate PSFCH resource for HARQ-ACK corresponding PSSCH for NR sidelink unlicensed operation.

- Proposal 2: PUCCH format 0 is considered as a starting point for RB-based interlace PSFCH transmission, but it should be further studied and investigated how to indicate PSFCH resource for HARQ-ACK corresponding PSSCH for NR sidelink unlicensed operation.

Conclusion

In this contribution, we have discussed channel access mechanism for NR sidelink on unlicensed spectrum. As a conclusion, we summarize our view as follows:

- Proposal 1: To meet OCB requirement, interlaced transmission for PSCCH, PSSCH on unlicensed spectrum should be supported.

  - The details how to support interlaced transmission such as RB based interlaced transmission or sub-channel based interlaced transmission should be further studied.

- Proposal 2: PUCCH format 0 is considered as a starting point for RB-based interlace PSFCH transmission, but it should be further studied and investigated how to indicate PSFCH resource for HARQ-ACK corresponding PSSCH for NR sidelink unlicensed operation.

References

[1] RP-220300, “WID revision: NR sidelink evolution”, OPPO