3GPP TSG RAN WG1 #109-e	Tdoc R1-2204554

e-Meeting, May 9th - 20th, 2022

Agenda Item:	7.1

Source:	Ericsson, Nokia, Nokia Shanghai Bell

Title:	HARQ-ACK multiplexing on PUSCH without PUCCH

Document for:	Discussion, Decision

# 1	Introduction

During last meetings the issue of HARQ-ACKs multiplexing on a PUSCH was discussed when the UE misses DL assignments corresponding to the HARQ-ACKs while the UL grant scheduling the PUSCH indicates UL-DAI for HARQ-ACK multiplexing [1][2][3].

For Rel-15, the following was concluded in RAN1#106-e:

Conclusion - For Rel-15, in the case of multiple overlapping PUSCHs with no overlapping PUCCH and if any UL-TDAI n.e. 4 (for Type 2 codebook) or UL-TDAI e.q. 1 (for Type 1 codebook) the UE behavior is left to UE implementation. Agreement - For Rel-15 with more than one non-overlapping PUSCH and no overlapping PUCCH within a span on one slot (both single carrier and UL CA) and if the UL-TDAI for the PUSCH UL-TDAI not equal to 4 (for Type 2 codebook) or UL-TDAI equal to 1 (for Type 1 codebook), the UE behavior is up to the UE implementation - For Rel-15 with one PUSCH and no overlapping PUCCH within a span of one slot and if the UL-TDAI for the PUSCH UL-TDAI not equal to 4 (for Type 2 codebook) or UL-TDAI equal to 1 (for Type 1 codebook), there is no consensus for any conclusion on one aligned UE behavior.

The continued discussions in RAN1#107-e, led to the following outcomes for Rel-16:

Agreement - For Rel-16 with multiple overlapping PUSCHs with no overlapping PUCCH with HARQ-ACK within a span of one PUCCH slot, if the UL-TDAI n.e. 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook) there is no consensus in RAN1 on Rel-16 UE behavior Agreement - For Rel-16 with one PUSCH and no overlapping PUCCH with HARQ-ACK within a span of one PUCCH slot (both single carrier and UL CA), if the UL-TDAI is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), the UE multiplexes HARQ-ACK following the UL-TDAI into the PUSCH. - Continue discussion on UE behavior with respect to multiplexing HARQ-ACK in PUSCH for the following case in Rel-16. - More than one non-overlapping PUSCH and no overlapping PUCCH with HARQ-ACK within a span on one PUCCH slot (both single carrier and UL CA), if for at least one of the PUSCHs the UL-TDAI is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook)

In RAN1#108-e, companies intensively discussed solutions for multiple PUSCHs including the possibility of a unified solution as summarized [3].

In this contribution we discuss our view on the UE behavior for HARQ-ACK multiplexing on multiple PUSCHs without overlapping PUCCH.

# 2	Discussions

## 2.1	Multiples PUSCHs cases

During the last meeting, different options were constructively discussed and analyzed [3]. The discussions established a proper foundation to investigate different options and revisit the topic in this meeting aiming for a reasonable decision. In the following, we explain our view on how we can reach to a unified solution based on further analysis and investigating of the different options and underlying constraints.

### 2.1.1	Underlying principles for decision on a solution

Firstly, it is important to emphasize that it is crucial to reach to a unified solution that addresses the multiple PUSCHs cases and not compromise the performance of UL CA deployments. It is in fact a responsible act from standardization point of view towards the industry. Furthermore, as explained previously, the UL DAI field in the DCI is intended to provide a control mechanism to ensure that the UE multiplexes a determined number of HARQ-ACK bits obtained from the indicated UL-TDAI in a PUSCH even if the UE misses one or all the corresponding DL assignments. Otherwise, there would be misalignment between the gNB and UEs on the multiplexed HARQ-ACK in a PUSCH and failure in decoding the PUSCH in case of DL miss detection. Therefore, ignoring HARQ-ACK multiplexing in a PUSCH when indicated by UL-TDAI in case of missing all the DL assignments defeats the purpose of UL-TDAI.

Reflecting on the discussion last meeting for candidate options, we observe that there is diverge views on a Rel-16 solution mainly motivated due to existing implementations, while majority favour to have a unified solution for Rel-17 that is aligned with the case that PUCCH would be present (i.e., Option 1). We also observe that the discussion led towards a direction that we as a group tried to have a Rel-16 solutions that would serve as a temporary solution and have a more proper solution for Rel-17 envisioning that the constraints with current implementations would be relaxed.

After further discussions regarding the options discussed last meeting, we would like to share our view on the way forward.

Firstly, we explain the NW behavior regarding this feature:

- NW does not change its behaviour for this feature whether the UE indicates Rel-15/16/17, etc.  That means that there is no value to make efforts on discussing different solutions for Rel-16 and Rel-17. Therefore, we need to avoid temporary solutions for Rel-16 since it would be of no use.

  - The NW would behave according to the specified behavior and does not change its behavior for this feature. The already existing UEs that their behaviour is unknown to the NW, would be anyway treated the same way as the new UEs. However, whether these UEs would cause issue or not, depends on specific implementation and is just not known.

- The earliest the behaviour is specified, helps to have less UEs with unknown behaviour in the NW and more “new UEs” with known behaviour which in turn contributes to improve overall operation across the NW.

Secondly, regarding the solution itself for this feature, it is important to keep in mind that we should not aim to optimize the solution for the case that PUCCH is absent due to DL miss-detection and specify a solution that diverges from the normal case when a PUCCH would be present. Any divergence increases unnecessarily the specification impact. Therefore, the envisioned optimizations should be solely pursued by implementation, and not specification.

Lastly, to move forward and address the concern of UE vendors regarding the existing implementations and/or facilitate implementation plans to implement the specified behavior, a capability corresponding the specified behavior could be introduced although the introduction of the capability is not preferred by NW vendors. As mentioned earlier, the NW behavior would be the same regardless of whether the UEs indicate the corresponding capability or not, since the behavior of incapable UEs is just unknown. Most importantly, the introduction of the capability hopefully does not encourage postponing the implementation of the specified behavior which is crucial for achieving a well-performing NW.

We summarize the above discussion by outlining the following observations as guidelines for considerations to reach to a unified solution:

Aiming for a unified solution including the multiple PUSCHs cases is crucial to not compromise the performance of UL CA deployments.

Different solutions for Rel16 and Rel-17 should be avoided. The NW would behave according to the specified behavior and does not change its behavior for this feature whether the UE indicates Rel-15/16/17, etc.

The unified solution to specify behavior for the case that PUCCH is absent should not diverge from the specified behavior when a PUCCH would be present. Any optimization is matter of implementation and not specification.

Introduction of capability for the solution can ease the UE implementations concern. While the earliest the behaviour is specified and implemented by UEs, improves the system performance due to less UEs with unknown behaviour in the NW.

### 2.1.2	The unified solution

Based on the discussion in the previous section and reflecting on the options that were discussed during the last meeting, it should be evident that the only viable way forward for a unified solution is to adopt Recommendation 1 (based on Option 1) for Rel-17 in [3] also for Rel-16/Rel-17 CR as shown below:

- Selection of the candidate PUSCH for multiplexing - Candidate PUSCHs - Alt-1: All the PUSCHs within the PUCCH slot are candidates - Alt-2: PUSCHs without UL-TDAI=4 in case Type 2 CB, and without UL-TDAI n.e. 1 in case of Type 1 CB within the PUCCH slot are candidates - N/W sets all TDAI values that overlap with PUCCH to the [same/different] value with TDAI n.e. 4 - Prioritization rules to select PUSCH for multiplexing. Prioritization rules are identical to 38.213 - Limitations for multiplexing - UE expects to multiplex HARQ-ACK on only 1 PUSCH selected based on step 2 in the PUCCH slot. - All the PUSCHs in the determined candidate set after step 1 have to satisfy Rel-15 UCI multiplexing timeline, defined with respect the starting symbol of the earliest PUSCH transmission in the candidate set. - gNB will not schedule any additional PUSCH in the PUCCH slot that does not satisfy the multiplexing timeline

Regarding the choice between Alt 1 and Alt 2 in step 1-a and the same or different UL-TDAI in step 1-b, we should aim not to diverge from the specified procedures for the case when PUCCH is present as explained previously. Therefore, only Alt-1 and no restriction on UL-TDAI would satisfy this goal as the following:

- Selection of the candidate PUSCH for multiplexing. - All the PUSCHs within the PUCCH slot are candidates. - Prioritization rules to select PUSCH for multiplexing. Prioritization rules are identical to 38.213 - Limitations for multiplexing - UE expects to multiplex HARQ-ACK on only 1 PUSCH selected based on step 2 in the PUCCH slot. - All the PUSCHs in the determined candidate set after step 1 have to satisfy Rel-15 UCI multiplexing timeline, defined with respect the starting symbol of the earliest PUSCH transmission in the candidate set. - gNB will not schedule any additional PUSCH in the PUCCH slot that does not satisfy the multiplexing timeline

Therefore, we propose the following as an attempt to find a compromise and resolution while having a meaningful outcome that helps the industry to move forwards for UL CA deployments plans.

For the case when multiple PUSCHs with no overlapping PUCCH with HARQ-ACK within a span of one PUCCH slot, the corresponding UE behavior is subject to indicating a capability and specified based on the following procedures (i.e. Option 1 with Alt-1 and no constraint on UL-TDAI):

- Selection of the candidate PUSCH for multiplexing. - All the PUSCHs within the PUCCH slot are candidates. - Prioritization rules to select PUSCH for multiplexing. Prioritization rules are identical to 38.213 - Limitations for multiplexing - UE expects to multiplex HARQ-ACK on only 1 PUSCH selected based on step 2 in the PUCCH slot. - All the PUSCHs in the determined candidate set after step 1 have to satisfy Rel-15 UCI multiplexing timeline, defined with respect the starting symbol of the earliest PUSCH transmission in the candidate set. - gNB will not schedule any additional PUSCH in the PUCCH slot that does not satisfy the multiplexing timeline

### 2.1.3	Proposed text proposals

The following agreement was made in RAN1#107-e for the single PUSCH case:

Agreement

- For Rel-16 with one PUSCH and no overlapping PUCCH with HARQ-ACK within a span of one PUCCH slot (both single carrier and UL CA), if the UL-TDAI is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), the UE multiplexes HARQ-ACK following the UL-TDAI into the PUSCH

The following text proposals implements the above agreement for Rei-16 specification that can be mirrored to Rel-17 specifications.

- Text proposals to implement agreement for single PUSCH case

=================== START of Text Proposal TS 38.213 =========================

9       UE procedure for reporting control information

<unchanged text omitted>

A UE does not expect to detect a DCI format scheduling a PDSCH reception or a SPS PDSCH release, a DCI format 1_1 indicating SCell dormancy, or a DCI format including a One-shot HARQ-ACK request field with value 1, and indicating a resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot if the UE previously detects a DCI format scheduling a PUSCH transmission in the slot and if the UE multiplexes HARQ-ACK information in the PUSCH transmission.

If a UE transmits a PUSCH scheduled by a DCI format that includes a DAI field on a serving cell in a slot with reference to slots for PUCCH transmissions, the UE multiplexes HARQ-ACK information in the PUSCH transmission if the UE has not received any PDCCH within the monitoring occasions for PDCCH with DCI format scheduling PDSCH receptions or SPS PDSCH release or indicating SCell dormancy on any serving cell c or the UE has any HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH.

If a UE multiplexes aperiodic CSI in a PUSCH and the UE would multiplex UCI that includes HARQ-ACK information in a PUCCH that overlaps with the PUSCH and the timing conditions for overlapping PUCCHs and PUSCHs in clause 9.2.5 are fulfilled, the UE multiplexes only the HARQ-ACK information in the PUSCH and does not transmit the PUCCH.

If a UE transmits multiple PUSCHs in a slot on respective serving cells that include first PUSCHs that are scheduled by DCI formats and second PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH, and the UE would multiplex UCI in one of the multiple PUSCHs, and the multiple PUSCHs fulfil the conditions in clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in a PUSCH from the first PUSCHs.

If a UE transmits multiple PUSCHs in a slot on respective serving cells and the UE would multiplex UCI in one of the multiple PUSCHs and the UE does not multiplex aperiodic CSI in any of the multiple PUSCHs, the UE multiplexes the UCI in a PUSCH of the serving cell with the smallest ServCellIndex subject to the conditions in clause 9.2.5 for UCI multiplexing being fulfilled. If the UE transmits more than one PUSCHs in the slot on the serving cell with the smallest ServCellIndex that fulfil the conditions in clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in the earliest PUSCH that the UE transmits in the slot.

<unchanged text omitted>

9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel

<unchanged text omitted>

A UE determines o0ACK, o1ACK,...,oOACK-1ACK HARQ-ACK information bits, for a total number of  HARQ-ACK information bits, of a HARQ-ACK codebook for transmission in a PUCCH or a PUSCH according to the following pseudo-code. In the following pseudo-code, if the UE does not receive a transport block or a CBG, due to the UE not detecting a corresponding DCI format, the UE generates a NACK value for the transport block or the CBG. The cardinality of the set  defines a total number  of occasions for PDSCH reception or SPS PDSCH release for serving cell  corresponding to the HARQ-ACK information bits.

<unchanged text omitted>

9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel

<unchanged text omitted>

If the UE transmits HARQ-ACK information in a PUSCH or a PUCCH in slot n with reference to slots for PUCCH transmission and for any PUCCH format, the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK  , for a total number of OACK HARQ-ACK information bits, according to the following pseudo-code:

<unchanged text omitted>

====================== End of Text Proposal TS 38.213 =========================

In the following, we update the proposed text proposal for single PUSCH case to incorporate the proposal for multiple PUSCHs case in the previous section. It is important to observe that the proposal for multiple PUSCHs case integrates well with the agreed behavior for the single PUSCH case as well as the specified behavior for the case when the PUCCH is present. One can observe easily that any divergence from Option 1 with Alt-1 and constraint in UL-TDAI for multiple PUSCHs case, would unnecessarily complicates the specifications and corresponding implementations.

Moreover, we provide the inter-operability analysis needed for the CR cover page to provide a fair assessment of the consequences and inter-operability.

Therefore, we propose the following:

Adopt the following Text proposals and corresponding descriptions for consequences and inter-operability analysis to implement the agreement for single PUSCH case as well as the proposal for multiple PUSCHs case for Rel-16 CR (being mirrored to Rel-17 CR)

- Text proposals to implement unified solution for single PUSCH and multiple PUSCHs case

==== Start of Text Proposal TS 38.213 for unified solution (single/multiple PUSCHs) ===========

9       UE procedure for reporting control information

<unchanged text omitted>

A UE does not expect to detect a DCI format scheduling a PDSCH reception or a SPS PDSCH release, a DCI format 1_1 indicating SCell dormancy, or a DCI format including a One-shot HARQ-ACK request field with value 1, and indicating a resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot if the UE previously detects a DCI format scheduling a PUSCH transmission in the slot and if the UE multiplexes HARQ-ACK information in the PUSCH transmission.

If a UE transmits a PUSCH scheduled by a DCI format that includes a DAI field on a serving cell in a slot with reference to slots for PUCCH transmissions, the UE multiplexes HARQ-ACK information in the PUSCH transmission if the UE has not received any PDCCH within the monitoring occasions for PDCCH with DCI format scheduling PDSCH receptions or SPS PDSCH release or indicating SCell dormancy on any serving cell c or the UE has any HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH.

If a UE multiplexes aperiodic CSI in a PUSCH and the UE would multiplex UCI that includes HARQ-ACK information in a PUCCH that overlaps with the PUSCH and the timing conditions for overlapping PUCCHs and PUSCHs in clause 9.2.5 are fulfilled, the UE multiplexes only the HARQ-ACK information in the PUSCH and does not transmit the PUCCH.

When a UE transmits multiple PUSCHs on respective serving cells in a slot with reference to slots for PUCCH transmission, the UE determines the PUSCH for UCI multiplexing by applying the following procedure:

- If the UCI includes HARQ-ACK information due to a DAI field in a DCI format scheduling at least one the multiple PUSCHs and the UE has not received any PDCCH within the monitoring occasions for PDCCH with DCI format scheduling PDSCH receptions or SPS PDSCH release or indicating SCell dormancy on any serving cell c and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, the UE multiplexes the UCI in the PUSCH if the UE indicates the corresponding capability [the name of the capability].

- If the multiple PUSCHs include first PUSCHs that are scheduled by DCI formats and second PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH, and the UE would multiplex UCI in one of the multiple PUSCHs, and the multiple PUSCHs fulfil the conditions in clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in a PUSCH from the first PUSCHs.

- If the UE would multiplex UCI in one of the multiple PUSCHs and the UE does not multiplex aperiodic CSI in any of the multiple PUSCHs, the UE multiplexes the UCI in a PUSCH of the serving cell with the smallest ServCellIndex subject to the conditions in clause 9.2.5 for UCI multiplexing being fulfilled. If the UE transmits more than one PUSCHs in the slot on the serving cell with the smallest ServCellIndex that fulfil the conditions in clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in the earliest PUSCH that the UE transmits in the slot.

<unchanged text omitted>

#### 9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

<unchanged text omitted>

A UE determines o0ACK, o1ACK,...,oOACK-1ACK HARQ-ACK information bits, for a total number of  HARQ-ACK information bits, of a HARQ-ACK codebook for transmission in a PUCCH or a PUSCH according to the following pseudo-code. In the following pseudo-code, if the UE does not receive a transport block or a CBG, due to the UE not detecting a corresponding DCI format, the UE generates a NACK value for the transport block or the CBG. The cardinality of the set  defines a total number  of occasions for PDSCH reception or SPS PDSCH release for serving cell  corresponding to the HARQ-ACK information bits.

<unchanged text omitted>

#### 9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

<unchanged text omitted>

If the UE transmits HARQ-ACK information in a PUSCH or a PUCCH in slot n with reference to slots for PUCCH transmissions and for any PUCCH format, the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK  , for a total number of OACK HARQ-ACK information bits, according to the following pseudo-code:

<unchanged text omitted>

==== End of Text Proposal TS 38.213 for unified solution (single/multiple PUSCHs) ===========

- Description on CR cover page for consequences and inter-operability analysis:

This CR completes an incompletely specified Rel-15 functionality for multiplexing HARQ-ACK when multiple PUSCHs overlap with the PUCCH slot. A UE implemented based on an earlier version of the specification, and thus not able to indicate the new UE capability, may still be compliant with this CR, while a UE not compliant with the CR has unknown behaviour. See isolated impact analysis.

Isolated impact analysis

- if a gNB is implemented according to the CR, but the UE is not, the UE behavior is unclear when HARQ-ACK is to be sent on a PUCCH slot where the UE transmits [on more than 1 carrier] more than 1 overlapping PUSCHs, potentially leading to loss of UCI as well as loss of PUSCH.

- If the UE is implemented according to the CR, but the gNB is not, the gNB assumption on where it expects to find the UCI is unclear when HARQ-ACK is to be sent on a PUCCH slot where the UE transmits [on more than 1 carrier] more than 1 overlapping PUSCHs, potentially leading to loss of UCI as well as loss of PUSCH.

## 2.2	PUSCH repetition cases

- During the last meeting the issue was also discussed for the case when a PUSCH scheduled by DCI is repeated and corresponding UL grant indicates UL-TDAI but a PUCCH with HARQ-ACK is absent throughout the PUSCH repetition. Three alternatives regarding the UE behaviour were discussed as the following:

- Alt 1-1: The HARQ-ACK corresponding to UL-TDAI is multiplexed in the first PUSCH of the repetition

- Alt 1-2: The HARQ-ACK corresponding to UL-TDAI is multiplexed in all PUSCHs of the repetition

- Alt 2: The HARQ-ACK corresponding to UL-TDAI is not multiplexed in any PUSCHs of the repetition

As we discussed in the previous meeting, ignoring multiplexing HARQ-ACK, brings us to the same original issue as discussed for PUSCH without repetition to allow a behavior that defeats the purpose of UL-TDAI. Therefore, we are not supportive of Alt-2.

Between Alt-1-1 and Alt-1-2, due to scheduling restrictions after UL grant and maximum configured K1 values, it is more realistic that the gNB could schedule PDSCHs such that the corresponding PUCCHs with HARQ-ACK would overlap with the earlier repetitions, e.g., the first two repetitions, specially in case of TDD. Therefore, among Alt 1-1 and Alt 1-2, Alt 1-2 would not be a typical scenario specially in case of TDD while Alt-1-1 seems to be the closest alternative in practice.

Therefore, we propose the following:

When a PUSCH scheduled by DCI is repeated and the corresponding UL grant indicates UL-TDAI but a PUCCH with HARQ-ACK is absent throughout the PUSCH repetition, the first PUSCH of the repetition is considered as a candidate PUSCH for HARQ-ACK multiplexing following the unified solution.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Aiming for a unified solution including the multiple PUSCHs cases is crucial to not compromise the performance of UL CA deployments.

Observation 2	Different solutions for Rel16 and Rel-17 should be avoided. The NW would behave according to the specified behavior and does not change its behavior for this feature whether the UE indicates Rel-15/16/17, etc.

Observation 3	The unified solution to specify behavior for the case that PUCCH is absent should not diverge from the specified behavior when a PUCCH would be present. Any optimization is matter of implementation and not specification.

Observation 4	Introduction of capability for the solution can ease the UE implementations concern. While the earliest the behaviour is specified and implemented by UEs, improves the system performance due to less UEs with unknown behaviour in the NW.

During the last meeting the issue was also discussed for the case when a PUSCH scheduled by DCI is repeated and corresponding UL grant indicates UL-TDAI but a PUCCH with HARQ-ACK is absent throughout the PUSCH repetition. Three alternatives regarding the UE behaviour were discussed as the following:

Based on the discussion in the previous sections we propose the following:

Proposal 1	For the case when multiple PUSCHs with no overlapping PUCCH with HARQ-ACK within a span of one PUCCH slot, the corresponding UE behavior is subject to indicating a capability and specified based on the following procedures (i.e. Option 1 with Alt-1 and no constraint on UL-TDAI):

Proposal 2	Adopt the following Text proposals and corresponding descriptions for consequences and inter-operability analysis to implement the agreement for single PUSCH case as well as the proposal for multiple PUSCHs case for Rel-16 CR (being mirrored to Rel-17 CR)

Proposal 3	When a PUSCH scheduled by DCI is repeated and the corresponding UL grant indicates UL-TDAI but a PUCCH with HARQ-ACK is absent throughout the PUSCH repetition, the first PUSCH of the repetition is considered as a candidate PUSCH for HARQ-ACK multiplexing following the unified solution.

# References

R1-2108647 Summary for [106-e-NR-7.1CRs-07] Discussion on HARQ-ACK multiplexing on PUSCH without PUCCH	Moderator (Apple)

R1-2112859		Summary for [107-e-NR-7.1CRs-6] Issue #10: Discussion on HARQ-ACK multiplexing on PUSCH, Moderator (Apple)

R1-2202835		Summary for [108-e-NR-CRs-06] Issue #8: Discussion on HARQ-ACK multiplexing on PUSCH, Moderator (Apple)