3GPP TSG RAN WG1 #109-e		R1-2204565

e-Meeting, May 9th – 20th , 2022

Source:	Panasonic

Title: 	Discussion on side control signals for network-controlled repeaters

Agenda Item:		9.8.1

Document for:	Discussion

# Introduction

The objectives of study item on network-controlled repeaters in Rel. 18 were defined [1]. The repeaters are supposed to provide coverage enhancements for FR1 FDD/TDD and FR2 TDD bands, while the FR2 bands may be prioritized during the study phase. In addition, both indoor and outdoor scenarios should be considered. One of the main objectives for the RAN1 study is to identify the essential side control signaling for the repeater, such as, the beamforming information, timing information, information on UL-DL TDD configuration, and power control information. In this contribution, we present our view on the required function for the network-controlled repeaters for the design of side control information.

# Discussions

For network-controlled repeaters, there could be different power control information that are need. These could be related to the limits on transmission powers, handling periodic traffic, and link-failures.

Limits on transmission powers

One of important features of the network-controlled repeaters is the ability of beam forming, particularly in FR2 bands. The beam forming enables emitting a higher transmission power on a specific direction. While this can improve the link quality significantly, it increases the chance that emitted signal is feedbacked to the repeater that causing self-oscillation. Conventionally, the self-oscillation is avoided using automatic gain control (AGC), located at the repeater side. The repeater's AGC could apply limits on transmission powers on specific beam(s)/direction(s). The network-controlled repeater may have the other limitation of the transmission power specific to its implementation. The gNB should be aware of that transmission power limitations of the network-controlled repeater. This helps the gNB to better schedule the UEs or avoid using certain beams. For this purpose, the repeater should report the maximum transmission power on each beam/direction determined by the repeater to the gNB.

Proposal 1: A repeater should determine the maximum transmission power on each beam/direction and report that to the gNB.

Periodic beam/power control pattern

The gNB can schedule the UEs in a TDM manner with periodic resources, for instance for SR, CG, SPS, and CQI report. This is especially the scenario of a limited number of UEs are served by a repeater. In such case, the gNB can configure the repeater with periodic patterns semi-statically for the beam selections and power allocations. Figure 1 illustrates a scenario that a repeater serves two UEs in downlink with periodic resources, e.g., SPS resources. The repeater is informed regarding the serving beams and associated transmission powers. The repeater applies the patterns for beam selections and transmission powers for downlink transmissions towards the UEs. Although the ideal operation might be gNB informs beam direction to the repeater for each transmission/reception, it requires instantaneous decoding of the control information at the repeater, which is complex or impossible. By using periodic patterns, such burden at the repeater can be resolved. In addition, the reduced signaling overhead compared to dynamic control signal delivery can be obtained.

Proposal 2: The periodic beam and transmission power patterns should be considered for serving periodic traffics.

Figure 1. Illustration of serving two UEs with periodic beam and power patterns.

Control information for link-failures

The network-controlled repeater needs to maintain the gNB-repeater link and repeater-UE link simultaneously [1]. In FR2 bands, the links are prone to blockage, which results in link failures. In such cases, the amplifying the link failed signal could increase the interference in the system. Hence, it is essential if the repeater could identify the link failures and stops amplifying in such occasions. For this purpose, the repeater could be configured with minimum signal power threshold for each beam/direction. If the repeater identifies that the perceived signal power is less than the predefined threshold value, it will consider that the link is blocked or there is no transmission from the UE/gNB. Then, it will temporarily stops amplifying to avoid introducing additional interference in the system. This approach would also reduce the power consumption of the repeater.

Proposal 3: A repeater should be defined with minimum power thresholds for the beams/directions. The repeater only amplifies the signal if the perceived power is above the threshold.

# Conclusion

In this contribution, we provide our view on control signals for network-controlled repeaters. We made following proposals.

Proposal 1: A repeater should determine the maximum transmission power on each beam/direction and report that to the gNB.

Proposal 2: The periodic beam and transmission power patterns should be considered for serving periodic traffics.

Proposal 3: A repeater should be defined with minimum power thresholds for the beams/directions. The repeater only amplifies the signal if the perceived power is above the threshold.

# Reference

[1] 	RP-213562, “Study on NR Smart Repeaters,” RAN#94-e.