3GPP TSG RAN WG1 Meeting #109-e                                		R1-2204568

e-Meeting, May 9th – 20th, 2022

Source:                TCL Communication

Title:                    Discussions on Sub-Use Cases in AI/ML for CSI Feedback Enhancement

Agenda item:      9.2.2.2

Document for:    Discussion and Decision

- Introduction

Artificial Intelligence (AI)/Machine Learning (ML) has exhibited its merits and superiority over the conventional practices in various walks of life. The study item on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface is established in RAN plenary meeting #94e for Rel.18 [1]. AI/ML will be an icon of the wireless communications for 5G advanced and afterwards.

According to current 5G Standard, the UE is continuously burdened by a large amount of data/bits for CSI feedback reporting to the gNB; especially, after the introduction of the codebook type II and the growth of the antenna ports to up to 32. That all leads to an increased amount of feedback data. Furthermore, the CSI feedback should be continuously sent to the gNB, e.g., for a MU-MIMO case, the feedback is very critical to the (BER) performance. The more delicate and in-time the feedback is, the better the scheduling will be. In the meantime, in AI/ML, the auto-encoder model is a good fit for overcoming the CSI feedback problem. The auto-encoder model is capable of compressing the input at the encoder side, and recover the input at decoder side. The compressed information can be delivered via the air interface. The channel measured by the CSI is sparse. The channel is sparse in a certain dimension under certain description, e.g., the tapped delay line model is sparse in time. Therefore, the measured channel is compressible. The auto-encoder model is able to complete the channel compression in an unknown higher dimension, then decode the output.

The gain of AI/ML based CSI feedback over the conventional codebook type I and codebook type II, has been shown by several companies. Such a gain comes from the fact that the conventional codebook type I and codebook type II are general designs applicable to various scenarios. Whereas, the auto-encoder model is a scenario-specific model. A scenario-specific model with a scenario-specific high dimension description can compress the measured channel better than a general design.

The CSI feedback will be discussed in this contribution.  We will provide possible sub use cases in time domain, frequency domain and spatial domain. The potential standard impacts will be discussed for each sub use case.

- Time Domain CSI Feedbacks

In this section, we will discuss the possible sub use cases of CSI feedback based on auto-encoder model. They are classified according to the input time range and the output time range as below.

  - Basis CSI Feedback Model

The basic auto-encoder based CSI feedback model is shown in Fig 1. There are two parts. The encoder resides in the UE side. And the decoder resides in the gNB side.

Fig 1. An auto-encoder model.

In a high level view, the CSI feedback based on codebook is a process of compression and decompression. The auto-encoder model is exactly able to compress a vector into lower dimension one and then recover it. The basic auto-encoder model is a replacement of the CSI feedback based on codebook at the UE side and at the gNB side, respectively.

Observation 1: In a high level view, the CSI feedback based on codebook is a process of compression and decompression. The auto-encoder model is exactly able to compress a vector into lower dimension one and then recover it.

The basic CSI feedback model based on auto-encoder is further explained according to Fig. 2.

At encoder side,

- The input is the CSI measurement of time instance t.

- The output of the encoder at UE side is the compressed CSI measurement of time instance t. This output is reported to gNB at time tp1.

At decoder side,

- The input is the compressed CSI measurement of time t, received via the air-interface.

- The output at the decoder side is decompressed. The output is obtained at time tp2.

Finally, the output of the decoder is applied to scheduling at time ts, for example, scheduling according to the calculated CQI.

Fig. 2 The compression and decompression of the CSI measurement of time t.

The merits of the basic CSI feedback model based on auto-encoder is the saved feedback bits through the air-interface, compared to the CSI feedback based on codebook. Whereas, the scheduling delay and processing delay are still there and left unhandled. Due to them, the scheduling policy at time ts is determined according to the measurement at time t, and is outdated.

Proposal 1: The basic CSI feedback model based on auto-encoder reduces feedback bits through the air-interface, compared to the CSI feedback based on codebook. It is a functional replacement of the CSI feedback based on codebook.

  - Predictive CSI Feedback Model

To make the CSI feedback in-time and truly reflect the channel conditions at the scheduling time, the CSI decoded at the gNB side is the predicted values at an intended future time. Thereby, the predictive CSI feedback model based on auto-encoder is obtained. It is described according to Fig. 3.

At encoder side,

- The input is the CSI measurements, e.g., from time t, to time t-n, where n is a positive integer.

- The output at the encoder side is the compressed output. The output is reported to gNB at tp1.

At decoder side,

- The input is the compressed CSI measurements received via the air-interface.

- The output at the encoder side is the predicted CSI at time ts.

Fig. 3 The compression and decompression of the CSI measurements with prediction.

This model utilizes the channel correlations to predict the channel conditions in a future time. The historical channel measurements are processed to generate the compressed feedback. The merits of this model is the reduced CSI feedback bits and predicted future CSI. Considering the UE should process multiple CSIs, the model complexity is higher than the basic CSI feedback model and the requirement on UE processing ability grows higher. The multiple historical CSI measurements are needed as input for the ML model to track the channel fluctuations and make the prediction.

Proposal 2: To fix the problem of outdated CSI feedback, the predictive CSI feedback model is necessary to predict CSI at scheduling time.

  - Predictive Multi-CSI Feedback

Since the channel is correlated in time, the measured CSIs are correlated in time too. Thus, the feedback CSIs can be further compressed in time domain. Multiple CSIs across a time range can be feedback to gNB at one report occasion. A schematic figure is shown in Fig. 4.

At encoder side,

- The input is the CSI measurements, e.g., from time t, to time t-n, where n is a positive integer.

- The output at the encoder side is the compressed output. The output is reported to gNB at tp1 at one report occasion.

At decoder side,

- The input is the compressed CSI measurements, received via the air-interface.

- The output at the encoder side are the predicted CSIs from time ts to time ts+m, where m is a positive integer.

Fig. 4 The feedback multi-CSI measurements in time domain.

The CSIs are further compressed along the time domain. Since the CSIs are correlated in time, the redundancy among them can be compressed. At one CSI report occasion, the compressed multiple CSIs are feedback to gNB. The model complexity is increased and the requirement on UE processing ability on the CSI compression grows higher.

Proposal 3: Multiple CSI measurements can be compressed together and feedback at one shot to further reduces the feedback overhead.

- CSI Feedback Compression in Spatial Domain

Previously, there were works about the CSI feedback. By measuring a sub set of antenna ports at time t, the UE/gNB gets the measurements of all antenna ports at time t . The spatial correlation among the antenna ports are utilized by the machine learning model [2].

The standard impact of this method is relatively less. The measurements of all the antenna ports can be reported to a gNB. The gNB is able to train and deploy the ML model by the gNB itself. The inputs are the measurements of a subset of the antenna ports. The ML model will generate the data of the measurements of all the antenna ports. Besides, the data collection should be carried out by reporting the CSI measurements of all antenna ports to the gNB. These data are utilized for ML model training. This kind of data collection is supported by the current CSI measurement and report framework.

Observation 2: The CSI feedback with compression in spatial domain has less standard impacts.

This method can be combined with any CSI feedback design in Section 2, to further reduce the CSI feedback overhead.

- CSI Feedback Compression in Frequency Domain

  - Sub-band CSI Feedback

In the current CSI feedback method based on codebook, the wideband PMI is generated at first. Accompanying it, the PMI for each the sub band is calculated. The Codebook Type II is enhanced in Rel.16, by compressing the CSI report in the frequency domain to reduce the CSI feedback overhead. The compression is performed across sub-bands.

To reduce the CSI-RS feedback, one possible way is to design much sparser CSI-RS patterns across the PRBs along the frequency dimension. The supported densities of CSI-RS in a DL channel is {3, 1, 0.5} according to Table 7.4.1.5.3-1 in 38.211 V17.0.0. The densities of CSI-RS in a DL channel can be further reduced. The PRB/sub-bands where there is no CSI-RS configured, is interpolated and extrapolated from neighboring CSI-RS. The interpolated and extrapolation can be completed by various methods. The ML model based interpolated and extrapolation is expected to give better performance, since the higher dimension information is captured.

The merit of this method is that the input vector size of the ML model for CSI feedback at the UE side is reduced. At the gNB side, the measurements of all the sub-bands with and without CSI-RS REs are recovered by an ML model. In the meantime, the resource utilization with sparser CSI-RS REs is improved by allocating more REs to data transmission.

Proposal 4: The CSI feedback compression along the frequency dimension, can be designed with new sparser CSI-RS patterns. The resource utilization is improved by allocating more REs to data transmission.

  - UL Channel Estimation from CSI-RS Feedback

In certain scenarios, e.g., the TDD case, the DL channel and the UL channel are correlated. This feature is termed as channel reciprocity in TDD. For more general cases, the DL channel and the UL channel can be correlated and there are minor differences between them. The machine learning model will cover this differences and deduce the UL channel estimations from DL CSI-RS.

As shown in Fig. 5, the UL channel is beside the DL channel. The CSI-RSs are configured along the boundary of the DL channel. At the inference stage, the input is the CSI-RSs of the DL channel. The output is the UL channel estimation which is given in Fig. 6. For data collections, the reference signals for DL channel and the UL channel are needed to be transmitted in pair. Dense reference signals can be configured for the UL channel to obtain the ground truth of the channel conditions

UL channel estimation from the CSI-RS feedback saves the reference signals in the UL channel at the inference stage. The resource utilization in the UL channel is improved. The processing burden of UL channel estimation is left to the gNB.

Fig. 5 The CSI-RSs are configured near the boundary of DL channel. The UL channel is beside the DL channel.

Fig 6. An ML model for UL channel condition estimation based on DL channel measurement. The input is the DL channel measurements. The output is the estimated uplink channel conditions.

Observation 3: UL channel estimation from CSI-RS feedback saves the reference signals for UL channel at the inference stage.

- Conclusion

In this contribution the following proposals been made:

Observation 1: In a high level view, the CSI feedback based on codebook is a process of compression and decompression. The auto-encoder model is exactly able to compress a vector into lower dimension one and then recover it.

Proposal 1: The basic CSI feedback model based on auto-encoder reduces feedback bits through the air-interface, compared to the CSI feedback based on codebook. It is a functional replacement of the CSI feedback based on codebook.

Proposal 2: To fix the problem of outdated CSI feedback, the predictive CSI feedback model is necessary to predict CSI at scheduling time.

Proposal 3: Multiple CSI measurements can be compressed together and feedback at one shot to further reduces the feedback overhead.

Observation 2: The CSI feedback with compression in spatial domain has less standard impacts.

Proposal 4: The CSI feedback compression along the frequency dimension, can be designed with new sparser CSI-RS patterns. The resource utilization is improved by allocating more REs to data transmission.

Observation 3: UL channel estimation from CSI-RS feedback saves the reference signals for UL channel at the inference stage.

References

- RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #94e, Dec. 6 - 17, 2021.

- RP-212011,	Study on AI/ML based air interface enhancement in Rel-18, vivo, RAN #93e meeting, Sep. 13 – 17, 2021.