3GPP TSG RAN WG1 Meeting #109-e                                R1-2204569

e-Meeting, May 9th – 20th, 2022

Source:               TCL Communication

Title:          	Discussions on Sub-Use Cases in AI/ML for Beam Management

Agenda item:      9.2.3.2

Document for:    Discussion and Decision

- Introduction

Artificial Intelligence (AI)/Machine Learning (ML) has the potential to be a landmark technology in 5G advanced. The gains from AI/ML have been investigated last year, especially in RAN plenary meetings #92e and #93e. The study item on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface is established in RAN plenary #94e meeting for Rel.18 [1].

Beam management was selected as one use case for study in Rel.18. The previous discussion related to beams was mainly try to utilize the prediction ability of AI/ML, such as choosing a proper beam predictively during the beam switching produces. The beam switching may cause delays. For example, if a beam is selected improperly, additional beam switching will be triggered which may bring more delays and processing overhead. Thus, choosing a beam with less beam switching/handover will ease the system maintenance and improve user experience. Besides, the prediction in ML is a kind of time series analysis in which the UE traces along the time are sampled into a time sequence by measurements. Due to the fact that, the moving traces of a certain kind of UEs, e.g., vehicles, satisfy such a certain prediction rule (i.e. the future data is correlated with historical data). That makes the prediction highly possible for the beam management use case.

The beam management covers several processing procedures, such as beam training, beam measurement, beam switching, and beam failure recovery. We will discuss the potential use cases accordingly. Their possible standard impacts will be provided.

- Discussions

In the objective of the SI [1], the use case of beam management is to focus on,

Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1].

The sub use cases of beam management based on ML are conceived with this objective in mind.

  - Beam Training Based on ML

One important function for beam management is to maintain the connection. The beam management in this section is discussed in two parts, i.e., during RACH procedures and after the connection established.

During the RACH procedures, the RACH occasions are associated with the SSB. The SSBs are transmitted with different beams to scan the cell coverage. Usually, at the initial access stage, the SSBs are transmitted uniformly in each beam. The UEs come into the cell coverage in different directions. The UE will connect to the beam with strongest signal strength measured from its received SSBs. At one hand, some beams will be frequently connected. On the other hand, some beams are rarely connected. Thus, the SSB beam scanning should be properly configured to reduce the connection latency. A straightforward method is to increase the scanning times of frequently connected beams, and to decrease the scanning times of less frequently connected beams. A schematic diagram of this method is shown in Fig. 1. The probability of a beam is connected by a new UE in the next future time unit is predicted by an ML model. The probability is time varying and related to the traffic patterns and UE activities.

Figure 1. The SSB beam scanning times are configured according to the probability of the beam is connected to a new UE.

The advantage of this method is to shorten the UE access latency at the cell level.

Proposal 1: The configuration of SSB beam scanning at initial access stage can be improved by ML.

After the connection is established, the UE and the gNB complete the beam pairing by P1, P2 and P3. The UE has to take MN times measurements at maximum, to find a beam properly paired with a gNB beam, where M indicates the number of beams of the gNB and N is the number of beams of the UE. To save the signaling overhead and improve the resource utilization, two subsets with reduced number of beams can be maintained both at the gNB and at the UE. For gNB, the cardinality of the subset of beams is denoted by M1, M1<M. For UE, the cardinality of the sub set of beams is N1, N1<N.

The problem behind is to find a mapping, from the current established beam (pair) to a new beam pair. This process is completed by beam training. To further simplify the problem, a mapping between the current established beam pair to two subset of beams, one at gNB, the other at UE, should be built for beam measurement. The ML is able to completed this task. A schematic figure is shown in Fig. 2.

Figure 2. ML assisted Beam Training. The connection with a gNB is established.  The gNB-UE beam pair is the input for the ML model to deduce the output. The output are two subsets of the beams for beam measurement at the gNB and the UE, respectively.

The subsets of beams are constructed with the correlation between the established beam pair and the beams in the subset. The overhead for beam training is reduced with the decreased number of beam measurement.

Proposal 2: The subsets of beams at the gNB side and UE side, can be constructed with an ML model to reduce the beam training overhead.

  - Predictive Beam Switching

For the fast moving UEs, a fast beam switching is necessary. The beam should be robust to reduce the probability of further beam switching. As UE moves, its measurements of the reference signals from different beams vary. The practice right now is to select the beam with the strongest signal strength. This is not the best policy for a moving UE. ML comes to help.

The ML model can be deployed at the gNB side.

- The input is the historical information, e.g., the historically connected beams.

- The output is the beam to be connected at next future time unit.

Figure 3. An ML model for predictive beam switching.

An intuition is that the beam switching is bounded to the UE positions. The UE’s next future movement can be predicted, so is the connected beam in the next future time unit. To some degree, the position information can reveal the user privacy. To avoid the privacy problem, the position information can be hidden, or not involved. The beam becomes narrow, as the frequency grows higher. The beam itself contains the spatial and direction information. A better choice for beam switching based on ML is using the historically connected beams as input, and output the beam to be connected at next future time unit, without predicting the UE position.

Proposal 3: The UE position information is not necessary for predictive beam switching.

This case can be further discussed in inter-cell beam switching and intra-cell beam switching, in order to benefit beam switching and reduce latency.

Proposal 4: The predictive beam switching shall be discussed in sub use cases of inter-cell beam switching and intra-cell beam switching for latency reduction.

  - Beam Failure Recovery

The beam management based on ML is able to select a proper and robust beam, to reduce unnecessary beam switching, even to reduce the beam failure occurrences. However, the beam failure may happen. The beam failure recovery mechanism is necessary. The current beam failure detection is relying on a counter. If the detected hypothetical BLER is continuously larger than a threshold within a time duration, the beam failure is considered to occur and the beam failure recovery request will be triggered.

The counter based beam failure detection is usually not accurate and is time consuming. Thus, it is helpful to improve the beam failure recovery with ML. An ML model for beam failure detection is shown in Fig. 4. The model can be deployed at the UE side, or at the gNB side.

An ML model for beam failure detection

- The input is historical measurements of the beams under beam failure detection.

- The output is the “0/1” indicating whether beam failure occurs with the beam.

Figure 4. The beam failure detection based on ML.

If the model is deployed at the UE side, one the standard impact comes from the ML model downloading. On the other hand, if the model is deployed at the gNB side, the foreseeable standard impacts are the measurement reporting and the new beam indication.

Proposal 5: The beam failure detection performance can be enhanced by an AI/ML model based on historical beam measurements.

More functions can be added into this simple ML model. Such as the candidate beam to switch to, which is usually denoted as qnew. An ML model with more functions is shown in Figure 5.

An ML model for beam failure recovery

- The input comprises two parts.

  - One is the historical measurements on the beam under beam failure detection.

  - The other part is historical measurements on the beams in the candidate beam sets.

- The output indicates whether beam failure occurs with beam under beam failure detection. If beam failure occurs, the suggested beam from the ML model output will be chosen as qnew.

Figure 5. The beam failure recovery based on ML.

qnew should be with good channel quality, which is often uncorrelated to the beam on which beam failure occurs. If qnew is correlated to the beam on which beam failure occurs, qnew is with bad channel conditions. The new candidate beam selection become meaningless. Thus, the independent relationship between qnew and the beam on which beam failure occurs, can be investigated by ML. That’s why they can be integrated together in Fig. 5.

Proposal 6: The new candidate beam qnew can be jointly determined by an ML model when beam failure occurs.

- Conclusions

In this contribution the following proposals been proposed:

Proposal 1: The configuration of SSB beam scanning at initial access stage can be improved by ML.

Proposal 2: The subsets of beams at the gNB side and UE side, can be constructed with an ML model to reduce the beam training overhead.

Proposal 3: The UE position information is not necessary for predictive beam switching.

Proposal 4: The predictive beam switching shall be discussed in sub use cases of inter-cell beam switching and intra-cell beam switching for latency reduction.

Proposal 5: The beam failure detection performance can be enhanced by an AI/ML model based on historical beam measurements.

Proposal 6: The new candidate beam qnew can be jointly determined by an ML model when beam failure occurs.

References

- RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #94e, Dec. 6 - 17, 2021.