3GPP TSG RAN WG1 #109	R1-2204570

e-Meeting, May 09th – May 20th, 2022

Agenda item:		9.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	ML terminology, descriptions, and collaboration framework

Document for:		Discussion and Decision

# Introduction

Rel-18 study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, agreed in [1], includes the following objectives:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced

In this contribution, we discuss details of ML terminology, descriptions, generic collaboration framework, and common evaluation methodology/KPIs.

# Discussion

## ML notation/terminology and description

As one may notice, there is a massive variety of terms related to ML. At the preliminary phase of the Rel.18 study, some companies used different terms assuming the same or similar meaning. Company-specific definitions for the same terms and different terms with the same meaning may notably complicate the discussion during the study.

Observation 1: Different understanding of terms and notations related to AI/ML may complicate the discussion in RAN1

The definitions for specific terms heavily depend on the understanding of underlying processes. Thus, final alignment on terminology may not be possible without sufficient progress in the use cases. However, it would be useful to create and maintain a list of temporary definitions throughout the discussion.

Proposal 1: RAN1 maintains a list of ML-related terms and definitions. Terminology in Annex A could be used as a starting point.

Some terms, like model deployment, could be misaligned with the discussion and agreements in other 3GPP groups, e.g., SA5. To avoid confusion, RAN1 could agree that terminology maintained during the study is only intended to facilitate understanding between companies. At the final stage of the study, RAN1 needs to align the terminology and the underlying assumptions with other related groups.

Proposal 2: RAN1 agrees that the terms used in this study are valid only for the air interface, at the final stage, some adjustments in terminology may be needed with other 3GPP groups.

An initial differentiation between the Reinforcement Learning (RL) -based algorithms and the other Supervised (SL) and Unsupervised algorithms might be needed in order to simplify the analysis of the ML-enabled solutions. This differentiation is motivated based on the use cases to be addressed [1] and also follows closely the ISO categorization [6]:

- The RL-based architectures operate typically with sequence(s) (time- or spatial- ordered series) of input data and generate output data in the form of sequence(s) of data and/or actions and/or data categories. These algorithms require an initial training/exploration, and during normal operation, the exploration and exploitation modes are run periodically, which leads to changes in the internal parameters of the algorithm.

- The Supervised CNN/DNN ML architectures typically operate with snapshots (time or spatial) of input data and generate output data in the form of snapshots of data categories and/or output data probabilities/likelihoods. These algorithms require training, and their structure (weight, layers) remains unchanged during the inference mode.

- The Unsupervised ML architectures do not require labeled training data. The training process includes continuous iterations over through an algorithm to identify similarities, patterns, or anomalies. Unsupervised algorithms typically operate with snapshots (time or spatial) of input data and generate output data in the form of snapshots of data categories and/or output data probabilities/likelihoods. ML often results in knowledge discovery.

Model generation typically starts from initialization. Initialization includes defining the model type and related hyperparameters, e.g., for ANN, those could be the number of network layers, the width of each layer, and the type of activation function.

For supervised and unsupervised algorithms, the next step after initialization is training. Training is a process of determining parameters of a machine learning model that best represent features and patterns in the training data. Training data is typically split into a training set and a validation set. Validation is a subprocess of training to evaluate the quality of a machine learning model using the validation set that helps ensure that the model’s performance generalizes beyond the training set.

Before inputting into the ML model, training data often needs to be pre-processed (e.g., data cleaning, formatting/normalization, and transformation). For example, if the ML model takes RSRP values as an input, those can be normalized in [0,1] instead of inputting values in dBm. Similar to inputs, the outputs of the model may need to be post-processed for their further utilization towards the real-life problem. According to [TR 37.813] ML model training function is responsible for data preparation. Data pre-/post- processing rules are model specific.

The training process for supervised learning relies on labelled datasets organized as pairs of input variables and “true” outputs. During the supervised learning process, the algorithm is fitted to the inputs and outputs to produce a model [6]. For unsupervised algorithms, the training process is based on continuous iterations over an algorithm. For example, for the K-means algorithm, samples are continuously iterated over the algorithm until the minimum distance from each sample to a centroid is achieved. In dimensionality reduction tasks using, for example, the principal component analysis (PCA) algorithm, the variance of each feature in the input data is calculated, and a predefined number of features with the highest variances are returned. [6]

Testing is a process of evaluating the performance of a model using a dataset different from the one used for model training and validation. Unlike the validation, the testing does not assume subsequent model tuning. Sometimes testing could be used in the training process to avoid ML model overfitting. Also, testing could be used to evaluate the performance of the model for the different contexts (i.e., different deployment scenarios). In such a case, testing allows deciding about the applicability of the model in another context.

Contrary to supervised and unsupervised algorithms, there is no explicit “training” in RL theory. Sometimes training can be mapped to the exploration of exploitation coefficients that are performed during runtime (inference). However, this is not an accurate mapping. For RL algorithms, the exploration can be a life-long (infinite) process. It is more accurate to say that RL training and inference are combined and start after the model is initialized. Nevertheless, RL algorithms which use a supervised model component, such as Deep RL, would need training similar to a classical supervised learning algorithm. In addition, the RL algorithms also require a proper initialisation of the internal parameters, which can be achieved for example via an extensive exploration phase (initial training).

A trained (for supervised and unsupervised algorithms) or initialized (for RL) model can be further used for inference. Inference is a process of making predictions by applying a model to the input data. Inference also requires the same data pre-/post- processing as training.

Naturally, there can also be ML-enabled solutions that use more than one ML algorithm and/ or a combination of the above-listed architecture types, for example, Deep RL.

Observation 2: Targeting one approach for all ML algorithms when studying functional architecture or collaboration options may negatively impact the clarity of deliverables and complicate the discussion.

Proposal 3: RAN1 at least to differentiate RL-based algorithms from other types of ML algorithms.

## Generic collaboration framework

As captured in [1], the application of AI/ML to wireless communications has been thus far limited to implementation-based approaches, both at the network and the UE sides. Such cases do not require specification changes, and hence they are out of the scope of this 3GPP study. There could still be some information exchange between network entities (i.e., UE, gNB) for the purpose of ML-enabled functions, but that is performed over existing interfaces and messages or at the application level.

The motivation for enabling in 3GPP a collaboration mode that requires specification should be based (if applicable) on its superior performance over both non-ML baseline and implementation-based ML (no specification change needed) with respect to agreed KPIs.

Proposal 4: RAN1 will support only the collaboration-based solutions if they outperform implementation-based ML solutions and/or non-ML baselines.

Figure 1 and Figure 2 provide high-level details of the possible collaboration options to be captured in the specifications. Depending on the use case, collaborating entities (i.e., UE, gNB) may use interfaces or host some of the functions captured in those figures.

Figure 1 ML-enabled function collaboration options (except RL)

1. Data collection from UE or gNB for the training of ML model or/and inference. The data collection can be controlled via RRC (e.g., the data collection control message could specify triggers, volume, periodicity, criteria, filtering, storage, formats, validation, etc.).

- NOTE: The inference output of one entity (e.g., UE) also could be communicated via data collection interface if it is used as inference input at another entity (e.g., gNB)

2. One entity (e.g., gNB) can share or configure the ML model in another entity (e.g., UE), and the feedback from either side can be collected.

3. Assistance information may include any data that support ML but is not used for training or inference. E.g., UE may indicate to gNB that some function is driven by ML or inform about the selected ML model.

4. One entity (e.g., gNB) may run ML and may impact an ML-enabled function at another entity (e.g., UE) via providing inference output.

For reinforcement learning, the collaboration options may be slightly different (Figure 2).

Figure 2 RL-enabled function collaboration options

1. ML configuration and initial exploration to configure the main functions and initialize the RL-based model.

2. Data collection (run-time)

- The data collection can be controlled via RRC (e.g., a data collection control message could specify triggers, volume, periodicity, criteria, filtering, storage, formats, validation, etc.).

- Different nodes might use a different source of the data, e.g., UE can use local measurements

- The data collection-exploration-exploitation functions need to be run in a continuous loop.

3. Exploration assistance information may include any data that support RL operation. E.g., multi-agent RL operation requires coordination of the exploration-inference operating modes.

4. One entity (e.g., gNB) that runs RL may impact an ML-enabled function at another entity (e.g., UE) via provided inference output.

NOTE: The collaboration options captured in Figure 1 and Figure 2 are not limited to UE-gNB collaboration and could also be applicable to collaboration between other entities, e.g., UE-UE (or UE-LMF for positioning case).

Collaboration options proposed for different use cases may target the same signaling interfaces. Their mapping to the high-level schemes captured above helps crystalize and generalize the specification impact among different proposals.

Proposal 5: RAN1 defines and maintains possible collaboration options and uses them to map the collaboration in the use-cases under study.

## Common evaluation methodology and datasets

### Documenting evaluation assumptions

Typically, for 3GPP RAN1 simulations, the configuration parameters and assumptions are provided by the contributing company. For the evaluation of ML-enabled function(s), a separate set of assumptions must be provided in addition to the usual system/link-level simulation assumptions. This includes information required to reproduce the ML-specific simulation/experiment and ultimately also to be able to interpret and compare the results from the different ML-enabled solutions (calibration and performance comparison).

For RAN1 evaluation purposes, it is beneficial to describe the proposed ML-enabled solutions using a common high-level approach. For example, a generic “ML-enabled function” can be defined, which includes the following main processing blocks which need to be described as part of the evaluation methodology:

- Input data acquisition and preparation/pre-processing (data collection, formatting, cleaning, feature engineering, etc.)

- ML-algorithm (algorithm selection, hyperparameters, the training & validation procedure, the inference procedure, etc.)

- Non-ML algorithm (optional algorithms such as measurement filtering, channel estimation, etc.)

- Output data processing (optional, combination of ML and non-ML algorithm outputs)

- Control mechanism (signaling and parameters used to configure the blocks 1-4)

When describing a particular (sub)use case (or ML-enabled function), details need to be provided for (at least) these 5 processing blocks. Furthermore, for the training and inference operating modes, some of these processing blocks have to be described separately. The exact format to document these processing blocks needs to be decided such that they are applicable to all targeted (sub-)use cases [1].

Observation 3: For RAN1 evaluation purposes, it is beneficial to describe the proposed ML-based solutions using a common high-level approach.

Proposal 6: RAN1 to adopt a high level description of the ML-based solutions using a defined set of processing blocks, including at least the description of their input and outputs data, type of algorithm, hyperparameters, and control mechanisms used.

### Complexity

The newly developed and studied ML-enabled solutions must also be analyzed in terms of their complexity. The higher performance disclosed in RAN system or link-level simulations needs to be balanced against the potential increase in computational complexity introduced with the use of ML algorithms.

Observation 4: It is difficult to compare the complexity of ML-enabled solutions vs. non-ML baseline solutions since the latter are rather implementation-specific.

Proposal 7: The RAN1 complexity comparison is to be performed between the different ML-enabled solutions proposed for the same function (sub-use case).

In general, the complexity (and energy consumption) of ML algorithms has been widely investigated in the literature e.g. [3]. These studies show that the correlation between the number of parameters and FLOPs for CNNs is 0.772, and the correlation for transformers is 0.994. Therefore, the FLOPs metric can be used as a reasonable indicator of the model complexity. It is, however, further highlighted that FLOPs for DNNs can sometimes be misleading due to underlying optimizations at the firmware, frameworks, memory, and hardware.

In the case of ML solutions running on embedded platforms with limited compute resources, like in the case of a UE, it is highlighted that the complexity evaluation/comparison procedure needs to consider: latency, accuracy, and energy consumption [4]. Furthermore, reference [4] also presents an established and detailed framework for the comparison of embedded ML solutions.

Based on the above findings, we analyze in the following how the complexity analysis would need to be carried out for the ML-enabled solutions developed and evaluated in RAN1.

First, for basic ML model analysis, comparability, and interoperability purposes, RAN1 may utilize the industry-adopted ONNX format [2].

Observation 5: For basic ML model comparability and interoperability purposes, RAN1 could use the ONNX format.

We consider the complexity analysis of an ML-enabled solution primarily to be an average estimation vs. time. For example, peak computes requirements can be estimated separately, depending on the needs of the specific (sub)use cases.

Observation 6: The complexity analysis of an ML-enabled solution primarily is an average complexity estimation vs. time.

It might also be beneficial to differentiate between the analysis of NG-RAN and UE side ML-enabled solutions such that the embedded system particularities can be described. For collaboration-based solutions (see Section 2.2), the analysis of the aggregated complexity of the algorithms at the NG-RAN and UE sides needs to be provided.

For an ML-enabled solution that requires training of the ML algorithm at the gNB or UE, the complexity analysis should consider the training and inference separately from operating modes. For example, when training of such algorithms is performed typically ‘off-line’ using dedicated, and high compute solutions, the inference is executed in NG-RAN and/or UE network nodes with more limited compute resources. Nevertheless, the training requires potentially large amounts of input data (cleaned, formatted, and labeled), which need to be collected by NG-RAN and/or UE, and these might imply a large signaling/traffic overhead on the air interface, which in the end limits the benefits of the ML-enabled solution.

Proposal 8: The RAN1 complexity estimation of an ML-enabled function should include the analysis of both training and inference operating modes.

The ML model/algorithm is just one building block of an ML-enabled function and proposed solution (see Section 2.3.1).  The complexity of an ML-enabled function is not only in the ML algorithm but also in additional signaling (air interface use), input/ output data pre-/post processing, and required configuration procedure (gNB and/or UE).

Observation 7: The overall system-level complexity of an ML-enabled function includes in addition to the complexity of the ML model itself (e.g. FLOPs), the estimated complexity of: signaling (air interface use, overhead), input/ output data pre-/post processing resources, and required configuration procedures for training and inference.

Proposal 9: RAN1 to consider including at least the following items in the complexity analysis of ML-enabled solutions:

- Training or (initial training/exploration for RL)

a.	Number of floating-point operations required for one iteration (forward-backward) of the ML-algorithm

b.	Number of required training iterations (steps and epochs) to reach the training performance/accuracy

c.	Alternatively, to a) and b), the floating-point operations per second needed to run the training

d.	Memory footprint of the ML algorithm (Mbit)

e.	Memory footprint of the potentially required input and output data storage (Gbit)

f.	Number of floating-point operations required to prepare (and format, convert) the input data in case these are not direct measurements or estimates readily available in the radio entity executing the ML-enabled function

g.	Estimated number and payload (bytes) of additional signalling messages required to convey the ML-input and ML-output information between the involved radio entities (gNB and UE)

- This might be complemented by the estimated required ML-input and ML-output data rates, i.e., factoring in the acceptable transmission delays

- Inference (or exploration/exploitation for RL)

a.	Number of floating point operations required for one forward pass of the ML-algorithm

b.	Alternatively, to a), the floating-point operations per second needed to run the ML algorithm for (X) seconds

c.	Number of floating-point operations required to prepare (and format, convert) the input data in case these are not direct measurements or estimates readily available in the radio entity executing the ML-enabled function

d.	Estimated number and payload (bytes) of additional signalling messages required to convey the ML-input and ML-output information between the involved radio entities (gNB, UE)

- This might be complemented by the estimated required ML-input and ML-output data rates, i.e., factoring in the acceptable transmission delays

### Datasets

During the Rel.18 preparations, some companies proposed to create common datasets for the study. Common datasets set assumes that companies open source some of their data. There are some benefits that could be envisioned in having common datasets in 3GPP for this study. Particularly, those can be used to calibrate different models (i.e., compare their performance). In some cases, common datasets may indicate the benefits of having a specific collaboration (e.g., between UE and gNB) in terms of accuracy and speed of training.

On the other hand, pursuing common datasets in 3GPP can be resource-demanding. Particularly, companies need to agree on the common format of the data and metadata, establish mechanisms for ensuring data quality (including potentially automated tests), and version control. Also, it is important to capture the assumptions/context behind the data, which can be difficult. Also, it is worth noting that not all ML algorithms can use offline data.

Observation 8: Generating common datasets proactively may waste resources, but it can be beneficial for some concrete sub-use cases later during the study.

For this study, we could rely on synthetic data generated with simulators. Simulation allows for faster prototyping and evaluation of solution candidates, which is crucial for the study. Alignment between companies on the assumptions behind the field data could be challenging. However, after sufficient progress with the use cases, field data or prototyping can be used for their advanced evaluation.

Proposal 10: RAN1 to use simulator data for the study, after sufficient progress and the convergence on the solutions, evaluation with field data can be discussed.

# Conclusion

This contribution provides details on ML terminologies, description, generic collaboration framework, and common assumptions for evaluations. Observations and proposals are as follows,

Observation 1: Different understanding of terms and notations related to AI/ML may complicate the discussion in RAN1

Proposal 1: RAN1 maintains a list of ML-related terms and definitions. Terminology in Annex A could be used as a starting point.

Proposal 2: RAN1 agrees that the terms used in this study are valid only for the air interface, at the final stage, some adjustments in terminology may be needed with other 3GPP groups.

Observation 2: Targeting one approach for all ML algorithms when studying functional architecture or collaboration options may negatively impact the clarity of deliverables and complicate the discussion.

Proposal 3: RAN1 at least to differentiate RL-based algorithms from other types of ML algorithms.

Proposal 4: RAN1 will support only the collaboration-based solutions if they outperform implementation-based ML solutions and/or non-ML baselines.

Proposal 5: RAN1 defines and maintains possible collaboration options and uses them to map the collaboration in the use-cases under study.

Observation 3: For RAN1 evaluation purposes, it is beneficial to describe the proposed ML-based solutions using a common high-level approach.

Proposal 6: RAN1 to adopt a high level description of the ML-based solutions using a defined set of processing blocks, including at least the description of their input and outputs data, type of algorithm, hyperparameters, and control mechanisms used.

Observation 4: It is difficult to compare the complexity of ML-enabled solutions vs. non-ML baseline solutions since the latter are rather implementation-specific.

Proposal 7: The RAN1 complexity comparison is to be performed between the different ML-enabled solutions proposed for the same function (sub-use case).

Observation 5: For basic ML model comparability and interoperability purposes, RAN1 could use the ONNX format.

Observation 6: The complexity analysis of an ML-enabled solution primarily is an average complexity estimation vs. time.

Proposal 8: The RAN1 complexity estimation of an ML-enabled function should include the analysis of both training and inference operating modes.

Observation 7: The overall system-level complexity of an ML-enabled function includes in addition to the complexity of the ML model itself (e.g. FLOPs), the estimated complexity of: signaling (air interface use, overhead), input/ output data pre-/post processing resources, and required configuration procedures for training and inference.

Proposal 9: RAN1 to consider including at least the following items in the complexity analysis of ML-enabled solutions:

- Training or (initial training/exploration for RL)

a.	Number of floating-point operations required for one iteration (forward-backward) of the ML-algorithm

b.	Number of required training iterations (steps and epochs) to reach the training performance/accuracy

c.	Alternatively, to a) and b), the floating-point operations per second needed to run the training

d.	Memory footprint of the ML algorithm (Mbit)

e.	Memory footprint of the potentially required input and output data storage (Gbit)

f.	Number of floating-point operations required to prepare (and format, convert) the input data in case these are not direct measurements or estimates readily available in the radio entity executing the ML-enabled function

g.	Estimated number and payload (bytes) of additional signalling messages required to convey the ML-input and ML-output information between the involved radio entities (gNB and UE)

- This might be complemented by the estimated required ML-input and ML-output data rates, i.e., factoring in the acceptable transmission delays

- Inference (or exploration/exploitation for RL)

a.	Number of floating point operations required for one forward pass of the ML-algorithm

b.	Alternatively, to a), the floating-point operations per second needed to run the ML algorithm for (X) seconds

c.	Number of floating-point operations required to prepare (and format, convert) the input data in case these are not direct measurements or estimates readily available in the radio entity executing the ML-enabled function

d.	Estimated number and payload (bytes) of additional signalling messages required to convey the ML-input and ML-output information between the involved radio entities (gNB, UE)

- This might be complemented by the estimated required ML-input and ML-output data rates, i.e., factoring in the acceptable transmission delays

Observation 8: Generating common datasets proactively may waste resources, but it can be beneficial for some concrete sub-use cases later during the study.

Proposal 10: RAN1 to use simulator data for the study, after sufficient progress and the convergence on the solutions, evaluation with field data can be discussed.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” 3GPP RAN #94-e.

- “Open Neural Network Exchange”, April 2022.

- Radosvet Desislavov et al, “Compute and Energy Consumption Trends in Deep Learning Inference”, 2021.

- Colby Banbury et al, “MLPerf Tiny Benchmark”, 2021.

- ISO/IEC “AWI 22989: Artificial Intelligence Concepts and Terminology”, Draft, April 2022.

- ISO/IEC “AWI 23053: Framework for Artificial Intelligence Systems Using Machine Learning” , Draft, April 2022.

# Annex A

- A multilayer perceptron (MLP) is a class of feedforward artificial neural network (ANN).

- Artificial Intelligence is a capability of an engineered system to acquire, process and apply knowledge and skills. Discipline which studies the engineering of systems with the capability to acquire, process and apply knowledge and skills. [ETSI GR ENI 004, ISO/IEC FDIS 22989]

- Batch (in NN) is a hyperparameter (batch size) that defines the number of samples to work through before updating the internal model parameters. A training dataset can be divided into one or more batches. A batch called mini-batch if the batch’s size is higher than one and lower than the size of training data.

- Catastrophic forgetting (or catastrophic interference phenomenon): happens during the continual acquisition of incrementally available information from non-stationary data distributions, leading to the interference of the new infor­mation with what the model has already learned.

- Continual learning: fundamental idea in machine learning in which models continuously learn and evolve based on the input of increasing amounts of data while retaining previously learned knowledge

- Convolutional layer. It is another kind of layer used in FFNs, and more precisely in CNNs. Similarly, to a fully-connected layer, it filters the input by a linear operation, namely a convolution, then applies a non-linearity, and finally forwards the result. However, each neuron needs not be connected to all neurons in the following layer.

- Data collection is a function providing data collected from the network nodes, management entity, or UE, as a basis for ML model training, data analytics, and inference. The examples of such data may include measurement reports, configurations, ML model parameters (e.g., for federated learning), inference output, etc.

- Deep learning (DL) is a subset of machine learning where an artificial neural network contains more than one hidden layer.

- Epoch means that learning algorithm worked through the entire training dataset. The number of epochs is an ML hyperparameter that defines how many times the learning algorithm work through the entire training dataset.

- Federated Learning is a distributed machine learning approach that trains machine learning models using decentralized examples residing on different devices (e.g., smartphones). In federated learning, a subset of devices downloads the current model from a central coordinating server. The devices use the examples stored on the devices to make improvements to the model. The devices then upload the model improvements (but not the training examples) to the coordinating server, where they are aggregated with other updates to yield an improved global model. After the aggregation, the model updates computed by devices are no longer needed, and can be discarded. (this definition adopted from [Google Inc., Machine learning glossary] with some adjustments).

- Feed-forward Neural Networks (FNN) are neural networks in which each neuron is connected only to the neurons in the following layer and thus the input data can only propagate forward, from the input layer to the output layer, without the possibility of any feedback loop.

- Fully-connected layer is the typical layer employed in FFNs, which is characterized by the fact that each neuron of the layer receives an input from all neurons of the preceding layer and is connected to all neurons of the following layer. The input data is first linearly processed, then passed through a non-linearity, and finally propagated to the following layer.

- Hyperparameters are characteristics of an ML algorithm that affect its learning process. Examples of hyperparameters for neural networks include the number of network layers, the width of each layer and the type of activation function [ISO/IEC FDIS 23053]

- Inference is a process of making predictions by applying the trained model to the input data.

- Input Data Labelling: Input data labelling is the process of identifying raw data (images, text files, videos, etc.) and adding one or more meaningful and informative labels to provide context so that a machine learning model can learn from it in training process.

- Machine learning (ML) is a process of using computational techniques to enable systems to learn from data or experience [ISO/IEC FDIS 23053]

- ML application is a deployable software package that implements ML-based application feature as an application version of the training and inference pipelines.

- ML framework is a programming and runtime interface to be used to build ML models, train and for running the inference. It implements the re-usable ML training algorithms and is used in runtime as a library or as a service.

- ML Inference is a process of using a trained ML model to make a prediction or guide the decision based on collected data. [3GPP TR 37.817]

- ML model applicability means for what product instances the model has been trained for. A model can be applied e.g., on only one cell or all gNBs of certain version where it performs most accurately. Thus, the model applicability depends solely on the Use Case/feature. Model can be tested with other data to check/extend its applicability beyond the context it has been trained for.

- ML model deployment is a function of initial deployment of a trained, validated, and tested AI/ML model to the Model Inference function or of delivering an updated model to the Model Inference function (adopted from [3GPP TR 37.817] with minor edits)

- ML model generation is a process that includes model initialization, training, and validation.

- ML model initialization is a step of ML generation to define type of model and its hyperparameters.

- ML model testing is a process of evaluating performance of a model using a dataset different from one used for model training and validation. Differently from ML model validation, testing do not assume subsequent tuning of the model.

- ML model training is an online or offline process to train an ML model by learning features and patterns that best present data and get the trained ML model for inference. [3GPP TR 37.817]

- ML model validation is a subprocess of training, to evaluate the quality of a machine learning model using the validation set, that helps ensure that the model’s performance generalizes beyond the training set (this definition adopted from [Google Inc., Machine learning glossary] with some adjustments).

- ML Model is a data driven algorithm by applying machine learning techniques that generates a set of outputs consisting of predicted information, based on a set of inputs [3GPP TR 37.817]

- ML-controlled process is a software/hardware module (e.g., antenna module) that takes action based on inference output and provides ML model performance feedback (optionally). Controlled process may also contain a default controller for the process (without ML), that can be used instead of ML (e.g., when ML fails).

- ML-enabled function is a PHY/MAC/RRC or entire RRM function, implemented in the gNB and/ or UE, which relies on any combination of non-ML (rule based) and ML-based algorithms.

- MLOps is a framework for developing, training, testing, deploying and maintaining ML algorithms life-cycle

- Neural network (NN) (or artificial neural network (ANN)) is a model that is composed of layers consisting of simple connected units (neurons) followed by nonlinearities. The information to map the inputs to outputs is stored to weights and biases and is learned through error reduction.

- Neural network hidden layer a synthetic layer in a neural network between the input layer (that is, the features) and the output layer (the prediction). Hidden layers typically contain an activation function (such as ReLU) for training. A deep neural network contains more than one hidden layer. [Google Inc., Machine learning glossary]

- Neural network input layer is the first layer in a neural network, that receives the input data. [Google Inc., Machine learning glossary]

- Neural network output layer is the "final" layer of a neural network. The layer containing the answer(s). [Google Inc., Machine learning glossary]

- Offline learning is a training process that uses static data in a non-operational environment i.e., in a software development phase. A baseline type of model may be produced at this stage that is used as is for the lifecycle of the application of re-trained in runtime.

- Online learning is a training process which is run in runtime to adapt to the monitored environment/ phenomenon using the new observations with the goal of improving the accuracy of the output of the inference. Note: the online training is optional in cases that the mapping function can be described in offline phase and there is no need for re-training during runtime.

- Open Neural Network Exchange (ONNX) is an open format built to represent machine learning models [onnx.ai]. ONNX helps to solve the challenge of hardware dependency related to AI models and enables deploying same AI models to several HW accelerated targets.

- Overfitting is creating a model which fits the training data too precisely and fails to generalize on new data. Overfitting can occur because the trained model has learned from non-essential features in the training data (i.e. features that do not generalize to useful outputs), excessive noise in the training data (e.g. excessive number of outliers) or because the model is too complex for the training data. Overfitting can be identified when there is a significant difference between errors measured on training data and on separate test and validation data. The performance of overfitted models is especially impacted when there is a significant mismatch between training data and production data. [ISO/IEC FDIS 23053]

- Pipeline is a set of processes that fulfils the application requirements for training and inference as a set of stereotypical functions that are implemented ideally as re-usable (whenever possible) software components. The pipeline is expected to have a structure a directed acyclic graph to allow building complex use-case scenarios. The implementation may be done explicitly as a set of dependent services or built as a set of libraries compiled in a single binary. Some ML frameworks offer Pipeline implementations to execute operation graphs.

- Pooling layer is a layer usually used in CNNs which operates by dividing the input data into blocks, and then selecting either the maximum element of each block or computing the average of the elements within each block.

- PyTorch an open source machine learning framework that accelerates the path from research prototyping to production deployment, including a rich ecosystem of tools and libraries

- Quantization is the process of approximating a neural network that uses floating-point numbers by a neural network of low bit width numbers. This reduces both the memory requirement and computational cost of the inference.

- Recurrent layer is the typical layer of RNNs. After performing an affine combination of the input and passing it through a non-linearity, the output is not just propagated forward, but a feedback loop is also present.

- Recurrent Neural Networks (RNN) are neural networks in which feedback loops are allowed, and the output of a neuron can become the input of the same neuron, as well as of other neurons in the same or in a previous layer.

- Reinforcement Learning (RL) is an area of machine learning concerned with how intelligent agents ought to take actions in an environment in order to maximize the notion of cumulative reward. Reinforcement learning is one of three basic machine learning paradigms, alongside supervised learning, and unsupervised learning.

- Supervised learning is a process of training a model from input and its corresponding labels. Here, the domain expert labels the data and the ML is capable to capture the human knowledge.

- Tensor is a data structure similar to arrays and matrices. In machine learning tensors are used to encode the inputs and outputs of a model, as well as the model’s parameters.

- Tensorflow an end-to-end open source machine learning platform, including an ecosystem of tools, libraries and community resources

- TinyML is a designation of ML model architectures suitable for execution of training and/or inference on resource (power and memory) constrained devices and edge implementation platforms [tinyML Foundation, https://www.tinyml.org/]

- Unsupervised learning is a process of training a model without labelled data e.g., clustering is a common example of this.