3GPP TSG RAN WG1 #109	R1-2204572

e-Meeting, May 09th – May 20th, 2022

Agenda item:		9.2.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Other aspects on AI/ML for CSI feedback enhancement

Document for:		Discussion and Decision

# Introduction

Rel-18 study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, agreed in [1], includes a use case for CSI feedback enhancement.

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment, model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced

In this contribution, we provide the details of two main sub-use cases within the use case of CSI feedback enhancement with AI/ML, namely CSI feedback compression with Autoencoders and CSI prediction.

# Discussion

## CSI feedback with Autoencoders

Autoencoders are, by definition, matching the problem of CSI feedback compression. Indeed, autoencoders are an unsupervised learning technique where a bottleneck is imposed in the network to force a compressed knowledge representation of the original input. The architecture of autoencoders is illustrated in Figure 1. The main challenge remains the reconstruction of the original input.

Before training the autoencoder, four hyperparameters, among others, need to be set: 1) the size of the codeword/bottleneck, 2) the number of layers, 3) the number of nodes per layer, and 4) the loss function to be used, e.g., mean squared error (MSE), cosine similarity. The number of nodes per layer typically decreases with each subsequent layer of the encoder and increases back in the decoder. The decoder is symmetric to the encoder in terms of the layer structure.

An autoencoder consists of three parts as described in Figure 1: 1) the encoder, 2) the bottleneck (codeword here), and 3) the decoder. The encoder aims at compressing the input data, in our case the channel matrix  H or the eigenvectors, into a codeword that is of dimension smaller than the original information. The bottleneck, in our case the codeword, is the compressed representation of the original information. The bottleneck is followed by the decoder, a module that decompress the codeword and reconstruct the data: the recovered information H. H is then compared to H. It is a lossy process, and the recovered matrix H will not be the same as H.

Figure 1: Autoencoder architecture.

In order to improve the encoding efficiency, the quantization of codewords is needed. Therefore, it is important to consider the quantization of the CSI after compression. The quantizer module is depicted Figure 2. The quantization process introduces quantization noise/distortion, it is then important to design an efficient quantizer that minimized the quantization noise. The methods proposed in the literature are not differentiable and cannot be included in the backpropagation of the training. To get optimal performance, the quantization module should be optimized with the overall structure of the neural network.

Figure 2: Lossy compression and recovery.

The autoencoder for CSI feedback in [2] can be considered a pioneering work in literature. Still, it needs a quantization step because no such quantization step is defined, and it only considers the dimensionality reduction of the feedback vector. There can be several ways to handle the quantization issue. A straightforward approach would be using a separate quantizer to quantize the feedback codeword, but this may lead to inferior performance. An alternative can be embedding the quantizer into the neural network and training the entire end-to-end chain. Some examples can be found in public reports [7] [8], which considers training tricks to handle the gradient vanishing issue for the embedded quantizer. Another proposed approach, VQ-VAE, uses vector quantized variational autoencoders (VQ-VAE) [5]. A vector quantization layer is explicitly defined in the scheme of VQ-VAE, and the VQ codebook is trained together with the neural network.

### Compressing channels

Figure 3: General conceptual structure of an Auto-Encoder based CSI feedback via compressing the full channel matrix H.

Considering a massive MIMO system with Nt transmit antennas at a gNB, Nr receive antennas at a UE, and Nc PRBs, the input size of the original channel matrix H is (Ns, Nc, Nt, 2Nr), where Ns is the number of samples. The real and imaginary parts of the channel matrix are extracted and stacked together (i.e., 2Nr) since ML models cannot handle complex inputs directly. The UEs measure and predict the channel matrix H, and apply ML models to compress and feedback the channel information to the gNB. The encoder model takes the Ns input channel matrices as images with a size of Nc×Nt pixels and 2Nr “feature maps” (as the RGB for color images). The encoder usually consists of a few convolutional layers to generate feature maps of the input channel matrix and then reshape the feature maps into a vector and compress it into a codeword. Then, the codeword is quantized and feedback to the gNB (as shown in Figure 3). At the gNB, the feedback codeword is the input to a pre-trained decoder model. The decoder decompresses the codeword and reconstructs the channel matrix H. Optimization algorithms like Adaptive Moment Estimation (ADAM) are used to update the set of parameters of the autoencoder-like model, where the loss function is the mean squared error (MSE) between the original and reconstructed channel matrices.

Instead of inputting the channel matrix H directly to the encoder at the UE side, pre-processing is needed to shorten the training period, compress the feedback bits, and improve the prediction accuracy. In our work, we first take a 3D inverse Discrete Fourier transform (IDFT) of the channel matrix across transmit antennas, receive antennas and frequency units, which transforms it from the spatial-frequency domain to the angle-delay domain and normalize the elements of H to a range of [0, 1]. The channel matrix in the angle-delay domain only has a small fraction of large components, and the other components are close to zero. Then we retain the first few rows of the channel matrix and remove the remaining rows (with large delays). At the gNB side, after the reconstruction of channel matrix H, zero padding is applied. Finally, inverse normalization and 3D DFT are applied to recover the reconstructed channel matrix back to its original scale.

### Compressing channel eigenvector(s)

We consider a typical MIMO system with Nt transmit antennas at the gNB and Nr receive antennas at UE side. Denoting K as the number of subbands consisting of KRB resource blocks (RBs) as the basic feedback granularity, the downlink channel can be written as H=[H1 , H2, …, HK], where Hk∈CNr×Nt, 1≤k≤K indicates the downlink channel of the kth subband.

With these definitions, one can refer to the conceptual block diagram of an Auto-Encoder based CSI feedback via compressing the full channel matrix H in Figure 3, where Normalized Mean-Squared Error is shown as an example objective function to be minimized, whereas other objective functions can also be used as discussed .

We can consider a single-stream downlink transmission and ideal channel estimation at the UE side, the corresponding eigenvector for the kth sub-band, denoted as vk∈CNt×1 with normalization ||vk|​2=1, will be directly used as the downlink precoding vector and can be calculated using eigenvector decomposition as HkHHkvk=λkvk, where λk represents the maximum eigenvalue of [HkHHk] and also indicates the precoding power gain obtained from MIMO system. Obviously, all K eigenvectors should be reported to the BS for downlink precoding. Hence total K×Nt complex coefficients should be compressed and recovered for each channel sample using various kinds of neural networks.

As the objective function to be minimized, the generalized cosine similarity (GCS) on the kth sub-band, denoted as ls,k, is utilized to evaluate the CSI feedback and recovery performance, which is written as ls,k=|vkHvk|vkvk where vk represents the recovered eigenvector for the kth sub-band. Furthermore, the average GCS on the whole band can be written as a loss function to be minimized during the optimization of the encoder’s weights by ls=-1Kk=1Kls,k. Obviously, we have 0≤ls,k≤1, and larger ls,k indicates better CSI feedback and recovery performance.

Figure 4 illustrates the general conceptual block diagram of an autoencoder-based CSI feedback with compressing the eigenvectors, where GCS is used as the objective function to be minimized in optimization of the auto-encoder.

Figure 4: General conceptual structure of NN (autoencoder) based CSI Feedback with Compressing the Eigenvectors.

The details of the computations of eigenvalues per sub-band are provided in Figure 5. For each sub-band, we form the covariance matrix snapshots by summing the products (HkHHk) over the PRBs included in each sub-band. Then we compute the eigenvalue decomposition and pick the eigenvector corresponding to the largest eigenvector of the covariance matrix snapshot matrix for each sub-band. In this manner, we obtain K (=number of sub-bands) eigenvectors of dimension Nt. In Figure 5 the number of sub-bands is taken as 13, and the number of PRBs per sub-band is taken as 4, and these parameter selections are provided as an example to facilitate the ease of illustration.

Figure 5: An illustration of the details of Eigenvector computation per sub-band.

Currently, there have been three major autoencoders proposed in [2],[3], and [4], for the aforementioned autoencoder blocks within Figure 3 and Figure 4. For that matter, [2] is one of the first major contributions that proposed autoencoder-decoder framework, as depicted in Figure 3, and the autoencoder block of [2] is called CsiNet. In CsiNet, conventional neural networks (CNNs) for the encoder and decoder are mainly utilized due to the fact that they can exploit local spatial correlation by enforcing a local connectivity pattern among the neurons of adjacent layers.

### Binary Variational CSI coding

A typical autoencoder-like implementation for CSI feedback includes an encoder implemented with a neural network on the UE side and a decoder implemented with another neural network on the gNB side. The channel feedback is the encoder output on the UE side. The decoder shall be driven by the gNB's received feedback from the uplink channel. As a result, the entire channel feedback process can be completed.

Quantization is a critical issue in NN-based CSI feedback solutions. The encoder output, which serves as channel feedback, must first be quantized before it can be sent to the gNB. The encoder's outputs are typically real-valued numbers. To digitize the CSI feedback, it is common practice to perform an additional quantization step. A high-resolution quantization will ensure that the CSI feedback information at the gNB is accurate. High-resolution quantization, on the other hand, increases feedback overhead, which degrades overall system-level performance. A low-resolution quantization, which has less overhead, may introduce a larger quantization error, affecting the performance of recovered CSI. There is a trade-off between channel information recovery quality and the associated overhead. This CSI quantization problem is, in general, a CSI compressing problem, which is the problem of maximizing CSI compression with the least amount of CSI feedback overhead while maintaining good CSI feedback quality.

It is possible to have a "built-in" quantizer as part of the encoder using the variational autoencoder (VAE) framework [6]. In fact, a binary latent variable can be used for VAE CSI coding thanks to the powerful approach of variational inference. In this contribution, we present a binary variational (biv) CSI coding scheme that can perform VAE operation and quantization at the same time.

The biv-CSI coding method can be thought of as an enhancement to the VAE algorithm for CSI compression. The VAE encoder can represent an input of CSI channel gains using binary CSI codes, where its dimension is a CSI compression parameter. The VAE decoder can reconstruct the channel information with the binary CSI codes as input. Neural networks will be used to implement both the VAE encoder and decoder. After training the biv VAE model, the VAE encoder will be deployed on the UE side, while the VAE decoder will be deployed on the gNB side. Based on the estimated channel gains, the UE can generate binary CSI codes using the VAE encoder. As CSI feedback, the dimension-reduced binary CSI codes must be sent back to gNB. The decoder is used on the gNB side to generate the reconstructed channel information. Figure 6 depicts the overall binary variational CSI coding.

Figure 6: Binary Variational (biv) CSI coding: overall structure.

Encoder, binary sampling, and decoder are the three basic components of biv CSI coding. Neural networks are used to implement the encoder and decoder. The biv CSI coding, like any neural-network-based AI/ML approach, has two operational phases: training and predicting. The encoder and decoder neural network models will be trained during the training phase. To complete the CSI feedback process, the predicting phase encodes a given CSI channel gain as a data sample into a binary CSI code, transmits the CSI code as CSI feedback, and decodes the CSI code into the reconstructed CSI.

During the training phase, the encoder takes an input x, as one data sample of CSI, which is usually a multi-dimension tensor. One typical example is a real-value tensor with a dimension of (nSubband, nTx, nRx, 2), where nSubband is the number of subbands, nTx is the number of Tx antenna ports, nRx is the number of Rx antenna ports, and 2 indicates the real and imaginary part of one complex value.

The output of the encoder is modelled as the log-likelihood ratio (LLR) vector, each component of which is an LLR value of one latent variable bit zj. The binary sampling unit converts the LLR vector into a sequence of binary output. The detailed operation follows a new approach called binary re-parameterizing trick. The output is a binary sequence, as binary CSI code, with each bit zj∈{0, 1}. With CSI feedback, the binary CSI code is fed back to the gNB for decoding. The decoder uses the CSI code z to generate the re-constructed CSI channel x.

The general approach in the biv CSI coding assumes that the latent variable z is a binary distribution (Bernoulli distribution). The encoder output uses the LLR value (or logit value) to represent the Bernoulli distribution z~Bernoulli(p, 1-p):

λ=logp1-p.

Note that the single LLR value λ determines the Bernoulli distribution.

The question is how to sample the Bernoulli distribution to yield a binary z from the λ. Direct sampling with λ will have problem in the gradient descent algorithm for neural network training because it is not easy to derive the gradient for a random variable. Instead, a binary re-parameterizing trick is derived to yield binary z from λ. The binary sampling operation converts the LLR values from the output of the encoder into a binary vector. This operation is defined as

z=sgnλ+σϵ,

where λ is the LLR value from the encoder, σ is a constant, ϵ is an independent gaussian random variable with zero mean and variance matrix as identity matrix ϵ~N(0,I), and sgn(⋅) is the sign function.

Figure 7: Computing of binary sampling: forward and backpropagation.

The general VAE is an optimization algorithm to maximize the evidence lower bound (ELBO) for the posteriori probability, as

logpθ(x)≥Lθ, ϕx=Ez~qϕzx-logqϕzx+logpθxz+logpθz.

With single data point, the VAE ELBO can be approximated as the sum of three items

Lθ, ϕx=logpθ(x|z)+logpθ(z)-logqϕ(z|x).

In this equation, we use parameter ϕ to indicate the parameterized encoder, and θ to indicate the parameterized decoder. The overall ML problem becomes an optimization problem: given one sample x, find parameters (θ,ϕ ) to maximize the objective function Lθ, ϕx. For the biv CSI coding, the data sample x captures the underlying channel and is usually a multi-dimension tensor.

First, we assume that the latent variable z is a Bernoulli distribution: z~Bernoulli(p, 1-p). At the second step, we use logit value (LLR) λ as the output to represent the Bernoulli distribution:

λ=logp1-p.

At the last step, the encoder can be calculated:

logqϕ(z|x)=jλjzj-log1+ejλ, where zj∈{0, 1}.

Note that λ and z are vectors with dimension of J. The dimension J of the binary latent variable defines the size of the CSI coding. A smaller J indicates a lower CSI feedback overhead.

The decoder from the objective function includes two terms, the conditional probability pθ(x|z) and the a priori probability of latent variable pθ(z). The conditional probability pθ(x|z) given one binary latent variable value can be obtained through the decoder’s output x. The difference between x and x shall determine the value. There are many choices to define the distance between the ground truth x and its reconstructed sample x. A common choice is to use the MSE to define the log probability as

logpθ(x|z)~-x-xl2σx2,

where |⋅​l2 is the l2 norm, and σx2 is used to scale the MSE.

The prior distribution of the binary variable z with dimension J is a Bernoulli distribution, as z~Bernoulli(p, 1-p). We can use the sum of z over its dimension, as z=(z1,⋯, zJ), where zj∈{0, 1}. The sum of all zj is a binomial distribution, as jzj=k~B(J, p). When J is large, the binomial distribution can be approximated with a Gaussian distribution. Assume p=12 for balanced binary variable zj, and all zj are considered independent. We have

Prjzj=k=BJ,12~N(J2,J4).

Therefore, the log probability of the a priori probability can be calculated as

logpθ(z)=logpθ(jzj=k)=-12log2π-12logJ4-2Jk-J22.

With the calculation of these three probabilities, the ELBO can be served as the objective function for SGD training of the autoencoder with binary latent variable. Both the encoder and decoder can be trained jointly. Once the autoencoder is trained, the binary latent variable values will serve as the CSI codes for CSI feedback. No additional quantizer is needed in this design. The diagram below illustrates the overall model training steps.

Figure 8: The model training for biv-CSI coding.

Proposal 1. Study channel- and eigenvector-based autoencoder-like machine learning solutions for CSI feedback compression.

Proposal 2. For the autoencoder-like machine learning solutions, study the impact of quantizers on CSI feedback compression.

## CSI prediction

Channel prediction is seen as a main enabler for more advanced use cases, which are sensitive to channel aging like MU MIMO precoding or coherent JT-CoMP as discussed for cell free massive MIMO. Furthermore, accurate channel prediction over a large prediction horizon can support high speed UEs and can be a suitable means to reduce the CSI reporting overhead over the state-of-the-art techniques like NR TYPE II. Such overhead reduction can be achieved by a reduced CSI reporting rate, which is then related to the channel prediction quality.

We should note that channel prediction also fundamentally impacts massive MIMO overhead for reference signals in FDD systems. Without channel prediction the usage of the reported CSI is limited by the coherence time and coherence frequency bandwidth. In case of a high number of antenna elements or antenna ports the related number of CSI reference signals might become a large portion of the resource elements in this coherence area of the radio channel. Therefore, it reduces the related number of resource elements for the user data rate, i.e., the PDSCH. With channel prediction the channel evolution might be reconstructed, thereby potentially might overcome this coherence related limitations.

Figure 9 illustrates the most basic idea for channel prediction, where the radio channel is measured over several time instances, typically for FDD systems based on regularly transmitted CSI RSs. Then the UE or the gNB input the estimated CSI, either directly or after some pre-processing, into the pre-trained ML model, which outputs the predicted CSI for one or several time steps.

Most useful is it to predict as CSI the explicit radio channel evolution in the time and/or the frequency domain as this will enable any type of precoding, will support any type of MU MIMO user grouping and scheduling and therefore is the basis for more advanced future concepts like extensive massive MIMO, or cell free massive MIMO. Alternatively, the CSI prediction might be close to current Type II CSI reporting and predicting parameters like PMI, RI, CQI, etc.

Note that for optimum channel estimation and channel prediction the CSI RSs should cover the full RF bandwidth as it is well known from theory that the observed frequency bandwidth affects the theoretical Cramer Rao Bound of the unbiased channel estimator. The Cramer Rao Bound itself is defined by the Fisher information contained in a certain signal used for the estimation of certain signal parameters like the delay, amplitude, or phase of a multipath component. This Fisher information is then increasing for an increasing number of CSI RSs and increasing RF bandwidth. This can be illustrated, for example, by assuming a single multipath component. The delay of such a multipath component is then related to a phase slope in the frequency domain and obviously the estimation of such a phase slope will be easier in case of a larger frequency bandwidth.

Helpful for optimum precoding is then to predict the evolution over time of the radio channel instead of the radio channel just for a single prediction time, or sequence of time instances. This provides full scheduling flexibility and the highest precoding performance, which might be especially relevant for larger prediction horizons.

Figure 9: Basic concept of channel prediction, where the radio channel is observed over a period of time and measured by CSI RSs and predicted into the future.

Proposal 3. Study channel prediction machine learning solutions.

Proposal 4. Compare channel prediction over broad bandwidth versus based on Type II CSI per sub-band.

### AI/ML model, terminology and description

In the following, some possible ML based channel prediction implementations will be given and compared to rule based Kalman filters as predictors. The model training, validation, and testing will be discussed exemplary for a most simple RNN consisting of LSTM neural networks. The possible inference operation using ML, rule based, or, ML enabled networks will be provided, together with the possible collaboration modes between UE and gNB. Then we shortly discuss version control and lifecycle management of the predictor models, and finally suggest a baseline reference mode of operation.

#### Model generation (training/validation/testing)

For channel prediction, we consider Kalman filter as PHY layer algorithm baseline while long-short term memory (LSTM) neural networks (NNs) might serve as the AI/ML baseline. Recurrent neural networks, LSTMs, and gated recurrent units (GRUs) are AI/ML models often used for applications where the data has some time dependency, e.g., speech recognition, natural language processing, and time forecast. Therefore, it is a reasonable idea to use LSTM as the AI/ML CSI prediction baseline. The LSTM learns the time evolution of the channel coefficients by means of supervised learning with the mean squared error (MSE) as typical loss function. The loss value guides the gradient descent optimization, which uses the backpropagation through time (BPTT) algorithm.

Note that channel prediction based on RNNs is here given more as a possible reference, while higher performance, lower complexity, or other benefits might suggest other more advanced NN implementations in the future.

Figure 10: Generic structure of a LSTM NN that receives as input channel coefficients of N past time instances and outputs the next N+p channel coefficients in the frequency domain for a subcarriers f1.

A LSTM neural network (NN) is designed to perform channel prediction of p subframes ahead based on knowledge of N past subframes, see Figure 10. For a SISO channel represented in the frequency domain, a few channel coefficients x = h1,h2,...hNt=1:N serve as input to the LSTM NN which is trained to output the channel coefficient in the next subframe y≈hN+1, or y≈hN+p subframes ahead. In Figure 11 we present an example where the LSTM NN receives for a single subcarrier information over three subframes as input and predicts the single-subcarrier channel coefficient at the next subframe. The LSTM NN can be expanded to predict channel coefficients for multiple subcarriers. Nonetheless, for SISO-OFDM, the number of parameters within the LSTM NN grows considerably as the number of subcarriers increases. Figure 12 presents an example for predicting for one time instance ahead with knowledge of the past three channel coefficients in the frequency domain. Prediction of multiple time steps ahead is also possible, but often limited to the number of time steps at the input of the LSTM NN.

Figure 11: Training configuration for a LSTM NN for SISO channel coefficient prediction.

Figure 12: Configuration for SISO-OFDM channel prediction.

During training of the LSTM NN, the channel measurements should be inputted to the model in a sequential way, i.e., the dataset (input and labels pairs) should not be shuffled. This is needed for the LSTM NN to learn the change dependencies among the frequency channel coefficients. For instance, the table in Figure 12 shows the order to provide the channel coefficients for the LSTM NN during training. After training the model parameters, the LSTM NN takes any sequence of three coefficients and outputs the next channel coefficient, regardless of where they happen. However, note that there are no performance guarantees if the trained LSTM model is used for predicting channel coefficients under different conditions, e.g., different Doppler frequency.

#### Inference operation

Figure 13 considers the basic UE sided inference operation based on a channel prediction model (CP model).  It uses n CSI RSs for observing the radio channel and infers from these n CSI estimates, the predicted CSI for the time instance tpredict. The inference of the predicted radio channel can similarly be implemented for pure PHY layer rule-based models, for pure ML models, and for ML enabled models combining specific PHY and ML blocks. Such PHY plus ML-based inference might have benefits with respect to the overall achievable performance, minimized latency, and moderate complexity.

Figure 13: possible implementations of the channel prediction with a CP model at the UE side, either as pure PHY based method, as ML neural network or as a combined PHY/ML model.

#### Collaboration between UE and gNB

The channel predictor can be an ML model located either at the UE or at the gNB. In the first case, the UE reports the predicted CSI - instead of the conventional instant NR Type II CSI feedback. Beneficial in one aspect might be an implementation at the gNB, as the gNB can be expected to have higher processing capabilities as well as can predict the CSI to any time instant of interest, or, might even predict the channel evolution over time (see Figure 15).

A channel predictor at UE side has potential benefits with respect to the direct access to the channel knowledge, while the gNB has to rely on the quantized and compressed reported CSI information (see Figure 14).

Figure 14: possible channel prediction with UE sided inference of the CSI for a predefined time t_predict. UE reports then at time t_observe the predicted CSI as well as the prediction time t_predict. Based on this information the gNB can make – possibly after some signal post processing - a prediction for the time instance t_predict.

Figure 15: possible channel prediction with gNB sided inference of the CSI for a predefined time t_predict. UE reports then at time t_observe n compressed CSI estimates as well as the related time stamps. Based on this information the gNB can infer a prediction for the time instance t_predict.

Proposal 5: Consider UE sided as well as gNB sided channel prediction, as well as potentially include combined prediction between UE and gNB.

#### Lifecycle management of AI/ML model

Lifecycle management of AI/ML models depends on the operation mode, i.e., UE-sided, gNB-sided or combined mode. In case of UE sided prediction, the UE vendor might have to ensure up to date and verified neural network model usage. Similarly, in case the prediction is done at gNB side then the gNB vendor or MNO might be responsible to keep models up to date.

In more advanced implementations, the ML and potentially the PHY layer models might be adapted to the general radio channel conditions. Eventually, this might include online training for fine-tuning of pre-trained ML models.

In the case of parallel models running on the UE and gNB side with an explicit exchange of model parameters, then a very detailed model selection, model verification, model training description will be needed.

### Baseline Scheme(s)

As baseline scheme for ML based channel prediction, we see the UE based channel prediction, where the CP model is realized mainly by an LSTM neural network potentially enhanced by some CNN layers. The scheme follows the setup in Figure 14, i.e., the gNB transmits n CSI RSs and the UE feeds the related n CSI estimates into the CP model.

We assume an OFDM signal with a regular grid of CSI RS in time and frequency and potentially enhanced by a first stage NN for noise reduction of the channel estimates. Typically, the neural network will directly use as input signal the normalized LMMSE estimates of the complex channel transfer functions, which are then split into the real and imaginary parts. The output signal is the predicted channel transfer function for a predefined prediction time tpredict.

In case the channel prediction is done at the gNB side, then the UE has to report the n CSI estimates on the PUCCH to the gNB. Then it might be beneficial as part of the CSI compression to transform the CSI from frequency into time domain. Furthermore, some channel prediction methods include as part of the data preprocessing such a frequency to time domain signal transformation. gNB sided channel prediction might allow for more complex and more advanced channel prediction methods, potentially, leading to an improved channel prediction horizon.

# Conclusion

In this contribution, we have discussed the details of two CSI sub-use cases. Our proposals are:

Proposal 1. Study channel- and eigenvector-based autoencoder-like machine learning solutions for CSI feedback compression.

Proposal 2. For the autoencoder-like machine learning solutions, study the impact of quantizers on CSI feedback compression.

Proposal 3. Study channel prediction machine learning solutions.

Proposal 4. Compare channel prediction over broad bandwidth versus based on Type II CSI per sub-band.

Proposal 5: Consider UE sided as well as gNB sided channel prediction, as well as potentially include combined prediction between UE and gNB.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” 3GPP RAN #94-e.

- C-K. Wen, W.-T. Shih, and S. Jin, “Deep Learning for Massive MIMO CSI Feedback,” IEEE Wireless Communications Letters, Vol. 7, Issue: 5, Page(s): 748 - 751, Oct. 2018.

- W. Liu, W. Tian, H. Xiao, S. Jin, X. Liu, and J. Shen, “EVCsiNet: Eigenvector-Based CSI Feedback Under 3GPP Link-Level Channels,” IEEE Wireless Communications Letters, Vol. 10, No. 12, December 2021.

- M. Chen, J. Guo, C.K. Wen, S. Jin, G. Y. Li, and A. Yang. “Deep Learning-based Implicit CSI Feedback in Massive MIMO,” IEEE Transactions on Communications, Vol. 70, No. 2, pp. 936-950, February 2022.

- van den Oord, A., Vinyals, O., & Kavukcuoglu, K. (2017, December). “Neural discrete representation learning,” in Proceedings of the 31st International Conference on Neural Information Processing Systems (pp. 6309-6318).

- Diederik P. Kingma and Max Welling. “Auto-encoding variational Bayes,” International Conference on Learning Representations (ICLR), 2014.

- L. Theis, W. Shi, A. Cunningham, and F. Huszar, “Lossy image compression with compressive autoencoders,” arXiv preprint arXiv: 1703.00395, 2017.

- Chen, Muhan, Jiajia Guo, Chao-Kai Wen, Shi Jin, Geoffrey Ye Li, and Ang Yang. "Deep Learning-based Implicit CSI Feedback in Massive MIMO." IEEE Transactions on Communications (2021).