3GPP TSG RAN WG1 #109	R1-2204573

e-Meeting, May 09th – May 20th, 2022

Agenda item:		9.2.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Evaluation on ML for beam management

Document for:		Discussion and Decision

# Introduction

Rel-18 study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, agreed in [1], includes a use case of beam management, and the exact objectives are as follows,

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced

In this contribution, we discuss the evaluation methodology for several sub-use cases (defined in [2]) including the details of simulation assumptions, baseline assumptions, and KPIs, and also provide some initial simulation results.

# Discussion

## Common assumptions for beam management use case

    - KPIs and corresponding requirements

There are KPIs that are specific to beam management use cases and have often been used in the past by 3GPP (50th percentile throughput, 5th percentile throughput and latency), and we suggest reusing them across all sub-use cases of beam management. In addition, a few ML-specific KPIs (prediction accuracy, RSRP error, etc.) can also be identified in Table 2.1-1 to further characterize the performance of sub-use cases of ML-based beam management.

Table 2.1-1: List of KPIs specific to beam management use cases.

| KPIs | KPI description | Notes |
| --- | --- | --- |
| Top-1 Prediction Accuracy | Proportion of samples that the predicted beam equals the actual best beam. | ATop-1=1Nt n=1Ntl(bn=bn) Where Nt is the number of data points for obtaining the ML model performance, l() is the indicator function and bn and bn are the predicted beam index and best beam index respectively. If bn=bn, then l()=1. Otherwise, l()=0. Best beam is selected based on bn=argmax(RSRPl) for l={1,…,M}, where M are all Tx beams. |
| Top-K Prediction Accuracy | Frequency for which one of the top-K predicted beams equals to the actual best beam. | ATop-K=1Nt n=1Ntk=1Kl(bk,n=bn) Where bn,k is the index of the k-th best predicted beam. |
| Top-1 Prediction Accuracy with 1 dB margin | Proportion of samples that the predicted beam equals the actual best beam. | ATop-1, 1 dB relax=1Nt n=1Ntl(RSRPbn-RSRPbn<1 dB) Where RSRPbn is the RSRP of the best beam and RSRPbn is the RSRP of the predicted beam. |
| RSRP Error | RSRP difference between the RSRP obtained with the predicted beam and the RSRP of the actual best beam | eRSRP=1Nt n=1NtRSRPbn-RSRPbn |
| BM measurement overhead | Fraction of beam measurements over the total number of beam measurements required by the exhaustive search. | OH[%]=NMif K=1N+K×#UEs/CellMif K>1 Where N is the number of beam measurements for a subset of beams with size N and M is the number of all beams. Whenever beam selection is based on the ML model output (case K>1), the overhead considers the UE specific beam measurements required to select the best beam among the top-K predicted beams. |
| Average cell throughput | Average cell throughput measured by deploying the ML model inside the system-level simulator and using the beam predicted by the ML model. | A similar mechanism as in Rel-16/17 MIMO BM simulations |
| 50th percentile throughput | 50-th percentile (median) of the effective user’s throughput measured by deploying the ML model inside the system-level simulator and using the beam predicted by the ML model. | A similar mechanism as in Rel-16/17 MIMO BM simulations |
| 5th percentile throughput | 5-th percentile of the effective user’s throughput measured by deploying the ML model inside the system-level simulator and using the beam predicted by the ML model. | A similar mechanism as in Rel-16/17 MIMO BM simulations |

ML-specific KPIs are considered for testing the ML model performance and for comparison of different types of ML models, different settings of the ML Model for training, etc.

Full validation of the ML model to check the RAN performance need to be carried out through simulations and by calculating UE and cell statistics such as throughput.

RAN1 shall consider the following KPIs for the ML-based beam management use case,

- Beam prediction accuracy (Top-1 Prediction, Top-K Prediction, Top-1 and Top-K Prediction with 1 dB margin)

- RSRP error/difference

- Beam management measurement overhead

- Cell throughput (average, 5%ile, 50%ile)

    - Assumptions for data collection/generation and simulations

Methodology is based on statistical models from TR 38.901. There are common assumptions in dataset construction for training, validation, and test for the selected use cases.

Table 2.1-2: General assumptions for data collection/simulations to be applied to different beam management sub-use cases.

|  | ## Beam Management Sub use cases |  |
| --- | --- | --- |
| Parameters | “Spatial Domain Beam Prediction”/ “Spatial-Temporal Domain Beam Prediction with Supervised Learning”/ “Spatial-Temporal Domain Beam Prediction with Local Online Learning” | ## “Beam Prediction in Spatial Domain for Improving the Throughput & Latency” |
| Scenario | 3D-UMi-street Canyon | 3GPP Dense Urban Macro |
| Deployment | Hexagonal grid, 7 BS sites, 3 sectors per site, 21 cells, BS antenna height 10m (downtilt = -10) | Hexagonal grid, 7 BS sites, 3 sectors per site, 21 cells |
| ISD | 100m | 200m |
| Carrier | 28 GHz | 30 GHz |
| System Bandwidth | 50 MHz | 50 MHz |
| Frame Structure | TDD, DL data frame only | FDD |
| Sub-carrier spacing | 120 kHz | 120 kHz |
| gNB antenna | (M, N, P, Mg, Ng) = (16, 16, 2, 1, 1) dual-polarized panel arrays | M, N, P, Mg, Ng) = (4, 8, 2, 1, 1) dual-polarized panel arrays |
| gNB Grid of Beam configuration | 64 Tx beams: Tx Beam Azimuth= {-56.25 -48.75 -41.25 -33.75 -26.25 -18.75 -11.25 -3.75 3.75 11.25 18.75 26.25 33.75 41.25 48.75 56.25} Tx Beam Elevation= {0 7.5 15 22.5} | 14 Tx beams Tx Beam Azimuth = {36.6, 50, 63.3, 76.6, 90, 103.3, 116.6, 130, 143.3, 42, 60, 90, 120, 138} Tx Beam Elevation = {-13, -10, -10, -10, -11, -10, -10, -10, -13, -30, -33, -36, -33, -30} |
| UE antenna panels | 4 dual-polarized panels with orientation {0, 90, 180, 270}, the panel with best received SNR is chosen for output metric. | 3 dual-polarized panels with orientation {0, 90, 270} |
| UE antenna configuration | (M, N, P) = (1, 1, 2) per panel, UE orientation uniformly distributed on [0,360] degree | (M, N, P) = (1, 1, 2) per panel, Ideal panel selection, UE orientation uniformly distributed on [0,360] degree |
| HARQ | No retransmission | Asynchronous with Chase-combining max. 6 retransmissions |
| Traffic Models | Traffic model: full buffer | Traffic model 1: full buffer Traffic model 2: FTP model 3 with 50 B packets and 1 ms mean inter-arrival time |
| UE distribution | 10 UEs/sector, randomly distributed over the simulation area 100% of UEs outdoor | Total number of UEs: 420 in high load and 210 in low load, randomly distributed over the simulation area 10% of UEs have traffic model 1 90% of UEs have traffic model 2 |

Methodology based on statistical models from 3GPP TR 38.901. There are common assumptions in dataset construction and simulations across different sub-use cases. Those common assumptions are in general the same for simulations unless sub-use case-specific conditions are specified.

The preliminary phase uses synthetic data generated from a system-level simulator that implements the 3GPP statistical channel models from 3GPP TR 38.901. The list of assumptions detailed in Table 2.1-2 can be considered for system-level simulations of different beam management sub-use cases.

For training/testing the supervised learning ML model, the number of scenarios/deployments and configurations of other simulation parameters can be extended and generalized beyond those detailed in Table 2.1-2.

## Spatial Domain Beam Prediction

This sub-use case is related to the supervised learning-based beam prediction provided in Section 2.1.2 & 2.2.1 of R1-2204574 [2].

### Evaluation Methodology & Results

#### Simulation Assumptions

For data collection, the parameters can be found in Table 2.1-2 under the “Spatial Domain Beam Prediction” sub-use case.

For training the ML model for Spatial Domain Beam Prediction the parameters can be found in Table 2.2-1.

Table 2.2-1: Spatial Domain Beam Prediction ML Model Training Parameters

| Parameter | Value |
| --- | --- |
| ML Model Type | Deep Neural Network (DNN), Convolutional Neural Network (CNN) |
| Number of layers | 2-4 |
| Number of neurons | 32-512 |
| Activation function | ReLu/Tanh/Leaky ReLu |
| Dropout | Enabled |
| Normalisation | Max-Min, layer normalization |
| Optimizer | Adam/SGD |
| Mini-batch size | 200 |
| Number of Epochs | 100-600 |
| Learning rate | Fixed or LR scheduler |
| Residual connections | Enabled |

Note:	Some of the ML model parameters are fine-tuned during the training.

#### Baseline assumption

The performance of the ML-based algorithm is compared to that of a state-of-the-art baseline considering the KPIs indicated in Table 2.1-1. The baseline specific for spatial domain beam prediction is detailed in Table 2.2-2 where a list of different alternatives and their descriptions is provided.

Table 2.2-2: Baseline for Spatial Domain Beam Prediction.

| Method | Description |
| --- | --- |
| Alternative 1 | UE measures 64 CSI-RS resources |
| Alternative 2 | UE measures a subset of CSI-RS resources with sizes {8,16 or 32} |
| Alternative 3 | UE measures 16 SSB resources |

Notes:	We consider the transmission of reference signals on SSB or CSI-RS resources for one or more gNB transmitter (Tx) beams. The UE measures and reports the RSRP measurement to the gNB. We consider 64 CSI-RS resources used for measuring the gNB Tx narrow beams and 16 SSB resources are used for measuring the gNB Tx wide beams.

#### Simulation Results

The ML model performance for spatial domain beam prediction can be found in Table 2.2-3, Figure 2.2-1 and Figure 2.2-2.

Table 2.2-3 includes the performance for both non-ML and ML-based methods. For non-ML based methods, beam selection is based on the maximum RSRP measured with different subsets of CSI-RS narrow beams. For ML-based methods, beam selection is based on the beam predicted by the ML models using different subsets of CSI-RS narrow beams as input. The ML-based methods may use the UE position to assist the prediction or may use a different pattern to select the CSI-RS narrow beams as input. Relevant KPIs described in Table 2.1-1 are evaluated by changing the ML model output, including the possibility of reporting the beam ranking, i.e., Top-K predicted beams.

Table 2.2-3: ML model performance for spatial domain beam prediction with CSI-RS narrow beams used as input of the ML model.

|  | Prediction Accuracy (1 dB RSRP Margin) [%] |  |  | Average RSRP Error [dB] |  |  | BM measurement overhead [%] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Method | Top-1 Single Beam | Top-2 Predicted beam | Top-4 Predicted beam | Top-1 Single Beam | Top-2 Predicted beam | Top-4 Predicted beam | Top-1 Single Beam | Top-2 Predicted beam | Top-4 Predicted beam |
|  | (K=1) | (K=2) | (K=4) | (K=1) | (K=2) | (K=4) | (K=1) | (K=2) | (K=4) |
| non-ML beam selection (64 CSI-RS) Alternative 1 | 100 | - | - | 0 | - | - | 100 | - | - |
| non-ML beam selection (32 CSI-RS) Alternative 2.1 | 55.226 | - | - | 3.717 | - | - | 50 | - | - |
| non-ML beam selection (16 CSI-RS) Alternative 2.2 | 28.63 | - | - | 7.65 | - | - | 25 | - | - |
| non-ML beam selection (8 CSI-RS) Alternative 2.3 | 15.59 | - | - | 12.57 | - | - | 12.5 | - | - |
| ML-based beam selection (32 CSI-RS Input) | 95.69 | 98.597 | - | 0.253 | 0.0602 | - | 50 | 81.25 | - |
| ML-based beam selection (16 CSI-RS Input) | 89.23 | 95.51 | 98.68 | 1.025 | 0.342 | 0.093 | 25 | 56.25 | 87.5 |
| ML-based beam selection (8 CSI-RS Input) | 74.00 | 87.27 | 95.70 | 3.80 | 1.48 | 0.43 | 12.5 | 43.75 | 75 |
| ML-based beam selection (32 CSI-RS Input with UE position) | 95.604 | 98.59 | - | 0.2586 | 0.0615 | - | 50 | 81.25 | - |
| ML-based beam selection (16 CSI-RS Input with UE position) | 94.13 | 97.779 | 99.47 | 0.405 | 0.110 | 0.0238 | 25 | 56.25 | 87.5 |
| ML-based beam selection (8 CSI-RS Input with UE position) | 79.813 | 90.3571 | 96.62 | 2.797 | 1.0585 | 0.3287 | 12.5 | 43.75 | 75 |
| ML-based beam selection (16 CSI-RS Input with pattern 2) | 94.86 | 98.275 | 99.5 | 0.37 | 0.105 | 0.021 | 25 | 56.25 | 87.5 |

Notes: BM measurement overhead refers to the percentage of measurements compared to an exhaustive search approach that requires 64 beam measurements. The BM measurement overhead considers cell overhead with 10 UEs/cell.

Figure 2.2-1: Accuracy (1 dB RSRP margin) vs BM measurement overhead for ML-based and non-ML methods.

Figure 2.2-2: Average RSRP Error vs BM measurement overhead for ML-based and non-ML based methods.

Table 2.2-4 shows the additional system configuration for system-level simulations of spatial domain beam prediction sub-use case.

The simulations throughput accounts for the overhead due to the transmission of control channels and RS such as DMRS, TRS, PDCCH, SSB resources, etc... For the assumptions detailed in Table 2.1-2, the common overhead is  30%, which is computed based on TR 37.910 for DenseUrban-eMBB scenario [3]. Another overhead for transmitting the CSI-RS resources for beam management is added to the common overhead. In our simulation setup, the CSI-RS resource overhead is about 0.24% for the case of 64 CSI resources, where we assume that each CSI-RS resource for BM is configured with frequency density 1/PRB, 1 CSI-RS resource(beam)/symbol, and 20ms periodicity.

Table 2.2-5 reports the system-level simulation results for the spatial domain prediction sub-use case, including the ML model results with methods using SSB wide beams and CSI-RS narrow beams as input of the ML model. For the SSB wide beams, two patterns to select the SSB beam measurements to be used as input of the ML model can be detailed as follows:

- SSB beams pattern 1 design is 1D flatten 4 conjugate narrow beams

- SSB beams pattern 2 design is 1D flatten 4 narrow beams with a grouping that benefits the beam prediction.

The SLS throughput gains are computed as 100% x (Throughput_Method - Throughput_Alt3)/ Throughput_Alt3, where Throughput_Method corresponds to the throughput of the ML-based method or the Throughput_Alt3 indicates the Alternative 3 method.

For the Alt3 method with SSB pattern 1, the UE attaches to the gNB based on the SSB measurements and selects the best SSB beam for data transmission.

For the ML method using as input the SSB wide beams, the UE attaches to the gNB based on the wide beam measurements over SSB resources and the predicted CSI-RS beam will be used for data transmission.

For the ML method using as input the CSI-RS narrow beams, the UE attaches to the gNB based on the SSB measurements, then a subset of CSI-RS measurements is configured and used as input of the ML model to predict the best CSI-RS beam for data transmission.

The last row of Table 2.2-5 shows the throughput gain computed for Alternative 1 (Ideal Case), in which the UE attaches to the gNB based on the SSB measurements, then selects the best beam with an exhaustive search across all the 64 CSI-RS beams. This method represents an ideal case and provides the upper bound throughput achievable by the ML methods.

Table 2.2-4: Additional system configuration for system-level simulations of the spatial domain beam prediction.

| Parameter | Value |
| --- | --- |
| UE antenna configuration | (M, N, P) = (2, 2, 2) per panel, UE orientation uniformly distributed on [0,360] degree |
| CSI feedback | Type-I CSI, rank-2 |
| UE speed | 30km/h |

Table 2.2-5: ML model accuracy results and corresponding throughput gains (obtained with system-level simulations) for the ML-based methods over the non-ML method (Alt-3 baseline).

| Method | Best CSI-RS Beam Prediction Accuracy (1 dB RSRP Margin) [%] | Average cell throughput Gain [%] | 5th percentile UE throughput Gain [%] |
| --- | --- | --- | --- |
|  | Top-1 Single Beam (K=1) | Top-1 Single Beam (K=1) | Top-1 Single Beam (K=1) |
| ML-based beam selection (16 SSB Input, SSB beam pattern 1) | 84% | 9,55% | -3,83% |
| ML-based beam selection (16 SSB Input, SSB beam pattern 2) | 94% | 13,13% | 29,12% |
| ML-based beam selection (32 CSI-RS Input) | 96% | 14,13% | 37,31% |
| ML-based beam selection (16 CSI-RS Input with pattern 2) | 94% | 13,34% | 26,33% |
| non-ML beam selection (64 CSI-RS) Alternative 1/Ideal Case | 100% | 14,53% | 40,51% |

Notes: The throughput Gains are computed as 100% x (Throughput_Method - Throughput_Alt3)/ Throughput_Alt3.

For spatial beam prediction based on supervised learning, the ML model with input RSRP measurements of a subset of Tx beams from the gNB Tx codebook enables a significant reduction of the beam measurements overhead while maintaining high-level accuracy and low RSRP error.

The ML model output reporting the beam ranking i.e., Top-K predicted beams may be used to improve the ML model performance.

Extra info such as UE position can be used as input for the ML model in addition to the RSRP measurements to improve the ML model beam prediction accuracy.

The selection of CSI-RS beam measurements to be used as input of the ML model has a large impact on the ML model performance.

For the use case of using the ML model taking SSB measurement as input and predicting CSI-RS beam, the design of the SSB beam pattern has a large impact on the ML model performance

The ML model performance has a large impact on the UE throughput, especially the lower percentiles of CDF of UE throughput.

Support RAN1 to study the ML model performance of spatial beam prediction based on supervised learning. The study should consider using as input of the ML model a subset of narrow beams and/or wide beams and related RSRP measurements as well as extra information such as UE position. The study should consider using as the output of the ML model the beam ranking or beam RSRP prediction.

## Beam Prediction in Spatial Domain for Improving the Throughput & Latency

This sub-use case is related to the reinforcement learning-based beam prediction provided in section 2.1.2/2.2.2 of R1-2204574 [2].

### Evaluation Methodology & Results

#### Simulation Assumptions

For simulations, the parameters can be found in Table 2.1-2 under the sub-use case for "Beam Prediction in Spatial Domain for Improving the Throughput & Latency”. Additional parameters specific to the sub-use case can be found in Table 2.3-1.

Table 2.3-1: Additional assumptions for simulations.

| Parameter | Value |
| --- | --- |
| CSI | Periodical with 5 ms periodicity and 2 ms reporting delay |

For ML model training, the parameters can be found in Table 2.3-2.

Table 2.3-2: ML Model Training Parameters

| Parameter | Value |
| --- | --- |
| ML Model Type | Double Deep Q Network with soft target network updates |
| Replay memory size | 10 000 latest samples |
| Batch size | 10 samples |
| Epochs per training interval | 1000 (reduced dynamically based on MSE reduction) |
| Sample storing to replay memory interval: | CSI reporting periodicity |

#### Sub-use case specific KPIs

In addition to throughput KPIs defined in section 2.1.1., packet delay KPI is used, this use case specific KPI is described in Table 2.3-3.

Table 2.3-3: Specific KPI for sub-use case Beam Prediction in Spatial Domain for Improving the Throughput & Latency.

| KPIs | KPI description | Notes |
| --- | --- | --- |
| Packet delay | One-way PDCP layer delay between gNB and UE |  |

#### Baseline assumption

We consider the beam selection based on the strongest RSRP measured by the UE. The benchmark used is detailed in Table 2.3-4.

Table 2.3-4: Baseline Beam Prediction in Spatial Domain for Improving the Throughput & Latency.

| Benchmarks | Method description |
| --- | --- |
| Alternative 1 | gNB schedules UE to the strongest beam based on RSRP reported by the UE |

#### Simulation Results

As shown in Figure 2.3-1, ML improves overall cell throughputs especially by finding beams to UEs that are discriminated the most due to QoS priorities in scheduling. Because scheduling prioritizes more latency critical data, ML finds beams for which there is a good trade-off between signal quality and probability of being scheduled.

Figure 2.3-1: Downlink throughput gain distribution obtained with ML-based beam selection for best effort UEs. ML is helping especially those UEs that are discriminated by the most due to scheduling prioritizing latency critical data and beams selected for those UEs. In both load scenarios, cell throughput gain was on average ~16%.

Simulated user throughput values, for the 5th and 50th percentile, are provided in Table 2.3-5. In both load scenarios overall cell throughput gain was ~16% on average.

Table 2.3-5: Traffic model 1 downlink user throughputs (5%ile & 50%ile)

| Beam Selection Method | Low load user throughputs [Mbps] |  | High load user throughputs [Mbps] |  |
| --- | --- | --- | --- | --- |
|  | 5 %ile | 50 %ile | 5 %ile | 50 %ile |
| RSRP-based | 4.9 | 30.1 | 0.5 | 14.6 |
| ML | 10.0 | 32.7 | 3.2 | 13.8 |

Furthermore, while improving system throughput, ML is also able to improve packet delays of latency-critical data. As shown in Figure 2.3-2, ML can find beams that minimize the latency with high reliability. Instead of always using the beam with the best signal quality, ML balances between scheduling delays and transmission reliabilities per UE beam management. With a lower load scenario, the RSRP-based beam selection overloads transmission queues at 0.9999 reliability, with the higher load scenario that happens already at 0.99 reliability.

Figure 2.3-2: Latency-critical traffic for different percentiles of the CDF of packet delays (Lower is better) for Traffic Model 1 and 2. ML-based beam selection selects the beams for UEs and provides a good trade-off between reliability and scheduling latency. At the lower of the CDF of packet delays, especially without ML, there are packets not being able to be scheduled in any reasonable time. Hence, the figure is capped to a 5 ms delay.

It was shown that it is beneficial to utilize ML to find beam selections that optimize QoS specific targets such as throughput or packet delay. Especially with practical antenna array configurations, having analog-only or hybrid architectures possessing only one or few RF chains, thus capable of utilizing only a single (or few) beams simultaneously, it is important to be able to take also other than signal quality specific beam selection criteria into account. Generating a non-ML algorithm for such a purpose can be rather hard

ML-based beam management methods may take into account the QoS metric. Beams may be selected based on QoS/DRB specific requirements in addition to signal quality measures.

The Reinforcement learning method can help the beam selection for UEs in the QoS/DRB class as it can be trained rewarding actions that favor a QoS-specific metric.

Support RAN1 to study ML models for beam prediction with Reinforcement learning and/or supervised learning based methods, considering QoS metrics in addition to CSI measurements.

Support RAN1 to study Reinforcement learning based methods for beam prediction in the space/time domain.

## Spatial-Temporal Domain Beam Prediction with Supervised Learning

This sub-use case is related to spatial-temporal domain beam prediction with supervised learning in section 3 of R1-2204574 [2].

### Evaluation Methodology & Results

#### Simulation Assumptions

For simulations, the parameters can be found in Table 2.1-2 under the sub-use case: "Spatial-Temporal Domain Beam Prediction”. Additional parameters specific to the sub-use case can be found in Table 2.4-1.

For ML model training, the parameters can be found in Table 2.4-2.

Table 2.4-1: Additional assumptions for spatial-temporal domain beam prediction.

| Parameter | Value |
| --- | --- |
| UE speed | 30km/h, 45km/h, 60km/h, 75km/h, 90km/h |
| UE trajectory model | Straight trajectory with random direction |
| CSI measurement/report periodicity | 80ms, 160ms |
| Channel Spatial consistency | Type A |

Table 2.4-2: ML Model Training Parameters

| Parameter | Value |
| --- | --- |
| ML Model Type | Recurrent neural network, Conv2D |
| Input layer Normalization | Amplitude scaling in [0,1] |
| Observation window length/prediction window length | 5/5 |
| Solver name | Adam, SGD |
| Mini-batch size | 64 |
| Number of Epochs | 5000 |
| Learning rate | StepLR |

As shown in table 2.4-1, the current UE mobility model for spatial-temporal beam prediction assumes a straight trajectory traveled by the UE with a random direction and channel spatial consistency type-A defined in TR 38.901. We understand that other UE mobility models can be assumed for evaluation. To align companies’ simulation results and to have a better study of spatial-temporal beam prediction, RAN1 should agree on a UE mobility model which is suitable for this use case.

A suitable UE mobility model is needed for the spatial-temporal domain beam prediction study.

#### Baseline assumption

The performance of the ML-based algorithm is compared to that of a state-of-the-art baseline considering the KPIs indicated in Table 2.1-1. The baseline specific for Spatial-Temporal Domain Beam Prediction is detailed in Table 2.2-2 where a list of different alternatives and their descriptions is provided.

Table 2.4-3: Baseline for Spatial-Temporal Domain Beam Prediction.

| Method | Description |
| --- | --- |
| Alternative 1 | UE measures 64 CSI-RS resources at all time instants. |
| Alternative 2 | UE measures 64 CSI-RS resources over time instants [T-M, T] and selects the CSI-RS beam based on RSRP measurement. UE maintains the selected CSI-RS beam over the time instants [T+1, T+N]. |

Baselines for supervised learning spatial-temporal domain beam prediction are needed. The following are possible candidates:

- UE measures all the CSI-RS resources with an exhaustive search at all time instants and selects the best beam at each time instant.

- UE selects the best beam based on the measurements of all the CSI-RS resources during the time instants within the observation window and maintains the selected beam during the time instants of the prediction window.

#### Simulation Results

In this simulation setup, we consider the CSI-RS observation window length is 5 and the CSI prediction window length is also 5. The CSI measurement/report is mainly L1-RSRP. Different CSI-RS measurement/report scenarios are shown in Figure 2.4-1.

Figure 2.4-1: CSI measurement/report scenario. In Alt.1 UE measures 64 CSI-RS resources in all the time instants. In Alt.2 UE measures 64 CSI-RS resources during [T-4, T] and maintains the best beam selected at time T for [T+1,T+5]. In the ML method, UE measures 64 CSI-RS resources during [T-4, T] and predicts 4 CSI-RS resources during [T+1, T+5].

The ML model performance for Spatial-Temporal Domain Beam Prediction with supervised learning can be found in Table 2.4-4.

Table 2.4-4:  ML model performance for spatial-temporal domain beam prediction.

| Method | UE speed[km/h] | Sampling interval [ms] | Top-4 Prediction Accuracy [%] |  |  |  |  | Cell BM measurement overhead [%] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T+1 | T+2 | T+3 | T+4 | T+5 | T+1 | T+2 | T+3 | T+4 | T+5 |
| Non-ML, Alt. 1 | - | - | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| Non-ML, Alt.2 | 30/45/60 | 80 | 81 | 70 | 59 | 51 | 44 | 0 | 0 | 0 | 0 | 0 |
|  |  | 160 | 68 | 45 | 35 | 27 | 21 |  |  |  |  |  |
|  | 75/90 | 80 | 68 | 47 | 34 | 27 | 22 |  |  |  |  |  |
|  |  | 160 | 50 | 31 | 20 | 14 | 11 |  |  |  |  |  |
| ML-based method | 30/45/60 | 80 | 99 | 99 | 98 | 97 | 95 | 63 | 63 | 63 | 63 | 63 |
|  |  | 160 | 98 | 96 | 92 | 90 | 86 |  |  |  |  |  |
|  | 75/90 | 80 | 97 | 96 | 93 | 90 | 89 |  |  |  |  |  |
|  |  | 160 | 93 | 87 | 83 | 78 | 73 |  |  |  |  |  |

Note: The Cell BM measurement overhead is calculated by UE measurement overhead * the number of UEs/cell, considering the 10 UEs/cell. The BM measurement overhead for each UE at the prediction window is 4/64 = 6.3%.

The supervised learning ML model can predict with high accuracy the best beam(s) in future time instants for UEs with mobility.

The UE speed has a large impact on the spatial-temporal beam prediction performance.

For supervised learning spatial-temporal beam prediction, the lengths of the observation window and the prediction window impact the prediction performance significantly.

The supervised learning spatial-temporal beam prediction provides a larger gain compared to the non-ML method for higher speed UEs than low-mid speed UE.

The CSI measurement/report periodicity (used as sampling interval) affects the performance of supervised learning spatial-temporal beam prediction.

Support RAN1 to further study supervised learning-based spatial-temporal domain beam prediction.

RAN1 should identify the scenario that most benefits the application of spatial-temporal domain beam prediction.

## Spatial-Temporal Domain Beam Prediction with Local Online Learning

This sub-use case is related to the spatial-temporal domain beam prediction using local online learning provided in Section 3 of R1-2204574 [2].

### Evaluation Methodology & Results

#### Simulation Assumptions

For simulations, the parameters can be found in Table 2.1-2 under the sub-use case for " Spatial-Temporal Domain Beam Prediction with Local Online Learning”. Additional parameters specific to the sub-use case can be found in Table 2.5-1.

For ML model training, the parameters can be found in Table 2.5-2.

Table 2.5-1: Additional assumptions for system-level simulations.

| Parameter | Value |
| --- | --- |
| UE speed | {30, 45, 60, 75, 90} km/h |
| Sampling interval | 80ms |
| UE trajectory model | Straight trajectory with random direction |
| Channel Spatial consistency | Type A |

Table 2.5-2: Online Learning Model Parameters

| Parameter | Value |
| --- | --- |
| Lower bound on the number of beams measured per time slot | 4 |
| Upper bound on the number of beams measured per time slot | 16, 32, 64 |

#### Baseline assumption

The performance of the ML-based algorithm is compared to that of a state-of-the-art baseline considering the KPIs indicated in Table 2.1-1. The baseline specific for local online learning beam prediction is detailed in Table 2.5-3 where two alternatives and their descriptions are provided.

Table 2.5-3: Baseline for Spatial-Temporal Beam Prediction with Local Online Learning.

| Benchmarks | Method description |
| --- | --- |
| Alternative 1 | At each time instant, UE measures a subset of CSI-RS resources. The subset is selected pseudo-randomly, sampling from a uniform distribution. The subset size is less than 64. |
| Alternative 2 | UE measures 64 CSI-RS resources at all time instants. |

Notes:	Alternative 1 is expected to provide low overhead, but large RSRP error. On the other hand, Alternative 2 is expected to provide high overhead but perfect beam prediction.

The benchmark methods do not utilize historical data and do not perform inference operations to select beams: for Alternative 1, a subset B⊂B :B=5,8,16,32,64 of beams are sampled and of those, the beam with the highest RSRP is selected; for Alternative 2, all beams, i.e. B=64, are sampled and the beam with the highest RSRP is selected. Results are averaged over all UEs.

#### Simulation Results

The simulation results for Spatial-Temporal Domain Beam Prediction with Local Online Learning can be found in Table 2.5-4.

Table 2.5-4: Simulation results for Spatial-Temporal Domain Beam Prediction with Local Online Learning.

|  | Prediction Accuracy (1 dB RSRP Margin) [%] |  |  |  |  | Average RSRP Error [dB] |  |  |  |  | UE overhead [%] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE Speed [km/h] | 30 | 45 | 60 | 75 | 90 | 30 | 45 | 60 | 75 | 90 | 30 | 45 | 60 | 75 | 90 |
| Method |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Alt1 (B=16) | 27.3 | 27.9 | 28.4 | 28.4 | 28.9 | 8.87 | 9.02 | 8.23 | 8.45 | 7.76 | 25 | 25 | 25 | 25 | 25 |
| Alt1 (B=32) | 52.2 | 52.3 | 53.3 | 53.7 | 54.3 | 4.37 | 4.41 | 4.06 | 4.11 | 3.78 | 50 | 50 | 50 | 50 | 50 |
| Alt2 (B=64) | 100 | 100 | 100 | 100 | 100 | 0 | 0 | 0 | 0 | 0 | 100 | 100 | 100 | 100 | 100 |
| Local online learning (4≤B≤ 16) | 93.4 | 91.2 | 87.7 | 86.9 | 84.5 | 0.27 | 0.42 | 0.62 | 0.70 | 0.84 | 21.3 | 21.5 | 22.1 | 21.8 | 22.2 |
| Local online learning (4≤B≤ 32) | 93.8 | 91.9 | 89.6 | 88.6 | 86.7 | 0.24 | 0.35 | 0.50 | 0.55 | 0.68 | 27.2 | 27.4 | 28.5 | 28.3 | 29.4 |
| Local online learning (4≤B≤ 64) | 94.0 | 92.2 | 89.4 | 88.7 | 86.9 | 0.24 | 0.33 | 0.49 | 0.55 | 0.68 | 28.4 | 28.6 | 29.8 | 29.6 | 30.7 |

Figure 2.5-1 Left: UE o	verhead for Alternative 2 (|B|=64) and Alternative 1 (|B|={4,8,16,32,64}) benchmarks (lower is better). Center: RSRP error for Alternative 2 (|B|=64) and Alternative 1 (|B|={4,8,16,32,64}) benchmarks (lower is better). Right: Accuracy with 1.

Figure 2.5-1 shows the overhead associated with the benchmark in terms of the number of beams sampled per slot in the left panel, while the center panel shows the RSRP error for the respective benchmarks. Trivially, overhead increases with |B| to a maximum of 100% for Alternative 1 (B=64). The RSRP error is null for Alternative 2--since every beam is sampled and therefore the optimal beam is always found through exhaustive search--and increases monotonically as |B| decreases. The RSRP error variance increases as |B| decreases, which we attribute to the pseudo-random (uniformly sampled) beam subset selection. The right-hand panel shows the accuracy, namely the fraction of time that the best predicted beam is within 1dB of the actual best beam.

The local online learning method samples a variably sized subset of beams at every slot, such that B⊂B : N≤B≤ N, with a prescribed lower (N) and upper (N) bounds on the beam set size. We keep the lower bound fixed at N=4 test 3 different upper bounds in the set N={16, 32, 64}. In each configuration, the method chooses an appropriate beam set size N≤B≤ N at every time slot. Unlike in the benchmark studies, where beam samples are selected randomly, the local online learning method uses past knowledge to select the next subset of beams.

Figure 2.5-3 Left: Overhead for the local online learning method for 4≤|B|≤64. Center: RSRP error for the local online learning method. Right: Accuracy with 1 dB RSRP margin for the local online learning method.

Figure 2.5-2 Left: Overhead for the local online learning method for a UE speed of 90km/h. Center: RSRP error for the local online learning method. Right: Accuracy with 1 dB RSRP margin for the local online learning method.

Figure 2.5-2 and Figure 2.5-3 show the performance of the local online learning method. Figure 2.5-2 separates performance by speed for the highest upper bound (4≤|B|≤64). Figure 2.5-3 separates performance by upper bound for the highest UE speed (90km/h). (We note that the upper bound on |B| has less effect for the lower UE speeds.) In both figures, we show overhead associated with the local online learning method for varying N in the left panel, while the center panel shows the RSRP error. At time t=0, nothing is known about the RSRP ‘landscape’ so the method favors exploration by keeping |B| large. This is evident from the overhead panel in Figure 2.5-2 where initially the cost is equal to N for each configuration. As time increases the model of the RSRP landscape improves, allowing B<N. Until about 320 ms (approximately 4 model iterations), the instantaneous overhead converges. At this point, the center panels of Figure 2.5-2 and Figure 2.5-3 show that the RSRP error converges to <1 dB for all configurations. This is an order of magnitude improvement over the Alternative 1 benchmark. The right panels of Figure 2.5-2 and Figure 2.5-3 show the accuracy, namely the fraction of time that the best predicted beam is within 1dB of the actual best beam.

The online local learning method performance can rapidly converge to low-RSRP error and high accuracy for UE moving at different speeds.

The online local learning method can accurately predict the actual future best beam for UE moving at different speeds and the beam management overhead hence can be reduced.

Support RAN1 to study local online learning methods for spatial-temporal domain beam prediction to train the beam prediction model online during system operation.

# Conclusion

In this contribution, we provide evaluation methodology, KPIs, baseline assumptions, and results for AI/ML for beam management. Observations and proposals of the contribution are,

- ML-specific KPIs are considered for testing the ML model performance and for comparison of different types of ML models, different settings of the ML Model for training, etc.

- Full validation of the ML model to check the RAN performance need to be carried out through simulations and by calculating UE and cell statistics such as throughput.

- RAN1 shall consider the following KPIs for the ML-based beam management use case,

- Beam prediction accuracy (Top-1 Prediction, Top-K Prediction, Top-1 and Top-K Prediction with 1 dB margin)

- RSRP error/difference

- Beam management measurement overhead

- Cell throughput (average, 5%ile, 50%ile)

Methodology based on statistical models from 3GPP TR 38.901. There are common assumptions in dataset construction and simulations across different sub-use cases. Those common assumptions are in general the same for simulations unless sub-use case-specific conditions are specified.

The preliminary phase uses synthetic data generated from a system-level simulator that implements the 3GPP statistical channel models from 3GPP TR 38.901. The list of assumptions detailed in Table 2.1-2 can be considered for system-level simulations of different beam management sub-use cases.

For training/testing the supervised learning ML model, the number of scenarios/deployments and configurations of other simulation parameters can be extended and generalized beyond those detailed in Table 2.1-2.

For spatial beam prediction based on supervised learning, the ML model with input RSRP measurements of a subset of Tx beams from the gNB Tx codebook enables a significant reduction of the beam measurements overhead while maintaining high-level accuracy and low RSRP error.

The ML model output reporting the beam ranking i.e., Top-K predicted beams may be used to improve the ML model performance.

Extra info such as UE position can be used as input for the ML model in addition to the RSRP measurements to improve the ML model beam prediction accuracy.

The selection of CSI-RS beam measurements to be used as input of the ML model has a large impact on the ML model performance.

For the use case of using the ML model taking SSB measurement as input and predicting CSI-RS beam, the design of the SSB beam pattern has a large impact on the ML model performance

The ML model performance has a large impact on the UE throughput, especially the lower percentiles of CDF of UE throughput.

Support RAN1 to study the ML model performance of spatial beam prediction based on supervised learning. The study should consider using as input of the ML model a subset of narrow beams and/or wide beams and related RSRP measurements as well as extra information such as UE position. The study should consider using as the output of the ML model the beam ranking or beam RSRP prediction.

ML-based beam management methods may take into account the QoS metric. Beams may be selected based on QoS/DRB specific requirements in addition to signal quality measures.

The Reinforcement learning method can help the beam selection for UEs in the QoS/DRB class as it can be trained rewarding actions that favor a QoS-specific metric.

Support RAN1 to study ML models for beam prediction with Reinforcement learning and/or supervised learning based methods, considering QoS metrics in addition to CSI measurements.

Support RAN1 to study Reinforcement learning based methods for beam prediction in the space/time domain.

A suitable UE mobility model is needed for the spatial-temporal domain beam prediction study.

Baselines for supervised learning spatial-temporal domain beam prediction are needed. The following are possible candidates:

- UE measures all the CSI-RS resources with an exhaustive search at all time instants and selects the best beam at each time instant.

- UE selects the best beam based on the measurements of all the CSI-RS resources during the time instants within the observation window and maintains the selected beam during the time instants of the prediction window.

The supervised learning ML model can predict with high accuracy the best beam(s) in future time instants for UEs with mobility.

The UE speed has a large impact on the spatial-temporal beam prediction performance.

For supervised learning spatial-temporal beam prediction, the lengths of the observation window and the prediction window impact the prediction performance significantly.

The supervised learning spatial-temporal beam prediction provides a larger gain compared to the non-ML method for higher speed UEs than low-mid speed UE.

The CSI measurement/report periodicity (used as sampling interval) affects the performance of supervised learning spatial-temporal beam prediction.

Support RAN1 to further study supervised learning-based spatial-temporal domain beam prediction.

RAN1 should identify the scenario that most benefits the application of spatial-temporal domain beam prediction.

The online local learning method performance can rapidly converge to low-RSRP error and high accuracy for UE moving at different speeds.

The online local learning method can accurately predict the actual future best beam for UE moving at different speeds and the beam management overhead hence can be reduced.

Support RAN1 to study local online learning methods for spatial-temporal domain beam prediction to train the beam prediction model online during system operation.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” 3GPP RAN #94-e.

- R1-2204574, “Other aspects of ML for beam management” Nokia, Nokia Shanghai Bell.

- TR 37.910 – B.4.1_eMBB_SE - SpectralEfficiency - 02 DenseUrban-eMBB