3GPP TSG RAN WG1 #109	R1-2204574

e-Meeting, May 09th – May 20th, 2022

Agenda item:		9.2.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Other aspects on ML for beam management

Document for:		Discussion and Decision

# Introduction

Rel-18 study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, agreed in [1], includes a use case of ML based beam management.

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced

In this contribution, we introduce details of several sub-use cases that can be considered within the use case of beam management.

# Spatial Domain Beam Prediction

AI/ML-based beam prediction involves predicting the best beam(s) or predicting the ranking or other quantities (i.e. RSRP(s)) of all beams based on a limited set of measurements. For spatial domain prediction, a limited set of beams are measured and reported by the UE, and the best beam for the UE is determined based on that limited set of measured beams. In spatial domain beam prediction, the limited set of measurements generally includes beam measurements that do not contain any historical information and therefore do not enable any tracking of the time dimension of the channel. Three examples of spatial domain beam prediction can be considered:

- The first example is predicting a best refined beam based on a set or subset of measurements made on a set of wide beams with or without extra information like UE position info, etc...

- The second example is predicting a best wide/refined beam based on a subset of measured wide/refined beams with or without the extra info.

- The third example is predicting a best beam for scheduling based on QoS metrics that can enable improved throughput or latency (depending on the QoS criteria).

For the first example, a set of SSB RSRP measurements and the extra info can be used to predict an optimal refined beam without the need for transmitting additional CSI-RS for RSRP and no need for the associated RSRP reporting. The result would be lower RS overhead and a lowering of the time required for determining a best refined beam.

For the second example, a best refined beam is predicted from a subset of measured refined beams, where the predicted refined beam may or may not belong to the subset of the measured refined beam. Extra info may also be used for the prediction. For example, the gNB could maintain a grid-of-beams containing a number of beams much larger than the maximum number of resources that the UE can be expected to measure. The gNB would transmit CSI-RS resources over a subset of beams, and the best refined beam would be determined based on the RSRP report from the UE for the subset of beams. The amount of overhead for CSI-RS for RSRP measurements would be reduced by this operation, which can also reduce the time required to determine a refined beam and potentially enable the determination of beam that is better than any of the beams in the reduced subset of measured beams.

In the third example, best beam is predicted in terms of QoS. This could mean e.g., throughput or latency – depending on the QoS class and its requirements. Shortly, UEs are sometimes scheduled to a beam that is not the strongest (signal strength wise) but a beam that can improve the UEs QoS metrics. This is especially important when limited number of beams can be scheduled during a transmission interval, and with higher number of spatially distributed users. In such situations, scheduling delays starts to increase if UE’s will be allocated always to the strongest reported beams. By offloading certain users to the ML-selected beams, scheduling delays can be reduced as well as throughputs increases, since a smaller number of beams has to be scheduled in time domain, thus frequency/time resources are filled more efficiently.

The potential use cases that can be beneficial from ML spatial domain beam prediction are:

- Use wide beam measurements to predict the best refined beam(s).

- Use a subset of wide/refined beam measurements to predict the best wide/refined beam(s).

- Utilize the QoS metric to assist the beam prediction for improving system throughput and reducing latency.

## ML Model Description and Training

### Beam prediction model with supervised learning

Figure 1 : ML-based spatial domain beam prediction with supervised learning. The ML model input is the all/subset of the beam RSRP for gNB GoB#1, the highlighted squares are the measured beams. The ML model outputs the prediction of the beam ranking or beam RSRP for gNB GoB#2, the highlighted square represents the predicted best beam.

ML model : The ML model proposed for spatial domain beam prediction is using a neural network (NN) with supervised learning. The NN can be constructed with multiple conv layers, dense layers and residual connections. Other NN structures are not exclusive. The NN model may include normalization layers, and neuron coefficient regulations. The activation function for the hidden layers can be ReLU or LeakyReLU, etc... The activation function for the output layer can be softmax or none depending on the targeting results.

Model input : The input of the model can be the RSRP measurements of a subset of the Tx beams from gNB GoB#1. Extra information such as UE position info, the GoB angles, etc., can be used in the model input to assist the prediction.

Model output : The NN model output can be the beam ranking, beam RSRP prediction for all/subset of Tx beams from gNB GoB#2. The GoB#2 can be the same as GoB#1 in one use case, i.e. single TRP measures and predicts beams from the same beam codebook. GoB#2 can be different to GoB#1 in another use case. i.e. GoB#1 and GoB#2 are from a single TRP where GoB#1 is a wide beam codebook and GoB#2 is a refined beam codebook.

Loss function : The loss function of the NN model should be different regarding different targeted outputs. For the beam rankings prediction, binary cross-entropy (BCE) can be used as the loss function. For RSRP prediction, the loss functions can one or more different L1 norm/MSE functions. Discriminator loss function from generative adversarial network (GAN) can be also used in part of the loss for RSRP prediction.

Data pre-processing : Regarding the pre-possessing of the measurements in the input data, amplitude scaling or batch/layer normalization can be used.

Data collection : The general data collection procedure is described in our other contribution [2]. In the spatial domain beam prediction use case, the training data will be the UE RSRP measurements and the extra information for the model input like the UE position information. Regarding the UE position info, if the data is collected in UE, the position information for the UE may identify the position or location of the UE by either, receiving its position information from a location management function (LMF) or other node or entity, or by calculating its position information based on received signals and/or received information). If the data is collected at the gNB, the gNB requests the UE location (UE position information) from the UE or with network-based positioning methods. The gNB may obtain UE position information of UE directly from UE, or from a location management function (LMF) of the network.

Model training : In the NN model training procedure, the collected RSRP for all/subset of GoB#1 Tx beams and the extra information are considered as input data, and the collected RSRP for all/subset of GoB#2 Tx beams are considered as label data. The data set that consists of all the input data and label data will be randomly subsampled to form a batch data set and the batch data is used to train the ML model based on the loss function. The training algorithm for the model can be Adam or stochastic gradient search (SGD) or other variants. Grid search of the model hyperparameters like learning rate, batch size, layers number, neurons number in each layer, etc., can be explored during the training process to optimize the model performance.

### Beam prediction model with reinforcement learning

ML model : The used model is DDQN (Double Deep Q Network), but it’s worth noting that it is one of the many possible RL algorithms that can be used for this problem. DDQN is basically a DQN algorithm but with additional target neural network to improve the stability of the training, but the basic principle stays the same.

Shortly, the main difference between reinforcement- and supervised learning is that RL methods collect the training data and labels online over time via trial-and-error, i.e., by trying what works and what doesn’t. While supervised learning methods are given the labels and training data for offline training, the RL agent interacts with the environment (e.g., the wireless network), applies an action based on the state (ML input), receives a reward (or RL label) measuring the goodness of the applied action in the given state, after which environment moves in a new state and the cycle repeats. RL agents are designed to maximize the cumulative reward over time. This is demonstrated with the Q-value function that this particular DDQN method is estimating

Qstate,action=Reward+gamma*maxactionQstatenew, action

The above equation states that the Q-value yielded from an action in a state is reward plus the estimated best action of the new state. The gamma value controls how much expected future reward will be taken into account. This way, the beam selection algorithm can learn the sequence of actions that will lead to a state that provides the highest cumulative reward.

Figure 2 : Reinforcement Learning basic principle

Model input : Input for the model (or RL state) is a) past N strongest beam RSRP measurements, b) cell level beam usage statistics, c) QoS flow or DRB ID for which the reward is given. But any additional inputs may be used that influences to a good decision such as data traffic characteristics. Having some knowledge on UEs signal quality, radio resource/beam usage, and traffic patterns, allow the ML model to learn which beams could be suggested as substitutes for the strongest beam to improve throughput and/or latency. It should be taken into account that the strongest beam input can be obtained from another model such as the one described in section 2.1.1.

Model output : The ML model outputs Q-value for all the beams in a cell. Thus, the ML model in this specific implementation example is predicting per gNB. When selecting the beam, the model makes a decision between exploration and exploitation. With this particular method, we have used the so-called epsilon-greedy algorithm, where explorative action (i.e., beam is selected randomly) is taken with probability ε, and exploitative action (selecting beam with highest Q-value) is taken with probability 1-ε. The episolon probability is changed over time, depending if the ML model have converged or not.

Figure 3 Example neural network of the RL based QoS-aware beam prediction, when ML model is at the gNB side. The same model works for UEs as well, but then additional beam usage information input neurons can be dropped out because the required statistics are not available on the UE side.

ML training : For the training, different rewards can be defined depending on the QoS classes and requirements. This is basically up to implementation, but as an example, any best effort type QoS flows or DRBs may be rewarded based on throughput, while latency critical QoS flows or DRBs can be rewarded based on the latency with additional penalty if packets have been dropped. This way, the ML model reinforces and starts to favor actions giving relatively higher rewards. ML models can be implemented either at the UE side or at gNB side. However, in case of the UE implementation some information such as past beam usage statistics cannot be assumed to be present. On the other hand, UE may have more extensive measurements available by implementation from gNB beams. If model is at the UE side, then UE is expected to report ML suggested beams within CSI.

Training of any RL model requires exploration. In practice, this can be done several ways, e.g., by using pretrained model without exploration, but optionally the model can be finetuned for the network with occasional exploration, or the exploration phase is executed at the beginning of model deployment. i.e., after deployment the model may need to perform random or semi-random actions (beam selection) to adjust the beam selection policy for the network. This is also the main difference from the supervised learning approaches, where beams can be labeled by measuring the signal strength. While with this QoS driven approach, it’s difficult (or even impossible) to label the beam selection decisions without exploring what is the actual performance in a beam. Since, the outcome of the beam selection is a function of several aspects such as spatial distribution of the UEs, traffic characteristics, scheduler implementation, interference fluctuations, etc.

It should be noted that the RL approach may also act as a complementary solution for the models predicting the strongest beams. For instance, supervised learning methods could improve the radio channel measurements that can be used as an input for the RL based approach that is having a different objective. Therefore, these two could coexist and complement each other; one improving the measurements while the other giving suggestions on which beam the data traffic should be steered.

With our simulation results [3], we understand that applying the ML method for spatial domain beam prediction can provide benefits in overhead saving for beam management, latency reduction and throughput improvement. Therefore, we have the following observations and proposals:

Supervised learning ML model beam prediction can be used for finding the highest RSRP wide/refined beam(s) by reducing the beam management overhead significantly.

In order to find beam selections that increase the QoS class specific performance metrics, reinforcement learning approaches can be used for searching the beams than the ones with highest RSRP.

For RL based methods, occasional exploration may be needed to train and validate the ML model.

Support RAN1 to further consider spatial beam prediction based on both supervised learning and reinforcement learning for

  - Beam prediction for reducing beam management resource overhead and latency.

  - QoS based beam prediction for improving system throughput and reducing latency.

For beam prediction based on supervised learning, the ML model consider the followings:

  - Model input: RSRP measurements of all/subset of Tx beams from gNB GoB#1, extra info can be included.

  - Model output: Prediction of beam ranking or beam RSRP for all/subset of Tx beams from gNB GoB#2.

  - Offline training for the ML model generation.

For beam prediction based on reinforcement learning, the ML model considers the following

  - Model input: Standard CSI measurements and optionally  beam usage statistics and scheduling information if available

  - Model output: best beam for QoS class or DRB

  - Optionally explorative data collection and training

## ML Model Inference

### Beam prediction model with supervised learning

The ML model inference procedure will be different depending on which node uses the model for inference. Different inference procedures can have different standard impacts.

#### ML model inference in gNB

Assuming the trained ML model is deployed at gNB side, the inference procedure can be the followings:

- gNB configures UEs to measure and report the RSRP of all/subset of Tx beams in GoB#1 through the existing CSI measurement/reporting framework. gNB can also use extra information for example the location information of ther user device for input of the ML model. The gNB may obtain UE position information of UE directicly from UE, or from a location management function (LMF) of the network.

- Together with step 1, gNB may also configure UEs to measure and report the RSRP of all/subset of Tx beams in GoB#2 through the existing CSI measurement/reporting framework. The collected RSRPs may be used for ML model validation by comparing to the prediction results. The collected RSRP from steps1&2 may be used for the NN model fine-tuning in gNB.

- After the model validation and/or model fine-tuning if they exist, with the collected input data from step 1, gNB can obtain the beam prediction results from the trained ML model. Then gNB may utilize the prediction results for the subsequence scheduling and the process may be transparent to UE.

To enable the supervised learning gNB-based beam inference operation, the existing CSI measurement/report framework can be sufficient.

#### ML model inference in UE

Assuming the model is trained by gNB or by UE, and the trained model is deployed at the UE side, further detail about gNB-UE collaboration scheme can be checked in [2]. In this case, the model inference procedure can be the followings:

- UE exchanges necessary signaling with gNB for driving the ML model inference. The signaling procedures may include but not limited to exchanging UE ML capability, part of input data for the ML model, gNB beam grid related info, info for ML models selection (i.e. prior info of channel statistic), etc... In this use case, the NN model input data will be the RSRP of Tx beams from gNB GoB#1, and the necessary extra info. With the NR framework, it is reasonable to assume the UE receive a CSI reporting configuration that indicating ML based beam reporting, and RS resources for measurements (e.g. Tx beams from gNB) and RS resources for predictions as two separate resource groups.

- UE may collect the RSRP of Tx beams in GoB#1 and GoB#2 for ML model validation and model fine-tuning. UE may receive the ML parameters from gNB for its ML model. UE may decide when to apply the ML inference or gNB can configure UE to proceed with the inference operation.

- After the UE measurements of Tx beams from gNB (e.g., corresponding to the RS resources for measurements) and ML model provides output considering all/sub-set of Tx beams from gNB (e.g. corresponding to the RS resources for measurements and predictions), the UE may report certain quantities for the predicted Tx beam(s) in GoB#2 alongside the measured beam(s) in GoB#1, and some other prediction-related metrics. With the NR framework, some adjustments on CSI reporting quantities and how the indexing is done across RS for measurement and prediction may be needed.

- Based on UEs’ reports, gNB configures the scheduling accordingly. The scheduling algorithm may be transparent to UE.

To enable the supervised learning UE-based beam inference operation

  - Additional gNB-UE collaboration and signaling may be needed for the ML model selection,  model input, model validation and model fine-tuning.

  - Enhanced CSI measurement/report framework may be needed.

For supervised learning spatial beam prediction, support RAN1 to further study both gNB-based and UE-based beam inference, including

  - The necessary collaboration and signal exchange between gNB and UE for ML model input, model validation, model fine-tuning and model version selection.

  - The necessary signaling for CSI reporting to enable ML model inference.

### Beam prediction model with reinforcement learning

#### ML model inference in gNB

When the ML implementation is at the gNB side, at least the following steps are required

ML input: UE is configured with standard CSI feedback that is input for the ML. gNB ML may take any additional information as input (up-to implementation) such as the QoS class, and scheduling information (e.g., the beam usage statistics).

ML inference: At every CSI reporting interval, ML inference at the gNB decides if a beam should be used that is not the strongest beam. gNB scheduler implementation takes the ML-beam as the beam to be used with the UE.

Applying ML action: Network may inform UE to expect a beam that is not the reported strongest beam.

To enable gNB-based QoS aware beam prediction, additional signaling could be introduced so that UEs can expect transmissions from a beam that is not with the highest RSRP.

## Other considerations

We expect ML model applied at the gNB to be considered a more implementation-specific option; therefore, an early discussion in RAN1 is needed to identify the possible collaboration frameworks to be prioritized over such implementation-specific variants. In such prioritization, if the UE applying a ML model get more attention in RAN1, it is required to discuss the level of controllability that gNB has over the exact functions/contexts of a ML model at the UE (as the gNB may want to get the inference outcome in different contexts). In particular to beam prediction of the spatial domain, gNB may use a subset of refined beam measurements to predict the best (strongest) refined beam(s) in some instances, while in another instance gNB may want to know the best-suited beams for improving system throughput and reducing scheduling latency. It is good to discuss whether ML models used at the UE can support such flexibility.

To enable flexibility of beam prediction in spatial domain, it may be required to have control over the ML model on the exact context of beam prediction in the spatial domain.

It may be feasible to reuse a ML model in different contexts, at least for beam prediction in spatial domain sub-use cases. For example, suppose the UE applies an ML model for the beam prediction to improve system throughput and reduce latency in the spatial domain. It may measure only a subset of refined beams and use the measurements to predict the best-suited beam(s) across a more extensive set of beams. Here, some input parameters are additionally used with the beam RSRP, e.g., beam usage information (as described in section 2.1.2). When the inputs parameters are controlled, e.g., beam usage information is set to equal or zero (to generalize that there is no difference on how the beams are used within the network), the prediction outcome could provide the strongest beam(s) across a larger set of beams. RAN1 shall consider such studies and have some controllability at the network to decide the context of beam prediction applied at the UE.

For spatial beam prediction, support RAN1 to further study UE-based beam inference, where beam predictions are performed according to the required context set by the gNB.

# Spatial-Temporal Domain Beam Prediction

In spatial-temporal domain beam prediction, the ML model can predict the best beam for a UE based on a set of limited measurements that includes historical information.  For example, the set of measurements could include a history of the best beam index selected by the UE with optional inclusion of the corresponding RSRP and/or UE position information. The intent for this prediction would be to lower RS overhead by narrowing down candidate best beam(s) mobile UEs, or by increasing the time period between the transmission of CSI-RS resource sets for beam refinement (e.g., CRI with RSRP feedback) for mobile UEs, which will also reduce reporting overhead by increasing the time period between UE reporting.

Spatial-temporal domain beam prediction can be used for reducing beam management resource overhead especially for mobile UE.

## ML Model Description and Training

Figure 4 Spatial-temporal beam prediction scheme. The ML model input is one or more best beam index(s) or RSRP measurement(s) from GoB#1 collected alongside a UE trajectory, the highlighted square is the measured best beam. The ML model output is the prediction of beam ranking or beam RSRP for GoB#2 for one or more future time instant(s), the highlighted square represents the predicted best beam.

ML model: We propose two different ML models for spatial-temporal beam prediction:

- Supervised Learning: One promising method is using NN model that is based on LSTM/convLSTM or other recurrent neural network (RNN) variations for this use case. “RNN” will be used to refer to any RNN variations in the following context. The main body of the NN model can have multiple RNN layers. There can be an encoder NN block connected to the input of the first RNN layer and a decoder NN block is connected to the output of the last RNN layer. The encoder and decoder are mainly for features extraction/compression/distraction and they can be constructed by Conv layers, dense layers, RNN layers, etc... The activation function for the hidden layers in the encoder/decoder can be ReLU or LeakyReLU. The activation function for the output layer in the decoder or the last RNN layer can be softmax or none depending on the targeting results.

- Local Online Learning: Another possible method is local online learning that the ML agent measures a limited number of beams in each time instant and updates an estimate of the correlations across beams and across time.

Both ML models will try to catch the correlation in the spatial-temporal domain, i.e., the correlation among the RSRP of beams across different time slots. Regarding the two ML models we proposed, we understand that: (1) the supervised learning model can be trained offline and it requires large reasonable UE trajectory training data. The model performance may face generalization issues over different UE trajectories/speeds/channels; (2) the Local Online Learning method, on the other hand, does training online for each UE during system operation, and hence there are no issues with offline training or model deployment.

Model input : The input of the ML model can be multiple best beam indexes or RSRP measurements collected alongside a UE trajectory. If the input is RSRP measurements, then each one of them is based on all/subset of the Tx beam from gNB GoB#1 and it may or may not include the actual best beam in GoB#1. For the supervised learning method, extra information, such as UE position info, etc., can be used in the input to assist the NN model prediction.

Model Output: The ML model output can be the prediction of the beam ranking or beam RSRP for all/subset of Tx beams from gNB GoB#2 for one or more future time instant(s). The assumptions for GoB#1 and GoB#2 are the same as the ones in section 2.1.1.

Loss function: The loss function of the supervised learning model should be different regarding different targeted outputs. For the beam ranking/RSRP prediction, the loss functions may be constructed with multiple different L1 norm/MSE functions. A Discriminator network can be also used in part of the loss functions.

Pre-processing: For supervised learning, amplitude scaling or batch normalization can be used for better learning convergence.

Data collection: The data collecting procedure for both supervised learning and local online learning methods will be similar to the related parts in section 2.1.1. but extend it to multiple time instants.

Training procedure:

- Supervised learning. For the NN method, each training data can be one or more RSRP measurements for all/subset of gNB Tx beams in GoB#1 over a UE trajectory, or the best beam indexes of gNB Tx beams in GoB#1 over a UE trajectory. Assuming each input data has (K+P) sample(s), after the pre-processing of the data, the first K sample(s), with the extra information (if it exists), will be considered as the input data, and the subsequence P sample(s) will be taken as label data. The rest training procedure will be the same as the procedure in section 2.1.1.

- Local online learning. For this method, the training data is a set of RSRP measurements at each time step. After each set of measurement results are received, the learning procedure updates an estimate of the correlations across beams and across time. These estimates are used to select a beam for each UE that maximizes the RSRP. Each set of measurements is chosen according to a tradeoff between maximizing the information learned about the correlations while also tracking the high-RSRP beams.

With our simulation results [3], we understand that applying both ML methods for spatial-temporal domain beam prediction can provide benefits in overhead saving for beam management and latency reduction for mobile UE. Therefore, we have the following observations and proposals:

Both supervised learning and local online learning provide a framework to track the best beam over time with reduced measurement overhead.

Support RAN1 to further consider the spatial-temporal beam prediction with ML methods, including:

  - Further study spatial-temporal beam prediction with supervised learning method

  - Further study spatial-temporal beam prediction local online learning method

  - Further study the tradeoffs between the supervised learning method and the online learning method.

## Inference operation

### ML model inference in gNB

Supervised learning : Assuming the ML model is deployed at gNB side, the inference procedure can be the followings:

- gNB configures UEs to measure and report the RSRP of all/subset of Tx beams from gNB GoB#1 for one or more time instant(s), or one or more best beam index(s) from gNB GoB#1 for one or more time instant(s), through the existing CSI measure/report framework, gNB can also configure UE to report for example the location information, etc. for the input of the ML model.

- Together with step 1, gNB may also configure UEs to measure and report the RSRP of all/subset of Tx beams from gNB GoB#2 for one or more time instant(s), ), or one or more best beam index(s) from gNB GoB#2 for one or more time instant(s), through the existing CSI measure/report framework. The collected RSRP(s)/beam index(s) may be used for ML model validation with the prediction results. The collected data from step 1&2 may be used for ML model fine-tuning.

- After the model validation and/or model fine-tuning if they exist, with the collected input data from step 1, gNB can obtain the beam prediction results from the NN model for one or more future time instant(s). Then gNB may utilize the prediction results for scheduling and the scheduling process may be transparent to UE.

Local online learning: In each time instant gNB configures UEs to measure and report the RSRP of a subset of Tx beams. The beam set is different for each UE and is designed to increase the gNB’s knowledge of the RSRP correlations across beams and across time, while also tracking the high-RSRP beams. After each set of measurements is received, the gNB selects a single beam to use for UE transmissions, along with a new set of beam measurement requests for the next time instant.

To enable the supervised learning or local online learning gNB-based spatial-temporal beam inference operation, the existing CSI measurement/report framework can be sufficient.

### ML model inference in UE

For both supervised learning and local online learning, assuming the model is trained by gNB or UE and the model is deployed at UE side, the inference procedure can be the following:

- UE exchanges necessary signaling with gNB for driving the ML model inference. The signaling procedures may include but not limited to exchanging UE ML capability, part of input data, gNB beam grid related info, info for ML model selection (i.e. prior info for channel statistic), etc... In this case, the ML model input data will be the RSRP measurement(s) for all/subset of the Tx beams from gNB GoB#1 form a UE over one or more pass time instant(s), and some extra info.

- UE may collect the RSRP measurements/best beam indexes of Tx beams from gNB GoB#1 and GoB#2 for one or more time instant(s) for ML model validation and fine-tuning. UE may receive ML parameters from gNB for its ML model. UE may decide when to apply the ML inference or gNB can configure UE to proceed with the inference operation.

- UE may report certain quantities for the predicted Tx beam(s) in GoB#2 alongside the measured beam(s) in GoB#1 for one or more future time instant(s). UE may report some other prediction-related metrics.

- Based on UEs’ reports, gNB configures the scheduling accordingly. The scheduling algorithm will be transparent to UE.

In some cases, it may be required to discuss how to trust the reliability of the beam predictions reported by the UE as there can be inaccuracies specially when the UE predict best beams for more than one-time instances. Determining the best possible prediction horizon may often need inference error checking or reliability checking of the ML model. In such cases, gNB may further configures one or more time instance(s) that the UE shall do both measurements and predications, where the performance of the ML model can be tracked such that measurement/prediction time instance(s) verify that the performance is in order, where any further updates on measurement or prediction time instances can be handled by the gNB according to the ML model performance level.

To enable the supervised learning or local online learning UE-based spatial-temporal beam inference operation

  - Additional gNB-UE collaboration and signaling may be needed for the ML model input, model validation, model fine-tuning, and model selection.

  - Enhanced CSI measurement/report framework may be needed.

  - Framework supporting model reliability may be needed.

For supervised learning or local online learning spatial-temporal beam prediction, support RAN1 to further study both gNB-based and UE-based beam inference, including

  - The necessary collaboration and signal exchange between gNB and UE for ML model input, model validation, model fine-tuning, model-reliability, and model version selection.

  - The necessary signaling for reporting the ML model output.

RAN4 spec impact should be considered for ML-based beam prediction.

# Conclusion

In this contribution, we discuss details of ML for beam management use case, and have following proposals and observations,

- The potential use cases that can be beneficial from ML spatial domain beam prediction are:

- Use wide beam measurements to predict the best refined beam(s).

- Use a subset of wide/refined beam measurements to predict the best wide/refined beam(s).

- Utilize the QoS metric to assist the beam prediction for improving system throughput and reducing latency.

Supervised learning ML model beam prediction can be used for finding the highest RSRP wide/refined beam(s) by reducing the beam management overhead significantly.

In order to find beam selections that increase the QoS class specific performance metrics, reinforcement learning approaches can be used for searching the beams than the ones with highest RSRP.

For RL based methods, occasional exploration may be needed to train and validate the ML model.

- Support RAN1 to further consider spatial beam prediction based on both supervised learning and reinforcement learning for

  - Beam prediction for reducing beam management resource overhead and latency.

  - QoS based beam prediction for improving system throughput and reducing latency.

For beam prediction based on supervised learning, the ML model consider the followings:

  - Model input: RSRP measurements of all/subset of Tx beams from gNB GoB#1, extra info can be included.

  - Model output: Prediction of beam ranking or beam RSRP for all/subset of Tx beams from gNB GoB#2.

  - Offline training for the ML model generation.

For beam prediction based on reinforcement learning, the ML model considers the following

  - Model input: Standard CSI measurements and optionally beam usage statistics and scheduling information if available

  - Model output: best beam for QoS class or DRB

  - Optionally explorative data collection and training

To enable the supervised learning gNB-based beam inference operation, the existing CSI measurement/report framework can be sufficient.

To enable the supervised learning UE-based beam inference operation

- Additional gNB-UE collaboration and signaling may be needed for the ML model selection,  model input, model validation and model fine-tuning.

- Enhanced CSI measurement/report framework may be needed.

For supervised learning spatial beam prediction, support RAN1 to further study both gNB-based and UE-based beam inference, including

  - The necessary collaboration and signal exchange between gNB and UE for ML model input, model validation, model fine-tuning and model version selection.

  - The necessary signaling for CSI reporting to enable ML model inference.

To enable gNB-based QoS aware beam prediction, additional signaling could be introduced so that UEs can expect transmissions from a beam that is not with the highest RSRP.

To enable flexibility of beam prediction in spatial domain, it may be required to have control over the ML model on the exact context of beam prediction in the spatial domain.

For spatial beam prediction, support RAN1 to further study UE-based beam inference, where beam predictions are performed according to the required context set by the gNB.

Spatial-temporal domain beam prediction can be used for reducing beam management resource overhead especially for mobile UE.

Both supervised learning and local online learning provide a framework to track the best beam over time with reduced measurement overhead.

Support RAN1 to further consider the spatial-temporal beam prediction with ML methods, including:

  - Further study spatial-temporal beam prediction with supervised learning method

  - Further study spatial-temporal beam prediction local online learning method

  - Further study the tradeoffs between the supervised learning method and the online learning method.

To enable the supervised learning or local online learning gNB-based spatial-temporal beam inference operation, the existing CSI measurement/report framework can be sufficient.

To enable the supervised learning or local online learning UE-based spatial-temporal beam inference operation

- Additional gNB-UE collaboration and signaling may be needed for the ML model input, model validation, model fine-tuning, and model selection.

- Enhanced CSI measurement/report framework may be needed.

- Framework supporting model reliability may be needed.

For supervised learning or local online learning spatial-temporal beam prediction, support RAN1 to further study both gNB-based and UE-based beam inference, including

  - The necessary collaboration and signal exchange between gNB and UE for ML model input, model validation, model fine-tuning, model-reliability, and model version selection.

  - The necessary signaling for reporting the ML model output.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” 3GPP RAN #94-e.

- R1-2204570 ML terminology, descriptions, and collaboration framework, Nokia, Nokia Shanghai Bell

- R1-2204573 Evaluation of ML for beam management, Nokia, Nokia Shanghai Bell