3GPP TSG RAN WG1 #109	R1- 2204575

e-Meeting, May 09th – May 20th, 2022

Agenda item:		9.2.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Evaluation on AI/ML for positioning accuracy enhancement

Document for:		Discussion and Decision

# Introduction

Rel-18 study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, agreed in [1], includes a use case of positioning accuracy enhancement, and objectives of the SI is as follows,

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced

In this contribution, we discuss two sub-use cases of AI/ML, which are positioning accuracy enhancement and positioning measurement reporting enhancement. We provide details on related evaluation methodology, KPIs, and initial simulation results.

# Discussion

## Common assumptions for positioning use case

### System-level Simulation Parameters

The system-level simulation assumption and scenarios are considered based on TR 38.857. In this contribution, we evaluate the positioning accuracy for heavy NLOS case focusing on InF-DH model. For positioning measurement reporting enhancements, we consider InF-SH model. Details considering different clutter density, height, and size are provided in Fig. 1 and Table 1.

Figure 1: Scenario layout [2]

Table 1: System-level simulation assumptions

|  |  | FR1 Specific Values |
| --- | --- | --- |
| Channel model |  | InF-DH (for accuracy improvement), InF-SH (positioning measurement reporting) |
| Layout | Hall size | InF-SH: (baseline) 300x150 m InF-DH: (baseline) 120x60 m |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m |
|  | Room height | 10m |
| Total gNB TX power, dBm |  | 24dBm |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 |
| gNB antenna radiation pattern |  | Single sector – Note 1 |
| Penetration loss |  | 0dB |
| Number of floors |  | 1 |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be at least the convex hull of the horizontal BS deployment. It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |
| UE antenna height |  | Baseline: 1.5m (Optional): FFS |
| UE mobility |  | 3km/h (Optional): FFS |
| Min gNB-UE distance (2D), m |  | 0m |
| gNB antenna height |  | Baseline: 8m (Optional): FFS |
| Clutter parameters: {density r, height hc,size d_clutter} |  | Low clutter density: ​{20%, 2m, 10m} High clutter density: - Baseline): {40%, 2m, 2m} for fixed UE antenna height and gNB antenna height |
| Note 1: According to Table A.2.1-7 in 3GPP TR 38.802 |  |  |

Table 2: Agreed simulation assumptions from RAN1#101-e on the NR PRS design [6]

| Parameter | Specific value |
| --- | --- |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Comb-6, 6 symbol NR PRS |
| Number of symbols used per slot per positioning estimate | 6 symbols per slot |
| Number of slots per positioning estimate | 1 slot |
| Power-boosting level | 6 dB |
| Network synchronization assumptions | Perfect Synchronization |

### Evaluation Methodology

#### Labeling of data for LOS/NLOS classification task using supervised learning

In our accompanying contribution [4], we present the LOS/NLOS classification as one of the representative sub use cases for positioning accuracy enhancements using ML.

To use supervised learning methods for classification tasks, data for training is required to be labeled. An agreement on the exact definitions of the labels and the related labeling is necessary. Specifically, in the case of LOS/NLOS classification task, the corresponding channel measurements need to be labeled (e.g., as LOS or NLOS). The real-world measurements, however, pose a challenge for the labeling task. To illustrate, an obstructed but not fully blocked LOS path results in an attenuated and delayed LOS signal due to the penetration.

A common approach in the literature is to label the channel measurements as NLOS whenever there is an object between the transmitter and the receiver. However, in reality, material of the blocking object can lead to different penetrability, e.g., signals can penetrate through a thin wooden object while not in a thick metal object.

Proposal 1: For LOS/NLOS classification tasks using supervised learning, the data labeling methodology should be further studied, especially in terms of whether there could be a common understanding that could be achieved within RAN1.

#### KPI for Positioning-related Classification Tasks

A common way to represent performance of classification tasks is to evaluate the confusion matrix. In case of binary classification, the matrix contains the values of true positive (TP), true negative (TN), false positive (FP) and false negative rates (FN). For the sake of easier representation, these values could be also combined to a single metric. Matthews correlation coefficient (MCC) is such a metric, which delivers a high score only if the classifier is good at classifying both positive and negative classes, e.g., LOS and NLOS, for both balanced and imbalanced datasets.

MCC is calculated as:

MCC = TP⋅TN - FP⋅FNTP+FPTP+FNTN+FPTN+FN

In the case of LOS/NLOS classification (if LOS is treated as the positive class) the following apply:

TP: correctly classified LOS, TN: correctly classified NLOS, FP: NLOS classified as LOS, FN: LOS classified as NLOS.

If any of the four factors in the denominator is 0, the denominator is set to 1. MCC score ranges between +1 and –1. An MCC score of +1 represents the case of a perfect classifier, -1 represents the case of a completely wrong classifier, and 0 represents the case of a classifier that is no better than a random classifier.

Proposal 2: For binary classification tasks, such as LOS/NLOS classification of the channel measurements for positioning, use MCC metric as to evaluate the performance of the ML model.

#### KPI for Positioning-related Regression Tasks

As highlighted in our accompanying contribution [4], AI/ML could be used to estimate the position information (including T(D)oA, range, or absolute location information) for accuracy improvement purposes. For this purpose, AI/ML models can be used for regression tasks. The performance of such regression task can be evaluated via accuracy in terms of absolute estimation error and two-norm for scalar, e.g., range, ToA, and vector, e.g., 2D, 3D absolute position, quantities, respectively, as:

ϵabs = Xest-Xtrue(or ϵabs = Xest-Xtrue2 )

where xtrue and xest represent the ground truth and the estimated value of the regressed quantity, such as T(D)oA, range, or horizontal location.

Proposal 3: For regression tasks, such as ranging estimation, use absolute estimation error, e.g., absolute horizontal positioning error, to evaluate the performance of the ML model.

#### KPIs for Positioning Measurement Reporting

For reporting measurements overhead reduction and error between original and de-compressed measurement needs to be used as possible KPIs.

Proposal 4: RAN1 to study positioning measurement and feedback enhancements considering overhead reduction and error between original and decompressed measurements as relevant KPIs.

#### KPIs for Positioning-related AI/ML model complexity

In our accompanying T-Doc on general aspects of ML [5], we propose various ways to evaluate the complexity of AI/ML algorithms, such as in terms of FLOPs and memory requirements. The same methodology could be applied to positioning use cases of AI/ML as well.

Observation 1: The analysis of complexity of AI/ML models used for the positioning use cases does not require any enhancements to KPIs currently considered such as FLOPs, etc.

## Evaluation Results for Positioning Use Case

### Ranging accuracy improvement using AI/ML

In this section, we provide our evaluation results on positioning accuracy improvement using ML-based methods. We propose a supervised ML algorithm to estimate the ToA (hence range) using CIR of the channel.

#### Evaluation Methodology

The considered ML method is a CNN composed of three convolutional layers followed by a fully connected layer. To train the algorithm, ground-truth ToA label along with each CIR is provided as input. During the online phase, i.e., inference, ToA can be estimated based on the CIR, which can be utilized to deduce the range as well as absolute position information.

As a benchmark, we consider the peak-detection based ToA estimation, which estimates the first peak above a predetermined noise threshold as ToA. In the evaluations, the noise threshold is modeled as a ratio of the maximum peak as

ηthreshold = maxCIRα

where ηthreshold and α  denotes noise threshold and the noise threshold ratio, respectively. α  is determined based on the training data to yield the lowest ToA error.

#### Simulation Results

Figure 2: Performance of ML-based estimation of positioning information

We provide our results on the performance of the proposed ML-based method as well as the benchmark method that uses peak detection, in Fig. 2. The results show the CDF of the accuracy of both algorithms in terms of absolute ranging error in m. It is observed that the ML algorithm can outperform conventional ToA estimators such as a peak detection-based estimator in terms of ranging accuracy.

In terms of complexity, the utilized ML model has ~3K parameters and the estimation task is performed with ~200K FLOPs.

Observation 2: ML-based methods can provide significant performance gains for positioning estimation in comparison to classical approaches.

Proposal 5: Include above evaluation results in the TR to highlight the benefits of ML in improving positioning estimation accuracy compared to conventional methods.

### Positioning measurement reporting

#### Evaluation Methodology

We propose a detailed description of the proposed approach using Principal Component Analysis (PCA) approach. In fact, PCA is a data analysis tool conventionally used to reduce the dimensionality (number of variables) of a large number of interrelated variables, while retaining as much of the information (variation) as possible. PCA calculates an uncorrelated set of variables (components or PCs). These components are ordered so that the first few retain most of the variation present in all of the original variables [3]. Thus, PCA will convert a set of possibly correlated measurements into a set of linearly uncorrelated variables with Y=W'X. The weights matrix W is determined based on historical data using eigenvalues decomposition method.

The PCA transform a data set of dimension N into a set of components of size K where each component yk can be formulated as a weighted sum of the N original data x :

yk =j=1Nwj,kxj

For example, considering the case of PCA with 2 components applied to a data set of size 18, each component is expressed as:

y1 =w1,1x1+w2,1x2+…..+w18,1x18

y2 =w1,2x1+w2,2x2+…..+w18,2x18

Figure 3 shows an example of data representation with only two PCA components represented in two-dimensional space (PC 1, PC 2). Each point in the figure represents 18 different features vector of [x1 , x2,…, x18]. To compute the final compressed value considering PC=2 means we need to have respective PCA component value y1 , y2 respectively. Figure 4 represents the respective weights wfeataure,PCA-component values for the data points considered in Figure 3.

Note that the above example is obtained with PCA function considering sklearn. When applying PCA, it is important to calculate the “explained variance” parameter (pca.explained_variance_ratio_) which shows the amount of information (variance) attributed to respective PCA component.

In our considered example, PC1 contains 83%, PC2 contains 10% of the variance respectively. In this case, two components gather 93% of the information in the original data set (whereas 4 components case achieve 99% variance). It is observed that increasing the number of components, increases the amount of captured information but at the cost of the reporting overhead.

Figure 3: Data representation with PCA 2 components

Figure 4: Example of PCA with 2 components

We perform an experimentation with PCA method considering simulation assumption in Table 1 and Table 2 using InF-SH scenario (300 x 150 m), channel model (InF-SH), clutter parameters (20%, 2m, 10m) depicted in Figure 5. For each position estimation, 18 time-of-arrival (ToA) measurements samples are considered. Total of 10k data samples considered which corresponds to unique position estimate and each sample composed of a vector of 18 measurements.

Figure 5: InF-SH scenario layout

The collected data is split into two sets:

  - 80% is used as historical data to fit the PCA

  - 20% is used to test the performance using the above fitted PCA.

In order to run the proposed approach, a historical data is used to train / fit the PCA. Based on the measurements received at LMF, the model can be trained and therefore, no standard impact can be noticed.

#### Simulation Results

In order to evaluate the performance of our proposed approach, we utilized minimum square error (MSE) error e as function of number of PCA components. Figure 6 shows the trade-off between number of PCA components and error.

For the baseline each UE report 18 different measurements, instead with use of our proposed PCA approach few PCA components can be used to re-construct the original measurements. Furthermore, Figure 6 shows the signalling gain versus number of PCA components.

For example, in case the PCA component is two, the error is about 2*10-15 between original and re-constructed measurements and the observed signalling gain is 88%. Considering PCA component 6, leads to 1,6*10-16 seconds error and observed signalling gain of 66% which is below as compared to the case when the value for the PCA component is two.

Allowed error threshold

Allowed error threshold

Figure 6: Performance with different PCA configurations

Observation 3: ML-based methods can provide significant performance gains for positioning measurement reporting in comparison to classical approaches.

Proposal 6: Include above evaluation results in the TR to highlight the benefits of ML in reducing positioning measurement reporting overhead in comparison with conventional methods.

# Conclusion

In this contribution we make the following observations and proposals:

Observation 1: The analysis of complexity of AI/ML models used for the positioning use cases does not require any enhancements to KPIs currently considered such as FLOPs, etc.

Observation 2: ML-based methods can provide significant performance gains for positioning estimation in comparison to classical approaches.

Observation 3: ML-based methods can provide significant performance gains for positioning measurement reporting in comparison to classical approaches.

Proposal 1: For LOS/NLOS classification tasks using supervised learning, the data labeling methodology should be further studied, especially in terms of whether there could be a common understanding that could be achieved within RAN1.

Proposal 2: For binary classification tasks, such as LOS/NLOS classification of the channel measurements for positioning, use MCC metric as to evaluate the performance of the ML model.

Proposal 3: For regression tasks, such as ranging estimation, use absolute estimation error, e.g., absolute horizontal positioning error, to evaluate the performance of the ML model.

Proposal 4: RAN1 to study positioning measurement and feedback enhancements considering overhead reduction and error between original and decompressed measurements as relevant KPIs.

Proposal 5: Include above evaluation results in the TR to highlight the benefits of ML in improving positioning estimation accuracy compared to conventional methods.

Proposal 6: Include above evaluation results in the TR to highlight the benefits of ML in reducing positioning measurement reporting overhead in comparison with conventional methods.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” 3GPP RAN #94-e.

- 3GPP TR 38.857, “Study on NR positioning enhancements,” v17.0.0, March 2021.

- I.T. Jolliffe, Principal Component Analysis, 2nd edition.

- R1-2204576, “Other aspects on AI/ML for positioning accuracy enhancement”, Nokia, Nokia Shanghai Bell, May 2022.

- R1- 2204570, “ML terminology, descriptions, and collaboration framework”, Nokia, Nokia Shanghai Bell, May 2022.

- R1-2009555, “Results on evaluation of achievable positioning accuracy and latency,” Nokia, Nokia Shanghai Bell, Nov. 2020.