3GPP TSG RAN WG1 #109	R1-2204577

e-Meeting, May 09th – May 20th, 2022

Agenda item:		9.2.5

Source:	Nokia, Nokia Shanghai Bell

Title:	On ML capability exchange, interoperability, and testability aspects

Document for:		Discussion and Decision

# Introduction

Rel-18 study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, agreed in [1], includes the following objectives:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced

The ML-based solutions being investigated in RAN1 may demonstrate non-deterministic behavior i.e., when the same input causes slightly different output. This may require a rethinking of their testability. According to the study item description [1], RAN4 only starts the work related to interoperability and testability aspects after sufficient progress in RAN1 and RAN2. Indeed, to start the work on the case-specific requirements and testing specifications, RAN4 needs to receive from RAN1 and RAN2 the details of the use cases. However, some generic requirements for non-deterministic systems could be discussed already in the RAN1 study phase.

In this contribution we discuss ML capability exchange, ML fallback mechanisms and testability aspects of ML-based solutions.

# Discussion

## Capability exchange and configuration

Most of the ML-based solutions to be introduced in 3GPP RAN1 rely on a certain degree of collaboration between the network nodes, between gNB and UEs, and/ or between gNBs. In both cases, the ML capabilities of the network nodes would need to be exposed (to a certain level) and corresponding capability exchange needs to be available.

Typically, between the gNB and UEs the capability exchange mechanisms ensure proper operation and RRM decisions taken at the network side. The same applies for best use of UE ML-based solutions.

Examples for information to be included in the ML-related capability exchange are:

- The type of RRM functions which has an ML-based implementation

- Collaboration mode

- The supported non-ML fall-back mode(s) for the RRM functions

- The supported configuration or control options and associated signaling type (DCI/MAC/RRC)

- Testing and monitoring capabilities

Proposal 1: RAN1 to include in the study of each ML-based solution (sub-use case) the type of ML-related capabilities (UE and gNB) which need to be exposed between the participating network nodes.

## Fallback mechanisms and automation control

One of the important questions that could be studied is human control in a ML automation system, especially in autonomous or self-learning systems. Safeguards against overconfidence/overreliance on the automation system must be established. For example, depending on the use case, there may be a need to have an option of human approval for the model update for automated model updates. The need for this could be studied in RAN1.

Proposal 2: RAN1 to consider necessity of involvement a human in a ML automation system

The performance of ML-enabled functions (at gNBs or UEs) may degrade (the performance drops in certain context or negatively impact performance of other UEs, gNBs). In that case RAN shall have an option to disable an ML-based function or replace it with a non-ML fallback.

Proposal 3: RAN1 to study mechanisms to disable an ML-based function in UEs or gNBs or replace it with a non-ML fallback

## Testability of UE ML-enabled solutions

All specified RRM functions supported by UE need to be testable (RAN4 & RAN5). Thus, in parallel with the development of the ML-enabled solutions which require collaboration between UE and gNB, there is also needed to specify how the UE conformance testing of such solution(s) are to be performed. In addition, MNOs would also test certain UE functionalities and performance, before allowing or activating RRM functions or UE/devices in their live networks.

Observation 1: Current 3GPP RAN4/5 tests are designed for ‘deterministic’ outcomes, while most of the UE ML-enabled solutions are likely to generate probabilistic output.

The performance of UEs with ML-enabled functions (collaboration between UE and gNB) might depend, for example, not only on current radio channel conditions, but also on the ‘historical’ measurements and past radio conditions. This is the case for solutions relying on continual learning or re-training, reinforcement learning or finetuning of the ML models. These are very different to conventional UEs with rule-based algorithms, which are assumed to be fixed for the rest of the UE lifetime. Therefore, a ML model which had been passed the conformance test in the beginning might behave differently than expected after some time. Thus, specific performance test procedures need to be designed for these UE ML-enabled functions.

Nevertheless, for both conformance and performance testing, there will certainly be also some ML-enabled functionalities which are completely ‘transparent’ and the testing of those RRM functionalities can follow the currently specified steps.

It is reasonable to assume that the UE specific implementation is always responsible for executing the ML model, or the combination of several ML models, which provide input to the RRM functionality under test.

Observation 2: The UE testing methodology should not aim at testing the ML model and algorithm implementation (input/output features, hyperparameters, etc.) – which are vendor specific – but rather at testing the output/outcome of the overall RRM function, which is supported or assisted by the ML algorithm.

Accordingly, for each proposed UE ML-enabled solution it is important to define the test conditions, test input(s) and test output(s). It is beneficial to start early the study of problems such as:

- How/if to accommodate different possible ML architecture solutions (RL, SL, USL) for the same RRM function?

- How to design a more common test framework for conformance testing and real-time performance testing (Over The Air) for ML-enabled function which involve collaboration between gNB and UE?

- How/if to distinguish if a UE’s output (action, report, parameter change, etc.) was rule based or ML based?

Proposal 4: For each proposed ML-enabled solution involving collaboration between gNB and UE, RAN1 to study the corresponding UE testing procedure (conditions, inputs and outputs) which will need to be designed in RAN4-5 to enable testing of the overall RRM function.

# Conclusion

In this contribution, we discuss other details related to Rel-18 AI/ML for Air-Interface. The following observations and proposals are made.

Proposal 1: RAN1 to include in the study of each ML-based solution (sub-use case) the type of ML-related capabilities (UE and gNB) which need to be exposed between the participating network nodes.

Proposal 2: RAN1 to consider necessity of involvement a human in a ML automation system

Proposal 3: RAN1 to study mechanisms to disable an ML-based function in UEs or gNBs or replace it with a non-ML fallback

Observation 1: Current 3GPP RAN4/5 tests are designed for ‘deterministic’ outcomes, while most of the UE ML-enabled solutions are likely to generate probabilistic output.

Observation 2: The UE testing methodology should not aim at testing the ML model and algorithm implementation (input/output features, hyperparameters, etc.) – which are vendor specific – but rather at testing the output/outcome of the overall RRM function, which is supported or assisted by the ML algorithm.

Proposal 4: For each proposed ML-enabled solution involving collaboration between gNB and UE, RAN1 to study the corresponding UE testing procedure (conditions, inputs and outputs) which will need to be designed in RAN4-5 to enable testing of the overall RRM function.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” 3GPP RAN #94-e.