3GPP TSG RAN WG1 #109	R1-2204589

e-Meeting, 16 – 27 May, 2022

Agenda item:		8.16.4

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for NR NTN

Document for:		Discussion and Decision

# Introduction

In RAN1#108-e the RAN1 UE features have been further developed [1, 2]. In this contribution we present our views on the UE features for NR NTN.

# Discussion

Detailed comments on UE features are listed below. For reference, the latest feature list from RAN1#108-e is available in the Annex.

- 26-1/4/5/6/6a/6b/8/9:

  - The following highlighted note can be found on these FGs:

  - [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported]

  - However, it is not clear how such a limitation would be applicable in practice.

    - Are the UEs expected to report support for the feature conditionally on the type of cell? This would not follow the UE capability framework for NR.

    - Another interpretation is that this would be intended at limiting the gNB implementation to prevent usage of some features that UEs have already reported to support. It is questionable what would be benefits of such approach, especially considering the said FGs are reported per band already.

  - Remove the note above in all these FGs.

- 26-1/6/6a/9:

  - These are basic features for UEs supporting NR over NTN.” Hence, they should be combined into a single FG, or at the very least carry a note that they must be indicated as supported by UEs supporting NR communication via satellite.

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for NR NTN, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Chair’s Notes RAN1#108-e, Mar. 2022

- R1-2202929, ”Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e　including remaining RAN1 issues”, Moderators (AT&T, NTT DOCOMO, INC.), Mar. 2022

# Annex

(See next page)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-1 | Uplink Time and Frequency pre-compensation and timing relationship enhancements | - UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - UE calculates common TA according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - UE pre-compensates the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window by UE-gNB RTT - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link - Determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated - Determining timing of the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated and determining the timing of PDCCH monitoring in recovery search space using K-mac during beam failure recovery procedure - UE receives cell-specific K_offset/K_mac in system information |  | No | No | Release 17 NR UE cannot communicate via satellite | per band | No | No | No | An NTN UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the serving satellite ephemeris [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling For UE supports NR communication via satellite, UE must indicate this FG is supported. |
| 26. NR_NTN_solutions | 26-4 | UE reporting of information related to TA pre-compensation | 1. Support UE reporting of information related to TA pre-compensation | 26-1 | Yes | No | UE does not support reporting of information related to TA pre-compensation for NR communication via satellite | Per band | No | No | No | Note: The exact content of UE reporting of information about the TA pre-compensation is up to RAN2 [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optionalwith capability signalling |
| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | 1. The maximal supported HARQ process number is X for UL and Y for DL |  | Yes | No | Increased number of HARQ processes is not supported for NR communication via satellite | Per band | No | No | No | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling supported] |
| 26. NR_NTN_solutions | 26-6 | Type-2 HARQ codebook enhancement | - Support of type-2 HARQ codebook enhancements when there are feedback-disabled HARQ processes | FFS | Yes | No | Type-2 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-6a | Type-1 HARQ codebook enhancement | - Support of Type-1 HARQ codebook enhancements when there are feedback-disabled HARQ processes | FFS | Yes | No | Type-1 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-6b | Type-3 HARQ codebook enhancement | - Support of Type-3 HARQ codebook enhancements when there are feedback-disabled HARQ processes | FFS | Yes | No | Type-3 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-8 | Support of polarization signalling in NR NTN | - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - Support polarization signalling for target serving cell in handover command message - Support polarization signalling for non-serving cell in RRM measurement configuration |  | No | No | UE is not able to take the advantage of polarization information to save power | Per band | No | No | No | [For UE supports NR communication via satellite, UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | - Reception of UE-specific K_offset via MAC-CE - Determining the timing of PUSCH, PUCCH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | 26-1, 26-4 | Yes | No | UE-specific K_offset reception is not supported for NR communication via satellite | Per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |