3GPP TSG RAN WG1 #109	R1-2204593

e-Meeting, 16 – 27 May, 2022

Agenda item:		8.16.8

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for NR coverage enhancement

Document for:		Discussion and Decision

# Introduction

In RAN1#108-e the RAN1 UE features have been further developed [1, 2]. In this contribution we present our views on the UE features for NR coverage enhancement.

# Discussion

Detailed comments on UE features are listed below. For reference, the latest feature list from RAN1#108-e is available in the Annex.

- 30-1:

  - Add 11-6 as prerequisite since the increased maximum number of PUSCH repetitions is supported for numberOfRepetitions, which was introduced in 11-6, and remove 5-17 as the increased maximum number of PUSCH repetitions is not supported for pusch-AggregationFactor.

- 30-2:

  - Replace list of pre-requisites with “one of {FG30-1 or 11-6}” as 30-1 already includes one of {5-14, 5-16, 5-17}.

- 30-3:

  - No need to have 11-6 as pre-requisite.

- 30-4:

  - The reporting of this feature should be so that either it includes all possible values supported in the specifications, or else UE should report a value of maximum duration from a set of candidate values. Further details can be found in our companion contributions in [3][4]. Therefore, the following should be added to the description of “Components”:

    - “	UE reports one value of maximum duration from the following set of candidate values:

      - For FDD, maximum duration = 4, 8, 16 or 32 slots.

      - For TDD, maximum duration = 2, 4, 8, [or 16] slots.”

- 30-4a/b/c/d:

  - Per band seems appropriate, considering the UE needs to maintaing consistency among DM-RS transmissions, and that might be band-dependent.

- 30-4e:

  - Confirm prerequisites one of {30-4a, 30-4b, 30-4c}.

  - Per UE is sufficient if prerequisites are agreed per band. No differentiation needed.

- 30-4g:

  - Description of "Feature group" should be (changes are highlighted in red):

    - "Restart DM-RS bundling after the dynamic events that violate power consistency and phase continuity"

  - Description of "Components" should be (changes are highlighted in red):

    - "Support restarting DM-RS bundling after the events triggered by DCI or MAC-CE that violate power consistency and phase continuity, except events which are triggered by DCI or MAC CE but regarded as semi-static events"

  - Description of "Consequence if the feature is not supported by the UE" should be (changes are highlighted in red):

    - "UE does not support restarting DM-RS bundling after the dynamic events that violate power consistency and phase continuity"

  - Per UE is sufficient if prerequisites are agreed per band. No differentiation needed.

- 30-4h:

  - It is not consistent if 30-4h is the only place where it is indicated that 30-4a/b/c/d are limited to back-to-back cases. The simpler solution seems to be to update the description of 30-4/a/b/c/d to indicate this restriction explicitly, in which case the yellow highlighted text in 30-4h can be confirmed as is.

  - Confirm the prerequisites

- 30-5:

  - Per UE

- 30-6:

  - Confirm FG description

  - Per UE

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for NR coverage enhancement, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#108-e, Mar. 2022

- R1-2202929, ”Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e　including remaining RAN1 issues”, Moderators (AT&T, NTT DOCOMO, INC.), Mar. 2022

- R1-2203611	“Remaining issues on joint channel estimation for PUSCH and PUCCH”, Nokia/NSB, May 2022

- R1-2203612	“Draft LS on description of RRC parameters for nominal time domain window length for PUSCH and PUCCH DMRS bundling”, Nokia/NSB, May 2022

# Annex

(See next page)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of PUSCH Type A repetitions | Maximum value of K (the number of repetitions) = 32 For DG PUSCH, the number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. For Type 1 CG PUSCH, the number of repetitions is indicated by repK-r17 For Type 2 CG PUSCH, the number of repetitions is indicated in a TDRA list or by repK-r17. | 5-14, 5-16, or 5-17 | Yes | N/A | UE does not support more than 16 repetitions PUSCH. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-2 | PUSCH Type A repetitions based on available slots | Transmission occasions for [K] repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots. | One of {5-14, 5-16, 5-17, [11-6, 30-1]} | Yes | N/A | UE does not support dynamic or configured grant PUSCH repetitions counted on the basis of available slots. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and CG in RRC connected mode. | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3a | Repetition of TB processing over multi-slot PUSCH | Support repetition of TB processing over multi-slot PUSCH in RRC connected mode. | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4 | The maximum duration for DM-RS bundling | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH | FFS | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4a | DM-RS bundling for PUSCH repetition type A | Support DM-RS bundling for PUSCH repetition type A | [30-4], [30-1] or [30-2] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4b | DM-RS bundling for PUSCH repetition type B | Support DM-RS bundling for PUSCH repetition type B | [30-4], [11-5] [30-1] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4c | DM-RS bundling for TB processing over multi-slot PUSCH | Support DM-RS bundling for TB processing over multi-slot PUSCH | [30-4], [30-3] | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFS | [No] | [N/A] | Note: If a UE reports support of FG 30-3a and 30-4c, the UE supports DMRS bundling for the repetitions of TBoMS | Optional with capability signalling |
| 30. NR_cov_enh | 30-4d | DMRS bunding for PUCCH repetitions | Support DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4 | [30-4], [4-23] | Yes | N/A | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4e | Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [30-4a] or [30-4b] or [30-4c] | Yes | N/A | UE does not support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4f | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [30-4d] | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4g | [Restart DM-RS bundling after the events that violate power consistency and phase continuity] | [Support restarting DM-RS bundling after the events that violate power consistency and phase continuity] | [30-4] | Yes | N/A | [UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity] | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4h | DM-RS bundling for non-back-to-back transmission | Support DM-RS bundling for [non-back-to-back transmission for consecutive slots] for PUSCH and PUCCH only for [corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d)] | [30-4a, 30-4b, 30-4c, or 30-4d] | Yes | N/A | UE does not Support DM-RS bundling for non-back-to-back transmission | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 | [4-23 and/or 25-2] | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | Per UE | No | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-6 | Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | [Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI] |  | Yes | N/A | UE does not support repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. | [Per UE] | [No] | [No] | [N/A ] |  | Optional with capability signalling |