3GPP TSG RAN WG1 #109-e	R1- 2204599

e-Meeting, May 9th – 20th, 2022

Agenda item:		8.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Initial access aspects

Document for:		Discussion and Decision

# Introduction

The work item on extending NR operation to frequency bands from 52.6 GHz to 71 GHz was updated at RAN#92-e [1]. Before that 3GPP carried out a study on required changes to NR using existing DL/UL waveform to support operation between 52.6 GHz and 71GHz [2]. In this contribution we continue the discussion on the following objectives of the WID (Note: changes compared to previous version highlighted):

- Supports 120kHz SCS for SSB and 120kHz SCS for initial access related signals/channels in an initial BWP. - Study and specify, if needed, additional SCS (240kHz, 480kHz, 960kHz) for SSB, and additional SCS(480kHz, 960kHz) for initial access related signals/channels in initial BWP. - Study and specify, if needed, additional SCS (480kHz, 960kHz) for SSB for cases other than initial access. - Note: coverage enhancement for SSB is not pursued. - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints: - Limited sync raster entry numbers - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n257 is 599). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz. - only 480kHz CORESET#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS. - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis. - 960 kHz numerology for the SSB is not supported by the UE for initial access in Rel-17. - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible - Note: 480 kHz is an optional SSB numerology for initial access for the UE. A UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access) - Note: Dependency or lack thereof for a UE supporting 480kHz and/or 960kHz numerology for data and control to also support 480kHz SSB numerology for initial access is to be tackled as part of UE capability discussion. - Support ANR and PCI confusion detection for 120, 480 and 960kHz SCS based SSB, support CORESET#0/Type0-PDCCH configuration in MIB of 120, 480 and 960kHz SSB - FFS: additional method(s) to enable support to obtain neighbour cell SIB1 contents related to CGI reporting - Only 1 CORESET#0/Type0-PDCCH SCS supported for each SSB SCS, i.e., (120, 120), (480, 480) and (960, 960). - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis. - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible - Note: From UE perspective, ANR detection for 480/960kHz SCS based SSB is not supported if the UE does not support 480/960 SCS for SSB. - Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification.

In this contribution we discuss the remaining open issues for initial access.

# Discussion

## Value range for NSSBQCL

In last meeting, the following the RAN2 agreements, ,  a working assumption was taken to limit the Q signalling in MIB to 1 bit, and use NSSBQCL value range of {32,64}:

Working assumption - Use 1 bit for Q in MIB - SubcarrierSpacingCommon field will be used to convey value of NSSBQCL={32, 64} for operation with shared spectrum channel access - Note that this is revising the working assumption made in RAN1#107-e on “use 2 bits for Q, {SubcarrierSpacingCommon, spare bit in MIB}”

To conclude this discussion it is proposed to confirm the afore working assumption.

Proposal 1: Confirm the working assumption from RAN1#107e to use one bit for Q in MIB (to convey NSSBQCL={32,64}.

## Type0-PDCCH CORESET and monitoring configuration

In RAN1#108e, the Type0-PDCCH CORESET configuration was discussion and a working assumption reached for the offsets:_

Working Assumption - The following table is used for set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120}, {480, 480}, and {960, 960} kHz for FR2-2. - FFS: whether/how to define X, if defined, candidate values {56, 76} - Text Proposal #4-2D (for 38.213, Section 13) in section 3 of R1-2202503 is endorsed - Note: this working assumption can be revisited once RAN4 finalizes the further details of the channelization. Index; SS/PBCH block and CORESET multiplexing pattern; Number of RBs NRBCORESET; Number of Symbols NsymbCORESET; Offset (RBs). 0; 1; 24; 2; 0. 1; 1; 24; 2; 4. 2; 1; 48; 1; 0. 3; 1; 48; 1; 14. 4; 1; 48; 1; 28. 5; 1; 48; 2; 0. 6; 1; 48; 2; 14. 7; 1; 48; 2; 28. 8; 1; 96; 1; 0. 9; 1; 96; 1; X. 10; 1; 96; 2; 0. 11; 1; 96; 2; X. 12; 3; 24; 2; -20 if kSSB=0 -21 if kSSB > 0. 13; 3; 24; 2; 24. 14; 3; 48; 2; -20 if kSSB=0 -21 if kSSB > 0. 15; 3; 48; 2; 48.

This agreement is reflected in TS38.213 as follows:

Table 13-10A: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz, {480, 480} kHz, or {960, 960} kHz for FR2-2 Index; SS/PBCH block and CORESET multiplexing pattern; Number of RBs NRBCORESET; Number of Symbols NsymbCORESET; Offset (RBs). 0; 1; 24; 2; 0. 1; 1; 24; 2; 4. 2; 1; 48; 1; 0. 3; 1; 48; 1; 14. 4; 1; 48; 1; 28. 5; 1; 48; 2; 0. 6; 1; 48; 2; 14. 7; 1; 48; 2; 28. 8; 1; 96; 1; 0. 9. 10; 1; 96; 2; 0. 11. 12; 3; 24; 2; -20 if kSSB=0 -21 if kSSB>0. 13; 3; 24; 2; -24. 14; 3; 48; 2; -20 if kSSB=0 -21 if kSSB>0. 15; 3; 48; 2; -48.

In RAN4#102e, RAN4 reached following agreements [3] for unlicenced and licenced for the synchronisation raster:

Unlicensed: Agreement: Use the fixed RF channel raster with the step size of 1680 (100.8 MHz) as baseline to define the channel raster for the unlicensed band, and accordingly provide the channel raster numbers - To Check if the above solution can support CA with different bandwidth combinations - If there is issue identified, then the above agreement can be revisited Agreement: Specify SSB SCS 960KHz in the GSCN and add a note to clarify that 960KHz SCS cannot be used for initial access. Licensed: Agreement: GSCN for licensed band - GSCN step sizes - 3 for 120 kHz - 12 for 480 kHz - 6 for 960 kHz - Those values will be further checked after agreeing on the SU values. Agreement: for RF channel raster for licensed band, SCS based channel raster is used for licensed bands, and the step size for channel raster is: - 2 for 120kHz - 8 for 480kHz - 16 for 960kHz

In this section we re-evaluate the RB offsets from the working assumption in last meeting considering the channelization assumption(s) taken by RAN4, as provided in Appendix A and floating channelization according to [1]. In the analysis we assume SCSs, CORESET#0 sizes and channel bandwidths as described in Table 1.

Table 1. Assumptions for used SCSs and channel bandwidths

| SCS [kHz] | CORESET#0 size [#PRB] | Channel bandwidth [#PRBs] (95 % SU) |
| --- | --- | --- |
| 120 | 24 | 66 (100 MHz ch.) |
|  | 48 | 66 (100 MHz ch.) |
|  | 96 | 264 (400 MHz ch.) |
| 480 | 24 | 66 (400 MHz ch.) |
|  | 48 | 66 (400 MHz ch.) |
|  | 96 | 132 (800 MHz ch.) |
| 960 | 24 | 33 (400 MHz ch.) |
|  | 48 | 66 (800 MHz ch.) |
|  | 96 | 132 (1600 MHz ch.) |

Range of ARFCN and GSCN for the floating channelization case as shown in Table 2. For the fixed channelization we select the GSCN so that each channel has at least one synchronization raster point.

Table 2. Assumptions for channel bandwidth and synchronisation and RF raster

| SCS [kHz] | CORESET#0 size [#PRB] | Channel bandwidth [#PRBs] (95 % SU) | Range of GSCN (First – <Step size> – Last) | Range of ARFCN (First – <Step size> – Last) |
| --- | --- | --- | --- | --- |
| 120 | 24 | 66 (100 MHz ch.) | 24153 < 3 > 24960 | 2563333 <2> 2794999 (57050.04 - 70950.00 MHz) |
|  | 48 | 66 (100 MHz ch.) |  |  |
|  | 96 | 264 (400 MHz ch.) |  |  |
| 480 | 24 | 66 (400 MHz ch.) | 24157 < 12 > 24949 | 2565835 <8> 2792499 (57200.16 - 70800.00 MHz) |
|  | 48 | 66 (400 MHz ch.) |  |  |
|  | 96 | 132 (800 MHz ch.) |  |  |
| 960 | 24 | 33 (400 MHz ch.) | 24160 < 6 > 24952 | 2565835 <16> 2792491 (57200.16 - 70799.52 MHz) |
|  | 48 | 66 (800 MHz ch.) |  |  |
|  | 96 | 132 (1600 MHz ch.) |  |  |

In Table 3 we provide the required RB offsets for CORESET#0 configuration in the considered scenarios for both fixed and floating channelization assumption.

Table 3. Required RB offsets for different CORESET#0 sizes and sub-carrier spacings.

| SCS [kHz] | CORESET#0 size [#PRB] | Required RB offsets fixed channel. | Required RB offsets floating channel. |
| --- | --- | --- | --- |
| 120 | 24 | 0, 4 | 0, 4 |
|  | 48 | 0, 14, 28 | 0, 14, 28 |
|  | 96 | 0 | 0 |
| 480 | 24 | 0, 4 | 0,4 |
|  | 48 | 0, 14, 28 | 0, 14, 28 |
|  | 96 | 0 | 0 |
| 960 | 24 | 0, 4 | 0, 4 |
|  | 48 | 0, 14, 28 | 0, 14, 28 |
|  | 96 | 0 | 0 |

One can observe from the results in Table 3 that required RB offsets for fixed channelization scheme are typically a subset of the ones required for floating channelization scheme. It can also be observed that the required values are covered by the working assumption made in last meeting. Thus, we make the following observations:

Observation 1: Required RB offsets for fixed channelization scheme are typically a subset of the ones required for floating channelization scheme.

Observation 2: The following RB offsets can support both fixed and floating channelization:

- 120 kHz SCS

  - 24 PRB CORESET#0: RB offsets 0, 4

  - 48 PRB CORESET#0: RB offsets 0, 14, 28

  - 96 PRB CORESET#0: RB offset 0

- 480 kHz SCS

  - 24 PRB CORESET#0: RB offsets 0, 4

  - 48 PRB CORESET#0: RB offsets 0, 14, 28

  - 96 PRB CORESET#0: RB offset 0

- 960 kHz SCS

  - 24 PRB CORESET#0: RB offsets 0, 4

  - 48 PRB CORESET#0: RB offsets 0, 14, 28

  - 96 PRB CORESET#0: RB offset 0

Observation 3: Working assumption made in RAN1#108-e is valid apart that for 96 PRB CORESET#0 only offset 0 is needed.

Proposal 2: Confirm the working assumption from RAN1#108-e on the CORESET#0 symbol, RB and offset configuration for 24 PRB and 48 PRB CORESET sizes.

Proposal 3: For 96 PRB CORESET#0 size, support only offset 0.

# Conclusion

In this contribution we discussed remaining open aspects related to the initial access mechanisms for beyond 52GHz deployments for NR. Based on the discussion we made the following observations and proposals.

Following the last meeting discussion we propose to confirm the NSSBQCL range:-

Proposal 1: Confirm the working assumption from RAN1#107e to use one bit for Q in MIB (to convey NSSBQCL={32,64}.

On the RB offsets for CORESET#0 configuration:

Observation 1: Required RB offsets for fixed channelization scheme are typically a subset of the ones required for floating channelization scheme.

Observation 2: The following RB offsets can support both fixed and floating channelization:

- 120 kHz SCS

  - 24 PRB CORESET#0: RB offsets 0, 4

  - 48 PRB CORESET#0: RB offsets 0, 14, 28

  - 96 PRB CORESET#0: RB offset 0

- 480 kHz SCS

  - 24 PRB CORESET#0: RB offsets 0, 4

  - 48 PRB CORESET#0: RB offsets 0, 14, 28

  - 96 PRB CORESET#0: RB offset 0

- 960 kHz SCS

  - 24 PRB CORESET#0: RB offsets 0, 4

  - 48 PRB CORESET#0: RB offsets 0, 14, 28

  - 96 PRB CORESET#0: RB offset 0

Observation 3: Working assumption made in RAN1#108-e is valid apart that for 96 PRB CORESET#0 only offset 0 is needed.

Proposal 2: Confirm the working assumption from RAN1#108-e on the CORESET#0 symbol, RB and offset configuration for 24 PRB and 48 PRB CORESET sizes.

Proposal 3: For 96 PRB CORESET#0 size, support only offset 0.

# References

- RP-211584, “Revised WID: Extending current NR operation to 71 GHz”, Qualcomm, Intel

- TR 38.808, “Study on supporting NR from 52.6GHz to 71 GHz", 3GPP

- R4-2206535, “WF on system parameters of FR2-2”, Intel Corporation

- R4-2208479, System parameters for a NR band in the range 52.6GHz – 71GHz, Nokia, Nokia Shanghai Bell

# Appendix A

For fixed channelization we used the following principles

- Select lowest possible ARFCN point for the first RF channel in the frequency range 57-71 GHz for each SCS and bandwidth pair

  - {120 kHz, 100 MHz}, {120 kHz, 400 MHz}

  - {480 kHz, 400 MHz}, {480 kHz, 800 MHz}, {480 kHz, 1600 MHz}

  - {960 kHz, 400 MHz}, {960 kHz, 800 MHz}, {960 kHz, 1600 MHz}, {960 kHz, 2000 MHz}

- Minimize distance between adjacent RF channels by having

  - Distance between center of the channels being integer multiple of 960 kHz

  - Guard bands of different RF channels are not overlapping

  - For 100MHz channel bandwidth the channel raster step is minimized

  - For 400MHz, 800MHz, 1600MHz and 2GHz channel bandwidths the channel raster locations are aligned to enable smooth CA operation and minimize the gap between channels

  - Note: Only the cell-defining SSB of the PCell is always on the sync raster.

Figure 1 Principle of channel allocation in fixed channelization.

Table 4 Center frequency of 1st channel in 57-71 GHz band and channel raster step size

| SCS [kHz] | Bandwidth [MHz] | 1st reference freq. [Hz] | Channel raster step [Hz] |
| --- | --- | --- | --- |
| 120 | 100 | 57050040000 | 100800000 |
| 120 | 400 | 57200040000 | 403200000 |
| 480 | 400 | 57200040000 | 403200000 |
| 480 | 800 | 57400020000 | 806400000 |
| 480 | 1600 | 57800040000 | 1612800000 |
| 960 | 400 | 57200040000 | 403200000 |
| 960 | 800 | 57400020000 | 806400000 |
| 960 | 1600 | 57800040000 | 1612800000 |
| 960 | 2000 | 58000020000 | 2016000000 |

Table 5 GSCNs in fixed channelization

| SCS | 120 kHz |  |  | 480 kHz |  |  |  | 960 kHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth | 100 MHz | 400 MHz |  | 400 MHz | 800 MHz | 1600 MHz |  | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
|  | 24153 | 24153 |  | 24157 | 24157 | 24157 |  | 24160 | 24160 | 24166 | 24166 |
|  | 24159 |  |  | 24181 |  |  |  | 24184 |  |  |  |
|  | 24165 |  |  | 24205 | 24205 |  |  | 24208 | 24208 |  |  |
|  | 24171 |  |  | 24229 |  |  |  | 24232 |  |  |  |
|  | 24177 | 24177 |  | 24253 | 24253 | 24253 |  | 24256 | 24256 | 24256 |  |
|  | 24183 |  |  | 24277 |  |  |  | 24280 |  |  | 24280 |
|  | 24189 |  |  | 24301 | 24301 |  |  | 24304 | 24304 |  |  |
|  | 24195 |  |  | 24325 |  |  |  | 24322 |  |  |  |
|  | 24201 | 24201 |  | 24349 | 24349 | 24349 |  | 24346 | 24346 | 24352 |  |
|  | 24207 |  |  | 24373 |  |  |  | 24370 |  |  |  |
|  | 24213 |  |  | 24397 | 24397 |  |  | 24394 | 24394 |  | 24400 |
|  | 24219 |  |  | 24421 |  |  |  | 24418 |  |  |  |
|  | 24225 | 24225 |  | 24445 | 24445 | 24445 |  | 24442 | 24442 | 24442 |  |
|  | 24231 |  |  | 24469 |  |  |  | 24466 |  |  |  |
|  | 24234 |  |  | 24493 | 24493 |  |  | 24490 | 24490 |  |  |
|  | 24240 |  |  | 24517 |  |  |  | 24514 |  |  | 24514 |
|  | 24246 | 24246 |  | 24529 | 24541 | 24541 |  | 24532 | 24538 | 24538 |  |
|  | 24252 |  |  | 24553 |  |  |  | 24556 |  |  |  |
|  | 24258 |  |  | 24577 | 24577 |  |  | 24580 | 24580 |  |  |
|  | 24264 |  |  | 24601 |  |  |  | 24604 |  |  |  |
|  | 24270 | 24270 |  | 24625 | 24625 | 24625 |  | 24628 | 24628 | 24628 | 24628 |
|  | 24276 |  |  | 24649 |  |  |  | 24652 |  |  |  |
|  | 24282 |  |  | 24673 | 24673 |  |  | 24676 | 24676 |  |  |
|  | 24288 |  |  | 24697 |  |  |  | 24700 |  |  |  |
|  | 24294 | 24294 |  | 24721 | 24721 | 24721 |  | 24724 | 24724 | 24724 |  |
|  | 24300 |  |  | 24745 |  |  |  | 24742 |  |  | 24748 |
|  | 24306 |  |  | 24769 | 24769 |  |  | 24766 | 24766 |  |  |
|  | 24312 |  |  | 24793 |  |  |  | 24790 |  |  |  |
|  | 24318 | 24318 |  | 24817 | 24817 | 24817 |  | 24814 | 24814 | 24814 |  |
|  | 24324 |  |  | 24841 |  |  |  | 24838 |  |  |  |
|  | 24330 |  |  | 24865 | 24865 |  |  | 24862 | 24862 |  |  |
|  | 24336 |  |  | 24889 |  |  |  | 24886 |  |  |  |
|  | 24339 | 24342 |  | 24913 | 24913 |  |  | 24910 | 24910 |  |  |
|  | 24345 |  |  | 24937 |  |  |  | 24934 |  |  |  |
|  | 24351 |  |  |  |  |  |  |  |  |  |  |
|  | 24357 |  |  |  |  |  |  |  |  |  |  |
|  | 24363 | 24363 |  |  |  |  |  |  |  |  |  |
|  | 24369 |  |  |  |  |  |  |  |  |  |  |
|  | 24375 |  |  |  |  |  |  |  |  |  |  |
|  | 24381 |  |  |  |  |  |  |  |  |  |  |
|  | 24387 | 24387 |  |  |  |  |  |  |  |  |  |
|  | 24393 |  |  |  |  |  |  |  |  |  |  |
|  | 24399 |  |  |  |  |  |  |  |  |  |  |
|  | 24405 |  |  |  |  |  |  |  |  |  |  |
|  | 24411 | 24411 |  |  |  |  |  |  |  |  |  |
|  | 24417 |  |  |  |  |  |  |  |  |  |  |
|  | 24423 |  |  |  |  |  |  |  |  |  |  |
|  | 24429 |  |  |  |  |  |  |  |  |  |  |
|  | 24435 | 24435 |  |  |  |  |  |  |  |  |  |
|  | 24441 |  |  |  |  |  |  |  |  |  |  |
|  | 24444 |  |  |  |  |  |  |  |  |  |  |
|  | 24450 |  |  |  |  |  |  |  |  |  |  |
|  | 24456 | 24456 |  |  |  |  |  |  |  |  |  |
|  | 24462 |  |  |  |  |  |  |  |  |  |  |
|  | 24468 |  |  |  |  |  |  |  |  |  |  |
|  | 24474 |  |  |  |  |  |  |  |  |  |  |
|  | 24480 | 24480 |  |  |  |  |  |  |  |  |  |
|  | 24486 |  |  |  |  |  |  |  |  |  |  |
|  | 24492 |  |  |  |  |  |  |  |  |  |  |
|  | 24498 |  |  |  |  |  |  |  |  |  |  |
|  | 24504 | 24504 |  |  |  |  |  |  |  |  |  |
|  | 24510 |  |  |  |  |  |  |  |  |  |  |
|  | 24516 |  |  |  |  |  |  |  |  |  |  |
|  | 24522 |  |  |  |  |  |  |  |  |  |  |
|  | 24528 | 24528 |  |  |  |  |  |  |  |  |  |
|  | 24534 |  |  |  |  |  |  |  |  |  |  |
|  | 24540 |  |  |  |  |  |  |  |  |  |  |
|  | 24546 |  |  |  |  |  |  |  |  |  |  |
|  | 24549 | 24552 |  |  |  |  |  |  |  |  |  |
|  | 24555 |  |  |  |  |  |  |  |  |  |  |
|  | 24561 |  |  |  |  |  |  |  |  |  |  |
|  | 24567 |  |  |  |  |  |  |  |  |  |  |
|  | 24573 | 24573 |  |  |  |  |  |  |  |  |  |
|  | 24579 |  |  |  |  |  |  |  |  |  |  |
|  | 24585 |  |  |  |  |  |  |  |  |  |  |
|  | 24591 |  |  |  |  |  |  |  |  |  |  |
|  | 24597 | 24597 |  |  |  |  |  |  |  |  |  |
|  | 24603 |  |  |  |  |  |  |  |  |  |  |
|  | 24609 |  |  |  |  |  |  |  |  |  |  |
|  | 24615 |  |  |  |  |  |  |  |  |  |  |
|  | 24621 | 24621 |  |  |  |  |  |  |  |  |  |
|  | 24627 |  |  |  |  |  |  |  |  |  |  |
|  | 24633 |  |  |  |  |  |  |  |  |  |  |
|  | 24639 |  |  |  |  |  |  |  |  |  |  |
|  | 24645 | 24645 |  |  |  |  |  |  |  |  |  |
|  | 24651 |  |  |  |  |  |  |  |  |  |  |
|  | 24654 |  |  |  |  |  |  |  |  |  |  |
|  | 24660 |  |  |  |  |  |  |  |  |  |  |
|  | 24666 | 24666 |  |  |  |  |  |  |  |  |  |
|  | 24672 |  |  |  |  |  |  |  |  |  |  |
|  | 24678 |  |  |  |  |  |  |  |  |  |  |
|  | 24684 |  |  |  |  |  |  |  |  |  |  |
|  | 24690 | 24690 |  |  |  |  |  |  |  |  |  |
|  | 24696 |  |  |  |  |  |  |  |  |  |  |
|  | 24702 |  |  |  |  |  |  |  |  |  |  |
|  | 24708 |  |  |  |  |  |  |  |  |  |  |
|  | 24714 | 24714 |  |  |  |  |  |  |  |  |  |
|  | 24720 |  |  |  |  |  |  |  |  |  |  |
|  | 24726 |  |  |  |  |  |  |  |  |  |  |
|  | 24732 |  |  |  |  |  |  |  |  |  |  |
|  | 24738 | 24738 |  |  |  |  |  |  |  |  |  |
|  | 24744 |  |  |  |  |  |  |  |  |  |  |
|  | 24750 |  |  |  |  |  |  |  |  |  |  |
|  | 24756 |  |  |  |  |  |  |  |  |  |  |
|  | 24759 | 24762 |  |  |  |  |  |  |  |  |  |
|  | 24765 |  |  |  |  |  |  |  |  |  |  |
|  | 24771 |  |  |  |  |  |  |  |  |  |  |
|  | 24777 |  |  |  |  |  |  |  |  |  |  |
|  | 24783 | 24783 |  |  |  |  |  |  |  |  |  |
|  | 24789 |  |  |  |  |  |  |  |  |  |  |
|  | 24795 |  |  |  |  |  |  |  |  |  |  |
|  | 24801 |  |  |  |  |  |  |  |  |  |  |
|  | 24807 | 24807 |  |  |  |  |  |  |  |  |  |
|  | 24813 |  |  |  |  |  |  |  |  |  |  |
|  | 24819 |  |  |  |  |  |  |  |  |  |  |
|  | 24825 |  |  |  |  |  |  |  |  |  |  |
|  | 24831 | 24831 |  |  |  |  |  |  |  |  |  |
|  | 24837 |  |  |  |  |  |  |  |  |  |  |
|  | 24843 |  |  |  |  |  |  |  |  |  |  |
|  | 24849 |  |  |  |  |  |  |  |  |  |  |
|  | 24855 | 24855 |  |  |  |  |  |  |  |  |  |
|  | 24861 |  |  |  |  |  |  |  |  |  |  |
|  | 24864 |  |  |  |  |  |  |  |  |  |  |
|  | 24870 |  |  |  |  |  |  |  |  |  |  |
|  | 24876 | 24876 |  |  |  |  |  |  |  |  |  |
|  | 24882 |  |  |  |  |  |  |  |  |  |  |
|  | 24888 |  |  |  |  |  |  |  |  |  |  |
|  | 24894 |  |  |  |  |  |  |  |  |  |  |
|  | 24900 | 24900 |  |  |  |  |  |  |  |  |  |
|  | 24906 |  |  |  |  |  |  |  |  |  |  |
|  | 24912 |  |  |  |  |  |  |  |  |  |  |
|  | 24918 |  |  |  |  |  |  |  |  |  |  |
|  | 24924 | 24924 |  |  |  |  |  |  |  |  |  |
|  | 24930 |  |  |  |  |  |  |  |  |  |  |
|  | 24936 |  |  |  |  |  |  |  |  |  |  |
|  | 24942 |  |  |  |  |  |  |  |  |  |  |
|  | 24948 |  |  |  |  |  |  |  |  |  |  |
|  | 24954 |  |  |  |  |  |  |  |  |  |  |