3GPP TSG RAN WG1 #109-e	R1- 2204604

e-Meeting, May 9th – 20th, 2022

Agenda item:		8.7.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Open issues on Idle/Inactive mode power saving

Document for:		Discussion and Decision

# Introduction

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1]

  - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1]

NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #100 meeting

  - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1]

NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required

The release 17 work item [1] on UE power saving includes the following objective:

In this contribution, we discuss about the remaining aspects related to the PEI and TRS occasion for IDLE/Inactive UEs.

# PEI configuration

## Type2A-PDCCH CSS configuration

In last meeting the necessary changes to introduce Type2A-PDCCH CSS to Section 10.4 of TS38.213 were discussed. The discussed changes were related to following text in Section 10.4:

## 10.1 UE procedure for determining physical downlink control channel assignment [text omitted] If a UE is provided a zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, or is not provided searchSpaceBroadcast, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set as described in clause 13, and the UE is provided a C-RNTI, the UE monitors PDCCH candidates only at monitoring occasions associated with a SS/PBCH block, where the SS/PBCH block is determined by the most recent of - a MAC CE activation command indicating a TCI state of the active BWP that includes a CORESET with index 0, as described in [6, TS 38.214], where the TCI-state includes a CSI-RS which is quasi-co-located with the SS/PBCH block, or - a random access procedure that is not initiated by a PDCCH order that triggers a contention-free random access procedure If a UE monitors PDCCH candidates for DCI formats with CRC scrambled by a C-RNTI and the UE is provided a non-zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, or monitors PDCCH candidates for DCI formats with CRC scrambled by a MCCH-RNTI or a G-RNTI and the UE is provided a non-zero value for searchSpaceBroadcast in pdcch-Config-MCCH and pdcch-Config-MTCH for a Type0/0B-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set, or of the Type0/0B-PDCCH set, respectively, based on the search space set associated with the value of searchSpaceID. [text omitted]

First, in order of appearance in 38.213, there was discussion whether to add Type-2A-PDCCH CSS to text in Section 10.1 (quoted below) that determines the PDCCH monitoring QCL relation when searchSpaceID in PDCCH-ConfigCommon equals zero and C-RNTI is provided to the UE. This text was introduced as an outcome of the RAN1#95 decisions related to adding the option of configuring a TCI-state to CORESET#0 (updating the previous agreement), quoted in Annex A. As per agreement this decision relates to the CONNECED mode behavour. Now the original text in 38.213v15.4.0 was defined so that the first part on Type0/0A/2-PDCCH CSS sets was separated from the latter part relating to C-RNTI configuration. In RAN1#98, a CR was endorsed [3], merging these two sencences but still differentiating the C-RNTI part via ‘if’ i.e. “, and if the UE is provided C-RNTI”. However in final approved version the ‘if’ was omitted [4]. As a result of this the described functionality seems to relate to CONNECTED mode, it does not appear necessary to introduce reference to the Type-2A-PDCCH CSS.

In context of SDT, it is good to note that this specification part does not cover Type1A-PDCCH CSS, nor Type1-PDCCH CSS.

Observation: It does not appear necessary to add reference to Type2A-PCCCH CSS to the text describing the UE behaviour when is configured with C-RNTI and is provided with SS ID=0 for searchSpaceID in PDCCH-ConfigCommon text in 38.213.

Proposal: Reference to Type2A-PCCCH CSS is not added to the text describing the UE behaviour when is configured with C-RNTI and is provided with SS ID=0 for searchSpaceID in PDCCH-ConfigCommon text in 38.213.

Second point (in order of appearance), whether to add Type-2A-PDCCH CSS to the text in Section 10.1 (quoted below) that determines PDCCH monitoring when UE is configured with C-RNTI and searchSpaceID in PDCCH-ConfigCommon has non-zero value was introduced based on agreement made in RAN1#94bis, quoted in Annex A.  As can be seen from the agreement, this was also focused on the UE assumptions in CONNECTED mode.

Observation: It does not appear necessary to add reference to Type2A-PCCCH CSS to the text describing the UE behaviour when is configured with C-RNTI and is provided non-zero SS ID for searchSpaceID in PDCCH-ConfigCommon text in 38.213.

Proposal: Reference to Type2A-PCCCH CSS is not added to the text describing the UE behaviour when is configured with C-RNTI and is provided non-zero SS ID for searchSpaceID in PDCCH-ConfigCommon text in 38.213.

## PEI sub-grouping configuration

In RAN2 meeting #117e, following agreement was made:

A PEI-capable UE stores its “last used cell” information. FFS on how to capture this in the specifications. Do not introduce an associated timer for the “last used cell” information stored by UE. The “lastUsedCellOnly” indication is a cell-level configuration and there is no per-subgroup indication. Introduce a one-bit indication of lastUsedCellOnly in PEI-Config. RAN2 clarifies the meaning of “last used cell only”: When a cell broadcasts “last used cell only”, a UE monitors PEI only if its last connection was released by this cell. A PEI-capable UE must support at least UEID-based subgrouping method. RAN2 confirms that “PEI without subgrouping” can be implemented by configuring PEI plus UEID subgrouping with one subgroup. “PEI without subgrouping” can be configured by only one method. When PEI is applied with eDRX, the UEID for UEID-based subgrouping is determined by 5G-S-TMSI mod 32768. No special handling or configuration is introduced for PEI monitoring with PTW (i.e., PEI is applicable to each PO within PTW) Send an informative LS to RAN3/SA2/CT1 for RAN2 agreements about PEI and paging subgrouping and ask question about mismatched understanding of “last cell” between network and UE. If a UE cannot find its subgroup ID with the PEI configurations in a cell, it monitors legacy paging.

As can be seen by the highlighted text, it was confirmed by RAN2 that in the configuration of sub-grouping, there is always one sub-group configured. This is also reflected in the CR [8], i.e. related fields in PEI-Config are mandatory.

Now in last meeting some changes were agreed to related parts in RAN1 specificiation, but further changes are needed to adopt the RAN2#117e agreements. Thus, following TPs are proposed to TS38.213 and TS38.212:

Proposal: Adopt following TP to TS38.213 Section 10.4:

## 10.4A PDCCH monitoring for early indication of paging A UE can be provided the following for detection of a DCI format 2_7 in RRC_IDLE state or in RRC_INACTIVE state [12, TS 38.331] - a search space set, by peiSearchSpace, to monitor PDCCH for detection of DCI format 2_7 according to a Type2A-PDCCH CSS set as described in clause 10.1 - a number of frames, by PEI-F_offset, from the start of a first paging frame of paging frames associated with a number of PDCCH monitoring occasions for DCI format 2_7 [17, TS 38.304] to the start of a frame - a number of symbols, by firstPDCCH-MonitoringOccasionOfPEI-O, from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 - a size, by payloadSizeDCI_format2_7 - a number of subgroups per paging occasion, NSGPO, by subgroupsNumPerPO - a number of paging occasions associated with the number of PDCCH monitoring occasions for DCI format 2_7, NPOPEI, by PONumPerPEI A paging indication field of DCI format 2_7 includes NPOPEI segments of K bits, where K=NSGPO if NSGPO>0 and K=1 if NSGPO is not provided. For a subgroup index iSG, 0≤iSG<K, a UE determines a value for the iPO⋅K+iSG bit in the paging indication field, where iPO=UE_IDmodN⋅NS+iSmodNPOPEI is a paging occasion index, and UE_ID, N, NS, iSG, and iS are defined in [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion. If NPOPEI<NS, the number of symbols from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 that is associated with paging occasion index iPO is the iSNPOPEI+1-th value from the NSNPOPEI values provided by firstPDCCH-MonitoringOccasionOfPEI-O.

Proposal: Adopt following TP to TS38.212 Section 7.3.1.3.8:

##### 7.3.1.3.8 Format 2_7 DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs. The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field – NPOPEINSGPO bit(s), where - NPOPEI is the number of paging occasions configured by higher layer parameter PONumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - NSGPOis the number of sub-groups of a paging occasion configured by higher layer parameter subgroupsNumPerPO, if subgroupsNumPerPO is configured; otherwise NSGPO is set to 1. - Each bit in the field indicates one UE subgroup of a paging occasion if subgroupsNumPerPO is configured; otherwise each bit in the field indicates the UE group of a paging occasion. - TRS availability indication – 1, 2, 3, 4, 5, or 6 bits if TRS-ResourceSetConfig is configured; 0 bits otherwise. The size of DCI format 2_7 is indicated by the higher layer parameter payloadSizeDCI_format2_7, according to Clause 10.4A of [5, TS 38.213]. The number of information bits in format 2_7 shall be equal to or less than the payload size of format 2_7. If the number of information bits in format 2_7 is less than the size of format 2_7, the remaining bits are reserved.

## Minimum time gap between PO and PEI-O

In past meetings it has been discussed whether there would be a need to define some minimum time gap between the PO and PEI to ensure that UEs have sufficient time to process the PEI and wake-up/prepare to paging reception. While it can be understood that this relates mainly to UE processing, it also has some system implications which would be good to consider.

For UE application delay perspective, considering also the needed synchronisation, the value of the minimum gap between PEI and PO could be considered to depend on the radio conditions experienced by the UE. As it has been discussed under power saving evaluations, UE’s in low SINR may require reception of several SSBs to ensure sufficient synchronisation to receive PDSCH, where at least 3 SSBs has been considered. Hence, it would appear that the minimum time gap between PO and PEI should be such that it allows sufficient number of SSB occasions to acquire sufficient synchronisation for paging message reception.

Observation: Minimum time gap between PO and PEI should enable UE to acquire synchronisation for paging message reception.

It has also been proposed that the minimum gap between PO and PEI-O would follow the same minimum time as assumed for DCP defined in Rel-16, i.e. 3ms. The purpose of this time would be to allow time for the UE to prepare for the reception of paging. This would not enable any SSB burst to be fitted between the PO and PEI-O.

Observation: From receiver preparation perspective minimum gap of 3ms could be considered.

# Behaviour and signaling related to TRS occassions

## Validity duration and reference point

In RAN1#108 some conclusions were made in relation to the validity duration reference point and L1 availability indication association:

Conclusion A UE can receive different TRS availability indication content from PEI and associated paging PDCCH. - Note: It is up to UE implementation whether UE receives both PEI and the associated paging PDCCH to receive TRS availability indication Conclusion No consensus to support - For PEI based availability indication, the reference time of the start of validity duration follows the reference time for associated monitoring occasion for the paging DCI.

It was also discussed, whether the reference point for validity duration is per MO or whether it is joint for PO or PEI-O. It was agreed to consider two alternative TPs for the reference point determination:

Agreement Down-select one of the TPs to clarify the current default DRX cycle used for determining the reference point for start of validity duration for an availability indication - Alt1: Adopt TP#1 - Alt2: Adopt TP#2 - Note: TRS availability indication transmitted in all PDCCH monitoring occasions of the same PEI-O or PO is same. - Note: Reference point for TRS availability indication in a given DCI is identical for all UEs regardless of UE specific DRX value. [TPs omitted, show in Annex]

Now as noted above, other TP, TP#2, aims to align the validity durations for different MOs (inside e.g. PO), while TP#1 maintains the earlier approach of the UE reference point definition to be MO specific. While it can be argued that TP#2 would be a simplification, it is not clear whether this is actually the case as UE (and NW) would in practise only keep track when given TRS resource will cease to be available, thus there does not appear to be any complexity difference. Thus, it is preferred to select TP#1 among the options.

Proposal: Adopt TP#1 for clarifying the reference point definition for current default DRX cycle.

## Rate matching behaviour in relation to TRS occasions

In last meeting the possible implications of the TRS occasions for the IDLE/Inactive mode UEs to the UE rate matching behaviour were discussed. As discussed in past meetings it would seem natural to keep the assumption of the IDLE/Inactive mode UEs similar as in earlier releases, so that UE does not expect to ratematch any RS’es (apart SSBs). As the legacy terminals do not expect any CSI-RS to overlap with the broadcast PDSCH, network cannot configure TRS on top of e.g. PDSCH carrying the paging message. As these are shared by legacy and Rel-17 UEs supporting the TRS occasions, it does not seem necessary to alter the IDLE/Inactive mode UE ratematching assumption. For MBS operation in IDLE/Inactive, as these are two different features, it would not be possible, without assuming that support of TRS occasions is mandatory for UEs that support MSB, to assume any overlap of MBS PDSCH and TRS occasions.

Observation: There appears to be no need to change the ratematching assumption for IDLE/Inactive mode UEs due to TRS occasions configuration.

Proposal: UE assumes that TRS occasions do not overlap with PDSCH.

For the handling of IDLE/Inactive mode TRS resource configuration, the TRS occasions configured by TRS-ResourceSet should not impact in any way to the UE CONNECTED mode operation. I.e. should not restrict or limit the CONNECTED mode configuration of CSI-RS or otherwise. In scope of rate matching, CONNECTED mode UEs will have a dedicated NZP-CSI-RS configuration(s), including TRS, which are accounted in the mapping to physical resources, as described in e.g. 38.211. Based on earlier agreements, no use for the TRS occasions (for IDLE/Inactive UEs) has been agreed for CONNECTED mode UEs either.

Observation: Configuration of TRS occasions for IDLE/Inactive UEs should not affect the configuration of CONNECTED mode UEs

The option to account this in the definition of available resource blocks for PDSCH in TS38.211 was discussed in RAN1. Among the referred RNTIs (C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI), apart MBS related RNTIs, relate to CONNECTED mode PDSCH scheduling, thus if RS configured by TRS-ResourceSetConfig are to be ignored in CONNECTED mode, there is no explicit need to capture this either in rate matching operation.

Thus as noted above, it is proposed:-

Proposal: TRS-ResourceSetConfig IE does not affect the UE behaviour in CONNECTED mode and can be ignored by UE in CONNECTED mode.

If it is seen necessary to have the rate matching behaviour specifically accounted, following TP could be considered:

#### 7.3.1.5 Mapping to virtual resource blocks The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS according to clause 7.4.1.5 if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource or except if the non-zero-power CSI-RS is TRS resources configured only via TRS-ResourceSetConfig in SIB; - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214].

## TRS configuration related

In the scope of the afore discussion, the TRS configuration for IDLE/Inactive UEs is done by TRS-ResourceSet-r17 IE (in SIBx) and not via NZP-CSI-RS-Resource IE. Hence, the description in TS38.211 appears not to cover the definition of TRS configuration for IDLE/Inactive UEs. Hence, it could be considered to account this also in the Section 7.4.1.5 of TS38.211.

Proposal: Adopt following TP to Section 7.4.1.5.1 of 38.211:

##### 7.4.1.5.1 General Zero-power (ZP) and non-zero-power (NZP) CSI-RS are defined - for a non-zero-power CSI-RS configured by the NZP-CSI-RS-Resource IE or by the CSI-RS-Resource-Mobility field in the CSI-RS-ResourceConfigMobility IE or by the TRS-ResourceSet IE, the sequence shall be generated according to clause 7.4.1.5.2 and mapped to resource elements according to clause 7.4.1.5.3 - for a zero-power CSI-RS configured by the ZP-CSI-RS-Resource IE, the UE shall assume that the resource elements defined in clause 7.4.1.5.3 are not used for PDSCH transmission subject to clause 5.1.4.2 of [6, TS 38.214]. The UE performs the same measurement/reception on channels/signals except PDSCH regardless of whether they collide with ZP CSI-RS or not.

In the running CR for TS38.331 the TRS resource set configuration is given as follows:

TRS-ResourceSet-r17 ::= SEQUENCE { powerControlOffsetSS-r17 ENUMERATED{db-3, db0, db3, db6}, scramblingID-Info-r17 CHOICE { scramblingIDforCommon-r17 ScramblingId, scramblingIDperResourceListWith2-r17 SEQUENCE (SIZE (2)) OF ScramblingId, scramblingIDperResourceListWith4-r17 SEQUENCE (SIZE (4)) OF ScramblingId, ... }, firstOFDMSymbolInTimeDomain-r17 INTEGER (0..9), startingRB-r17 INTEGER (0..maxNrofPhysicalResourceBlocks-1), nrofRBs-r17 INTEGER (24..maxNrofPhysicalResourceBlocksPlus1), ssb-Index-r17 SSB-Index, periodicityAndOffset-r17 CHOICE { slots10 INTEGER (0..9), slots20 INTEGER (0..19), slots40 INTEGER (0..39), slots80 INTEGER (0..79) }, frequencyDomainAllocation-r17 BIT STRING (SIZE (4)), indBitID-r17 INTEGER (0..5), nrofResources-r17 ENUMERATED{2,4}, ... }

Part of the discussions in RAN1 it was agreed that not all NZ-CSI-RS related parameters need to be provided as they can be implicitely derived from the restrictions defined in TS38.214 following from the assumption that trs-info is configured (while not provided).

Agreement: Configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only, including following limitations - Configuration parameters that are necessary to provide configuration of periodic TRS for idle/inactive UEs - Applicable values that are necessary to provide configuration of periodic TRS for idle/inactive UEs - If the configuration is provided, idle/inactive UEs can always implicitly assume that trs-info is configured. - The parameter trs-info does not need to be provided in the configuration

Hence, as can be seen from the running CR for TS38.331 not all the parameters needed to complete the NZP-CSI-RS configuration are provided, but can be derived e.g. from TS38.214. To align TS38.211, following changes could be considered.

Proposal: Adopt following TP to Section 7.4.1.5.3 of TS38.211:

##### 7.4.1.5.3 Mapping to physical resources [text omitted] The value of is given by the higher-layer parameter density in the CSI-RS-ResourceMapping IE or the CSI-RS-CellMobility IE and the number of ports X is given by the higher-layer parameter nrofPorts. For NZP CSI-RS configured by the TRS-ResourceSet IE, the density =3 and number of ports X =1. The UE is not expected to receive CSI-RS and DM-RS on the same resource elements. The UE shall assume for a non-zero-power CSI-RS where is selected such that the power offset specified by the higher-layer parameter powerControlOffsetSS in the NZP-CSI-RS-Resource IE or in the TRS-ResourceSet IE, if provided, is fulfilled. The quantities k', l', , and are given by Tables 7.4.1.5.3-1 to 7.4.1.5.3-5 where each k,l in a given row of Table 7.4.1.5.3-1 corresponds to a CDM group of size 1 (no CDM) or size 2, 4, or 8. The CDM type is provided by the higher layer parameter cdm-Type in the CSI-RS-ResourceMapping IE. For NZP CSI-RS configured by the TRS-ResourceSet IE, the CDM type is ‘noCDM’. The indices k' and l' index resource elements within a CDM group. The time-domain locations l0∈0,1, …, 13 and l1∈2, 3, …, 12 are provided by the higher-layer parameters firstOFDMSymbolInTimeDomain and firstOFDMSymbolInTimeDomain2, respectively, in the CSI-RS-ResourceMapping IE or the CSI-RS-ResourceConfigMobility IE and defined relative to the start of a slot. For NZP CSI-RS configured by TRS-ResourceSet IE, the time-domain locations l0∈0,1, …, 13 and l1∈2, 3, …, 12 are provided by the higher-layer parameter firstOFDMSymbolInTimeDomain and firstOFDMSymbolInTimeDomain+4. The frequency-domain location is given by a bitmap provided by the higher-layer parameter frequencyDomainAllocation in the CSI-RS-ResourceMapping IE or, the CSI-RS-ResourceConfigMobility IE or the TRS-ResourceSet IE with the bitmap and value of ki in Table 7.4.1.5.3-1 given by - , ki-1=fi for row 1 of Table 7.4.1.5.3-1 - , ki-1=fi for row 2 of Table 7.4.1.5.3-1 - , ki-1=4fi for row 4 of Table 7.4.1.5.3-1 - , ki-1=2fi for all other cases where is the bit number of the bit in the bitmap set to one, repeated across every 1ρ of the resource blocks configured for CSI-RS reception by the UE. The starting position and number of the resource blocks in which the UE shall assume that CSI-RS is transmitted are given by the higher-layer parameters freqBand and density in the CSI-RS-ResourceMapping IE for the bandwidth part given by the higher-layer parameter BWP-Id in the CSI-ResourceConfig IE or given by the higher-layer parameters nrofPRBs in the CSI-RS-CellMobility IE where the the startPRB given by csi-rs-MeasurementBW is relative to common resource block 0. For NZP CSI-RS configured by TRS-ResourceSet IE, the starting position and number of the resource blocks in which the CSI-RS can be transmitted are given by the higher-layer parameters nrofRBs, and startingRB in the TRS-ResourceSet IE, where startingRB is relative to common resource block 0 and density =3.

It is good to note that, with density set to 3 and number of ports limited to 1, CDM type could be derived implicitely.

# Conclusion

In this contribution we have discussed the the aspects related PEI and TRS occasions for IDLE/INACTIVE-mode UEs.

In section 2 we discussed aspects related to PEI and make following observations and proposals:-

Observation: It does not appear necessary to add reference to Type2A-PCCCH CSS to the text describing the UE behaviour when is configured with C-RNTI and is provided with SS ID=0 for searchSpaceID in PDCCH-ConfigCommon text in 38.213.

Proposal: Reference to Type2A-PCCCH CSS is not added to the text describing the UE behaviour when is configured with C-RNTI and is provided with SS ID=0 for searchSpaceID in PDCCH-ConfigCommon text in 38.213.

Observation: It does not appear necessary to add reference to Type2A-PCCCH CSS to the text describing the UE behaviour when is configured with C-RNTI and is provided non-zero SS ID for searchSpaceID in PDCCH-ConfigCommon text in 38.213.

Proposal: Reference to Type2A-PCCCH CSS is not added to the text describing the UE behaviour when is configured with C-RNTI and is provided non-zero SS ID for searchSpaceID in PDCCH-ConfigCommon text in 38.213.

Proposal: Adopt following TP to TS38.213 Section 10.4:

## 10.4A PDCCH monitoring for early indication of paging A UE can be provided the following for detection of a DCI format 2_7 in RRC_IDLE state or in RRC_INACTIVE state [12, TS 38.331] - a search space set, by peiSearchSpace, to monitor PDCCH for detection of DCI format 2_7 according to a Type2A-PDCCH CSS set as described in clause 10.1 - a number of frames, by PEI-F_offset, from the start of a first paging frame of paging frames associated with a number of PDCCH monitoring occasions for DCI format 2_7 [17, TS 38.304] to the start of a frame - a number of symbols, by firstPDCCH-MonitoringOccasionOfPEI-O, from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 - a size, by payloadSizeDCI_format2_7 - a number of subgroups per paging occasion, NSGPO, by subgroupsNumPerPO - a number of paging occasions associated with the number of PDCCH monitoring occasions for DCI format 2_7, NPOPEI, by PONumPerPEI A paging indication field of DCI format 2_7 includes NPOPEI segments of K bits, where K=NSGPO if NSGPO>0 and K=1 if NSGPO is not provided. For a subgroup index iSG, 0≤iSG<K, a UE determines a value for the iPO⋅K+iSG bit in the paging indication field, where iPO=UE_IDmodN⋅NS+iSmodNPOPEI is a paging occasion index, and UE_ID, N, NS, iSG, and iS are defined in [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion. If NPOPEI<NS, the number of symbols from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 that is associated with paging occasion index iPO is the iSNPOPEI+1-th value from the NSNPOPEI values provided by firstPDCCH-MonitoringOccasionOfPEI-O.

Proposal: Adopt following TP to TS38.212 Section 7.3.1.3.8:

##### 7.3.1.3.8 Format 2_7 DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs. The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field – NPOPEINSGPO bit(s), where - NPOPEI is the number of paging occasions configured by higher layer parameter PONumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - NSGPOis the number of sub-groups of a paging occasion configured by higher layer parameter subgroupsNumPerPO, if subgroupsNumPerPO is configured; otherwise NSGPO is set to 1. - Each bit in the field indicates one UE subgroup of a paging occasion if subgroupsNumPerPO is configured; otherwise each bit in the field indicates the UE group of a paging occasion. - TRS availability indication – 1, 2, 3, 4, 5, or 6 bits if TRS-ResourceSetConfig is configured; 0 bits otherwise. The size of DCI format 2_7 is indicated by the higher layer parameter payloadSizeDCI_format2_7, according to Clause 10.4A of [5, TS 38.213]. The number of information bits in format 2_7 shall be equal to or less than the payload size of format 2_7. If the number of information bits in format 2_7 is less than the size of format 2_7, the remaining bits are reserved.

Observation: Minimum time gap between PO and PEI should enable UE to acquire synchronisation for paging message reception.

Observation: From receiver preparation perspective minimum gap of 3ms could be considered.

In section 3 we discussed the remaining open items relared to the TRS occasions for the IDLE/INACTIVE mode UEs:-

Proposal: Adopt TP#1 for clarifying the reference point definition for current default DRX cycle.

Observation: There appears to be no need to change the ratematching assumption for IDLE/Inactive mode UEs due to TRS occasions configuration.

Proposal: UE assumes that TRS occasions do not overlap with PDSCH.

Observation: Configuration of TRS occasions for IDLE/Inactive UEs should not affect the configuration of CONNECTED mode UEs

Proposal: TRS-ResourceSetConfig IE does not affect the UE behaviour in CONNECTED mode and can be ignored by UE in CONNECTED mode.

Proposal: Adopt following TP to Section 7.4.1.5.1 of 38.211:

##### 7.4.1.5.1 General Zero-power (ZP) and non-zero-power (NZP) CSI-RS are defined - for a non-zero-power CSI-RS configured by the NZP-CSI-RS-Resource IE or by the CSI-RS-Resource-Mobility field in the CSI-RS-ResourceConfigMobility IE or by the TRS-ResourceSet IE, the sequence shall be generated according to clause 7.4.1.5.2 and mapped to resource elements according to clause 7.4.1.5.3 - for a zero-power CSI-RS configured by the ZP-CSI-RS-Resource IE, the UE shall assume that the resource elements defined in clause 7.4.1.5.3 are not used for PDSCH transmission subject to clause 5.1.4.2 of [6, TS 38.214]. The UE performs the same measurement/reception on channels/signals except PDSCH regardless of whether they collide with ZP CSI-RS or not.

Proposal: Adopt following TP to Section 7.4.1.5.3 of TS38.211:

##### 7.4.1.5.3 Mapping to physical resources [text omitted] The value of is given by the higher-layer parameter density in the CSI-RS-ResourceMapping IE or the CSI-RS-CellMobility IE and the number of ports X is given by the higher-layer parameter nrofPorts. For NZP CSI-RS configured by the TRS-ResourceSet IE, the density =3 and number of ports X =1. The UE is not expected to receive CSI-RS and DM-RS on the same resource elements. The UE shall assume for a non-zero-power CSI-RS where is selected such that the power offset specified by the higher-layer parameter powerControlOffsetSS in the NZP-CSI-RS-Resource IE or in the TRS-ResourceSet IE, if provided, is fulfilled. The quantities k', l', , and are given by Tables 7.4.1.5.3-1 to 7.4.1.5.3-5 where each k,l in a given row of Table 7.4.1.5.3-1 corresponds to a CDM group of size 1 (no CDM) or size 2, 4, or 8. The CDM type is provided by the higher layer parameter cdm-Type in the CSI-RS-ResourceMapping IE. For NZP CSI-RS configured by the TRS-ResourceSet IE, the CDM type is ‘noCDM’. The indices k' and l' index resource elements within a CDM group. The time-domain locations l0∈0,1, …, 13 and l1∈2, 3, …, 12 are provided by the higher-layer parameters firstOFDMSymbolInTimeDomain and firstOFDMSymbolInTimeDomain2, respectively, in the CSI-RS-ResourceMapping IE or the CSI-RS-ResourceConfigMobility IE and defined relative to the start of a slot. For NZP CSI-RS configured by TRS-ResourceSet IE, the time-domain locations l0∈0,1, …, 13 and l1∈2, 3, …, 12 are provided by the higher-layer parameter firstOFDMSymbolInTimeDomain and firstOFDMSymbolInTimeDomain+4. The frequency-domain location is given by a bitmap provided by the higher-layer parameter frequencyDomainAllocation in the CSI-RS-ResourceMapping IE or, the CSI-RS-ResourceConfigMobility IE or the TRS-ResourceSet IE with the bitmap and value of ki in Table 7.4.1.5.3-1 given by - , ki-1=fi for row 1 of Table 7.4.1.5.3-1 - , ki-1=fi for row 2 of Table 7.4.1.5.3-1 - , ki-1=4fi for row 4 of Table 7.4.1.5.3-1 - , ki-1=2fi for all other cases where is the bit number of the bit in the bitmap set to one, repeated across every 1ρ of the resource blocks configured for CSI-RS reception by the UE. The starting position and number of the resource blocks in which the UE shall assume that CSI-RS is transmitted are given by the higher-layer parameters freqBand and density in the CSI-RS-ResourceMapping IE for the bandwidth part given by the higher-layer parameter BWP-Id in the CSI-ResourceConfig IE or given by the higher-layer parameters nrofPRBs in the CSI-RS-CellMobility IE where the the startPRB given by csi-rs-MeasurementBW is relative to common resource block 0. For NZP CSI-RS configured by TRS-ResourceSet IE, the starting position and number of the resource blocks in which the CSI-RS can be transmitted are given by the higher-layer parameters nrofRBs, and startingRB in the TRS-ResourceSet IE, where startingRB is relative to common resource block 0 and density =3.

# References

- Mediatek, ” New WI: UE Power Saving Enhancements”, RP-193239, December 2019

- R1-1814052, Draft CR on TS38.213 on CORESET 0, NTT DOCOMO, INC.

- R1-1909632, Draft CR on monitoring occasion of SS#0, ZTE

- RP-191942, Maintenance of NR -38.213, RAN1

- R1-2112943, Introduction of Rel-17 UE power saving enhancements, CR to 38.212

- R1-2112936, Introduction of UE power savings enhancements in NR, CR to 38.213

- R1-2112957, Introduction of NR UE power savings enhancements, CR to 38.214

- R2-2204122, Introduction of ePowSav in TS 38.331, CATT

# Annex A: Agreements related to Type2A-CSS

RAN1#95 agreement (yellow highlight added by tdoc author):

Agreements: When the CORESET ID field is 0, the TCI state ID field in the MAC-CE indicates a TCI-state in the PDSCH-Config. - For CORESET#0, - A UE follows the indicated TCI state or the QCL-D of the SSB of the active BWP which is selected through the random access procedure with a PRACH transmission not initiated by a PDCCH order that triggers a non-contention based random access procedure or not initiated by link recovery procedure, whichever occurs more recently - MAC CE based TCI indication delay for CORESET 0 is the same as MAC CE based TCI indication delay for other CORESETs - A UE is expected to be configured only with TCI state of CSI-RS/TRS which is QCLed with an SSB based on the corresponding QCL type defined in section 5.1.5 in 38.214 for the active BWP. - If different reference signals are configured in the TCI state, they should be QCLed with the same SSB with corresponding QCL type. - Note: this does not require new way of handling QCL chain between the CSI-RS/TRS in the TCI-state and the QCLed SSB. - It is up to NW whether to indicate the TCI-state for the CORESET#0 by the MAC-CE. - If the TCI-state for the CORESET#0 is not indicated by the MAC-CE, the UE shall follow the SSB selected in the most recent random access procedure with a PRACH transmission not initiated by a PDCCH order that triggers a non-contention based random access procedure or not initiated by link recovery procedure to receive any PDCCH with any RNTI on the CORESET#0. - For CONNECTED state, if Type0/0A/2-CSS is SS#0, the UE monitors the common search space on the PDCCH monitoring occasions determined based on the association between an SSB index and PDCCH monitoring occasions according to Section 13 of TS38.213 (which is called default association), where; - the SSB is the one QCLed to the CSI-RS/TRS in the TCI-state indicated for the CORESET#0 or is the one selected through the random access procedure with a PRACH transmission not initiated by a PDCCH order that triggers a non-contention based random access procedure or not initiated by link recovery procedure, whichever comes the recent. - RAN1 will ask RAN2 about handling of commonControlResourceSet if Option 1 is agreed. - TP for 213 section 5 will be discussed in RLM session. - For component 1 of FG2-4, if a UE reports X active TCI state(s), the UE is expected to be configured/activated with X active QCL assumption(s) for any PDSCH and any CORESETs for a given BWP of a serving cell. - TCI states applicable to CORESET#0 are (1) up to the first 64 sorted by TCI-state IDs, and (2) which contains CSI-RS sourced by SSB. - QCL assumption for CORESET#0 is updated by q_new after RACH procedure for BFR/RLM. - Note: In RRC connected mode, for CORESET other than #0, MAC-CE indicates a TCI-state in the PDSCH-Config RRC IE with the restriction of the TCI-state indexes configured for the CORESET RRC IE - Note: it is gNB responsibility whether/how to ensure the performance of broadcast PDCCH whose QCL-TypeD is TRS - Note: The active TCI state for the CORESET#0 is counted as 1 in the UE capability signalling - For the case when a PRACH transmission is initiated by link recovery procedure, FFS the timing aspect – aim to conclude by Tuesday TP to be checked on Tuesday – Fred (DCM), R1-1814052 Agreements: - The two draft CRs in R1-1814052 (38.213 & 38.214) are endorsed

RAN1#94bis agreement (yellow highlight added by tdoc author):

Agreements: - Note: Previously it was agreed that for IDLE/INACTIVE state, if Type-0/0A/2-CSS is SS#0, the UE monitors the common search space on the PDCCH monitoring occasions determined based on the association between an SSB index and PDCCH monitoring occasions according to Section 13 of TS38.213 (which is called default association). - Note: Previously it was agreed that IDLE/INACTIVE state, if Type-0A/2-CSS is other than SS#0, the UE monitors the common search space on the PDCCH monitoring occasions determined based on the association between an actually transmitted SSB and PDCCH monitoring occasions according to Section 7.1 of TS38.304 and Section 5 of TS38.331 (which is called non-default association). - For CONNECTED state, if Type0/0A/2-CSS is SS#0, the UE monitors the common search space on the PDCCH monitoring occasions determined based on the association between an SSB index and PDCCH monitoring occasions according to Section 13 of TS38.213 (which is called default association). - To capture in 38.213 - For CONNECTED state, if Type0/0A/2-CSS is other than SS#0, - Regardless of whether TCI-state is configured/indicated or not, the UE monitors the common search space based on the PDCCH monitoring occasions according to the configuration of SearchSpace RRC IE. - To capture in 38.213

# Annex B: Agreements from RAN1#108e

Agreement from RAN1#108

Agreement Down-select one of the TPs to clarify the current default DRX cycle used for determining the reference point for start of validity duration for an availability indication - Alt1: Adopt TP#1 - Alt2: Adopt TP#2 - Note: TRS availability indication transmitted in all PDCCH monitoring occasions of the same PEI-O or PO is same. - Note: Reference point for TRS availability indication in a given DCI is identical for all UEs regardless of UE specific DRX value. ============================= start of TP#1========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame within the DRX cycle that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#1========================================== ============================= start of TP#2========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame of the DRX cycle associated with the first PDCCH MO of the PEI-O or PO that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#2==========================================.