3GPP TSG RAN WG1 #109-e	R1-2204605

e-Meeting, May 9th – 20th, 2022

Agenda item:		8.7.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Open issues on PDCCH monitoring adaptation for UE power saving

Document for:		Discussion and Decision

# Introduction

- Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4]

  - Study and specify, if agreed, extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time for an active BWP, including PDCCH monitoring reduction when C-DRX is configured [RAN1]

- NOTE: Rel-15 and Rel-16 available power saving solutions should be supported by the UE and included in the evaluation. RAN1 will ask the confirmation from RAN2 that Rel-15 and Rel-16 available power saving solutions are properly utilized.

The release 17 work item on UE power saving enhancements [1] includes the following objective:

In this contribution we discuss on the PDCCH monitoring adaptation design.

# Timer based adaptation and related configuration for PDCCH monitoring

In RAN#108e meeting following agreement was reached:

Agreement If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE - Resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by - Working Assumption: C-RNTI/CS-RNTI/MCS-C-RNTI - Otherwise, decrease the timer value by one after each slot. - Note: There is no timer for SSSG switching is running for UE in SSSG#0

As expressed, when UE detects a DCI format with the SSSG switching field (i.e. DCI 0_1/2 or 1_1/2 scrambled with C-RNTI based on current agreements) and the field does not indicate SSSG switch, it would imply that network wants the UE to stay in current SSSG. Network can choose the field value and by not indicating change, UE should be expected to stay in current SSSG, hence timer should be reset.

Correspondingly, UE could also be scheduled with fallback DCI (e.g. DCI format 1_0 scrambled with C-RNTI) which does not carry a field for SSSG switching in the monitored search space(s). As network would have had typically a choice whether to use e.g. DCI format 1_1 instead, it could be understood that also in this case network does not intent the UE to change the SSSG. Thus, the timer should be also reset in this case. This would align the UE behaviour to the C-DRX operation where scheduling restarts the inactivity timer.

Proposal: Confirm the working assumption in agreement:

Agreement If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE - Resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by - Working Assumption: C-RNTI/CS-RNTI/MCS-C-RNTI - Otherwise, decrease the timer value by one after each slot. - Note: There is no timer for SSSG switching is running for UE in SSSG#0

# PDCCH monitoring adaptation behaviour with MO triggers

In RAN1#108e RAN1 discussed the RAN2 LS[2] and provided a reply in [3].

The RAN2 LS send to RAN1 quoted the related agreements:

Following agreements were made in RAN2#116bis-e: - UE ignores PDCCH skipping while the SR is pending. - If PDCCH skipping is applied to RNTI(s) monitored during RAR/MsgB window, the UE ignores PDCCH skipping during the RAR/MsgB window. - UE ignores PDCCH skipping while contention resolution timer is running.

In context of these, RAN1 made following agreement in RAN1#108e:

Agreement Answer 2 for R1-2200884 (R2-2201960): RAN1 would discuss and conclude how to capture the above RAN2 agreements in RAN1 specification. o It is RAN1 understanding PDCCH skipping is not applied to perform PDCCH monitoring during RAR window/MsgB window/contention resolution timer or when SR is pending. - FFS: If the UE is indicated skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE can stops PDCCH skipping and monitor PDCCH for UL grant (i.e., PDCCH skipping is not activated ).

As noted, it was left for further discussion how to capture the intended behaviour to specification.

Furthermore, in context of same functionality, in RAN2#117e, RAN2 made following clarifications to earlier agreements:

RAN2 clarifications to earlier RAN2 decisions: P1: UE ignores PDCCH skipping on all serving cells of the corresponding CG while SR is pending (FFS if “all” can be further restricted). P2: If PDCCH skipping is applied to RNTI(s) monitored during RAR/MsgB window, UE ignores PDCCH skipping on SpCell. P3: UE ignores PDCCH skipping on SpCell while contention resolution timer is running.

Firstly, considering the SR, UE can transmit SR in PUCCH (or UCI in PUSCH) if resources are available, or if e.g. timing alignment timer has expired, via RACH procedure. In case of PUCCH based UE should monitor PDCCH scheduling the UL grant (e.g. DCI format 0_0/0_1 with scrambled with C-RNTI) and in case when RACH is triggered, UE should monitor Msg2 scheduling (scrambled with RA-RNTI). Thus, as per agreements in RAN2 and RAN1, PDCCH skipping is not to be applied when SR is pending. The procedure related to SR triggering is covered by RAN2 specification, while RAN1 specification covers the physical layer aspects e.g. UCI multiplexing. Apart from UCI reporting in PUCCH, RAN1 specification does not explicitly identify SR related behaviour. For RACH the SR related triggering is not differentiated from other causes. Thus, in order to capture the UE behaviour when SR in pending and PDCCH skipping is configured it would be simplest to refer to RAN2 specifications. Also, both cases need to be covered in terms of timeline, skipping triggered before or after SR. As quoted above, RAN2 made an agreement that the skipping should not be applied in any cell belonging to same cell group as the serving cell where SR is pending:

Thus following proposal is made:-

Proposal: Capture following text to TS38.213 Section 10.4:

If UE has pending SR(s) in a serving cell [14, TS38.321], the UE is not allowed apply PDCCH monitoring skipping in cells belonging to the cell group of the serving cell.

Now, as discussed in last meeting for the RAN2 LS reply, skipping applies to USS and Type3-PDCCH CSS (during the Active time), thus the scenarios that would need to be accounted fall to both CBRA and CFRA procedures, and in both Type-1 and Type-2 RA procedure.

Based on the last meeting agreement, it could be defined that that skipping is not applied during the related time windows/timers, i.e. ra-ResponseWindow, msgB-ResponseWindow and ra-ContentionResolutionTimer. However, as discussed below, it could be beneficial to consider that when RACH is triggered or that when aforementioned timers are running, the PDCCH monitoring adaptation field indicating skipping is ignored.

Observation: It could be considered, for simplicity and system robustness that PDCCH skipping is cancelled when RACH procedure is triggered and PDCCH monitoring adaptation field indicating skipping is ignored when ra-ResponseWindow, msgB-ResponseWindow or ra-ContentionResolutionTimer is running.

In CFRA BFR, after BFR resolution carried via C-RNTI or MCS-C-RNTI based scheduling during the recovery window UE continues to monitor PDCCH on recoverySearchSpaceId until the UE receives MAC CE configuring UE with new PDCCH beam. While MAC CE could have provided the new PDCCH beam, further action maybe required, thus it would be good to ensure that UE would monitor the PDCCH in USS.

Observation: In case of BFR via CFRA, PDCCH skipping should not affect the PDCCH monitoring in recovery search space to enable completion of procedure. Also, to enable further beam management actions, UE should resume normal monitoring after BFR.

In this relation, RAN2 has also agreed (and RAN1 confirmed) that skipping should not be applied during RAR/MsgB window, thus CBRA based BFD is covered until contention resolution. As discussed, to ensure completion of beam management procedures, it would be preferable that the skipping is cancelled upon transmission of RACH. Thus as noted above, in relation to RA procedure RAN1 could just agree that the skipping is cancelled upon triggering the RA procure and indications ignored while related timers are running. As discussed also in last meeting, this would ensure robust completion of related procedures and cover the contention resolution as well. (Note that this would cover also the RACH based SR).

Proposal: Agree that when RACH procedure is triggered or running, UE should to stop the PDCCH skipping and not apply the skipping if indication is received while RACH procedure is running and adopt following TP to TS38.213 Section 10.4:

## 10.4 Search space set group switching and skipping of PDCCH monitoring [text omitted]. If random access procedure is initiated as described in Section 8, and UE is skipping the PDCCH monitoring for a duration, the UE shall stop the skipping of PDCCH monitoring. iWhen ra-ResponseWindow, msgB-ResponseWindow or ra-ContentionResolutionTimer is running [14, TS38.321], UE shall ignore the PDCCH monitoring adaptation field in DCI indicating PDCCH monitoring skipping for a duration.

Observation: If it is not acceptable to cancel skipping or ignore skipping indications upon/during RACH procedure, stopping the skipping for the duration of the timers can be considered as per following TP to TS38.213 Section 10.4:

## 10.4 Search space set group switching and skipping of PDCCH monitoring [text omitted]. If ra-ResponseWindow, msgB-ResponseWindow or ra-ContentionResolutionTimer is running [14, TS38.321], the UE shall not apply skipping in PDCCH monitoring.

# Application delay

The application delay options together with interaction with HARQ and UL TB has been discussed in past meetings, and in last meeting agreements were reached on this:

Working Assumption - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B), - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - Note: Pswitch is defined in Table 10.4-1 in TS38.213 Agreement - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), select one of the following schemes - Alt 1a: the UE applies Beh 1A on an active BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - FFS: whether the UE will monitor PDCCH when DRX Retransmission timer is running

For SSSG switching, the need to account the minimum scheduling slot offset restriction was discussed. The argument was that to enable reduced PDCCH processing (to obtain some power saving) with minimum applicable scheduling slot offset restriction, further relaxation would be needed. Some consideration can be done assuming that the application delay of minimum scheduling slot offset restriction covers both PDCCH processing and change in configuration. When considering this, it can be seen that the value for application delay for minimum scheduling offset restriction is lower bound by Zµ. It could be considered that this is mostly set by the delay due to the configuration change. This value results always lower value than minimum of SSSG switching delay, Pswitch. approximately by 11 symbols. I.e. if the configuration change delay is on same level, the PDCCH processing could be reduced for the remaining. Now as the model nor power saving benefit of PDCCH processing reduction due to increased minimum scheduling slot offset was not agreed, it is hard to quantify whether there is any non-negligible power saving benefit. It is also good to note that applied value for Pswitch is configurable via searchSpaceSwitchDelay IE ({10…52}symbols). As the SSSG application delay, Pswitch, can configured, it does not seem necessary to account the minimum scheduling slot offset in the SSSG switching delay, and if it is later seen that the benefit due to the PDCCH processing reduction is significant, the delay can be configured to be longer.

Proposal: The SSSG switching delay is not affected by minimum scheduling slot offset restriction.

As noted above the SSSG switching delay, Pswitch, is configurable via searchSpaceSwitchDelay IE. The range is currently set to {10…52} symbols. The minimum value for the switching delay is given by Table 10.4.-1 in TS38.213, quoted below in Table 1. As can be seen from the table the minimum value for the switching delay, Pswitch, for sub-carrier spacing of 480kHz and 960kHz is 160 symbols and 320 symbols, respectively. Evidently this is beyond the range that can be currently signalled.

Table 1: Minimum value of Pswitch [symbols] [TS38.213]

| μ | Minimum Pswitch value for UE processing capability 1 [symbols] | Minimum Pswitch value for UE processing capability 2 [symbols] |
| --- | --- | --- |
| 0 | 25 | 10 |
| 1 | 25 | 12 |
| 2 | 25 | 22 |
| 3 | 40 | - |
| 5 | 160 | - |
| 6 | 320 | - |

Therefore it would seem appropriate to adjust the range of searchSpaceSwitchDelay at least for the higher sub-carrier spacings. In context of adjusting different parameter values in Ag. 8.2, scaling of the parameter value for 120kHz sub-carrier spacing by 4x and by 8x, for 480khz and 960khz sub-carrier spacings, respectively, has been assumed. E.g. as agreed in RAN1#108e for switching timer:

Agreement In unit of slots, the supported values for searchSpaceSwitchTimer-r17 and PDCCHSkippingDuration for 480 kHz and 960 kHz are respectively 4x and 8x of their supported values for 120 kHz.

Thus scaling factor could be applied in case of 480kHz or 960kHz sub-carrier spacing, to increase the range of searchSpaceSwitchDelay above the minimum value set by the UE processing capability. Additional aspect that could be accounted is that the SSSG switch has been agreed to occur in slot group boundary, i.e. happening at granularity of 4 or 8 slots:

Agreement For multi-slot PDCCH monitoring for 480/960 kHz SCSs, the boundary of SSSG switching is always aligned with the boundary of a slot group.

Exact realization of the application delay depends naturally on the SS set configuration (i.e. point of scheduling), but having multiple values that fall within e.g. 4 slots range is not needed. Thus scaling the value range of searchSpaceSwitchDelay would seem appropriate solution.

The applied value of the scaling factor could be further considered. As noted above, the alignment to slot boundary results implicit granularity to the realized application delay. Thus, it might be preferable to assume scaling of 8x and 16x for 480kHz and 960kHz sub-carrier spacing respectively, instead of 4x and 8x, to result some range beyond the minimum capability. We raise this aspect also in [4].

Proposal: Agree that the supported values for SSSG switching delay, searchSpaceSwitchDelay, for 480kHz and 960kHz, are scaled by factor the supported values for 120kHz. The scaling factor is selected among {[4x and 8x]} or by {[8x and 16x]} for 480kHz and 960kHz respectively.

It is good to note, as agreed in RAN1#109e under A.g. 8.2 (quoted below), Rel-16 and Rel-17 SSSG switching parameters are not expected to be configured simultaneously, and as searchSpaceSwitchDelay is carried by SearchSpaceSwitchConfig-r16, new parameter may need to be introduce Rel-17 SSSG switching delay.

Agreement - A UE does not expect to be configured with Rel-16 SSSG switching parameters (such as searchSpaceSwitchTimer and SearchSpaceSwitchTrigger) and Rel-17 SSSG switching parameters (such as searchSpaceSwitchTimer-r17 and searchSpaceGroupIdList-r17) per cell simultaneously. - Note: This behaviour is reflected in a RAN2 power saving CR for 38.331 (R2-2203058) and may therefore not require additional specification text.

Observation: New parameter may be needed for Rel-17 SSSG switching delay to differentiate the configuration from Rel-16 SSSG switching delay.

# Interaction with HARQ and UL scheduling

The handling of the HARQ re-transmissions has been discussed in past meeting. In addition the HARQ handling can be considered to have also a relation how the application delay is determined. It was agreed in last meeting that PDCCH skipping duration is started in the following slot upon reception of the indication:

Agreement - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), select one of the following schemes - Alt 1a: the UE applies Beh 1A on an active BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - FFS: whether the UE will monitor PDCCH when DRX Retransmission timer is running

The UE behaviour was left open whether UE would monitor the PDCCH during the re-transmission timers (drx-RetransmissionTimerDL/UL). As noted also in context of BSR, this would allow network to handle also the arrival of data to new logical channel by scheduling a new UL resource.

Proposal: If PDCCH skipping duration overlaps with drx-RetransmissionTimerDL/UL, UE should stop skipping and resume normal PDCCH monitoring for the duration of drx-RetransmissionTimerDL/UL.

It would also be good to clarify what would be the expected UE behaviour in context of possible scheduling during the configured re-transmission window. Evidently, if the DL transmission or UL transmission has been successful and no new scheduling DCI is received, it would be expected that UE can continue the PDCCH skipping for the remainder of PDCCH skipping duration after expiry of the re-transmission window timer. When a (DL/UL) scheduling DCI is detected during the re-transmission window different behaviours could be considered. When the scheduling DCI does not indicate skipping (Behv1), it could be considered that the further skipping is cancelled, and that UE should continue to monitor the PDCCH normally. Or, if indicated, carry out a SSSG change. This would allow network to stop the skipping i.e. if BSR indicates need to handle some high priority traffic. When UE would detect a scheduling DCI indicating skipping (Beh1A) during the re-transmission timer it would seem natural that UE would re-initiate the skipping. It would be simplest to assume that the skipping duration is reset in this case (to align the behaviour)

Proposal: Clarify the UE behaviour, in case of PDCCH monitoring during the drx-RetransmissionTimerDL/UL during PDCCH skipping duration, is determined by the PDCCH monitoring adaptation indication received in DCI during the re-transmission timer.

# Interaction with C-DRX

The work item description determines that the scope of the work is in DRX Active Time, thus it would be good to confirm this and account this in specification work.

Proposal: As the work item determines the scope to be DRX Active Time, it should be confirmed that Rel-17 PDCCH monitoring adaptation is configured only together with C-DRX.

In context of the LS reply to RAN2 [3], RAN1 made following agreement in RAN1#108e:

Agreement  It is RAN1 understanding that for DRX operation, - As DCP is monitored only outside active time, it cannot be missed due to PDCCH skipping as skipping applies only in active time. Note: RAN1 is discussing whether PDCCH skipping duration can apply to outside active time.

The agreement that PDCCH skipping is not applied outside Active time would need to be also reflected in RAN1 specification. To achieve this, following TP is proposed:-

Proposal: Adopt following TP to Section 10.4 of TS38.213 to reflect the agreement on PDCCH monitoring skipping outside Active Time:

## 10.4 Search space set group switching and skipping of PDCCH monitoring [text omitted] When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. The PDCCH monitoring skipping can be applied only during Active Time [11, TS 38.321].

The behaviour of the skipping duration timer was left open. The simplest approach would be to define that skipping duration expires upon expiry of inactivity timer (drx-InactivityTimer). Other approach is, that while PDCCH skipping itself is not applied during the active time the skipping duration timer would be decremented outside the Active Time. In this case, when the skipping duration is not expired at expiry of the inactivity timer, to avoid possible ambiguity of the expiry of skipping duration between UE and network, it would be most straight forward that skipping duration timer is kept independent of the C-DRX, during/outside Active Time. Thus, there are to possible options for the handling for the skipping duration; either the skipping duration is expired when inactivity timer (drx-InactivityTimer) expires or the skipping duration timer is kept running and decremented independent of the C-DRX Active Time.

Proposal: Agree either that PDCCH skipping duration expires upon expiry of inactivity timer (drx-InactivityTimer) or that PDCCH skipping duration timer is decremented independently of C-DRX Active time.

The current duration definition in TS38.213 is not affected by C-DRX Active time, thus if it is agreed to expire the skipping duration upon expiry of inactivity timer, following TP could be adopted:

Proposal: If it is agreed that PDCCH skipping duration expires upon expiry of inactivity timer (drx-InactivityTimer), adopt following TP to Section 10.4 of TS38.213:

## 10.4 Search space set group switching and skipping of PDCCH monitoring [text omitted] When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. The duration for PDCCH monitoring skipping expires when drx-InactivityTimer expires [11, TS 38.321].

# Conclusion

This contribution we have presented power saving evaluations for SS set groups switching and PDCCH skipping in different scenarios.

In Section 2 we discuss the looked the timer related behaviour and concluded as follows:

Proposal: Confirm the working assumption in agreement:

Agreement If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE - Resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by - Working Assumption: C-RNTI/CS-RNTI/MCS-C-RNTI - Otherwise, decrease the timer value by one after each slot. - Note: There is no timer for SSSG switching is running for UE in SSSG#0

In Section 3 we considered other open aspects related to the PDCCH monitoring adaptation for certain MO originated behaviours, in light of the RAN2 LS [2], we observe and propose in Section 2.2:

Proposal: Capture following text to TS38.213 Section 10.4:

If UE has pending SR(s) in a serving cell [14, TS38.321], the UE is not allowed apply PDCCH monitoring skipping in cells belonging to the cell group of the serving cell.

Observation: It could be considered, for simplicity and system robustness that PDCCH skipping is cancelled when RACH procedure is triggered and PDCCH monitoring adaptation field indicating skipping is ignored when ra-ResponseWindow, msgB-ResponseWindow or ra-ContentionResolutionTimer is running.

Observation: In case of BFR via CFRA, PDCCH skipping should not affect the PDCCH monitoring in recovery search space to enable completion of procedure. Also, to enable further beam management actions, UE should resume normal monitoring after BFR.

Proposal: Agree that when RACH procedure is triggered or running, UE should to stop the PDCCH skipping and not apply the skipping if indication is received while RACH procedure is running and adopt following TP to TS38.213 Section 10.4:

## 10.4 Search space set group switching and skipping of PDCCH monitoring [text omitted]. If random access procedure is initiated as described in Section 8, and UE is skipping the PDCCH monitoring for a duration, the UE shall stop the skipping of PDCCH monitoring. iWhen ra-ResponseWindow, msgB-ResponseWindow or ra-ContentionResolutionTimer is running [14, TS38.321], UE shall ignore the PDCCH monitoring adaptation field in DCI indicating PDCCH monitoring skipping for a duration.

Observation: If it is not acceptable to cancel skipping or ignore skipping indications upon/during RACH procedure, stopping the skipping for the duration of the timers can be considered as per following TP to TS38.213 Section 10.4:

## 10.4 Search space set group switching and skipping of PDCCH monitoring [text omitted]. If ra-ResponseWindow, msgB-ResponseWindow or ra-ContentionResolutionTimer is running [14, TS38.321], the UE shall not apply skipping in PDCCH monitoring.

In Section 4, for the application delay we make following proposal:-

Proposal: The SSSG switching delay is not affected by minimum scheduling slot offset restriction.

Proposal: Agree that the supported values for SSSG switching delay, searchSpaceSwitchDelay, for 480kHz and 960kHz, are scaled by factor the supported values for 120kHz. The scaling factor is selected among {[4x and 8x]} or by {[8x and 16x]} for 480kHz and 960kHz respectively.

Observation: New parameter may be needed for Rel-17 SSSG switching delay to differentiate the configuration from Rel-16 SSSG switching delay.

The operation in relation to HARQ feedback and pending UL transmissions is discussed in Section 5 and we observe and propose as follows:-

Proposal: If PDCCH skipping duration overlaps with drx-RetransmissionTimerDL/UL, UE should stop skipping and resume normal PDCCH monitoring for the duration of drx-RetransmissionTimerDL/UL.

Proposal: Clarify the UE behaviour, in case of PDCCH monitoring during the drx-RetransmissionTimerDL/UL during PDCCH skipping duration, is determined by the PDCCH monitoring adaptation indication received in DCI during the re-transmission timer.

To conclude the interaction with C-DRX operation in Section 6 we make following observations and proposals:-

Proposal: As the work item determines the scope to be DRX Active Time, it should be confirmed that Rel-17 PDCCH monitoring adaptation is configured only together with C-DRX.

Proposal: Adopt following TP to Section 10.4 of TS38.213 to reflect the agreement on PDCCH monitoring skipping outside Active Time:

## 10.4 Search space set group switching and skipping of PDCCH monitoring [text omitted] When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. The PDCCH monitoring skipping can be applied only during Active Time [11, TS 38.321].

Proposal: Agree either that PDCCH skipping duration expires upon expiry of inactivity timer (drx-InactivityTimer) or that PDCCH skipping duration timer is decremented independently of C-DRX Active time.

Proposal: If it is agreed that PDCCH skipping duration expires upon expiry of inactivity timer (drx-InactivityTimer), adopt following TP to Section 10.4 of TS38.213:

## 10.4 Search space set group switching and skipping of PDCCH monitoring [text omitted] When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. The duration for PDCCH monitoring skipping expires when drx-InactivityTimer expires [11, TS 38.321].

# References

- RP-193239, New WID: UE Power Saving Enhancements, Mediatek

- R1-2200884, LS on PDCCH Skipping in RRC_CONNECTED, RAN2

- R1-2202905, LS reply on PDCCH skipping, RAN1

- R1-2204603, Supported values for SSSG switching delay for 480 kHz and 960 kHz SCS, Nokia, Nokia Shanghai Bell