3GPP TSG RAN WG1 #109-e			R1-2204610

e-Meeting, May 9th – 20th, 2022

Source:	CAICT

Title:	Discussion on side control information to enable NR network-controlled repeaters

Agenda Item:	9.8.1

Document for:	Discussion and Decision

- Introduction

The study item scope of NR Network-controlled Repeaters includes the following set of objectives:

- Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1]

- Beamforming information

- Timing information to align transmission / reception boundaries of network-controlled repeater

- Information on UL-DL TDD configuration

- ON-OFF information for efficient interference management and improved energy efficiency

- Power control information for efficient interference management (as the 2nd priority)

- Study and identify L1/L2 signalling (including its configuration) to carry the side control information [RAN1]

In this contribution, we share our views on side control information to enable NR network-controlled repeaters.

- Discussion of side control information

  - Beamforming information

We think beamforming information is the first prioritization of side control information, which is necessary to be informed to network-controlled repeater, so that the amplification function of repeater can be used targeting to the specific UEs for coverage enhancement. The beamforming side control information is transmitted between gNB and network-controlled repeater to control the transmitting beam direction between repeaters and UEs. There are two types of channels between repeaters and UEs, one is NR common control channels and the other is dedicated control and data channels. Considering NR common control channels such as synchronization channels, beam sweeping is necessary for UEs detecting the best beam among different beams and send PRACH mapped to a specific beam, so the side beamforming control information should be signaled to network-controlled repeater in a periodic TDM pattern. On the other hand, for NR dedicated control channels and data channels, the beamforming information should be dynamically or semi-dynamically signaled to network-controlled repeaters just as the normal procedure of beam management, the UEs measure the signal quality of reference signals corresponding to different beams to obtain the optimal beam and the gNB control the repeater to transmit dedicated control signaling and data to specific UEs by using the optimal beams.

  - ON-OFF information

Side control ON-OFF information of network-controlled repeater is necessary to be specified to improve energy efficiency and realize better interference management. On one hand, from energy efficiency perspective, the ON-OFF state of the network-controlled repeaters should be flexibly controlled by side control information in case of different using scenarios, for example, the network-controlled repeater can be controlled in OFF state in the time period of low UE loads with coverage holes. On the other hand, from interference management perspective, introduction of network-controlled repeater may also introduce interference to other UEs or other cells, it is necessary to flexibly control the ON-OFF state of network-controlled repeater to better realize the interference management. We think the ON-OFF information of side control is also with high prioritization.

  - Timing information

Timing information to align transmission/reception boundaries of network-controlled repeater is also needed to be included in side control information. There are four timing relationship from network-controlled repeater perspective, DL Rx and UL Tx between gNB and network-controlled repeater, DL TX and UL Rx between network-controlled repeater and UEs. Considering the switching time between transmission and reception of network-controlled repeater, guard symbols are needed. Another aspect needed to be considered is timing alignment, since network-controlled repeater is transparent to UEs, the information of timing advance cannot be acquired by it, so the TA cannot be compensated by the network-controlled repeater.

  - Information on UL-DL TDD configuration

TDD-pattern is needed for network-controlled repeater to ensure the proper DL/UL behavior, the UL slots or UL symbols are used for UEs to gNB transmission direction through the network-controlled repeater forwarding, and the DL slots or DL symbols are used for gNB to UEs transmission direction through the network-controlled repeater forwarding. It is for further study whether the NR TDD configuration signaling including semi-static and dynamic signaling can be reused or not.

  - Power control information for efficient interference management

Power control information could be signaled to network-controlled repeater for efficient interference management. The output power of the network-controlled repeater is determined by the RF amplification and the maximum output power of repeater, the specific power information is needed to be informed to network-controlled repeater considering the interference management. As described in ON-OFF information, interference can also be managed by controlling ON-OFF state of network-controlled repeater, whether to support both power control and ON-OFF information could firstly be analyzed and evaluated.

Proposal: The beamforming and ON-OFF information of side control should be considered with high priority, and whether power control information should also be considered needs FFS.

- Conclusion

In this contribution, we give following proposals on side control information to enable NR network-controlled repeaters.

Proposal: The beamforming and ON-OFF information of side control should be considered with high priority, and whether power control information should also be considered needs FFS.

References

- New SI: Study on NR Network-controlled Repeaters, ZTE