3GPP TSG RAN WG1 #109-e			R1-2204611

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.2.1

Source: 	LG Electronics

Title: 	Remaining issues of initial access aspects to support NR above 52.6 GHz

Document for:	Discussion and decision

- SS/PBCH block

Working Assumption - The following table is used for set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120}, {480, 480}, and {960, 960} kHz for FR2-2. - FFS: whether/how to define X, if defined, candidate values {56, 76} - Text Proposal #4-2D (for 38.213, Section 13) in section 3 of R1-2202503 is endorsed - Note: this working assumption can be revisited once RAN4 finalizes the further details of the channelization. Index; SS/PBCH block and CORESET multiplexing pattern; Number of RBs NRBCORESET; Number of Symbols NsymbCORESET; Offset (RBs). 0; 1; 24; 2; 0. 1; 1; 24; 2; 4. 2; 1; 48; 1; 0. 3; 1; 48; 1; 14. 4; 1; 48; 1; 28. 5; 1; 48; 2; 0. 6; 1; 48; 2; 14. 7; 1; 48; 2; 28. 8; 1; 96; 1; 0. 9; 1; 96; 1; X. 10; 1; 96; 2; 0. 11; 1; 96; 2; X. 12; 3; 24; 2; -20 if kSSB=0 -21 if kSSB > 0. 13; 3; 24; 2; 24. 14; 3; 48; 2; -20 if kSSB=0 -21 if kSSB > 0. 15; 3; 48; 2; 48.

One remaining issue of initial access for FR2-2 is whether and how to define RB-level offset between SSB and 96-RB CORESET#0. The followings are relevant RAN4 agreements on synchronization and channel raster points for 60 GHz licensed band.

Agreement: for RF channel raster for licensed band, SCS based channel raster is used for licensed bands, and the step size for channel raster is - 2 for 120kHz - 8 for 480kHz - 16 for 960kHz Agreement: for GSCN for licensed band, - GSCN step sizes - 3 for 120 kHz - 12 for 480 kHz - 6 for 960 kHz - Those values will be further checked after agreeing on the SU values.

According to RAN4 agreements, it is noted that floating channelization is adopted for 60 GHz licensed band. Thus, with 0 RB offset between SSB and 96-RB CORESET#0, one can find out a channel that fully contains 96-RB CORESET#0. With this regard, 0 RB offset between SSB and 96-RB CORESET#0 seems sufficient without defining X as in RAN1 working assumption.

Observation: Based on RAN4 agreement on RF channel raster and GSCN for 60 GHz licensed band, considering that floating channelization design is adopted, gNB can configure a channel corresponding to any of GSCN such that 96-RB CORESET#0 with 0 RB offset can be confined within the channel.

Agreement: Use the fixed RF channel raster with the step size of 1680 (100.8 MHz) as baseline to define the channel raster for the unlicensed band, and accordingly provide the channel raster numbers - To Check if the above solution can support CA with different bandwidth combinations - If there is issue identified, then the above agreement can be revisited

In RAN4#102bis-e meeting, the above agreement was made on channel and sync raster arrangement for un-licensed operation within FR2-2. Even though RAN4 adopted fixed RF channel raster design for unlicensed band as the baseline, still there is a possibility that the agreement can be revisited if any critical issue will be identified. Furthermore, based on RF channel raster and GSCN design proposed in our companion paper [1], 0 RB could be enough as the offset between SSB and 96-RB CORESET#0.

Proposal: Defer the decision on whether/how to define X (i.e., additional RB offset between SSB and 96-RB CORESET#0) until RAN4 finalize RF channel raster and GSCN designs.

- Conclusions

In this contribution, we discussed SS/PBCH block and initial BWP to support NR in high frequency range (FR2-2) from 52.6 GHz to 71 GHz, and the following observation and proposal were suggested.

Observation: Based on RAN4 agreement on RF channel raster and GSCN for 60 GHz licensed band, considering that floating channelization design is adopted, gNB can configure a channel corresponding to any of GSCN such that 96-RB CORESET#0 with 0 RB offset can be confined within the channel.

Proposal: Defer the decision on whether/how to define X (i.e., additional RB offset between SSB and 96-RB CORESET#0) until RAN4 finalize RF channel raster and GSCN designs.

- References

R4- 2208649, “60GHz channel and synchronization raster,” LG Electronics, May 2022