3GPP TSG RAN WG1 #109-e			R1-2204613

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.2.3

Source: 	LG Electronics

Title: 	Remaining issues of PDSCH/PUSCH enhancements to support NR above 52.6 GHz

Document for:	Discussion and decision

- Introduction

In this contribution, we discuss and provide views on multi-PDSCH/PUSCH scheduling and corresponding HARQ operation and time line related aspects adapted to each of 480 kHz and 960 kHz SCSs, to support NR in high frequency range from 52.6 GHz to 71 GHz.

- Minimum applicable scheduling offset for 480/960 kHz SCS

At the last RAN2 e-meeting, it was agreed to adopt the value of 64 for maxK0-SchedulingOffset-r17 and maxK2-SchedulingOffset-r17 for 480 kHz and 960 kHz SCS as below. Although the relevant LS has not been sent to RAN1, it is worth discussing whether RAN1 impact can occur by adopting these new values.

Agreement (RAN2 e-meeting) - To adopt 64 for maxSchedulingK0/2-SchedulingOffset-r17 for SCS 480 and 960 kHz - Can come back to this in the next meeting if there are issues with RAN1 with these values (no LS sent but companies should check with the RAN1 colleagues). Add editor's note about this to RRC CR.

Accordingly, the following update can be found in 38.331 [1].

maxK0-SchedulingOffset-r16	INTEGER ::= 16	-- Maximum number of slots configured as min. scheduling offset (K0)

maxK2-SchedulingOffset-r16	INTEGER ::= 16	-- Maximum number of slots configured as min. scheduling offset (K2)

maxK0-SchedulingOffset-r17	INTEGER ::= 64	-- Maximum number of slots configured as min. scheduling offset (K0)

maxK2-SchedulingOffset-r17	INTEGER ::= 64	-- Maximum number of slots configured as min. scheduling offset (K2)

First of all, we agree on the need for these values to increase. Through the previous RAN1 meetings, the maximum value of k0 and k2 were increased from 32 to 128 for 480/960 kHz SCS. Since the minimum scheduling offset restriction introduced in Rel-16 can be applied for these SCSs, it is reasonable to also increase the value ranges of maxK0-SchedulingOffset and maxK2-SchedulingOffset according to the increased k0 and k2.

Proposal #1: Support to adopt 64 for maxK0-SchedulingOffset-r17 and maxK2-SchedulingOffset-r17.

Meanwhile, according to the 38.214 spec, for aperiodic CSI reporting and aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology, a UE is not expected to be triggered by CSI triggering state indicated by the CSI request field in DCI in which CSI-RS triggering offset is smaller than the currently applicable minimum scheduling offset restriction K0min when the minimum scheduling offset restriction is applied. If the minimum applicable value of scheduling offset increases to 64 for 480/960 kHz as agreed by RAN2 above, the value range of CSI-RS triggering offset for 480/960 kHz should also be increased to provide a CSI-RS triggering offset not less than the applicable minimum scheduling offset. Considering that the maximum value of the minimum applicable scheduling offset for 480/960 kHz has increased by 4 times the value for 120k, we propose the supported values of CSI-RS trigger offset for 480/960kHz are also 4x of the supported values for 120 kHz. By increasing the maximum value of CSI-RS triggering offset in this way, the ratio of these values can be maintained at a similar level in Rel-16 and Rel-17.

Proposal #2: Increase the supported values (i.e., {0, 1x4, 2x4, …, 31x4) for CSI-RS triggering offset for 480/960 kHz by 4 times of those (i.e., {0, 1, 2, …, 31}) for 120 kHz.

- Multi-PDSCH/PUSCH scheduling

Agreement: (RAN1#106bis-e) For two multi-PDSCH (or two multi-PUSCH) scheduling DCIs, UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling. - FFS: whether to allow OOO scheduling for the following two cases: - for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH) - for the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV - Note: The above FFS aspect applies only to multi-PDSCH and multi-PUSCH scheduling with single DCI Conclusion (RAN1#107bis-e) - UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCIs to lead to out-of-order scheduling, also for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH). - This may not have specification impact. - Note: It is separately discussed whether the scheduled PDSCHs (or PUSCHs or SLIV) is based on configured SLIV or valid SLIV. Conclusion (RAN1#107bis-e) UE does not expect any of the received PDSCHs (including SPS PDSCH) and the resource for the HARQ-ACK transmission to lead to out-of-order scheduling, for any scheduling DCIs (including multi-PDSCH scheduling DCI). Agreement (RAN1#108-e) - The case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) schedulings have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, is considered as out-of-order scheduling and is not expected by UE. - This applies also when one of two DCIs is single-PDSCH (or single-PUSCH) scheduling DCI, including the case that one DCI schedules multi-slot PDSCH (or PUSCH repetition type A or B). - Note: This doesn’t apply when each of two DCIs schedules multi-slot PDSCH (or PUSCH repetition type A or B) as in Rel-15/Rel-16 - Note: This doesn’t apply when each of the two DCIs schedules single PDSCH (or single PUSCH) as in Rel-15/Rel-16 - Note: It is separately discussed whether the scheduled SLIV is based on configured SLIV or valid SLIV.

In RAN1#108-e meeting, it was discussed whether the scheduled PDSCHs (or PUSCHs or SLIV) highlighted in the above agreements/conclusions are determined based on valid ones or configured ones. Furthermore, it was pointed out that the impact to type-1 HARQ-ACK codebook generation should be analysed to resolve this OOO scheduling issue.

Our view is that it would be reasonable to consider only valid PDSCH (or PUSCHs or SLIV) to determine the rule for OOO scheduling because validity of PDSCH (or PUSCHs or SLIV) is semi-statically determined and does not lead to an ambiguity issue between gNB and UE, and also because it can provide more scheduling flexibility to gNB.

Proposal #3: Do not consider any invalid PDSCH or PUSCH (which is collided with semi-static UL or DL symbol(s)) to check out-of-order scheduling.

As a result of Proposal #3, if we do not consider invalid PDSCH for OOO scheduling, this may lead to an additional impact to type-1 HARQ-ACK codebook pseudo code especially when time domain bundling is configured. For example, as shown in Figure 1, since slots #N-4 and #N-3 are excluded to check OOO scheduling, PDSCHs scheduled in slots #N-2 and #N-5 via M-DCI #1 and #2, respectively, can be considered as in-order scheduling. It is also noted that configured K1 values are assumed as {2, 3}. Provided that time domain bundling is configured and type-1 HARQ-ACK codebook generation mechanism in current specification is applied, there could be an ambiguity on how to perform HARQ-ACK bundling corresponding to a PDSCH reception occasion m on slot #N-3. This is because current specification is based on the fact that there is only one scheduled PDSCH for a slot but there are more than one PDSCH scheduled on slot #N-3 in this case. To resolve this issue, current type-1 HARQ-ACK codebook pseudo code needs to be modified such that if there are more than one scheduled PDSCH (i.e., PDSCH#1 scheduled by M-DCI #1 and PDSCH#2 scheduled by M-DCI #2 on slot #N-3, in Figure 1) for a PDSCH reception occasion m, then HARQ-ACK bundling is performed for the PDSCH associated with the last SLIV (i.e., PDSCH#2 scheduled by M-DCI #2 on slot #N-3).

Figure 1. Example of type-1 HARQ-ACK codebook generation

Proposal #4: If type-1 HARQ-ACK codebook is configured with time domain bundling and OOO scheduling is determined based on valid PDSCHs,

- If there are more than one scheduled PDSCH for a PDSCH reception occasion m, then bundled HARQ-ACK information is generated for the PDSCH associated with the last SLIV.

Agreement (RAN1#108-e) When a DCI format indicates TCI state update without scheduling PDSCH reception, the PDCCH indicates a TDRA row index including only one SLIV.

In RAN1#108-e meeting, it was agreed that DCI indicating a TDRA row index associated with multiple SLIVs is not allowed for TCI state update, for the simplicity. Accordingly, the same principle can be extended to the case of SCell dormancy indication. To be specific, DCI format indicating SCell dormancy without scheduling a PDSCH reception can indicate a TDRA row index including only one SLIV. For this case, it is clarified that 1-bit NDI and 2-bit RV are re-interpreted for SCell dormancy indication.

Proposal #5: A DCI format indicating SCell dormancy without scheduling PDSCH reception can indicate a TDRA row index including only one SLIV.

Proposal #6: It is clarified that 1-bit NDI and 2-bit RV for transport block 1 are used for SCell dormancy indication when a DCI (including multi-PDSCH scheduling DCI) indicates SCell dormancy without scheduling a PDSCH reception.

- Conclusions

In this contribution, multi-PDSCH/PUSCH scheduling and corresponding HARQ operation and time line related aspects adapted to each of 480 kHz and 960 kHz SCSs, to support NR in the frequency range from 52.6 GHz to 71 GHz were discussed. The derived proposals are as follows,

Proposal #1: Support to adopt 64 for maxK0-SchedulingOffset-r17 and maxK2-SchedulingOffset-r17.

Proposal #2: Increase the supported values (i.e., {0, 1x4, 2x4, …, 31x4) for CSI-RS triggering offset for 480/960 kHz by 4 times of those (i.e., {0, 1, 2, …, 31}) for 120 kHz.

Proposal #3: Do not consider any invalid PDSCH or PUSCH (which is collided with semi-static UL or DL symbol(s)) to check out-of-order scheduling.

Proposal #4: If type-1 HARQ-ACK codebook is configured with time domain bundling and OOO scheduling is determined based on valid PDSCHs,

- If there are more than one scheduled PDSCH for a PDSCH reception occasion m, then bundled HARQ-ACK information is generated for the PDSCH associated with the last SLIV.

Proposal #5: A DCI format indicating SCell dormancy without scheduling PDSCH reception can indicate a TDRA row index including only one SLIV.

Proposal #6: It is clarified that 1-bit NDI and 2-bit RV for transport block 1 are used for SCell dormancy indication when a DCI (including multi-PDSCH scheduling DCI) indicates SCell dormancy without scheduling a PDSCH reception.

- References

3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification (Release 17)”, March 2022