3GPP TSG RAN WG1 #109-e		                         R1-2204615

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.2.5

Source: 	LG Electronics

Title: 	Remaining issues of beam management to support NR above 52.6 GHz

Document for:	Discussion and decision

- Introduction

In this contribution, we discuss and provide views on timing associated with beam-based operation and QCL assumptions for multiple PDSCHs scheduled by a single DCI, to support NR in high frequency range from 52.6 GHz to 71 GHz.

- Beam switching time

Different from Rel-15/16 beam management operation, beam switching time (i.e., required time to change phase shifters) should be considered at least for 960 kHz SCS, and in addition, for 480 kHz SCS when applicable. Even though RAN4 tentatively agreed on 59 ns beam switching time at gNB side [1], UE may require beam switching time longer than gNB or have different beam switching time capabilities depending on how to implement. Therefore, it should be discussed how to handle beam switching time or how to allocate symbol-level gap between DL/UL signals/channels. One possible approach could be to define UE capabilities for beam switching time and corresponding UE behaviour when receiving adjacent DL signals/channels (or transmitting adjacent UL signals/channels) considering the reported beam switching time.

For instance, at least one symbol gap can be allocated (except for long CP symbol every half sub-frame) between two adjacent CORESETs if those are associated with different TCI states. For another instance for consecutive UL signal/channel transmissions as shown in Figure 1, if CG-PUSCH is configured with beam A at slot N+3 and DG-PUSCH is scheduled with beam B at slots N+4 and N+5, UE may drop CG-PUSCH at slot N+3 or drop DG-PUSCH at slot N+4 depending on required beam switching time between CG-PUSCH and DG-PUSCH and/or UL cancellation time defined for CG-PUSCH.

Figure 1. Consecutive CG-PUSCH and DG-PUSCH transmissions with different beams

Proposal #1: Support a UE capability for Rx and Tx beam switching time between adjacent DL signals/channels and adjacent UL signals/channels, respectively.

Proposal #2: Define UE behavior (e.g., drop CG-PUSCH or drop DG-PUSCH depending on PUSCH cancellation time) when CG-PUSCH and DG-PUSCH transmissions are consecutively assigned and the reported beam switching time between CG-PUSCH and DG-PUSCH is not guaranteed.

- QCL assumptions for multiple PDSCHs

Agreement (RAN1#108-e) TP#1 below is endorsed for TS 38.214 - RAN1 continues discussion to down-select between the following Alt1 and Alt2. - Alt1) The applied TCI states can be updated using unified TCI framework within the span of multi-PDSCH - Alt2) The applied TCI states cannot be updated using unified TCI framework within the span of multi-PDSCH - Note: coordination with Rel-17 MIMO WI is allowed as necessary TP#1 5.1.5 Antenna ports quasi co-location *** Unchanged text omitted *** When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH(s). When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. *** Unchanged text omitted ***.

In RAN1#108-e meeting, it was agreed that multi-slot PDSCH and multiple PDSCHs scheduled by a single DCI have the same behaviour in that the activated TCI states (each of which is associated with a code-point of TCI field in a scheduling DCI) shall not be changed in the middle of the span of PDSCH(s) scheduled by the DCI. In addition, it was discussed 1) whether unified TCI framework is relevant with the agreed TP#1 above or not, and 2) whether the applied TCI states can be updated using unified TCI framework within the span of multi-PDSCH or not.

First of all, based on the following paragraph (cited by TS 38.214), the applied TCI state is not indicated by a DCI scheduling multi-slot PDSCH but determined after beam application time from HARQ-ACK transmission. Meanwhile, TP#1 describes how UE applies TCI state based on information from scheduling DCI. Therefore, we can conclude that the above agreed TP#1 is irrelevant with unified TCI framework introduced in Rel-17.

### 5.1.5 Antenna ports quasi co-location <Unrelated parts are omitted> When the UE would transmit the last symbol of a PUCCH with HARQ-ACK information corresponding to the DCI carrying the TCI State indication and without DL assignment, or corresponding to the PDSCH scheduling by the DCI carrying the TCI State indication, and if the indicated TCI State is different from the previously indicated one, the indicated DLorJointTCIState or UL-TCIstate should be applied starting from the first slot that is at least BeamAppTime_r17 symbols after the last symbol of the PUCCH. The first slot and the BeamAppTime_r17 symbols are both determined on the carrier with the smallest SCS among the carrier(s) applying the beam indication.

Next, according to the above captured paragraph, it is observed that the applied TCI states using unified TCI framework can be updated within the span of multi-slot PDSCH. Therefore, with this understanding, it would be straight-forward to adopt Alt 1 also for multi-PDSCH scheduling case.

Proposal #3: Adopt Alt1 (i.e., the applied TCI states can be updated using unified TCI framework within the span of multi-PDSCH).

- No additional specification change is needed.

- Conclusions

In this contribution, beam management enhancements to support NR from 52.6 GHz to 71 GHz were discussed, and the followings were proposed.

Proposal #1: Support a UE capability for Rx and Tx beam switching time between adjacent DL signals/channels and adjacent UL signals/channels, respectively.

Proposal #2: Define UE behavior (e.g., drop CG-PUSCH or drop DG-PUSCH depending on PUSCH cancellation time) when CG-PUSCH and DG-PUSCH transmissions are consecutively assigned and the reported beam switching time between CG-PUSCH and DG-PUSCH is not guaranteed.

Proposal #3: Adopt Alt1 (i.e., the applied TCI states can be updated using unified TCI framework within the span of multi-PDSCH).

- No additional specification change is needed.

- References

R1-2106425, “LS on 60 GHz time-related issues,” RAN4, Apple, RAN1#106-e