3GPP TSG RAN WG1 #109-e			R1-2204616

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.3.1

Source: 	LG Electronics

Title: 	Remaining issues on HARQ-ACK feedback enhancements

Document for:	Discussion and decision

Introduction

In the last meeting, following proposal has been discussed but haven’t been concluded.

Proposed Conclusion 6.12.1: There is no consensus to support the joint operation of PUCCH repetition and (dynamic or semi-static) PUCCH cell switching in Rel-17. - A UE is not expecting to be configured with dynamic or semi-static PUCCH cell switching and PUCCH repetitions for any PUCCH resource of any PHY priority within a PUCCH cell group.

In this contribution, we provide our views on the above proposals and also discuss on other aspects

Discussion

After the last meeting, UE feedback enhancement is considered as completed overall. However, few remaining issues haven’t been concluded yet, due to matter of time. In this section, we provide our views on the remaining issues and propose possible solutions to solve these issues.

dynamic/semi-static PUCCH cell switching + PUCCH repetition

Currently, PUCCH cell switching is applied only for a slot. If UE is configured with PUCCH resource having repetition factor, the UE would determine PUCCH occasions over multiple slots. In addition, if the UE is also configured with PUCCH cell switching based on semi-static pattern, it is necessary to define how to handle multiple PUCCH occasions over multiple slots and the PUCCH cell switching pattern which may or may not be aligned.

There were two alternatives which had been discussed for this issue. The alternatives are listed in below.

- Alt. 1: The target cell is determined for each PUCCH repetition individually (i.e., switching within the repetition bundle supported):

- Alt. 2 : For semi-static PUCCH cell switching, a PUCCH repetition transmission on a different target PUCCH cell from the PUCCH cell of the first PUCCH repetition is not supported

  - Alt. 2A : Postpone PUCCH mapped to different PUCCH cell from the first PUCCH occasion

  - Alt. 2B : Drop PUCCH mapped to different PUCCH cell from the first PUCCH occasion

  - Alt. 2C : PUCCH carrier switching within PUCCH repetition bundle is not expected at UE

First of all, it should be noted that PUCCH carrier switching and PUCCH repetition have different technical purpose and domain. PUCCH carrier switching is basically to increase PUCCH availability in TDD CA configuration. PUCCH repetition is to ensure coverage and/or reliability up to requirement.

In case of the issue between PUCCH repetition and SPS HARQ-ACK deferral, we understood that PUCCH repetition can replace SPS HARQ-ACK deferral to resolve UL/DL collision. On the other hand, PUCCH carrier switching and PUCCH repetition have totally different goals, the side effect of the case where joint operation is not supported would be larger than other issues.

According to the discussion in the previous meetings, the majority view was that PUCCH resource is given by target carrier (i.e., the carrier with the first PUCCH occasion) configuration, so the PUCCH repetition would be done within the target carrier. There are also different views, for example, PUCCH repetition would be performed between different cells for the diversity gain. In our view, it is unclear how to determine PUCCH resources in the cell not indicated as target carrier and the benefit and gain hasn’t been identified yet.

Considering dynamic PUCCH carrier switching, it would be straightforward that carrier indication in DL assignment applies to all repetition bundle, if joint configuration with PUCCH repetition is supported. It means that, UE may not switch PUCCH carrier within repetition bundle at least for the case of dynamic PUCCH carrier switching. Thus, if Alt. 2 is adopted, UE behaviors between dynamic and semi-static can be aligned.

From those point of views, we would like to prioritize simplicity rather than unidentified gain at this late stage, like other simultaneous configuration. We cannot see the reason to make a difference for this case. Among Alt. 2 family, our preference is Alt. 2A

Proposal #1: For semi-static PUCCH cell switching, Adopt Alt. 2 as a baseline.

- Among Alt. 2A/B/C, down-select one until RAN1#109-e.

Proposal #2: For dynamic PUCCH cell switching and PUCCH repetitions, indicated PUCCH cell applies to all repetition.

- All repetitions have same target PUCCH cell so that a PUCCH repetition transmission on a different target PUCCH cell from the PUCCH cell of the first PUCCH repetition is not supported.

In our view, the advantage of Alt. 1 are diversity and reduced total transmission time, which are meaningful for URLLC. Considering that, it could be considered to support Alt. 1 additionally to Alt. 2. If both of Alternatives are supported, UE can report capable UE behavior via UE capability signaling and UE can be configured with one of PUCCH repetition method.

If Alt. 1 is supported, it should be specify how to determine PUCCH resource in different cells and how to handle different subcarrier spacing between cells. To minimize issues, we would like to propose to support Alt. 1 only for PUCCH cells having same subcarrier spacing, i.e., equal PUCCH slot length.

Proposal #3: For semi-static PUCCH cell switching and PUCCH repetitions, if Alt. 1 is supported additionally to Alt. 2, Alt. 1 is limited only when PUCCH cells have the same subcarrier spacing.

Editorial for Type-3 HARQ-ACK codebook determination

In the previous meeting, an issue on spec description for HARQ-ACK codebook determination was raised in [2]. The issue is that the two different ways to configure one enh. Type 3 CB (using either pdsch-HARQ-ACK-enhType3perCC or pdsch-HARQ-ACK-enhType3perHARQ) are currently not captured in the specifications.

The reason of the problem is that discrete cell and HARQ process sets are not considered in pseudo code. In order to solve this problem, we propose following text proposal. In the text proposal, we re-define set c and set h according to the indicated enhanced type-3 sub-codebook, and change declaration position of set h so that set h can be re-defined for each serving cell.

Proposal #4: The following TP to 38.213 is endorsed for the editor’s CR:

### 9.1.4 Type-3 HARQ-ACK codebook determination If a UE is provided pdsch-HARQ-ACK-OneShotFeedback, the UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits, of a Type-3 HARQ-ACK codebook according to the following procedure. If the UE is provided pdsch-HARQ-ACK-enhType3List and a DCI format scheduling PDSCH reception and triggering the Type-3 HARQ-ACK codebook includes a Type3-subcodebook-index field that provides a value for pdsch-HARQ-ACK-enhType3Index, the UE determines a number of indicated serving cells NcellsDL,ind and a number of indicated HARQ processes NHARQ,cDL,ind for each indicated serving cell c from the entry in pdsch-HARQ-ACK-enhType3List corresponding to the pdsch-HARQ-ACK-enhType3Index value. If the DCI format does not include the Type3-subcodebook-index field, the pdsch-HARQ-ACK-enhType3Index value is zero. Set NcellsDL to the number of configured serving cells or, when applicable, to NcellsDL,ind Set NHARQ,cDL to the value of nrofHARQ-ProcessesForPDSCH for serving cell c, if provided; else, set NHARQ,cDL=8 . When applicable, set NHARQ,cDL to NHARQ,cDL,ind Set NTB,cDL to the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided and NDIHARQ=0, or if harq-ACK-SpatialBundlingPUCCH is not provided, or if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c; else, set NTB,cDL=1 Set NHARQ-ACK,cCBG/TB,max to the number of HARQ-ACK information bits per TB for PDSCH receptions on serving cell c as described in clause 9.1.1 if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c and pdsch-HARQ-ACK-OneShotFeedbackCBG or pdsch-HARQ-ACK-enhType3CBG is provided; else, set NHARQ-ACK,cCBG/TB,max=0 Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI or pdsch-HARQ-ACK-enhType3NDI is provided; else set NDIHARQ=1 Set c=0 – serving cell index : lower indexes correspond to lower RRC indexes of corresponding cell in the indicated entry in pdsch-HARQ-ACK-enhType3List if the UE is provided pdsch-HARQ-ACK-enhType3List Set h=0 – HARQ process number Set t=0 – TB index Set g=0 – CBG index Set j=0 while c<NcellsDL Set h=0 – HARQ process number : lower indexes correspond to lower HARQ process number of corresponding HARQ process for serving cell c in the indicated entry in pdsch-HARQ-ACK-enhType3List if the UE is provided pdsch-HARQ-ACK-enhType3List --------------------------------------------------------End text proposal----------------------------------------------------------

In addition to this, the name of DCI field determining type-3 sub codebook hasn’t been aligned between 38.213 and 38.212. To align those, we also provide text proposal to rename “Type3-subcodebook-index” to “Enhanced Type 3 codebook indicator”

Proposal #5: The following TP to 38.213 is endorsed for the editor’s CR:

### 9.1.4 Type-3 HARQ-ACK codebook determination If a UE is provided pdsch-HARQ-ACK-OneShotFeedback, the UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits, of a Type-3 HARQ-ACK codebook according to the following procedure. If the UE is provided pdsch-HARQ-ACK-enhType3List and a DCI format scheduling PDSCH reception and triggering the Type-3 HARQ-ACK codebook includes a Type3-subcodebook-index Enhanced Type 3 codebook indicator field that provides a value for pdsch-HARQ-ACK-enhType3Index, the UE determines a number of indicated serving cells NcellsDL,ind and a number of indicated HARQ processes NHARQ,cDL,ind for each indicated serving cell c from the entry in pdsch-HARQ-ACK-enhType3List corresponding to the pdsch-HARQ-ACK-enhType3Index value. If the DCI format does not include the Type3-subcodebook-index Enhanced Type 3 codebook indicator field, the pdsch-HARQ-ACK-enhType3Index value is zero. ## *** Unchanged text is omitted *** the DCI format provides a request for a Type-3 HARQ-ACK codebook report and does not schedule a PDSCH reception. If the UE is provided pdsch-HARQ-ACK-enhType3List and the DCI format includes a Type3-subcodebook-index Enhanced Type 3 codebook indicator field that provides a value for pdsch-HARQ-ACK-enhType3Index, the UE determines a number of indicated serving cells NcellsDL,ind and a number of indicated HARQ processes NHARQ,cDL,ind for each indicated serving cell c from the entry in pdsch-HARQ-ACK-enhType3List corresponding to the pdsch-HARQ-ACK-enhType3Index value. If the DCI format does not include the Type3-subcodebook-index Enhanced Type 3 codebook indicator field, the pdsch-HARQ-ACK-enhType3Index value is provided by the value of MCS field in the DCI format. The UE is expected to provide HARQ-ACK information in response to the request for the Type-3 HARQ-ACK codebook after N symbols from the last symbol of a PDCCH providing the DCI format, where the value of N for μ=0,1,2 is provided in clause 10.2 by replacing "SPS PDSCH release" with "DCI format". --------------------------------------------------------End text proposal----------------------------------------------------------

Conclusions

In this contribution, remaining issues on HARQ-ACK feedback enhancements for URLLC were discussed, and the followings are proposed.

Proposal #1: For semi-static PUCCH cell switching, Adopt Alt. 2 as a baseline.

- Among Alt. 2A/B/C, down-select one until RAN1#109-e.

Proposal #2: For dynamic PUCCH cell switching and PUCCH repetitions, indicated PUCCH cell applies to all repetition.

- All repetitions have same target PUCCH cell so that a PUCCH repetition transmission on a different target PUCCH cell from the PUCCH cell of the first PUCCH repetition is not supported.

Proposal #3: For semi-static PUCCH cell switching and PUCCH repetitions, if Alt. 1 is supported additionally to Alt. 2, Alt. 1 is limited only when PUCCH cells have the same subcarrier spacing.

Proposal #4: The following TP to 38.213 is endorsed for the editor’s CR:

### 9.1.4 Type-3 HARQ-ACK codebook determination If a UE is provided pdsch-HARQ-ACK-OneShotFeedback, the UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits, of a Type-3 HARQ-ACK codebook according to the following procedure. If the UE is provided pdsch-HARQ-ACK-enhType3List and a DCI format scheduling PDSCH reception and triggering the Type-3 HARQ-ACK codebook includes a Type3-subcodebook-index field that provides a value for pdsch-HARQ-ACK-enhType3Index, the UE determines a number of indicated serving cells NcellsDL,ind and a number of indicated HARQ processes NHARQ,cDL,ind for each indicated serving cell c from the entry in pdsch-HARQ-ACK-enhType3List corresponding to the pdsch-HARQ-ACK-enhType3Index value. If the DCI format does not include the Type3-subcodebook-index field, the pdsch-HARQ-ACK-enhType3Index value is zero. Set NcellsDL to the number of configured serving cells or, when applicable, to NcellsDL,ind Set NHARQ,cDL to the value of nrofHARQ-ProcessesForPDSCH for serving cell c, if provided; else, set NHARQ,cDL=8 . When applicable, set NHARQ,cDL to NHARQ,cDL,ind Set NTB,cDL to the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided and NDIHARQ=0, or if harq-ACK-SpatialBundlingPUCCH is not provided, or if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c; else, set NTB,cDL=1 Set NHARQ-ACK,cCBG/TB,max to the number of HARQ-ACK information bits per TB for PDSCH receptions on serving cell c as described in clause 9.1.1 if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c and pdsch-HARQ-ACK-OneShotFeedbackCBG or pdsch-HARQ-ACK-enhType3CBG is provided; else, set NHARQ-ACK,cCBG/TB,max=0 Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI or pdsch-HARQ-ACK-enhType3NDI is provided; else set NDIHARQ=1 Set c=0 – serving cell index : lower indexes correspond to lower RRC indexes of corresponding cell in the indicated entry in pdsch-HARQ-ACK-enhType3List if the UE is provided pdsch-HARQ-ACK-enhType3List Set h=0 – HARQ process number Set t=0 – TB index Set g=0 – CBG index Set j=0 while c<NcellsDL Set h=0 – HARQ process number : lower indexes correspond to lower HARQ process number of corresponding HARQ process for serving cell c in the indicated entry in pdsch-HARQ-ACK-enhType3List if the UE is provided pdsch-HARQ-ACK-enhType3List --------------------------------------------------------End text proposal----------------------------------------------------------

Proposal #5: The following TP to 38.213 is endorsed for the editor’s CR:

### 9.1.4 Type-3 HARQ-ACK codebook determination If a UE is provided pdsch-HARQ-ACK-OneShotFeedback, the UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits, of a Type-3 HARQ-ACK codebook according to the following procedure. If the UE is provided pdsch-HARQ-ACK-enhType3List and a DCI format scheduling PDSCH reception and triggering the Type-3 HARQ-ACK codebook includes a Type3-subcodebook-index Enhanced Type 3 codebook indicator field that provides a value for pdsch-HARQ-ACK-enhType3Index, the UE determines a number of indicated serving cells NcellsDL,ind and a number of indicated HARQ processes NHARQ,cDL,ind for each indicated serving cell c from the entry in pdsch-HARQ-ACK-enhType3List corresponding to the pdsch-HARQ-ACK-enhType3Index value. If the DCI format does not include the Type3-subcodebook-index Enhanced Type 3 codebook indicator field, the pdsch-HARQ-ACK-enhType3Index value is zero. ## *** Unchanged text is omitted *** the DCI format provides a request for a Type-3 HARQ-ACK codebook report and does not schedule a PDSCH reception. If the UE is provided pdsch-HARQ-ACK-enhType3List and the DCI format includes a Type3-subcodebook-index Enhanced Type 3 codebook indicator field that provides a value for pdsch-HARQ-ACK-enhType3Index, the UE determines a number of indicated serving cells NcellsDL,ind and a number of indicated HARQ processes NHARQ,cDL,ind for each indicated serving cell c from the entry in pdsch-HARQ-ACK-enhType3List corresponding to the pdsch-HARQ-ACK-enhType3Index value. If the DCI format does not include the Type3-subcodebook-index Enhanced Type 3 codebook indicator field, the pdsch-HARQ-ACK-enhType3Index value is provided by the value of MCS field in the DCI format. The UE is expected to provide HARQ-ACK information in response to the request for the Type-3 HARQ-ACK codebook after N symbols from the last symbol of a PDCCH providing the DCI format, where the value of N for μ=0,1,2 is provided in clause 10.2 by replacing "SPS PDSCH release" with "DCI format". --------------------------------------------------------End text proposal----------------------------------------------------------

References

RAN1 chairman notes, RAN1#108-e

R1-2201090, Remaining issues on HARQ-ACK enhancements for Rel-17 URLLC, vivo, RAN1#108-e