3GPP TSG RAN WG1 Meeting #109-e			R1-2204619

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.6.1

Source: 	LG Electronics

Title: 	Remaining aspects of UE complexity reduction for RedCap

Document for:	Discussion and decision

- Introduction

Since the WI titled “Support of reduced capability NR devices” was approved in RAN#90-e meeting [1], there has been active discussion to support RedCap in Rel-17. As of RAN1#107-e meeting, it is considered that most of the essential/critical issues have been resolved from RAN1 perspective, but at the same time, there are a few remaining issues related to UE complexity reduction of RedCap to be further discussed/clarified during Rel-17 maintenance phase. The WI objectives relevant to those remaining issues are copied below from latest version of the WID [2].

## 4 Objective ### 4.1 Objective of Core part WI This WI has the following objectives: - Specify support for the following UE complexity reduction features [RAN1, RAN2, RAN4]: - Reduced maximum UE bandwidth: - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz. - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz - … - Duplex operation: - HD-FDD type A with the minimum specification impact (Note that FD-FDD and TDD are also supported.) - …

- Discussion

In this contribution, we present our views on the remaining aspects of the UE complexity reduction for RedCap.

  - Remaining issues on reduced maximum UE bandwidth of RedCap UEs

Regarding reduced maximum UE bandwidth of RedCap UEs, one of the remaining open issues is on the presence of SSB transmission in separate initial DL BWP in connected mode for BWP#0 configuration option 1. There has already been very active discussion on this topic in RAN1#108-e meeting, so we can start off from the two options (copied below) from the latest FL’s proposal in RAN1#108-e meeting [3].

High Priority Proposal 3-1h: Down select between the following options during RAN1#108-e: - Option 1: - For FR1, for BWP#0 configuration option 1, - A RedCap UE in connected mode does not expect to operate in a separate initial DL BWP that does not include CD-SSB and the entire CORESET#0. - For FR2, for BWP#0 configuration option 1, - A RedCap UE in connected mode does not expect to operate in a separate initial DL BWP that does not include CD-SSB and the entire CORESET#0. - Option 2: - For FR1, for BWP#0 configuration option 1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - During a random access procedure in connected mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - For FR2, for BWP#0 configuration option 1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - During a random access procedure in connected mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - For BWP#0 configuration option 1, upon successful completion of the random access procedure, a RedCap UE supporting FG 6-1 only (but not FG 6-1a) in connected mode is not required to operate on a separate initial DL BWP that does not contain SSB. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP.

According to the agreements so far, for an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), a RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB.

For BWP#0 configuration option 1, the BWP#0 is not considered to be an RRC-configured BWP, so the agreement above does not guarantee the transmission of the NCD-SSB in the BWP#0 per se. On the other hand, for BWP#0 configuration option 2, as the BWP#0 is considered to be an RRC-configured BWP, the NCD-SSB is transmitted in the BWP#0 according to the agreement.

A RedCap UE can also optionally support the case where the NCD-SSB transmission is not needed for RedCap UEs in the RRC-configured active DL BWP in connected mode by reporting optional capabilities of relevant operation based on [FG 6-1a] with or without supporting CSI-RS as agreed in RAN1#108-e meeting. In this case, for both BWP#0 configuration option 1 and option 2, a RedCap UE may NOT expect NCD-SSB in the separate initial DL BWP.

With the clarifications above, firstly we think there is no problem allowing the combination of the separate initial DL BWP and the BWP#0 configuration option 1. And in this case, we prefer Option 2 in the High Priority Proposal 3-1h above, because it allows to share the RACH configurations between idle/inactive UEs and connected UEs based on RedCap UE capability.

Proposal 1: For the remaining open issues on the presence of SSB transmission in separate initial DL BWP in connected mode for BWP#0 configuration option, support Option 2 in the High Priority Proposal 3-1h from the latest FL’s proposal in RAN1#108-e meeting.

The other remaining issue on the separate initial DL BWP for RedCap UEs is the clarification of UE behavior when separate initial DL BWP is not configured. According to the agreement made in RAN1#107-e meeting (copied below), a separate initial DL BWP may not be configured for RedCap UEs when an initial DL BWP for non-RedCap UEs is larger than max RedCap UE bandwidth.

Agreement (107-e) - For both FR1 and FR2, for a cell that allows a RedCap UE to access, network can configure a separate initial DL BWP for RedCap UEs in SIB. At least the case when the separate initial DL BWP includes CD-SSB and the entire CORESET#0 is supported - It can be used in idle/inactive mode (including paging) and during and after initial access, when applicable - It is no wider than the maximum RedCap UE bandwidth. - This applies to both TDD and FDD (including FD FDD and HD FDD) cases.

In our view, if a separate initial DL BWP for RedCap is not configured when an initial DL BWP for non-RedCap UEs is larger than maximum UE bandwidth of the RedCap, the RedCap UE may assume the MIB-configured CORESET#0 bandwidth as the initial DL BWP. In this case, same as for the non-RedCap UEs, the center frequencies b/w the separate initial UL BWP and the MIB-configured CORESET#0 may or may not be aligned.

Alternatively, the RedCap UE may assume as the initial DL BWP the SIB-configured initial DL BWP for non-RedCap UEs if configured. In this case, the center frequencies b/w the separate initial UL BWP and the SIB-configured initial DL BWP for non-RedCap UEs may or may not be aligned.

To minimize frequency retuning for RedCap UEs in the case above, whether to apply some restrictions on the possible configurations of the separate initial UL BWP for RedCap, for instance to align the center frequencies between the separate initial UL BWP and the MIB-configured CORESET#, can be further discussed. If we fail to reach a consensus after further discussion, our understanding on the default assumption is that there is no restriction on the center frequencies of the initial UL BWP and the MIB-configured CORESET#0 as long as the bandwidth spanned by the initial UL BWP and the MIB-configured CORESET#0 together does not exceed the max RedCap UE bandwidth.

Proposal 2: If a separate initial DL BWP for RedCap UEs is not configured when the initial DL BWP for non-RedCap UEs is larger than the max UE bandwidth of the RedCap, the RedCap UE may assume the MIB-configured CORESET#0 bandwidth as the initial DL BWP.

- In this case, same as for the non-RedCap UEs, center frequencies b/w the separate initial UL BWP and the MIB-configured CORESET#0 may or may not be aligned.

Proposal 3: There is no restriction on the center frequencies of the initial UL BWP and the MIB-configured CORESET#0 as long as the bandwidth spanned by the initial UL BWP and the MIB-configured CORESET#0 together does not exceed the max RedCap UE bandwidth.

  - Remaining issues on duplex operation of RedCap UEs

Since RAN1#104-e meeting, the basic principle to deal with the DL/UL collision cases relevant for HD-FDD type A operation in FDD band has been discussed. Refer to the following agreement for the collision cases that has been studied.

Agreements: (RAN1#104-e) - For HD-FDD operation for RedCap UEs, collisions may be addressed or alleviated with proper scheduling. The following cases of potential collisions can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: Configured SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 8: Dynamic or semi-static DL vs. valid RO - Case 9: Collision due to direction switching

And as a follow-up, collision handling rules have been discussed and the corresponding agreements have been made on each of the collision cases. Currently, most of the cases have been addressed, but there is one outstanding remaining issue under the Case 5 Configured SSB vs. dynamically scheduled or configured UL transmission. For Case 5, the following agreements have been made so far.

Agreement: (106e) - For Case 5 of SSB overlaps with in configured UL transmission, re-use the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over configured UL transmission - The configured UL transmission includes CG-PUSCH, or SRS - FFS: Confirm that PUCCH is included Agreement (106e) - For Case 5 of SSB overlaps with configured UL transmission, the configured UL transmission includes PUCCH transmission configured by higher layers - Note: The UL transmission indicated by DCI is supposed to be dynamic UL transmission. Agreement (106e) - For Case 5 of dynamically scheduled UL transmission vs. SSB, one or both of the following options to be determined till next meeting: - Option 1: Dynamically scheduled UL transmission is prioritized over SSB - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission Agreement (107e) - For Case 5 of dynamically scheduled UL transmission vs. SSB, support Option 2 at least for dynamically scheduled UL transmission other than Msg3 (re)transmission and PUCCH for Msg4 - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission

As highlighted in yellow above, the decision on how to handle the collision case of the Msg3 (re)transmission and PUCCH for Msg4 vs. SSB is pending. For the Case 5 of Msg3 or PUCCH in response to Msg4/MsgB vs. SSB, we prefer Option 2 (Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission). In this way, the same collision handling is applied for both the configured and the dynamic including the Msg3 (re)transmission and PUCCH for Msg4. If different collision handling rules were created, then the available slots for PUSCH transmissions would be different for different cases making the UE behaviour unnecessarily complicated.

Proposal 4: For Case 5 of dynamically scheduled UL transmission vs. SSB, support Option 2 for the case of Msg3 (re)transmission and PUCCH for Msg4 vs. SSB.

- Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission.

- Conclusion

In this contribution, we presented our views on the remaining aspects of the UE complexity reduction for RedCap.

Proposal 1: For the remaining open issues on the presence of SSB transmission in separate initial DL BWP in connected mode for BWP#0 configuration option, support Option 2 in the High Priority Proposal 3-1h from the latest FL’s proposal in RAN1#108-e meeting.

Proposal 2: If a separate initial DL BWP for RedCap UEs is not configured when the initial DL BWP for non-RedCap UEs is larger than the max UE bandwidth of the RedCap, the RedCap UE may assume the MIB-configured CORESET#0 bandwidth as the initial DL BWP.

- In this case, same as for the non-RedCap UEs, center frequencies b/w the separate initial UL BWP and the MIB-configured CORESET#0 may or may not be aligned.

Proposal 3: There is no restriction on the center frequencies of the initial UL BWP and the MIB-configured CORESET#0 as long as the bandwidth spanned by the initial UL BWP and the MIB-configured CORESET#0 together does not exceed the max RedCap UE bandwidth.

Proposal 4: For Case 5 of dynamically scheduled UL transmission vs. SSB, support Option 2 for the case of Msg3 (re)transmission and PUCCH for Msg4 vs. SSB.

- Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission.

- References

RP-202933	New WID on support of reduced capability NR devices, 			Ericsson

RP-211574	Revised WID on support of reduced capability NR devices, 		Ericsson

R1-2202532	FL summary #4 on reduced maximum UE bandwidth for RedCap	Moderator (Ericsson)